3GPP RAN WG4 Meeting #103-e		R4-2207681

Online, May 9th – 20th, 2022

Agenda item:	13.2.7

Source:	Apple

Title:	Reply to RAN5 LS on SCell Dropping in FR2 RF UL-CA tests

WI/SI:	NR_newRAT-Core

Release:	Rel-15

Document for:	Approval

# 1	Introduction

The SCell power reduction or dropping in UL CA maximum output power condition was first identified in FR2 as a RAN5 conformance test issue which was brought up in RAN4 #96-e meeting [1,2]. The issue was further discussed in the next few RAN4 meetings with the focus on whether equal PSD condition for intra-band UL CA should be maintained for testing maximum output power associated requirements [3]. And the conclusion was drawn in an LS to RAN5 with RAN4’s recommendation that equal PSD is a preferred test condition to verify the UL CA requirements, but the testing details are up to RAN5 [4].

To resolve the FR2 intra-band UL CA maximum output power testability issue with achieving equal or close to equal PSD condition for all carriers, RAN5 has been considering the usage of a conformance-only test function to test FR2 RF UL-CA conformance tests, for 3GPP Release 16 and forward. Such a conformance-only test function approach includes the usage of a parameter that defines the power back-off needed to be applied to the PCell to potentially avoid SCell drop in FR2 UL-CA conformance test cases [5-7]. With that RAN5 sent an LS to RAN4 [8] to request responses from RAN4 on the following questions:

- Whether RAN4 sees a need to define within TS 38.101-2, the aforementioned power backoff parameter which will be used by conformance-only test function?

- Whether RAN4 can share guidance on any impact on absolute and relative power tolerance accuracy that needs to be factored because of usage of such a conformance-only test function to apply power limits/back-off?

In this contribution, we provide our views and responses to RAN5’s questions above in conjunction with a draft reply LS attached for consideration.

# Discussion

In response to the first question “Whether RAN4 sees a need to define within TS 38.101-2, the aforementioned power backoff parameter which will be used by conformance-only test function?”, in our view, there is a clear distinction between the conformance test and field operation under the maximum output power condition. In conformance test, the tester would continue sending the TPC “UP” commands to the DUT till its output power saturates which would be an indication of reaching UE’s PCMAX. Unlike in real network, there is no UE power headroom (PHR) reporting back to the tester during the TPC “UP” processing in the conformance test. As a result, UE would continue scaling back its SCell output power to leave more power to PCell according to the prioritization rule till SCell is dropped. The conformance-only test function is to complement the maximum output power test procedure with pre-calculated power back-off for PCell so that it would not consume all the power headroom under UL CA operation to cause SCell dropping upon contiguous TPC “UP” commands. There is no need to define this power back-off parameter in RAN4 specifications as in field operation, the UL power is prioritized for PCell where SCell can be dropped or de-activated when PCell is running out of power headroom.

Observation 1: The conformance-only test function is to complement the maximum output power test procedure with pre-calculated power back-off for PCell so that it would not consume all the power headroom under UL CA operation to cause SCell dropping upon contiguous TPC “UP” commands.

Observation 2: There is no need to define the said power back-off parameter in RAN4 specifications as in field operation, the UL power is prioritized for PCell where SCell can be dropped or de-activated when PCell is running out of power headroom.

Based on the above assessment and observations, we propose the following response to RAN5’s first question:

Response to Question a): RAN4 sees no need to define within TS 38.101-2 the power back-off parameter which will be used by conformance-only test function.

In response to the second question “Whether RAN4 can share guidance on any impact on absolute and relative power tolerance accuracy that needs to be factored because of usage of such a conformance-only test function to apply power limits/back-off?”, first of all, we would like to clarify that applying power limits is less likely feasible for FR2 as UE nominal maximum output power could vary between minimum peak EIRP and maximum peak EIRP. And since UE is not required to report its nominal maximum output power, it would not be possible for the tester to properly instruct the UE what power limit should be applied.

Observation 3: Applying power limits is less likely feasible for FR2 as UE nominal maximum output power could vary between minimum peak EIRP and maximum peak EIRP. And since UE is not required to report its nominal maximum output power, it would not be possible for the tester to properly instruct the UE what power limit should be applied.

The power back-off on the other hand can be used by UE internally to calculate the power headroom for both PCell and SCell(s) without needing to know UE’s nominal maximum output power. The absolute and relative power tolerance shall have no influence on the back-off parameter for power headroom calculation which is processed with absolute accuracy.

Observation 4: The absolute and relative power tolerance shall have no influence on the back-off parameter for power headroom calculation which is processed with absolute accuracy.

Based on the above assessment and observations, we propose the following response to RAN5’s second question:

Response to Question b): RAN4 concludes that there is no impact on absolute and relative power tolerance accuracy that needs to be factored because of usage of the conformance-only test function to apply power back-off.

Proposal: Approve the attached draft reply LS with the proposed responses above.

# 3	Conclusion

In this contribution, we provide our views and responses to the questions in RAN5’s LS on SCell dropping in FR2 RF UL-CA tests. A draft reply LS is also attached for consideration.

Observation 1: The conformance-only test function is to complement the maximum output power test procedure with pre-calculated power back-off for PCell so that it would not consume all the power headroom under UL CA operation to cause SCell dropping upon contiguous TPC “UP” commands.

Observation 2: There is no need to define the said power back-off parameter in RAN4 specifications as in field operation, the UL power is prioritized for PCell where SCell can be dropped or de-activated when PCell is running out of power headroom.

Observation 3: Applying power limits is less likely feasible for FR2 as UE nominal maximum output power could vary between minimum peak EIRP and maximum peak EIRP. And since UE is not required to report its nominal maximum output power, it would not be possible for the tester to properly instruct the UE what power limit should be applied.

Observation 4: The absolute and relative power tolerance shall have no influence on the back-off parameter for power headroom calculation which is processed with absolute accuracy.

Response to Question a): RAN4 sees no need to define within TS 38.101-2 the power back-off parameter which will be used by conformance-only test function.

Response to Question b): RAN4 concludes that there is no impact on absolute and relative power tolerance accuracy that needs to be factored because of usage of the conformance-only test function to apply power back-off.

Proposal: Approve the attached draft reply LS with the proposed responses above.

# 4	References

R4-2009656 “NR SCC UL power drop behavior with EN-DC UE in FR2”, Anritsu Corporation, 3GPP TSG-RAN WG4 Meeting #96-e, August 17th – 27th, 2020

R4-2011695 “WF on NR SCC UL power drop behavior in FR2”, Anritsu Corporation, 3GPP TSG-RAN WG4 Meeting #96-e, August 17th – 27th, 2020

R4-2016994 “WF on NR SCC UL power drop behavior in FR2”, Ericsson, 3GPP TSG-RAN WG4 Meeting #97-e, November 2nd – 13th, 2020

R4-2103124 “LS on SCell dropping”, OPPO, 3GPP TSG-RAN WG4 Meeting #98-e, January 25th – February 5th, 2021

R5-221920 “Add new messages and procedure for test function to limit Pcell Power”, Apple, 3GPP TSG-RAN5 Meeting #94-e, February 21st – March 4th, 2022

R5-221921 “Addition of new test function to limit Pcell power”, Apple, 3GPP TSG-RAN5 Meeting #94-e, February 21st – March 4th, 2022

R%-221922 “implement test function approach to avoid Scell Drop in FR2 UL-CA”, Apple, 3GPP TSG-RAN5 Meeting #94-e, February 21st – March 4th, 2022

R5-221617 “LS on SCell Dropping in FR2 RF UL-CA tests”, TSG RAN WG5, 3GPP TSG-RAN5 Meeting #94-e, February 21st – March 4th, 2022

3GPP TSG-RAN WG4 Meeting #103-e	R4-22xxxxx

# Online, May 9th – 20th, 2022

Title:	[draft] Reply LS on SCell dropping in FR2 RF UL-CA tests

Response to:	(R5-221617) LS on SCell dropping in FR2 RF UL-CA tests

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	RAN4

To:	RAN5

Cc:

Contact Person:

- Name:	James Wang

- E-mail Address:	 fucheng_wang@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

None

1. Overall Description:

RAN4 would like to thank RAN5 on informing the development of a conformance-only test function to test FR2 RF UL-CA conformance tests which includes the usage of a parameter that defines the power back-off needed to be applied to the PCell to potentially avoid SCell drop in FR2 UL-CA conformance test cases.

Regarding RAN5’s request to RAN4 for the responses to the following questions:

- Whether RAN4 sees a need to define within TS 38.101-2, the aforementioned power backoff parameter which will be used by conformance-only test function?

- Whether RAN4 can share guidance on any impact on absolute and relative power tolerance accuracy that needs to be factored because of usage of such a conformance-only test function to apply power limits/back-off?

RAN4 had discussed and concluded with the following responses:

Response to Question a): RAN4 sees no need to define within TS 38.101-2 the power back-off parameter which will be used by conformance-only test function.

Response to Question b): RAN4 concludes that there is no impact on absolute and relative power tolerance accuracy that needs to be factored because of usage of the conformance-only test function to apply power back-off.

2. Actions:

To: RAN5

RAN4 respectfully asks RAN5 to take the above RAN4 responses into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

- 3GPP RAN4 #104							Aug 22nd – 26th, 2022					   Toulouse

- 3GPP RAN4 #104bis-e						Oct 10th – 19th, 2022					        Online