3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207737

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2287 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Power Saving CA Handling Clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 04/25/2022 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SSB based RLM and CSI-RS based BFD on Scell case is missing from relaxation criterion definition on CR from last meeting. Description for BFD on which cells counted into good serving cell condition is also vague due to the language of “serving cell” and “a serving cell”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clairfy CA handling according to the agreed WF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA Handling is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1, 8.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

#### 8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria

For the UE supports [connected mode power saving] and configured with explicit signaling [TBD], the relaxed requirements defined in clause 8.1.2.4 for SSB based radio link monitoring and the relaxed requirements defined in clause 8.1.3.4 for CSI-RS based radio link monitoring should apply to the UE:

-	for the serving cells in intra-band carrier aggregation configured with CSI-RS based BFD on SCell when

-	UE has fulfilled good serving cell quality criterion defined in [TBD] based on RLM and BFD measurements configured on all the serving cells in the intra-band CA if the low mobility criteria is not configured, or

-	UE is also configured with low mobility criterion defined in [TBD] and UE has fulfilled both good serving cell quality criterion defined in [TBD] based on RLM and BFD measurements configured on all the serving cells in the intra-band CA and low mobility criterion defined in [TBD].

-	for other serving cells when

-	UE has fulfilled good serving cell quality criterion defined in [TBD] based on RLM and BFD measurements configured on the serving cell if the low mobility criteria is not configured, or

-	the UE is configured with low mobility criterion defined in [TBD], and UE has fulfilled both good serving cell quality criterion defined in [TBD] based on RLM and BFD measurements configured on the serving cell and low mobility criterion defined in [TBD].

otherwise, UE shall apply the requirements defined in clause 8.1.2.2 for SSB based radio link monitoring and the requirements defined in clause 8.1.3.2 for CSI-RS based radio link monitoring.

The UE is not allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met:

-	The UE sends out-of sync indications to the higher layers,

-	The UE has triggered T310 timer

<End of Change #1>

<Start of Change #2>

#### 8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria

For the UE supports [connected mode power saving] and configured with dedicated signaling [TBD], the relaxed requirements defined in clause 8.5.2.4 for SSB based beam failure detection and the relaxed requirements defined in clause 8.5.3.4 for CSI-RS based beam failure detection should apply to the UE:

-	for the cells in intra-band carrier aggregation configured with CSI-RS based BFD on SCell when:

-	UE has fulfilled good serving cell quality criterion defined in [TBD] based on RLM and BFD measurements configured on all the serving cells in the intra-band CA if the low mobility criteria is not configured, or

-	UE is also configured with low mobility criterion defined in [TBD] and UE has fulfilled both good serving cell quality criterion defined in [TBD] based on RLM and BFD measurements configured on all the serving cells in the intra-band CA and low mobility criterion defined in [TBD].

-	for other serving cells when:

-	UE has fulfilled good serving cell quality criterion defined in [TBD] based on RLM and BFD measurements configured on the serving cell if the low mobility criteria is not configured, or

-	the UE is configured with low mobility criterion defined in [TBD], and UE has fulfilled both good serving cell quality criterion defined in [TBD] based on RLM and BFD measurements configured on the serving cell and low mobility criterion defined in [TBD].

otherwise, UE shall apply the requirements defined in clause 8.5.2.2 for SSB based beam failure detection and the requirements defined in clause 8.5.3.2 for CSI-RS based beam failure detection.

Note: 	the explicit signalling indicates that the low mobility state of the UE, if the low mobility criteria is not configured

The scenario and RS resource configurations in the set  defined in section 8.5.1 apply for this section.

The UE is not allowed to apply the relaxed link recovery procedures provided that at least one of the following conditions is met:

The UE sends a beam failure instance indication to the higher layers,

The UE has triggered T310 timer.

<End of Change #2>