3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207768

# Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.9.2.1

Source:	Apple

Title:	On test case list for SRS antenna port switching

Document for:	Approval

# Introduction

In last RAN #95e meeting, the core part of R17 FeRRM WI has been completed [1]. Based on the TU budget plan [RP-221019], the performance part work shall start from this RAN4 #103e meeting. In this contribution the test case list for SRS antenna port switching is proposed.

# Test case list for SRS antenna port switching

For SRS antenna port switching, as discussed in WI summary [2], RAN4 has specified interruption requirement for SRS antenna port switching as well as the impact to other existing RRM requirements:

- Interruption requirements were defined for two scenarios:

  - Scenario 1: when X=1 SRS symbol is configured in a slot for SRS antenna port switching, the configured number of SRS symbols is used as SRS transmission time

  - Scenario 2: otherwise, using X=6 SRS symbols in a slot as assumption of SRS transmission time

- RAN4 specified:

  - Interruption requirement (symbol-level) for scenario 1 sync case

  - Interruption requirement (slot-level) for scenario 1 async case

  - Interruption requirement (slot-level) for scenario 2

RAN4 also had pre-meeting discussion on the test case list, and following test case list and candidate company list is proposed:

| TC index | TC | DC/CA mode | Purpose | Section ID | Candidate companies |
| --- | --- | --- | --- | --- | --- |
| 1 | E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in synchronous EN-DC | EN-DC tests with all NR cells in FR1 | To verify the interruption of FR1 NR SRS antenna switching on LTE PCC and FR1 NR SCC for scenario 1 sync case | A.4.5.2.x1 | Nokia |
| 2 | E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in asynchronous EN-DC | EN-DC tests with all NR cells in FR1 | To verify the interruption of FR1 NR SRS antenna switching on LTE PCC and FR1 NR SCC for scenario 1 async case | A.4.5.2.x2 | CATT |
| 3 | E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with more than 1 SRS symbol in asynchronous EN-DC | EN-DC tests with all NR cells in FR1 | To verify the interruption of FR1 NR SRS antenna switching on LTE PCC and FR1 NR SCC for scenario 2 | A.4.5.2.x3 | Huawei |
| NA | No test is needed due to FR1+FR2 testing issue (SRS on FR1 and interruption on FR2). | EN-DC tests with one or more NR cells in FR2 | NA | NA |  |
| 4 | NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in NR-CA | NR standalone tests with all NR cells in FR1 | To verify the interruption of FR1 NR SRS antenna switching on FR1 NR SCC for scenario 1 (only sync case) | A.6.5.2.x1 | vivo |
| 5 | NR FR1 interruptions at NR SRS antenna port switching with more than 1 SRS symbol in NR-CA | NR standalone tests with all NR cells in FR1 | To verify the interruption of FR1 NR SRS antenna switching on FR1 NR SCC for scenario 2 | A.6.5.2.x2 | Intel |
| NA | No test is needed due to FR1+FR2 testing issue (SRS on FR1 and interruption on FR2). | NR standalone tests with one or more NR cells in FR2 (NR CA and NR-DC) | NA | NA |  |
| 6 | NR FR1 - E-UTRAN interruptions at NR SRS antenna port switching with 1 SRS symbol in synchronous NE-DC | NE-DC with all NR cells in FR1 | To verify the interruption of FR1 NR SRS antenna switching on LTE PSCC and FR1 NR SCC for scenario 1 sync case | A.4A.3.1.x1 | OPPO |
| 7 | NR FR1 - E-UTRAN interruptions at NR SRS antenna port switching with 1 SRS symbol in asynchronous NE-DC | NE-DC with all NR cells in FR1 | To verify the interruption of FR1 NR SRS antenna switching on LTE PSCC and FR1 NR SCC for scenario 1 async case | A.4A.3.1.x2 | Xiaomi |
| 8 | NR FR1 - E-UTRAN interruptions at NR SRS antenna port switching with more than 1 SRS symbol in asynchronous NE-DC | NE-DC with all NR cells in FR1 | To verify the interruption of FR1 NR SRS antenna switching on LTE PSCC and FR1 NR SCC for scenario 2 | A.4A.3.1.x3 | MTK |

# Conclusion

In this contribution we discuss the test case list for SRS antenna port switching.

Proposal: Agree the test case list for SRS antenna port switching in this contribution.

# References

[1] RP-220443, SR of further RRM enhancement for NR and MR-DC, Apple, RAN #95-e meeting

[2] RP-220443, Summary for WI: Further RRM enhancement for NR and MR-DC, Apple, CATT, RAN #95-e meeting