3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207799

# Electronic Meeting, May 09 - May 20, 2022

Agenda item:	9.11.2.1.1

Source:	Apple Inc.

Title:	Simulation results for PDSCH requirements in Intercell Interference

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#102-e PDSCH demodulation requirements with MMSE-IRC for inter-cell interference was discussed and way forward [1] was agreed. In this contribution we discuss the open issues and present simulation results for PDSCH demod requirements in ICI.

# 2. Discussion

One of the open issues in RAN4#102-e for PDSCH demod requirements in ICI was Interference modelling in PDCCH region. The following options were discussed in the last meeting captured in [1]:

  - Considered options

    - Option 1: Reuse the LTE PDSCH IRC testing approach. NR interference model to have unallocated RE’s in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports in the requirement scenario.

    - Option 2: Assume PDCCH transmission from interference cells

    - Option 3: Assume PDCCH transmission from interference cells and use non-overlapping PDCCH configurations. Use parameters in Table 2-4 from R4-2205789 as PDCCH configurations

    - Option 4: Assume Option 3 when SSB is non-colliding and Option 2 when SSB is colliding.

  - Companies are encouraged to check the impact of PDCCH decoding on PDSCH performance

  - Companies are encouraged to check whether there are any RRM related issues (RLM, RLF) for one of considered options

In the real network scenario PDCCH would typically see interference from PDCCH from other cells. It would be unrealistic to assume that PDCCH is interference free. Hence, Option 3 would be impractical. With option 4, we don’t think the network would co-ordinate PDCCH transmission to reduce interference if SSB transmission is non-overlapping.

Observation #1: It would be unrealistic to assume that PDCCH would be interference free in ICI scenarios or that network would co-ordinate PDCCH transmission if SSB are non-overlapping

There was also a concern in-sync/ out-of-sync evaluation might be affected if there is interference on PDCCH and no interference on SSB. If SSB is used as RLM-RS and is non-overlapping with other SSB, it may be interference free or see interference from other channel from other cells. In case its interference free the SSB based measurements at the UE might be optimistic. For the SINR operating range considered for ICI requirements, all > -4dB, we don’t expect any issue with in-sync/ out-of-sync evaluation and the SNR threshold of out-of-sync is much lower. Also, PDCCH is a robust channel and for the operating SINR used for ICI requirements, we don’t expect to see PDCCH decoding errors.

Observation #2: We don’t expect to see any problem with out-of-sync/in-sync evaluation if there is no interference on SSB and interference on PDCCH for the SINR ranges considered for ICI requirements.

Observation #3: We don’t expect to PDCCH decoding error in the SINR ranges considered for ICI requirements.

Based on the above observations, it would be suitable to introduce either PDCCH transmission from interference cells or have QPSK modulated random symbols in PDCCH region. For all channels – TRS, NZP CSI-RS, PDSCH we have the same signal transmitted from interference cell as the target cell. For easier test setup it would be suitable to also have PDCCH transmitted from interference cell in PDCCH region.

Proposal #1: For interference modelling in PDCCH assume PDCCH transmission from interference cells.

# 3. Simulation Results

For the simulation assumptions agreed for PDSCH demodulation requirements in ICI, we present simulation results below.

Table 1: Performance results for PDSCH demod in ICI

| Ant. Config | Duplex | Num. Inf Cells | Deployment | Prop. Channel | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 2x2 | FDD | 2 | HomoNet | TDLC300-100 | 14.4 | 16.4 |
|  |  | 1 | HetNet | TDLA30-10 | 11 | 13 |
|  | TDD | 2 | HomoNet | TDLC300-100 | 14.6 | 16.6 |
|  |  | 1 | HetNet | TDLA30-10 | 10.9 | 12.9 |
| 2x4 | FDD | 2 | HomoNet | TDLC300-100 | 10.7 | 12.7 |
|  |  | 1 | HetNet | TDLA30-10 | 6.2 | 8.2 |
|  | TDD | 2 | HomoNet | TDLC300-100 | 11.1 | 13.1 |
|  |  | 1 | HetNet | TDLA30-10 | 6.6 | 8.6 |

# 4. Conclusion

In this paper, we provide our views on the open issues for requirements definition for PDSCH demod with ICI and simulation results for requirements definition. Our observations and proposals are captured below:

Observation #1: It would be unrealistic to assume that PDCCH would be interference free in ICI scenarios or that network would co-ordinate PDCCH transmission if SSB are non-overlapping

Observation #2: We don’t expect to see any problem with out-of-sync/in-sync evaluation if there is no interference on SSB and interference on PDCCH for the SINR ranges considered for ICI requirements.

Observation #3: We don’t expect to PDCCH decoding error in the SINR ranges considered for ICI requirements.

Proposal #1: For interference modelling in PDCCH assume PDCCH transmission from interference cells.

# Reference

- R4-2207242, “WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel.