3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207806

# Electronic Meeting, May 09 -  May 20, 2022

Agenda item:	9.18.2.1

Source:	Apple Inc.

Title:	Discussion on RRM requirements for Unified TCI

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#102-e requirements for intercell beam management were discussed and way forward [1] was agreed.  In this contribution we present our views on open issues on requirements for unified TCI.

# 2. Discussion

## UL TCI State Switching Requirements

Condition for DCI based switch

For DCI based DL TCI state switch it is agreed that the target TCI state is known and in active TCI state list. But for UL TCI state switch it is FFS:

Issue 1-2-1 Requirement applicability of DCI based DL and UL TCI state switching delay

- target TCI state is known

- Note: UL TCI known state will be further clarified

- target TCI state is in active TCI state list for DL and joint TCI switch

- FFS: target TCI state is maintained for UL and joint TCI state switch.

In unified TCI framework, we have a activated DL TCI – active Dl TCI state list and activated UL TCI - UL TCI state list. The UE is expected to track the TCI states in activated DL TCI and this is also reflected in requirements from Rel-15 on DL TCI state switch. Also, the UE is expected to maintain the PL-RS in or associated with the activated UL TCI states – or UL TCI state list.

The following agreement in RAN1#105-e also supports this:

DCI based TCI state switch is expected to be fast without any additional measurements required at the UE prior to the switch, hence we propose that for DCI based UL TCI state switch, the requirements are applicable if the target TCI state is known and in activated UL TCI state.

Proposal #1: DCI based UL TCI state switch shall be applicable when target TCI state is known and in activated UL TCI state list.

Requirements when PL-RS is SSB in FR2

In the last meeting there was discussion on UL TCI state switching delay when PL-RS is SSB:

Issue 1-2-5 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2

- Proposals

  - Option 1:

    - When a SSB is indicated as PL-RS in a UL TCI state, the scaling factor for beam sweeping needs to be introduced for PL-RS measurement time in FR2.

    - In FR2, the MAC-CE based UL TCI state switching delay need to be separately defined for SSB based PL-RS.

    - In FR2, when a SSB is indicated as PL-RS in a UL TCI state, the MAC-CE based UL TCI state switching delay for both known case and unknown case can be defined as:

    - THARQ + 3ms + NM*(5*TL1-RSRP_SSB + 2ms) with the assumption of M=1.

    - Where NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise.

  - Option 1a: The delay is

    - TFirstSSB + 39*TSSB

  - Option 2:

    - TL1-RSRP_SSB as (M*P)*TSSB

  - Option 3: Define generic requirement

  - Option 4: Further clarification is needed

In case the PL-RS in or associated with the UL TCI state is SSB and the PL-RS is not maintained, additional time for PL-RS measurement is needed. In FR2 all SSB based measurements account for Rx beam sweeping time, so the PL-RS measurement in FR2 if based on SSB should also account for Rx beam sweep time.

Hence, for MAC-CE based UL TCI state switch in FR2, when PL-RS for UL TCI is SSB the switching delay should be defined as:

THARQ + 3ms + NM*( TFirstSSB  + 39*TSSB + 2ms)

Proposal #2: When PL-RS of target TCI state is SSB, in FR2 the time for PL-RS measurement shall also include Rx beam sweeping time. Define the MAC-CE based TCI state switching delay as: THARQ + 3ms + NM*( TFirstSSB  + 39*TSSB + 2ms)

Requirements for UL active TCI state list update

The UE receives MAC-CE for TCI states to be activated. Once the TCI states are activated in UL TCI state list, the UE can receive a DCI based indication for any of the activated TCI states to transmit on the UL with that TCI. Hence, the delay should be defined for when the UE is able to receive PDCCH for DCI based indication. The DCI can be with or without DL assignment.

Observation#1: After the TCI is activated in UL (or DL) TCI state list, the UE can receive DCI with TCI indication for the activate TCI state.

Proposal #3: The active TCI state list update delay specifies the time for UE to be able to receive DCI for TCI indication.

## Common TCI

In RAN4#102-e there was discussion on known condition for CA scenario:

Issue 1-4-1 Known condition in CA scenario

- Proposals

  - Option 1(ZTE):

    - Reuse the existing known condition. Once the source RS of target TCI state is known for each CC in the intra-band CC group, which means the known condition is satisfied.

  - Option 2 (Intel, Nokia, vivo, Ericsson, Samsung, MTK, ZTE):

    - If the associated RS in common TCI state provides QCL-TypeD, the known condition can only consider whether the associated RS in the reference CC is known or not.

  - Option 3(Apple, ZTE):

    - The known condition should be dependent on shared RS or different RS.

For common TCI indication introduced in R17, the CCs can be indicated a common TCI ID. Hence the common TCI switch for CA can be one of 2 cases:

Case 1: The TCI states associated with common TCI ID is the same or shared RS

Case 2: The TCI states associated with common TCI ID are different RS

If the TCI states are associated with different RS, the TCI state switching delay would apply for each CC separately depending on the known condition, target TCI in active TCI list and whether PL-RS is maintained. There is no necessity to specify a switching delay for this case.

Observation #2: If the TCI states associated with common TCI ID are different RS then the switching delay would apply for each CC separately.

If the TCI states associated with common TCI ID is the same or shared RS, then a common TCI state switching delay can be defined.

Observation #3: If the TCI states associated with common TCI ID are the same or shared RS then a common switching delay can be defined.

We propose that we only define common TCI switching requirements for TCI states associated with common TCI ID is the same or shared RS.

Proposal #4: Define requirements for common RS case only – when the target TCI state ID is associated with the same RS for all CCs

In the approved draft CRs for unified TCI, the following was the wording used for potentially capturing requirements for common TCI:

Common TCI is not indicated by simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 . These IEs were introduced in Rel-16 for simultaneous TCI update and not in Rel-17 for common TCI update. RAN2 is discussing the IE to indicate common TCI. From the running CR, the IE used to indicate common TCI is simultaneousU-TCI-UpdateListn

Hence we propose to delete [all CCs with a common indicated TCI-State in the same CC list configured by simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2] in DCI based TCI switch delay.

Proposal #5: Delete the wording in “[ ]”:  [all CCs with a common indicated TCI-State in the same CC list configured by simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2]  for DCI based downlink TCI state switch delay

We propose to further discuss how the common TCI update is captured in requirements, for shared RS.

Proposal #6: Further discuss how to capture common TCI switch with common RS if necessary.

# 3. Conclusion

In this paper, we provide our views on open issues on requirements for unified TCI. Our observations and proposals are captured below:

UL TCI state switching requirements

Proposal #1: DCI based UL TCI state switch shall be applicable when target TCI state is known and in activated UL TCI state list.

Proposal #2: When PL-RS of target TCI state is SSB, in FR2 the time for PL-RS measurement shall also include Rx beam sweeping time. Define the MAC-CE based TCI state switching delay as: THARQ + 3ms + NM*( TFirstSSB  + 39*TSSB + 2ms)

Observation#1: After the TCI is activated in UL (or DL) TCI state list, the UE can receive DCI with TCI indication for the activate TCI state.

Proposal #3: The active TCI state list update delay specifies the time for UE to be able to receive DCI for TCI indication.

Common TCI Indication

Observation #2: If the TCI states associated with common TCI ID are different RS then the switching delay would apply for each CC separately.

Observation #3: If the TCI states associated with common TCI ID are the same or shared RS then a common switching delay can be defined.

Proposal #4: Define requirements for common RS case only – when the target TCI state ID is associated with the same RS for all CCs

Proposal #5: Delete the wording in “[ ]”:  [all CCs with a common indicated TCI-State in the same CC list configured by simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2]  for DCI based downlink TCI state switch delay

Proposal #6: Further discuss how to capture common TCI switch with common RS if necessary.

# Reference

- R4-2206943, “WF on FeMIMO RRM impact for unified TCI state”, Intel.