3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207809

# Electronic Meeting, May 09 - May 20, 2022

Agenda item:	9.18.2.3

Source:	Apple Inc.

Title:	Discussion on other RRM requirements for FeMIMO

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#102-e requirements for FeMIMO were discussed and way forward [1] was agreed.  In this contribution we present our views on requirements for TRP specific BFR.

# 2. Discussion

In [1] the agreements for TRP specific BFR were:

- RRM requirements for TRP-specific BFR should be specified for FR1 in R17 and do not introduce sharing factor PTRP in FR1; - Introduce PTRP = 2 in FR2 for overlapping resources for equal sharing between BFD/ CBD resources (SSB and CSI-RS) between the 2 TRPs. - FFS the conditions on applying PTRP = 2 in FR2

In FR2 we need to define a sharing factor for the case when BFD-RS and CBD -RS from 2 TRPs are overlapped in time, since we don’t assume simultaneous reception with different QCL Type D in Rel-17.

In multi-TRP scenario the UE is receiving from 2 TRPs simultaneously and the TRP specific BFR is to enable the beam quality for each TRP to be separately monitored. In case the resources are overlapping for BFD and CBD in FR2, the UE should have equal opportunities to measure resources from each TRP. The sharing factor of 2 is already agreed in last meeting.

The condition for the sharing factor is if resources are overlapping in FR2. In addition, if the UE is capable of simultaneous reception with different QCL type D, it might not need this sharing factor, but since we are not introducing requirements with simultaneous QCL Type D in Rel-17, we need not consider that case. The baseline assumption for minimum requirements is that UE is doesn’t support simultaneous reception with different QCL Type D.

Proposal #1: The baseline assumption in Rel-17 for RAN4 minimum requirements is that UE doesn’t support simultaneous reception with different QCL Type D.

Proposal #2: Condition for PTRP = 2 is overlapping BFD/CBD resources in FR2.

In the agreed big CR for FeMIMO requirements [2], we have the following:

For the first FFS in “[ ]”, we don’t see the use case in multi-TRP scenario where the UE is receiving from TRPs with the same QCL-Type D. Hence, we don’t think the first FFS should be captured in spec.

The second line in “[ ]” is not possible in our understanding. The requirements are defined to specify TRP specific BFD requirements. UE will evaluate the quality of BFD RS on the two sets q0,0 and q0,1in the specified measurement period. We don’t see how PTRP=1 condition is possible with evaluation on q0,0 alone. Hence, we propose to delete the sentence in square brackets.

Proposal #3: Delete the sentences in square brackets for exceptions to PTRP=2 in TRP specific BFD requirements.

# 3. Conclusion

In this paper, we provide our views on requirements for TRP specific BFR. Our observations and proposals are captured below:

Proposal #1: The baseline assumption in Rel-17 for RAN4 minimum requirements is that UE doesn’t support simultaneous reception with different QCL Type D.

Proposal #2: Condition for PTRP = 2 is overlapping BFD/CBD resources in FR2.

Proposal #3: Delete the sentences in square brackets for exceptions to PTRP=2 in TRP specific BFD requirements.

# Reference

- R4-2206935, “WF on other RRM requirements for FeMIMO”, Huawei, HiSilicon.

- R4-2207123, “Big CR: RRM requirements for Rel-17 NR feMIMO”, Samsung.