3GPP TSG RAN WG4 Meeting #103-e	R4-2207848

Online, 9th – 20th May 2022

Agenda item:	8.15.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-21: EN-DC_41_n1-n3

Document for:	Approval

1	Introduction

EN-DC of 1B LTE and 2B NR of DC_41_n1-n3 was approved in RAN [1]. This TP is to capture the basic aspects for the EN-DC.

2	Text Proposal

[Unchanged Parts Skipped]

6.X	DC_41_n1-n3

6.X.1	Operating bands for DC

Table 6.X.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41_n1-n3 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |

6.X.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Channel bandwidth [MHz] | Maximum aggregated bandwidth For DL [MHz] |
| DC_41A_n1A-n3A | DC_41A_n1A DC_41A_n3A | 41 | 5, 10, 15, 20 | 70 |
|  |  | n1 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
| DC_41C_n1A-n3A | DC_41A_n1A DC_41A_n3A | 41 | CA_41C | 90 |
|  |  | n1 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |

6.X.3	Co-existence studies

Based on the co-existence studies of DC_41A-n1A and DC_41A-n3A, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.X.4	∆TIB and ∆RIB values

For DC_41_n1-n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n1-n3 | 41 | 0.53 |
|  |  | 0.84 |
|  | n1 | 0.5 |
|  | n3 | 0.5 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41_n1-n3 | 41 | 03 |
|  |  | 0.54 |
|  | n1 | 0 |
|  | n3 | 0 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

6.X.5	MSD

As mentioned in 6.X.3, there is no need to specify additional MSD requirement for this UL DC configuration.

[Unchanged Parts Skipped]

3	Reference

[1]		RP-211750 Revised WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17, LG Electronics