3GPP TSG RAN WG4 Meeting #103-e	R4-2207849

Online, 9th – 20th May 2022

Agenda item:	8.7.2

Source:	SoftBank Corp.

Title:	TP for TR 38.717-04-01: CA_n1-n3-n41-n257

Document for:	Approval

1	Introduction

This contribution is a text proposal for TR 38.717-04-01 to include NR CA_n1-n3-n41-n257 as defined in WID [1].

2	Text Proposal

---Start of changes---

5.x	CA_n1-n3-n41-n257

5.x.1	Operating bands for CA

Table 5.xx.1-1: Inter-band CA operating bands of CA_n1-n3-n41-n257

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n1-n3-n41-n257 | n1, n3, n41, n257 |

5.x.2	Channel bandwidths per operating bands for CA

Table 5.x.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n41A-n257A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n41A-n257G | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n41A-n257H | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n41A-n257I | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n257 | CA_n257I |  |

5.x.3	∆TIB,c and ∆RIB,c values

For four simultaneous DLs and one UL of Band combinations n1-n3-n41-n257, the TIB,c and RIB,c  values are shown in table 5.x.3-1 and  table 5.x.3-2, respectively.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n41-n257 | n1 | 0.5 |
|  | n3 | 0.5 |
|  | n41 | 0.35 |
|  |  | 0.86 |
|  | n257 | 0 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n41-n257 | n1 | 0 |
|  | n3 | 0 |
|  | n41 | 05 |
|  |  | 0.56 |
|  | n257 | 0 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

5.x.4	REFSENS requirements

MSD requirements are captured in the lower order combinations.

---End of changes---

3	Reference

[1] 		RP-220827, “Revised WID: Rel-16 NR inter-band CA for 4 bands DL with 1 band UL”, Ericsson