3GPP TSG-RAN WG4 Meeting #103e	R4-2207862

Electronic Meeting, May 09 - May 20, 2022

Source:	China Telecom

Title:	Maintenance of phase continuity requirements for DMRS bundling

Agenda Item:	9.17.1.1

Document for:	Discussion

- Introduction

In RAN4 #102e meeting, the WF on issues for maintenance of NR coverage enhancements was approved in [1]. This contribution provides our views on the maintenance of phase continuity requirements for DMRS bundling.

- Discussion

2.1	Phase continuity requirement applicability for bands capable of UL-MIMO or TxD

The following issue was listed in the WF [1]:

Combing DMRS bundling feature with TxD and UL MIMO feature

To further confirm the phase continuity tolerance requirement applicability on the feature of

1. TxD

2. UL MIMO

As we know, DMRS bundling and 2Tx with 2 PAs (1-layer UL-MIMO or TxD) are two techniques for coverage enhancement and can be used together. For bands such as n77/78, 2Tx is one popular UE implementation, and UL-MIMO or TxD has already been supported in these bands for some UEs. The coverage performance from network perspective would be sacrificed if we artificially prevent the use of DMRS bundling for the bands capable of UL-MIMO or TxD.

Therefore, we support to clarify how the phase tolerance requirements are applicable to the bands capable of UL-MIMO or TxD. Also, the intention is not to add any new requirements, but just clarify the requirement applicability. Otherwise, if UE declares the support of UL-MIMO or TxD, and also the support of DMRS bundling in some bands, there is no requirement for testing.

In the last meeting, the main concern on clarifying the phase continuity requirement applicability for bands capable of UL-MIMO or TxD is that it is not quite sure whether the same phase continuity tolerance requirements can be directly applied to 2Tx case.

In our view, in the simulation for tolerable phase offset, only the number of DMRS ports matters, regardless of how many Tx chains are equipped at UE. In coverage limited scenario, one DMRS port will be configured for UEs with either 1Tx or 2Tx, so the same phase continuity requirement can be applied for UEs with 1Tx and 2Tx.

Observation 1: The coverage performance from network perspective would be sacrificed if we artificially prevent the use of DMRS bundling for the bands capable of UL-MIMO or TxD.

Observation 2: Without the addition of the requirement applicability in the spec, if UE declares the support of UL-MIMO or TxD, and also the support of DMRS bundling in some bands, there is no requirement for testing.

Observation 3: In the simulation for tolerable phase offset, only the number of DMRS ports matters, regardless of how many Tx chains are equipped at UE, and one DMRS port will be configured for UEs with either 1Tx or 2Tx in coverage limited scenario.

Proposal 1: The phase continuity requirement agreed in RAN4 #102e can be applied for the bands capable of UL-MIMO or TxD.

2.2	Restriction on modulation order

In the RAN1 LS on updated Rel-17 RAN1 UE features list for NR in [2], the following issue was raised:

For NR_cov_enh, based on RAN4 feedback, FG 30-4 is defined without distinction of the modulation order. RAN1 has no intention to revert any existing RAN4 agreements on maximum duration. It is up to RAN4 to decide whether FG 30-4 is applicable only to QPSK and lower modulation order.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH | FFS | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

We support to add the restriction on the modulation order in the UE feature list. For deriving the phase continuity requirements within certain maximum duration, only modulation orders up to QPSK have been simulated in RAN4. For other higher modulation orders (if considered), probably tighter requirements shall be required to obtain the gain from JCE. So, we don’t think if UE passes the RAN4 Rel-17 requirements for DMRS bundling within a certain maximum duration, it means UE can support the DMRS bundling for higher modulation orders within the same maximum duration. To make the specification clearer, from RAN4 perspective, it is beneficial to add the restriction on the modulation order in the column of “note” for FG 30-4 in the UE feature list.

Observation 4: The RAN4 Rel-17 requirements for DMRS bundling are derived based on the simulations for up to QPSK modulation with certain maximum duration. For other higher modulation orders (if considered), probably tighter requirements shall be required to support DMRS bundling and to obtain the JCE gain.

Proposal 2: For FG 30-4 in RAN1 UE feature list, add in the column of “note” that: This is applicable for QPSK or lower modulation orders.

Based on the above observation and proposal, a LS to RAN1 and RAN2 is prepared in [3].

2.3	Removal of bracket on phase difference requirement

In the CRs agreed in the last RAN4 meeting, the requirement values for phase difference are in square bracket. We support to remove the brackets on [25] degrees and [30] degrees in this meeting.

Proposal 3: For the allowable phase difference requirement, remove the brackets on [25] degrees and [30] degrees.

- Conclusion

This contribution presented our views on the maintenance of phase continuity requirements for DMRS bundling, with the following observations and proposals:

Considering the phase continuity requirement applicability for bands capable of UL-MIMO or TxD:

Observation 1: The coverage performance from network perspective would be sacrificed if we artificially prevent the use of DMRS bundling for the bands capable of UL-MIMO or TxD.

Observation 2: Without the addition of the requirement applicability in the spec, if UE declares the support of UL-MIMO or TxD, and also the support of DMRS bundling in some bands, there is no requirement for testing.

Observation 3: In the simulation for tolerable phase offset, only the number of DMRS ports matters, regardless of how many Tx chains are equipped at UE, and one DMRS port will be configured for UEs with either 1Tx or 2Tx in coverage limited scenario.

Proposal 1: The phase continuity requirement agreed in RAN4 #102e can be applied for the bands capable of UL-MIMO or TxD.

Considering the restriction on modulation order:

Observation 4: The RAN4 Rel-17 requirements for DMRS bundling are derived based on the simulations for up to QPSK modulation with certain maximum duration. For other higher modulation orders (if considered), probably tighter requirements shall be required to support DMRS bundling and to obtain the JCE gain.

Proposal 2: For FG 30-4 in RAN1 UE feature list, add in the column of “note” that: This is applicable for QPSK or lower modulation orders.

Considering the removal of bracket on phase difference requirement:

Proposal 3: For the allowable phase difference requirement, remove the brackets on [25] degrees and [30] degrees.

- References

R4-2206593, WF on issues for maintenance of NR coverage enhancements, Ericsson, RAN4 #102e, Feb - Mar 2022.

R1-2202927, LS on updated Rel-17 RAN1 UE features list for NR, NTT DoCoMo, AT&T, RAN1 #108-e, Feb - Mar 2022.

R4-2207864, Reply LS on modulation order for DMRS bundling, China Telecom, RAN4 #103-e, May 2022.