3GPP TSG-RAN WG4 Meeting # 103-e														R4-2207940

Electronic Meeting, May 09 – 20, 2022

Source:	Verizon, Samsung

Title:	TP for TR 37.717-31-11: DC_2-13-66_n2

Agenda Item:	8.13.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11to include DC_2A-13A-66A_n2A according to the request in [1].

# 2. Reference

[1] RP-220826: Revised WID: Rel-17 Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)

# 3. Text Proposal

<Start of Text Proposal>

### 5.1.x	DC_2-13-66_n2

#### 5.1.x.1	Configuration for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-13A-66A_n2A | DC_66A_n2A |
| DC_2A-13A-66A-66A_n2A | DC_66A_n2A |

#### 5.1.x.2	∆TIB and ∆RIB values

Table 5.1.x.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13-66_n2 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.x.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-13-66_n2 | 2 | 0.3 |
|  | 13 | 0.0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

#### 5.1.1.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

<End of Text Proposal>