3GPP TSG-RAN WG4 Meeting #103-e 		R4-2208020

Electronic meeting, 9th – 20th May, 2022

Source:	Ericsson

Title:	Discussion on PUCCH demodulation requirements for NR extended to 71GHz

Agenda item:	9.15.10.3.2

Document for:	Discussion

# Introduction

In the last RAN4 meeting, companies have discussions on the NR extended to 71GHz demodulation requirements. Following WF [3] was agreed on the PUCCH demodulation requirement part.

Issue 2-4-1: Scope of PUCCH performance requirements

Define performance requirements for the enhanced PUCCH formats 0, 1, and 4.

Define new requirements for PUCCH formats 2 and 3.

Issue 2-4-2a: Detailed PUCCH test setup for PUCCH format 0

Number of PRBs:

- Option 1: 16 (enhanced format)

- Option 2: 10 (enhanced format)

Number of OFDM symbols:

- Option 1: 1

- Option 2: 1 and 2

Intra-slot frequency hopping: enabled if number of OFDM symbols higher than 1

Test metric: ACK missed detection rate < 1%

Issue 2-4-2b: Detailed PUCCH test setup for PUCCH format 1

Number of PRBs:

- Option 1: 1

- Option 2: 16 (enhanced format)

- Option 3: 10 (enhanced format)

Number of OFDM symbols:

- Option 1: 14

Intra-slot frequency hopping: enabled

Test metric: ACK missed detection rate < 1% and NACK to ACK probability < 0.1%

Issue 2-4-2c: Detailed PUCCH test setup for PUCCH format 2

Number of PRBs:

- Option 1: 9

Number of OFDM symbols:

- Option 1: 2

- Option 2: 1 and 2

Intra-slot frequency hopping: enabled

Test metric: ACK missed detection rate < 1%, UCI BLER < 1%

Issue 2-4-2d: Detailed PUCCH test setup for PUCCH format 3

Number of PRBs:

- Option 1: 1 and 3

Number of OFDM symbols:

- Option 1: 14 and 4

- Option 2: 4

Additional DMRS

- Option 1: With and without

- Option 2: Without

Intra-slot frequency hopping: enabled

Test metric: UCI BLER < 1%

Issue 2-4-2e: Detailed PUCCH test setup for PUCCH format 4

Number of PRBs:

- Option 1: 16 (Enhanced)

- Option 2: 10 (Enhanced)

Number of OFDM symbols:

- Option 1: 14

Additional DMRS

- Option 1: With and without

Intra-slot frequency hopping: enabled

Test metric: UCI BLER < 1%

In this contribution, open issues for PUCCH demodulation requirements will be further analyzed.

# Discussion

## 2.1 	 General

PRB numbers for multi-PRB PUCCH format

The maximum of multiple PRB for enhanced PUCCH is 16. Based on our simulation [4], there is no issue for 16 PRBs and the performance gain is 2dB compared to 10 PRBs which is aligned with the bandwidth gain. To check the performance, it would be better to define the requirement with the largest configuration.

On the other hand, FR2-2 band could be applied for both licensed and unlicensed deployment, then the UE power regulation for FR2-2 in different regions should also be considered. After checking TR38.807 [2], the regulations on 60GHz as shared access spectrum are captured in following Table 2.1-1. The calculation on UE power limitation can be summarized in Table 2.1-2.

Table 2.1-1 Regional Power Limitations

| Region | Maximum Conducted Power, Pmax (dBm) |
| --- | --- |
| US | Conducted power limit due to EIRP limit: Pmax_EIRP = 40 dBm – TxBF ["FCC EIRP"] Conducted power limit as a function of PUCCH BW per hop: Pmax_P = 27 dBm – max(0, 10*log10(100 / BW)) ["FCC 100 MHz"] Combined limit: Pmax = min(Pmax_P, Pmax_EIRP) |
| Europe | Conducted power limit due to EIRP limit: Pmax_EIRP = 40 dBm – TxBF ["ETSI EIRP"] Conducted power limit due to PSD limit (assumes N_RB contiguous RBs with all REs allocated per PRB): Pmax_PSD = 23 dBm/MHz + max(0, 10*log10(BW)) – TxBF ["ETSI PSD"] Combined limit: Pmax = min(Pmax_PSD, Pmax_EIRP) |
| South Korea | Conducted power limit due to EIRP limit: Pmax_EIRP = 43 dBm – TxBF when an equipment is >=300m from an astronomical antenna Pmax_EIRP = 27 dBm – TxBF when an equipment is <300m from an astronomical antenna ["SK EIRP"] Conducted power limit due to PSD limit (assumes N_RB contiguous RBs with all REs allocated per PRB): Pmax_PSD = 13 dBm/MHz + max(0, 10*log10(BW)) – TxBF ["SK PSD"] Combined limit: Pmax = min(Pmax_PSD, Pmax_EIRP) |
| TxBF: Transmit antenna beamforming gain BW: PUCCH bandwidth per hop in MHz |  |

Table 2.1-2 UE Power Limitations

| UE Power Limitations expressed in terms of maximum conducted power | Conducted power limit due to UE_EIRP: Pmax_EIRP = UE_EIRP – TxBF ["UE-Limit-EIRP"] Conducted power limit due to UE_P: Pmax_P = UE_P – backoff ["UE-Limit-P“] Combined limit: Pmax = min(Pmax_P, Pmax_EIRP) |
| --- | --- |
| TxBF: Transmit antenna beamforming gain backoff: Backoff required due to fluctuations in power, approximated with cubic metric UE_EIRP: UE equivalent isotropic radiated power UE_P: UE maximum allowed transmit power |  |

If we assume UE_EIRP = 25dBm, UE_P = 21dBm and TxBF = 6dBi as an example, the maximum useful bandwidth and number of PRBs for each SCS can be summarized in Table 2.1-3. In this example, the maximum PRBs for Europe would be very limited for all SCSs.

Table 2.1-3 Example for maximum useful bandwidth and number of RBs

| Region | Maximum useful bandwidth | Maximum number of RBs for 120 kHz/ 480 kHz /960 kHz |
| --- | --- | --- |
| US | 15.8 MHz | 11 / 3 / 2 |
| Europe | 1.6 MHz | 2 / 1 / 1 |
| South Korea | 15.8 MHz | 11 / 3 / 2 |

Observation 1: Different maximum PRBs would be applied for different SCS regarding to power regulation on shared access spectrum.

Observation 2: It will be very limited PRBs could be applied in some regions.

The requirements are expected to cover both licensed and unlicensed deployments if possible, so smaller PRBs might be suitable for this expectation.

Proposal 1: Further discussion on how many PRBs applied for multiple PRB PUCCH format requirements is needed regarding cover both licensed and unlicensed deployment.

As analysis above, the multi-PRB feature might not be applied in some region. Furthermore, the multiple PRB feature for PUCCH is optional and single PRB feature is basic feature. To avoid no requirement for FR2-2 PUCCH format 0/1/4, single PRB requirement would be necessary.

Proposal 2: Requirement for single PRB should also be introduced for PUCCH format 0/1/4 to avoid no requirement condition when multi-PRB feature can’t be applied.

## 2.2 	Configurations

The two OFDM symbol would check the frequency hopping. Considering FR2-1 have defined requirements for both 1 and 2 symbols, it would be better to follow FR2-1 to keep the same test coverage for FR2-2.

If both single PRB and multi-PRB requirements would be introduced for PF0, only single PRB requirement including 2 symbols configuration would be enough.

Proposal 3: Define PUCCH format 0 requirement with both 1 symbol and 2 symbols for single PRB feature to keep the same coverage as FR2-1, and only 1 symbol requirement for multi-PRB feature.

For format 2 and format 3, the PRB numbers should also be reviewed as Proposal 1 mentioned above. The other configurations, such as OFDM symbol number and DM-RS configuration, could follow FR2-1 to keep the same test coverage.

Proposal 4: Review PRB number for PUCCH format 2/3 and keep the same test coverage as FR2-1.

# Conclusions

Observation 1: Different maximum PRBs would be applied for different SCS regarding to power regulation on shared access spectrum.

Observation 2: It will be very limited PRBs could be applied in some regions.

Proposal 1: Further discussion on how many PRBs applied for multiple PRB PUCCH format requirements is needed regarding cover both licensed and unlicensed deployment.

Proposal 2: Requirement for single PRB should also be introduced for PUCCH format 0/1/4 to avoid no requirement condition when multi-PRB feature can’t be applied.

Proposal 3: Define PUCCH format 0 requirement with both 1 symbol and 2 symbols for single PRB feature to keep the same coverage as FR2-1, and only 1 symbol requirement for multi-PRB feature.

Proposal 4: Review PRB number for PUCCH format 2/3 and keep the same test coverage as FR2-1.

# References

[1]	TR38.808 v17.1.0, Study on supporting NR from 52.6 GHz to 71 GHz (Release 17).

[2]	TR38.807 v16.1.0, Study on requirements for NR beyond 52.6 GHz (Release 16).

[3]	R4-2207223, WF for NR extended to 71GHz demodulation, Intel, RAN4#102-e

[4] 	R4-2208023, Simulation results for FR2-2 PUCCH demodulation requirement, Ericsson