3GPP TSG-RAN WG4 Meeting #103-e 		R4-2208022

Electronic meeting, 9th – 20th May, 2022

Source:	Ericsson

Title:	Simulation results for FR2-2 PUSCH demodulation requirements

Agenda item:	9.15.10.3.1

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the NR extended to 71GHz demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

Table 2-1 Simulation parameters for FR2-2 PUSCH demodulation

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS:3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | 120kHz SCS: 100M |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2, Disabled |
| configuration | Time density (LPT-RS) | 1, Disabled |
| Test metric | Normalized throughput | 70% |
| Antenna | Tx and Rx configuration | 1Tx 2Rx |
| Channel model |  | TDLA20_200 / TDLA20_2000 |
| MCS | 64QAM MCS table index | 4,16, 20, 22 |
| Phase noise | Model sets in TR38.808 | Set#1 |
| Note1: Companies deliver two sets of ideal simulation results for requirement discussion. Result set#1 is without phase noise and set#2 is with phase noise. • No Tx phase noise is modelled • Rx Phase noise is modelled only to find feasible FRC configuration (i.e. achieve maximum throughput and loss in comparison to scenarios without Rx phase noise is less than 1 dB) • 70 GHz carrier frequency is assumed Note2: CPE w/wo ICI compensation |  |  |

Table 2-1 Simulation results for FR2-2 PUSCH demodulation, 120kHz SCS 100MHz bandwidth

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | MCS | PN Model | Phase Compensation | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA20-200 Low | 70 % | 4 | No | No | No | -3.6 |
|  |  |  |  |  |  | Set1 | No | No | -3.5 |
|  |  |  |  |  |  | Set1 | CPE | Yes | -3 |
|  |  |  |  |  |  | Set1 | CPE+ICI | Yes | -3.2 |
|  |  | Normal | TDLA20-200 Low | 70 % | 16 | No | No | No | 7.1 |
|  |  |  |  |  |  | Set1 | No | No | 7.6 |
|  |  |  |  |  |  | Set1 | CPE | Yes | 8.1 |
|  |  |  |  |  |  | Set1 | CPE+ICI | Yes | 8.1 |
|  |  | Normal | TDLA20-200 Low | 70 % | 20 | No | No | No | 9.9 |
|  |  |  |  |  |  | Set1 | No | No | 11.5 |
|  |  |  |  |  |  | Set1 | CPE | Yes | 11.3 |
|  |  |  |  |  |  | Set1 | CPE+ICI | Yes | 11.3 |
|  |  | Normal | TDLA20-200 Low | 70 % | 22 | No | No | No | 12.1 |
|  |  |  |  |  |  | Set1 | No | No | 15 |
|  |  |  |  |  |  | Set1 | CPE | Yes | 14.2 |
|  |  |  |  |  |  | Set1 | CPE+ICI | Yes | 14.2 |
| 1 | 2 | Normal | TDLA20-2000 Low | 70 % | 4 | No | No | No | -3.3 |
|  |  |  |  |  |  | Set1 | No | No | -3.4 |
|  |  |  |  |  |  | Set1 | CPE | Yes | -2.6 |
|  |  |  |  |  |  | Set1 | CPE+ICI | Yes | -2.8 |
|  |  |  |  |  | 16 | No | No | No | 8.5 |
|  |  |  |  |  |  | Set1 | No | No | 9.1 |
|  |  |  |  |  |  | Set1 | CPE | Yes | 9.5 |
|  |  |  |  |  |  | Set1 | CPE+ICI | Yes | 9.4 |
|  |  |  |  |  | 20 | No | No | No | 11.4 |
|  |  |  |  |  |  | Set1 | No | No | 13.2 |
|  |  |  |  |  |  | Set1 | CPE | Yes | 13.1 |
|  |  |  |  |  |  | Set1 | CPE+ICI | Yes | 13.1 |
|  |  |  |  |  | 22 | No | No | No | 13.6 |
|  |  |  |  |  |  | Set1 | No | No | 17.1 |
|  |  |  |  |  |  | Set1 | CPE | Yes | 16.3 |
|  |  |  |  |  |  | Set1 | CPE+ICI | Yes | 16.3 |

Observation 1: For low modulation level cases (QPSK and 16QAM), phase noise impact could be ignored.

Observation 2: For 64QAM or higher modulation, peak throughput might not be achieved if no compensation is implemented.

Observation 3: CPE and ICI won’t help too much on the performance when SNR is low (<10dB) or number of PRB is small.

Observation 4: Peak throughput can still be achieved after CPE compensation when MCS <=20.  ICI won’t help when in most of cases.

# Conclusions

Simulation results are delivered for FR2-2 PUCCH demodulation further discussion.

Observation 1: For low modulation level cases (QPSK and 16QAM), phase noise impact could be ignored.

Observation 2: For 64QAM or higher modulation, peak throughput might not be achieved if no compensation is implemented.

Observation 3: CPE and ICI won’t help too much on the performance when SNR is low (<10dB) or number of PRB is small.

Observation 4: Peak throughput can still be achieved after CPE compensation when MCS <=20.  ICI won’t help when in most of cases.

# References

[1]	R4-2207223, WF for NR extended to 71GHz demodulation, Intel, RAN4#102-e

# Annex A Simulation result figures

- (b)

(c) 						(d)

Figure A-1 Simulation results for FR2-2 PUSCH demodulation with TDLA20-200. (a) MCS4, (b) MCS16, (c) MCS20, (d) MCS22

(a)  						(b)

(c)  						(d)

Figure A-2 Simulation results for FR2-2 PUSCH demodulation with TDLA20-2000. (a) MCS4, (b) MCS16, (c) MCS20, (d) MCS22,