3GPP TSG-RAN WG4 Meeting #103-e		R4-2208042

Electronic Meeting, May 09 – May 20, 2022

Agenda item:	11.2.3.4

Source:	Intel Corporation

Title:	TP to TR 38.844: Method comparison

Document for:	Discussion

# 1	Introduction

This contribution includes TP of method comparisons for section 7 of TR 38.844.

# Discussion

We previously agreed that for any particular UE method there would be no requirement for a particular BS method.  However, in the comparison table several of the items list limitations that are assuming the BS is limited to only one BS method.  Updates are proposed to correct this and leave the UE methods independent of the BS methods.

The specific changes include:

- Proposed feature comparison table of section 7.1

# 7	Conclusion

<<START OF CHANGES>>

#### 7.1 Comparison Between Different Schemes

This section provides a comparison of the proposed schemes using different criteria.

The comparison between different schemes is summarized Table 7.2.1.

Table 7.2.1. Comparison of different schemes

| Comparison Criteria | Overlapping CA (two cells) – described in Section 6.2.3 | Combined UE CBW (One cell) – described in Section 6.2.2 | Overlapping UE CBW from Network Perspective – described in Section 6.2.1 | Wider CBW (one cell) – described in Section 6.1 |
| --- | --- | --- | --- | --- |
| Regulatory requirement | No issue | No issue | No issue | No issue |
| UE performance degradation relative to minimum requirements | Legacy UE: Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. New UE: requirements could be defined | Legacy: Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. New UE: requirements could be defined | No issue | Legacy UE: Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. New UE: UE ACS and in-band blocking degradation in scenarios with multiple blockers |
| gNB complexity | gNB has to support CA and schedule the data without collision in the two CC’s overlapping PRBs | gNB has to support splitting the signal into 2 RF carriers with a predefined phase relationship | gNB has to support the irregular channel BW (can also be implemented through RF combining of 2 channels) | gNB has to support the irregular channel BW ( can be implemented through RF combination of 2 channels or next larger CBW method) |
| UE complexity | UE has to support intra-band NC CA. 1 less CC can be supported when irregular BW is used in combination from other bands, or more total CCs needed. | UE has to support RF architecture as in intra-band NC CA. Needs new capability to aggregate 2 RF channels in baseband. Complexity higher than CA because the baseband will need a new "combiner" module. 1 less CC can be supported when irregular BW is used in combination from other bands, or more total CCs needed. | No changes needed, supported by legacy UEs | For asymmetric BW combinations, UE needs to support when UL CBW is Smaller CBW and CL CBW is next Wider CBW |
| UE throughput | UEs supporting the feature can use the entire spectrum allocation, legacy UEs can use an already supported channel BW | UEs supporting the feature can use the entire spectrum allocation, legacy UEs can use an already supported channel BW | UE throughput based on existing channel BWs (5MHz for holdings <10MHz, 10MHz for holdings <15MHz, etc) | UE throughput based on how many RBs can be used |
| Spectral utilization | Channel edge guardband based on the aggregated channel BW (5MHz for <10MHz, 10MHz for <15MHz, etc), 2 SSBs are needed | Channel edge guardband based on the aggregated RF carrier BW (5MHz for <10MHz, 10MHz for <15MHz, etc), single SSB needed | Channel edge guardband based on the actual holding (can be same as Overlapping CA), 2 SSBs are needed | Depends on the usable number of RBs, only single SSB needed |
| Cell Spectral utilization | Entire spectrum holding can be used even only with legacy UEs | Entire spectrum holding can be used even only with legacy UEs for some scenarios depending on whether a single SSB can be used to configure legacy channels at both edges of the spectrum holding. Otherwise, entire spectrum can be used only by new UEs, all legacy UEs have to use the same regular channel BW part of the spectrum holding. | Entire spectrum holding can be used even only with legacy UEs | Entire licensed spectrum block, but with wider guard bands than in the other methods, can be used even only with legacy UEs |
| Network capacity | Entire spectrum can be used by multiplexing different UEs(even legacy UEs) | Entire spectrum can only be used for new UEs, | Entire spectrum can be used by multiplexing different UEs in the frequency domain | Entire spectrum can be used by any UE in DL direction. UL is smaller channel BW. |
| Legacy UE support | Legacy UEs supported, can use one of the aggregated CCs | Legacy UEs can use part of the spectrum that contains the SSB | Legacy UEs supported | Legacy UEs supported |
| RAN1/2/4 Specification impact | RAN1/2 – new UE capabilities needed, RAN4 – new band combinations, changes to channel spacing definition, Overlapping CA reqs applicability, new demod requirements for UEs | RAN2 – impact on new capability RAN4 –core requirements equivalent to new channel BW for BS, new demod requirements for UEs | RAN4 – BS requirements for new channel BW | New asymmetric bandwidth combinations are needed. Handled on a band-by-band bases and is done as the existing process. |

<<END OF CHANGES>>

# 4	References

RP-202103, "New SID: Study on Efficient utilization of licensed 	spectrum that is not aligned with existing NR channel bandwidths", T-Mobile USA, Ericsson