3GPP TSG-RAN WG4 Meeting # 103-e		R4-2208155

# Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.8.3

Source: 	CATT

Title: 	Discussion on RRM test cases for FR2 HST

Document for:	Discussion

Introduction

In RAN4#102-e meeting, the core requirements of Rel-17 NR FR2 HST are completed. This contribution provides discussion on the RRM test case list.

Discussion

According to the work plan for performance part, the test cases should be designed and specified in performance part.

The enhanced RRM requirements for NR FR2 HST are summarized as below:

- Idle mode: cell reselection for intra-frequency cells

- Connected mode: RRC re-establishment

- Tq is changed from 2.5 *64*Tc to 4.5 *64*Tc

- One shot large UL timing adjustment

- RLM/BFD requirements for reduced N for set1 and set2

- MAC-CE based TCI state switch delay

- Intra-frequency cell identification when without MG

- L1-RSRP measurement

- L1-SINR scheduling availability

In general, RAN4 should design test cases to verify new defined requirements.

Proposal 1: the test case list is proposed as following table:

| Core requirements | Test cases |
| --- | --- |
| Idle mode: cell reselection for intra-frequency cells | Define test case in A. 7.1.1.X for cell reselection for HST FR2 to verify cell reselection delay. DRX= 0.32 |
| Connected mode: RRC re-establishment | Add one test case in A.7.3.2.X to verify re-establishment delay |
| Tq is changed from 2.5 *64*Tc to 4.5 *64*Tc | In current test cases, the Tp and Tq adjustment rate are not tested for exact Tq. There is no need to add new test case to verify Tq enhancement in HST FR2. |
| One shot large UL timing adjustment | The capability of one shot large UL timing adjustment is still FFS. Prefer to add one test case. |
| RLM/BFD requirements for reduced N for set1 and set2 | For RLM, add one test to verify Out-of-sync Test for set1 and DRX = 40ms in A.7.5.1.X For BFD, add one test to verify BFD and link Recovery Test based on SSB for set2 and DRX = 40ms in A.7.5.5.X |
| MAC-CE based TCI state switch delay | Add one test in A.7.5.8.X |
| Intra-frequency cell identification when without MG | Add SA event triggered reporting test case in A.7.6.1.2.X to verify cell search requirements for [set1] and [set2]. DRX = 40ms |
| L1-RSRP measurement | Add one test case in A.7.6.3.2.X for SSB based with DRX = 40ms |
| L1-SINR scheduling availability | Do not add test case |

Conclusion

In this contribution, we provide our consideration of test case list for FR2 HST. It is proposed to define the test cases according to the following list:

Proposal 1: the test case list is proposed as following table:

| Core requirements | Test cases |
| --- | --- |
| Idle mode: cell reselection for intra-frequency cells | Define test case in A. 7.1.1.X for cell reselection for HST FR2 to verify cell reselection delay. DRX= 0.32 |
| Connected mode: RRC re-establishment | Add one test case in A.7.3.2.X to verify re-establishment delay |
| Tq is changed from 2.5 *64*Tc to 4.5 *64*Tc | In current test cases, the Tp and Tq adjustment rate are not tested for exact Tq. There is no need to add new test case to verify Tq enhancement in HST FR2. |
| One shot large UL timing adjustment | The capability of one shot large UL timing adjustment is still FFS. Prefer to add one test case. |
| RLM/BFD requirements for reduced N for set1 and set2 | For RLM, add one test to verify Out-of-sync Test for set1 and DRX = 40ms in A.7.5.1.X For BFD, add one test to verify BFD and link Recovery Test based on SSB for set2 and DRX = 40ms in A.7.5.5.X |
| MAC-CE based TCI state switch delay | Add one test in A.7.5.8.X |
| Intra-frequency cell identification when without MG | Add SA event triggered reporting test case in A.7.6.1.2.X to verify cell search requirements for [set1] and [set2]. DRX = 40ms |
| L1-RSRP measurement | Add one test case in A.7.6.3.2.X for SSB based with DRX = 40ms |
| L1-SINR scheduling availability | Do not add test case |

Reference