3GPP TSG-RAN WG4 Meeting #103-e                                                                                      R4-2208227

Electronic Meeting, May 09 – May 20, 2022

Source: 	CATT

Title: 	Discussion on the BS TX RF requirements for 52.6-71GHz

Agenda item:	9.15.4.1

Document for:	Approval

- Introduction

In RAN4#102-e, a WF [1] on BS RF TX requirements for 52.6 – 71 GHz was approved. The FFT sizes and EVM window length need to be further discussed

In this contribution, we provide our views on the remaining issues in the WF.

- Discussion

FFT sizes

The selection of FFT sizes needs to meet the following two conditions:

- The sampling frequency () needs to be larger than or equal to the channel bandwidth (). So we have.

- The FFT sizes () needs to be the nearest power of two of.

Based on above two conditions, the FFT sizes can be calculated using the following equation 1.

(Equation 1)

Where,

nextpow2(N) returns the first P such that 2.^P >= abs(N).

The FFT sizes for FR2-2 are shown in the following table 2-1 using equation 1.

Table 2-1: FFT sizes for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | FFT size | FFT size | FFT size | FFT size | FFT size |
| 120 | 1024 | 4096 | N/A | N/A | N/A |
| 480 | N/A | 1024 | 2048 | 4096 | N/A |
| 960 | N/A | 512 | 1024 | 2048 | 4096 |

Proposal 1: To adopt the FFT sizes for FR2-2 in the following table 2-1.

Table 2-1: FFT sizes for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | FFT size | FFT size | FFT size | FFT size | FFT size |
| 120 | 1024 | 4096 | N/A | N/A | N/A |
| 480 | N/A | 1024 | 2048 | 4096 | N/A |
| 960 | N/A | 512 | 1024 | 2048 | 4096 |

EVM window length

As per the WF [1], the remaining issues concerning EVM window length are shown as below:

2.2 EVM Agreements: - Add new tables with information only agreed values and remaining FFS 480 kHz and 960 kHz as described in Table 2.4-3 and Table 2.4-4. - Put 50% EVM window length within [] until conformance test feasibility issues are resolved. - Note: The value on EVM window length can be further discussed and updated if needed. - FFT sizes to be also double checked by companies.

Using above proposed FFT sizes for FR2-2, according to Table 2.4-3 and Table 2.4-4 in moderator summary 2207445.zip[2], the EVM window length for FR2-2 can be calculated as the following Table 2-2 and Table 2-3.

Table 2-2: EVM window length for normal CP, FR2-2, 480 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (%) |
| --- | --- | --- | --- | --- |
| 400 | 1024 | 72 | 36 | 50 |
| 800 | 2048 | 144 | 72 | 50 |
| 1600 | 4096 | 288 | 144 | 50 |

Table 2-3: EVM window length for normal CP, FR2-2, 960 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (%) |
| --- | --- | --- | --- | --- |
| 400 | 512 | 36 | 18 | 50 |
| 800 | 1024 | 72 | 36 | 50 |
| 1600 | 2048 | 144 | 72 | 50 |
| 2000 | 4096 | 288 | 144 | 50 |

Proposal 2: To adopt EVM window length for FR2-2 in the following Table 2-2 and Table 2-3.

Table 2-2: EVM window length for normal CP, FR2-2, 480 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (%) |
| --- | --- | --- | --- | --- |
| 400 | 1024 | 72 | 36 | 50 |
| 800 | 2048 | 144 | 72 | 50 |
| 1600 | 4096 | 288 | 144 | 50 |

Table 2-3: EVM window length for normal CP, FR2-2, 960 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (%) |
| --- | --- | --- | --- | --- |
| 400 | 512 | 36 | 18 | 50 |
| 800 | 1024 | 72 | 36 | 50 |
| 1600 | 2048 | 144 | 72 | 50 |
| 2000 | 4096 | 288 | 144 | 50 |

- Conclusion

This contribution provides analysis on BS TX RF requirements for 52.6-71GHz. The following proposals and observations are concluded as follows:

Proposal 1: To adopt the FFT sizes for FR2-2 in the following table 2-1.

Table 2-1: FFT sizes for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | FFT size | FFT size | FFT size | FFT size | FFT size |
| 120 | 1024 | 4096 | N/A | N/A | N/A |
| 480 | N/A | 1024 | 2048 | 4096 | N/A |
| 960 | N/A | 512 | 1024 | 2048 | 4096 |

Proposal 2: To adopt EVM window length for FR2-2 in the following Table 2-2 and Table 2-3.

Table 2-2: EVM window length for normal CP, FR2-2, 480 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (%) |
| --- | --- | --- | --- | --- |
| 400 | 1024 | 72 | 36 | 50 |
| 800 | 2048 | 144 | 72 | 50 |
| 1600 | 4096 | 288 | 144 | 50 |

Table 2-3: EVM window length for normal CP, FR2-2, 960 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (%) |
| --- | --- | --- | --- | --- |
| 400 | 512 | 36 | 18 | 50 |
| 800 | 1024 | 72 | 36 | 50 |
| 1600 | 2048 | 144 | 72 | 50 |
| 2000 | 4096 | 288 | 144 | 50 |

- References

- R4-2207216，WF on >52.6 GHz BS RF Tx Requirements，Nokia, Nokia Shanghai Bell，RAN4#102-e

- R4-2207445, Email discussion summary for [102-e][312] NR_exto71GHz_BSRF, Moderator (Nokia), RAN4#102-e