3GPP TSG-RAN WG4 Meeting #103-e                                                                                      R4-2208230

Electronic Meeting, May 09 – May 20, 2022

Source: 	CATT

Title: 	Discussion on the BS receiver characteristics conformance testing for 52.6-71GHz

Agenda item:	9.15.5.3

Document for:	Approval

- Introduction

In RAN4#102-e, a WF [1] on >52.6 GHz BS RF conformance aspects was approved. There are some the remaining issues in the WF need to be further discussed

In this contribution, we provide our views on the remaining issues in the WF.

- Discussion

Frequency range for RX spurious emission and out-of-band blocking

As per the WF [1], the remaining issues concerning frequency range for TX spurious emission are shown as below:

Issue 3-2-3 Frequency range considerations Agreement: Study and define practical upper point for frequency range for both spurious emissions and out-of-band blocking. WF: Companies are encouraged to bring study result and proposal in next meeting.

For FR2-1, for receiver spurious emission and out-of-band blocking, 60GHz is defined as max limit frequency of frequency range (Fstep,6    min(2nd harmonic of the upper frequency edge of the DL operating band in GHz; 60 GHz) ).  60GHz is slightly larger than 59.5GHz maximum Fstep,6 of all FR2-1 band.

For FR2-2, Step frequencies for FR2-2 are shown in the following table. The Fstep,6 of n263 is 127GHz, according to FR2-1, we can choose a frequency slightly larger than Fstep,6 (127GHz) as max limit frequency point, e.g., 130GHz.

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n263 | 18 | 43 | 53.5 | 74.5 | 85 | 127 |

Proposal 1: Choose a frequency slightly larger than Fstep,6 (127GHz) as max limit frequency point, e.g., 130GHz.

FR2 Interferer signal step size for blocking

From Table 7.5.2.4.2-1 of TS 38.141-2, the measurement step size for FR2 interferer signal step size for OTA in-band blocking and OTA out-of-band blocking varied with minimum supported BS channel bandwidth.

Table 7.5.2.4.2-1: FR2 Interferer signal step size Minimum supported BS channel bandwidth (MHz); Measurement step size (MHz). 50; 15. 100; 30. 200; 60. 400; 60.

The minimum supported BS channel bandwidth for 52.6-71GHz will be larger than 400MHz, such as 800MHz, 1600Hz, and 2000MHz.To reduce testing time, 120 MHz can be considered as measurement step size for interferer signal step size for 800MHz and 1600MHz.

Proposal 2: 120 MHz can be considered as measurement step size for interferer signal step size for 800MHz, 1600MHz, and 2000MHz CBW for OTA in-band blocking and OTA out-of-band blocking.

- Conclusion

This contribution provides analysis on BS receiver conformance testing for 52.6-71GHz. The following proposals and observations are concluded as follows:

Proposal 1: Choose a frequency slightly larger than Fstep,6 (127GHz) as max limit frequency point, e.g., 130GHz.

Proposal 2: 120 MHz can be considered as measurement step size for interferer signal step size for 800MHz, 1600MHz, and 2000MHz CBW for OTA in-band blocking and OTA out-of-band blocking.

- References

- R4-2207218, WF on >52.6 GHz BS RF conformance aspects, Keysight Technologies, RAN4#102-e