3GPP TSG-RAN WG4 Meeting # 103-e 		                       R4-2208242

Electronic Meeting, May 9th – 20th, 2022

Title: 	Remaining issue for BS RF requirement in 6GHz band

Source: 	CATT

Agenda item:	8.1.4

Document for:	Discussion

- Introduction

In the last RAN4 meeting, WF on BS RF requirements was approved in [1]. The following remaining issues are to be solved in this meeting.

- ΔfOBUE

- ΔfOOB

- Out-of-band blocking

This paper will discuss these open issues and give our proposals.

- Discussion

2.1 ΔfOBUE/ΔfOOB

The following options were discussed for ΔfOBUE and ΔfOOB in the last RAN4 meeting. However no consensus was reached.

Option 1: Define ΔfOBUE = 100 MHz and ΔfOOB = 100 MHz for BS type 1-C, type 1-H and type 1-O. (Supported by Huawei, China Unicom, CATT, Spark NZ)

Option 2: Define ΔfOBUE = 100 MHz for BS type 1-H and type 1-O, and ΔfOBUE = 40 MHz and ΔfOOB = 60 MHz for BS type 1-C. (Supported by Huawei, China Unicom, ZTE, Spark NZ)

Option 3: Define ΔfOBUE = 40 MHz and ΔfOOB = 60 MHz at least for MR and LA BS type 1-C and type 1-H, and further consider it for WA BS type 1-C and 1-H. (Supported by Nokia)

Option 4: Postpone decision on ΔfOBUE and ΔfOOB values for all BS classes to next RAN4#103-e meeting. (Supported by Nokia, Ericsson)

We believe 100MHz is appropriate value for  ΔfOBUE/ΔfOOB in this band at least for BS type 1-H and BS type 1-O. Regarding whether to define different requirement for BS type 1-C (Option 2) or to define different requirement for MR/LA BS (Option 3), we prefer to keep the same value for all BS types and all BS classes.

Proposal 1: it is proposed to adopt Option 1, e.g.

- Define ΔfOBUE = 100 MHz and ΔfOOB = 100 MHz for BS type 1-C, type 1-H and type 1-O.

2.2 Out-of-band blocking

The following proposals were discussed with no consensus reached.

-	Proposal 1: The blocking level is set to a level of -35 dBm for the frequency range (FUL_low -500) to (FUL_low -ΔfOOB) and (FUL_high +ΔfOOB) to (FUL_high +500). (yes: Nokia, Huawei; no: ZTE; FFS: Ericsson)

-	Proposal 2: To add the new 6GHz licensed band into the operating band list in table 7.5.2-1a of TS 38.104 instead of adding a new NOTE (to include text in proposal 1) in table 7.5.2-1. (yes: Nokia, Huawei, Spark NZ; no: ZTE; FFS: Ericsson)

The proposal 1 comes from the requirement for band n46 and n96. While the current out-of-band blocking requirement for FR1 NR licensed band are -15dBm applicable from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band for BS supporting FDD.

We see no reason to relax the out-of-band blocking requirement for licensed operation in 6GHz to proposal 1. It is proposed to specify the same -15dBm/MHz out-of-band blocking requirement as other licensed band.

Proposal 2: It is proposed to specify the same -15dBm/MHz out-of-band blocking requirement as other licensed band.

- Conclusion

This paper discussed the remaining issue for BS operation in 6425-7125 MHz band. The following proposals are concluded,

Proposal 1: it is proposed to adopt Option 1, e.g.

- Define ΔfOBUE = 100 MHz and ΔfOOB = 100 MHz for BS type 1-C, type 1-H and type 1-O.

Proposal 2: It is proposed to specify the same -15dBm/MHz out-of-band blocking requirement as other licensed band.

- Reference

[1]	R4-2206374, WF on BS RF requirements Nokia