3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208318

# Electronic Meeting, May 9 – 20, 2022

Source:	LG Electronics

Title:	Discussion on test cases for SL enhancement demodulation requirements

Agenda item:	9.14.6

Document for:	Discussion

Introduction

In the last RAN4 meeting, general work plan and scope were discussed and the WF [1] was approved. In this document, based on WF, we provide our views and initial simulation results for opened test cases for sidelink performance requirements.

Discussion

Test case for 256QAM demodulation

Even though 256QAM demodulation is optional feature for sidelink communication, this feature is very useful for advanced V2X service, especially unicast scenarios where high SNR can be achieved. Meanwhile, in RF specification, the requirements related 256QAM were specified, e.g., maximum input level, MPR, and A-MPR. Therefore, the test case for 256QAM demodulation requirement should be introduced. To avoid increasing the number of test cases for single link demodulation, 256QAM demodulation test can replace the 64QAM demodulation test by defining applicability rule if a UE supports Rx 256QAM.

- Proposal 1: Introduce test case for 256QAM demodulation requirement

Due to the limited time for performance phase, simulation assumption for 64QAM single link demodulation can be reused for 256QAM demodulation performance as Table 1 and Table 2.

Table 1 simulation parameters for 256QAM demodulation

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| Bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Sidelink UE 1 | Sidelink transmissions |  | PSCCH + PSSCH |
|  | PSSCH DMRS pattern (Note 1) |  | {2,2} |
|  | Index of sub-channel allocation |  | [0] |
|  | Timing offset (Note 2) | s | CP/2-12*64*Tc |
|  | Frequency offset (Note 3) | Hz | +600 |
|  | Synchronization |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
| PSFCH resource period |  | Slot | 4 |
| MinTimeGapPSFCH |  | Slot | 3 |
| Propagation condition |  |  | TDLA30-180 |
| Note 1: {x, y}: x and y means the number of DMRS symbols for slot with PSFCH transmission and without PSFCH transmission, respectively. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS referring timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. |  |  |  |

Table 2 Reference measurement channel for 256QAM demodulation

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Allocated resource blocks |  | RB | 10 |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  |  | 9 |
| CP-OFDM symbols for slot without PSFCH |  |  | 12 |
| Modulation order |  |  | 256QAM |
| MCS index |  |  | 20 (table 2) |
| Number of MIMO layers |  |  | 1 |
| Number of DMRS REs |  |  | 12 |
| Number of REs for SCI format 1-A |  |  | 240 |
| 2nd stage SCI format 2-A configuraion | Payloads | Bits | 35 |
|  | α |  | 1 |
|  | βoffset |  | 5 |
| Overhaed for TBS determination |  |  | 0 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC), PSFCH symbols, and guard symbols. |  |  |  |

Based on the Table 1 and Table 2, Fig.1 shows initial simulation result for 256QAM demodulation. The SNR at 10% PSSCH BLER is 22.6 dB.

Figure 1 PSSCH demodulation performance for 256QAM

CSI reporting

The CSI reporting in NR sidelink is mandatory feature if a UE supports NR sidelink, and the UE reports CQI and RI based on aperiodic CSI-RS.

For RI, depending on the capability of the number of antenna port(s), RI reporting is applicable for the UE supporting rank 2. To verify the basic feature, CSI reporting requirements should be introduced. However, since there are limited time for performance works and RAN4 agreed not to introduce Rank 2 demodulation requirements, we propose one simple test case for CQI reporting under AWGN to verify CSI reporting in NR sidelink.

- Proposal 2: Introduce only one test case for CQI reporting performance under AWGN.

Table 3 shows the initial results for CQI reporting under AWGN based on simulation parameters in Table 4.

Table 3 Initial results for CQI reporting under AWGN

| SNR (dB) | (1)Percentage of reported CQI index in {median CQI -1, median CQI +1} (%) | (2)BLER with following median CQI - 1 | (2)BLER with following median CQI + 1 | Median CQI index |
| --- | --- | --- | --- | --- |
| 4 | 100 | 0 | 0.91 | 7 |
| 5 | 100 | 0 | 1 | 8 |
| 12 | 100 | 0 | 1 | 12 |
| 13 | 100 | 0 | 1 | 12 |

Table 4 simulation parameters for CQI reporting

| Parameter |  | Unit | Test 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |  |  |  |
| Subcarrier spacing |  | kHz | 30 |  |  |  |
| Resource pool configuration |  |  |  |  |  |  |
| Sidelink UE1 | sidelink transmission |  | PSCCH+PSSCH |  |  |  |
|  | PSSCH DMRS pattern |  | {2,2} |  |  |  |
|  | Index of sub-channel allocation |  | [0,1] (20PRB) |  |  |  |
|  | synchronization |  | GNSS |  |  |  |
|  | timing/frequency offset |  | 0 |  |  |  |
| PSFCH resource period |  | slot | 4 |  |  |  |
| MinTimeGapPSFCH |  | slot | 3 |  |  |  |
| SNR |  | dB | 4 | 5 | 12 | 13 |
| Propagation channel |  |  | AWGN |  |  |  |
| Antenna configuration |  |  | 1×2 with static channel specified in Annex B.1 (TS38.101-4) |  |  |  |
| sl-CSI-Acquistion |  |  | enable |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |  |  |
|  | Number of CSI-RS ports (X) |  | 1 |  |  |  |
|  | Density (ρ) |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 2, (4) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |  |  |
|  | aperiodicTriggeringOffset | slot | 5 |  |  |  |
| ReportConfigType |  |  | Aperiodic |  |  |  |
| CQI-table |  |  | Table 1 |  |  |  |
| Latency Bound CSI Report |  | slot | 4 |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  |
| Subchannel size |  | RB | 10 |  |  |  |
| Maximum number of HARQ transmission |  |  | 1 |  |  |  |
| CQI delay |  | ms | 5 |  |  |  |

Conclusion

In this contribution, we provide our views on test cases for NR sidelink demodulation and CSI reporting and initial simulation results, and we propose

- Proposal 1: Introduce test case for 256QAM demodulation requirement.

- Proposal 2: Introduce only one test case for CQI reporting performance under AWGN.

Reference

[1] R4-2207224, “WF on SL enhancement demodulation,” LG Electronics.