3GPP TSG-RAN WG4 Meeting #103-e	R4-2208327

Electronic Meeting, 9th – 20th May 2022

Agenda Item:	9.15.10.2.2

Source:	Ericsson

Title:	Simulation results for PDCCH and PBCH demodulation in FR2-2

Document for:	Information

# 1	Introduction

In this paper, we present the initial results on the PDSCH performance in the frequency range 52.6 GHz to 71 GHz. Following the WF [1], we carried out our simulations using the agreed channel models and delay spread during RAN4 meeting #102-e, given by

The channel models have been decided to be TDLA and TDLD, where both using the following delay spread RMS: - SCS = 120 KHz, 10 ns. - SCS = 480 KHz, 10 ns / 5 ns. - SCS = 960 KHz, 10 ns / 5 ns.

However, several needed related parameters have not been agreed yet, such as

Issue 1-3-3: Max Doppler frequency Consider 3 km/h UE speed (200 Hz). FFS on higher UE speed. - Option 1: 10 km/h (650 Hz) - Option 2: 30 km/h (2000 Hz) Issue 3-1-1: SCS for DL requirements definition Consider the following SCS for DL requirements definition: - 120, 480 kHz - FFS 960 kHz Issue 3-1-2: CBW for DL requirements definition 120 kHz: - 100, 400 MHz 480 MHz: - 400 MHz - FFS on 1600 MHz 960 kHz: - 400 MHz - FFS on 2000 MHz Issue 3-4-1: Performance requirements for multi-slot PDCCH monitoring Option 1: Define performance requirements for multi-slot PDCCH monitoring for 480 and 960 kHz SCS Option 2: Do not define performance requirements for multi-slot PDCCH monitoring for 480 and 960 kHz SCS Issue 3-4-2: PDCCH simulation assumptions Use following assumptions for PDCCH performance test. - Antenna configuration: 1T2R and 1T4R Aggregation level: 2 and 4 for 1T2R; 8 and 16 for 2T2R PBCH simulation assumptions (if introduced pending on outcome of issues Issue 1-2-2) SSB Index: - Option 1: Only with not known SSB index - Option 2: Both known and not known SSB index - Option 3: Only with known SSB index - Option 4: Not known SSB index for 120 kHz and known SSB index for 480/960 kHz.

# 2	PDCCH Simulation Results

## 2.1	Simulation setup

As per WF [1], and due to the time limitation, we carried out simulations using a delay spread of 10 ns for both agreed channel models TDLA and TDLD, while considering low and moderate velocity.

We carried out our simulation using the following table where it is worth noting that a TDD pattern should be defined/agreed for the coming test assessments.

Test Parameters (reference from TS 38.101-4 Table Table 7.3.2.2-1)

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | TBD |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1-1 6 for test 1-2 | 2 |
| Interleaver size |  | 3 for test 1-1 2 for test 1-2 | 3 |
| Shift index |  | 0 |  |

Furthermore, we kept the same number of payload bits (without CRC) as 56 bits for both CORESET time duration (1 OFDM symbol and 2 OFDM symbol durations), and we used the following general parameters:

- Numerologies:

- SCS = 120 KHz, CBW = 100 MHz (66 PRB) and 400 MHz (264 PRB).

- SCS = 480 KHz, CBW = 400 MHz (66 PRB) and 1600 MHz (264 PRB).

- Max Doppler frequency/DMRS

- UE speed at 3 Km/h (f_D = 200 Hz).

- UE speed at 10 Km/h (f_D = 650 Hz).

- Frequency Offset: 0 ppm.

## 2.2	Simulation results

In this section, we present the PDCCH performance in terms of the probability of miss-detection of the downlink scheduling grant (Pm-dsg) as function of SNR (dB). The PDCCH performance metric is defined as the needed SNR (dB) for a Pm-dsg (BLER) at 1%. In the sequel, we provide the obtained results summarized in the following tables. Moreover, and for the sake of the presentation quality, we moved the figures in the Appendix A.1.

SCS 120 KHz/CBW 100 MHz

| 2 | Interleaver size | REG bundle size | CORE SET RB | CORE SET duration | Aggrega-tion level | Antenna config | Propagation condition | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | 1x2 ULA Low | TDLA10-200 | 7.55 |
|  |  |  |  |  |  |  | TDLD10-200 | 4.10 |
|  |  |  |  |  |  |  | TDLA10-650 | 7.53 |
|  |  |  |  |  |  |  | TDLD10-650 | 4.09 |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | 1x2 ULA Low | TDLA10-200 | 3.2834 |
|  |  |  |  |  |  |  | TDLD10-200 | -0.47696 |
|  |  |  |  |  |  |  | TDLA10-650 | 3.3274 |
|  |  |  |  |  |  |  | TDLD10-650 | -0.49662 |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | 2x2 ULA Low | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | 1.4743 |
|  |  |  |  |  |  |  |  | 0.39724 |
|  |  |  |  |  |  |  |  | 1.4583 |
|  |  |  |  |  |  |  |  | 0.40487 |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | 2x2 ULA Low | TDLA10-200 TDLD10-200 TDLA10-650 TDLD10-650 | -0.52723 |
|  |  |  |  |  |  |  |  | -4.8284 |
|  |  |  |  |  |  |  |  | -0.62699 |
|  |  |  |  |  |  |  |  | -4.8846 |

Observation 1: For SCS 120 KHZ/CBW 100 MHz, we kept same test setting as in FR2-1.

SCS 120 KHz/CBW 400 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORE SET duration | Aggrega-tion level | Antenna config | Propagation condition | SNRBB (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 240 | 1 | 2 | 1x2 ULA Low | TDLA10-200 | 5.5047 |
|  |  |  |  |  |  |  | TDLD10-200 | 2.4794 |
|  |  |  |  |  |  |  | TDLA10-650 | 5.6704 |
|  |  |  |  |  |  |  | TDLD10-650 | 2.5587 |
| Test 1-2 | 2 | 6 | 240 | 1 | 4 | 1x2 ULA Low | TDLA10-200 | 1.5302 |
|  |  |  |  |  |  |  | TDLD10-200 | -1.8142 |
|  |  |  |  |  |  |  | TDLA10-650 | 1.5925 |
|  |  |  |  |  |  |  | TDLD10-650 | -1.731 |
| Test 1-3 | 3 | 2 | 240 | 1 | 8 | 2x2 ULA Low | TDLA10-200 | -0.73425 |
|  |  |  |  |  |  |  | TDLD10-200 | -2.8253 |
|  |  |  |  |  |  |  | TDLA10-650 | -0.68344 |
|  |  |  |  |  |  |  | TDLD10-650 | -2.7754 |
| Test 1-4 | 3 | 2 | 240 | 1 | 16 | 2x2 ULA Low | TDLA10-200 | -3.1529 |
|  |  |  |  |  |  |  | TDLD10-200 | -5.0606 |
|  |  |  |  |  |  |  | TDLA10-650 | -3.0767 |
|  |  |  |  |  |  |  | TDLD10-650 | -5.0406 |

| Test case | Interleaver size | REG bundle size | CORESET RB | CORE SET duration | Aggrega-tion level | Antenna config | Propagation condition | SNRBB (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 240 | 2 | 2 | 1x2 ULA Low | TDLA10-200 | 5.9579 |
|  |  |  |  |  |  |  | TDLD10-200 | 2.9305 |
|  |  |  |  |  |  |  | TDLA10-650 | 6.0846 |
|  |  |  |  |  |  |  | TDLD10-650 | 2.9564 |
| Test 1-2 | 2 | 6 | 240 | 2 | 4 | 1x2 ULA Low | TDLA10-200 | 1.1069 |
|  |  |  |  |  |  |  | TDLD10-200 | -2.2737 |
|  |  |  |  |  |  |  | TDLA10-650 | 1.34 |
|  |  |  |  |  |  |  | TDLD10-650 | -2.2035 |
| Test 1-3 | 3 | 2 | 240 | 2 | 8 | 2x2 ULA Low | TDLA10-200 | -0.050953 |
|  |  |  |  |  |  |  | TDLD10-200 | -2.4337 |
|  |  |  |  |  |  |  | TDLA10-650 | -0.045395 |
|  |  |  |  |  |  |  | TDLD10-650 | -2.3816 |
| Test 1-4 | 3 | 2 | 240 | 2 | 16 | 2x2 ULA Low | TDLA10-200 | -2.6833 |
|  |  |  |  |  |  |  | TDLD10-200 | -4.7019 |
|  |  |  |  |  |  |  | TDLA10-650 | -2.6499 |
|  |  |  |  |  |  |  | TDLD10-650 | -4.6363 |

Observation 2: For SCS 120 KHz with max CBW such as 400 MHz, we proposed PDCCH modulation performance where 240 PRB have been allocated to the CORESET and all the aggregation levels have been either limited to 1 OFDM symbol duration or extended to 2 OFDM symbol duration.

SCS 480 KHz/CBW 400 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORE SET duration | Aggrega-tion level | Antenna config | Propagation condition | SNRBB (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | 1x2 ULA Low | TDLA10-200 | 6.9331 |
|  |  |  |  |  |  |  | TDLD10-200 | 4.7086 |
|  |  |  |  |  |  |  | TDLA10-650 | 7.1172 |
|  |  |  |  |  |  |  | TDLD10-650 | 4.6782 |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | 1x2 ULA Low | TDLA10-200 | 1.6345 |
|  |  |  |  |  |  |  | TDLD10-200 | -0.50965 |
|  |  |  |  |  |  |  | TDLA10-650 | 1.9466 |
|  |  |  |  |  |  |  | TDLD10-650 | -0.59185 |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | 2x2 ULA Low | TDLA10-200 | 0.72284 |
|  |  |  |  |  |  |  | TDLD10-200 | 0.54033 |
|  |  |  |  |  |  |  | TDLA10-650 | 0.87117 |
|  |  |  |  |  |  |  | TDLD10-650 | 0.75742 |

| Test case | Interleaver size | REG bundle size | CORESET RB | CORE SET duration | Aggrega-tion level | Antenna config | Propagation condition | SNRBB (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 2 | 2 | 1x2 ULA Low | TDLA10-200 | 5.6885 |
|  |  |  |  |  |  |  | TDLD10-200 | 3.0671 |
|  |  |  |  |  |  |  | TDLA10-650 | 5.7156 |
|  |  |  |  |  |  |  | TDLD10-650 | 2.9406 |
| Test 1-2 | 2 | 6 | 60 | 2 | 4 | 1x2 ULA Low | TDLA10-200 | 0.40407 |
|  |  |  |  |  |  |  | TDLD10-200 | -2.3855 |
|  |  |  |  |  |  |  | TDLA10-650 | 0.4336 |
|  |  |  |  |  |  |  | TDLD10-650 | -2.3967 |
| Test 1-3 | 3 | 2 | 60 | 2 | 8 | 2x2 ULA Low | TDLA10-200 | -1.1746 |
|  |  |  |  |  |  |  | TDLD10-200 | -2.6161 |
|  |  |  |  |  |  |  | TDLA10-650 | -0.97186 |
|  |  |  |  |  |  |  | TDLD10-650 | -2.6106 |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | 2x2 ULA Low | TDLA10-200 | -3.2821 |
|  |  |  |  |  |  |  | TDLD10-200 | -4.954 |
|  |  |  |  |  |  |  | TDLA10-650 | -3.0075 |
|  |  |  |  |  |  |  | TDLD10-650 | -4.9059 |

Observation 3: For SCS 480 KHz with min CBW such as 400 MHz, we proposed PDCCH modulation performance where 60 PRB have been allocated to the CORESET and

- The aggregation level 8 has been allocated either 1 or 2 OFDM symbol durations.

- The aggregation level 16 has been allocated 2 OFDM symbol duration.

SCS 480 KHz/CBW 1600 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORE SET duration | Aggrega-tion level | Antenna config | Propagation condition | SNRBB (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 240 | 1 | 2 | 1x2 ULA Low | TDLA10-200 | 6.4268 |
|  |  |  |  |  |  |  | TDLD10-200 | 2.3395 |
|  |  |  |  |  |  |  | TDLA10-650 | 6.4994 |
|  |  |  |  |  |  |  | TDLD10-650 | 2.3257 |
| Test 1-2 | 2 | 6 | 240 | 1 | 4 | 1x2 ULA Low | TDLA10-200 | 1.2678 |
|  |  |  |  |  |  |  | TDLD10-200 | -2.1898 |
|  |  |  |  |  |  |  | TDLA10-650 | 1.312 |
|  |  |  |  |  |  |  | TDLD10-650 | -2.134 |
| Test 1-3 | 3 | 2 | 240 | 1 | 8 | 2x2 ULA Low | TDLA10-200 | -1.1793 |
|  |  |  |  |  |  |  | TDLD10-200 | -2.4263 |
|  |  |  |  |  |  |  | TDLA10-650 | -1.1617 |
|  |  |  |  |  |  |  | TDLD10-650 | -2.4645 |
| Test 1-4 | 3 | 2 | 240 | 1 | 16 | 2x2 ULA Low | TDLA10-200 | -4.1403 |
|  |  |  |  |  |  |  | TDLD10-200 | -5.0915 |
|  |  |  |  |  |  |  | TDLA10-650 | -3.9475 |
|  |  |  |  |  |  |  | TDLD10-650 | -5.1748 |

Observation 4: For SCS 480 KHz with max CBW such as 1600 MHz, we proposed PDCCH modulation performance where 240 PRB have been allocated to the CORESET and all the aggregation levels 8 and 16 have been limited to 1 OFDM symbol duration.

# 3	PBCH Simulation Results

## 3.1	Simulation setup

As per WF [1], and due to the time limitation, we carried out simulations using a delay spread of 10 ns for both agreed channel models TDLA and TDLD, while considering low and moderate velocity.

We carried out the simulations using the following table where it is worth noting that we kept same target coding rate for both numerologies, SCS 120 KHz and SCS 480 KHz.

Table A.3.4.2-1: PBCH Reference Channel

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channels |  | R.PBCH.5 | R.PBCH.6 |
| SS/PBCH block subcarrier spacing | kHz | 120 | 240 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 56/864 | 56/864 |
| Payload (without CRC and timing related PBCH payload bits) | bits | 24 | 24 |

## 3.2    Simulation results

In this section, we present the PBCH performance in terms of the probability of miss-detection of the PBCH (Pm-bch), given by

Pm-bch=1-AB

where A is the number of correctly decoded MIB PDUs and B is the number of transmitted MIB PDUs, as function of SNR (dB). The Pm-bch is derived with the assumption that UE combines the PBCH symbols of the same SS/PBCH block index within the MIB TTI (80ms). Therefore, we have 4 transmissions for each MIB as per the following parameters’ table.

Table (TS 38.101-4 7.4.2.2-1): Test parameters for PBCH

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| TDD UL-DL pattern |  | TBD |
| Note 1: as specified in clause 4.1 of TS 38.213 |  |  |

Observation 5: Pm-bch is derived in terms of BLER for every MIB (4 SSB Transmissions).

Tables 1 and 2 gather the obtained results.

| Test number | SSB SCS | Antenna configuration | Propagation condition | SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120 KHz | 1x2 Low | TDLA10-200 | -6.37 | unknown |
|  |  |  | TDLD10-200 | -10.00 |  |
|  |  |  | TDLA10-650 | -6.20 |  |
|  |  |  | TDLD10-650 | -10.17 |  |
| 1-2 |  |  | TDLA10-200 | -9.13 | known |
|  |  |  | TDLD10-200 | -10.68 |  |
|  |  |  | TDLA10-650 | -9.31 |  |
|  |  |  | TDLD10-650 | -10.86 |  |

| Test number | SSB SCS | Antenna configuration | Propagation condition | SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 480 KHz | 1x2 Low | TDLA10-200 | -6.37 | unknown |
|  |  |  | TDLD10-200 | -10.34 |  |
|  |  |  | TDLA10-650 | -6.55 |  |
|  |  |  | TDLD10-650 | -10.34 |  |
| 1-2 |  |  | TDLA10-200 | -8.96 | known |
|  |  |  | TDLD10-200 | -10.68 |  |
|  |  |  | TDLA10-650 | -8.96 |  |
|  |  |  | TDLD10-650 | -10.86 |  |

# 6	Summary

In this paper, we carried out simulations for PDCCH and PBCH in FR2-2. Here we propose some observations that need further discussions:

Observation 1: For SCS 120 KHZ/CBW 100 MHz, we kept same test setting as in FR2-1.

Observation 2: For SCS 120 KHz with max CBW such as 400 MHz, we proposed PDCCH modulation performance where 240 PRB have been allocated to the CORESET and all the aggregation levels have been either limited to 1 OFDM symbol duration or extended to 2 OFDM symbol duration.

Observation 3: For SCS 480 KHz with min CBW such as 400 MHz, we proposed PDCCH modulation performance where 60 PRB have been allocated to the CORESET and

- The aggregation level 8 has been allocated either 1 or 2 OFDM symbol durations.

- The aggregation level 16 has been allocated 2 OFDM symbol duration.

Observation 4: For SCS 480 KHz with max CBW such as 1600 MHz, we proposed PDCCH modulation performance where 240 PRB have been allocated to the CORESET and all the aggregation levels 8 and 16 have been limited to 1 OFDM symbol duration.

Observation 5: Pm-bch is derived in terms of BLER for every MIB (4 SSB Transmissions).

# References

- R4-2207223, “WF on demodulation performance requirements definition for 52.6 - 71 GHz”, Intel.

# Appendix	Simulation assumption

## A.1	PDCCH Performance Results

(a) AL2/1 OFDM Symbol                      (b) AL4/1 OFDM Symbol

(c) AL8/1 OFDM Symbol                      (d) AL16/2 OFDM Symbols

Fig 1 – PDCCH performance in terms of BLER as function of SNR using (SCS,CBW) = (120 KHz, 100 MHz) in {TDLA 10-200, TDLD 10-200, TDLA 10-650, TDLD 10-650} channels, where 60 RBs are allocated for the CORESET.

(a) AL2/1 OFDM Symbol                      (b) AL4/1 OFDM Symbol

(c) AL8/1 OFDM Symbol                      (d) AL16/1 OFDM Symbol

Fig 2 – PDCCH performance in terms of BLER as function of SNR using (SCS,CBW) = (120 KHz, 400 MHz) in {TDLA 10-200, TDLD 10-200, TDLA 10-650, TDLD 10-650} channels, where 240 RBs are allocated for the CORESET. The CORESET duration is 1 OFDM symbol for all aggregation levels.

(a) AL2/2 OFDM Symbols                      (b) AL4/2 OFDM Symbols

(d) AL16/2 OFDM Symbols

Fig 3 – PDCCH performance in terms of BLER as function of SNR using (SCS,CBW) = (120 KHz, 400 MHz) in {TDLA 10-200, TDLD 10-200, TDLA 10-650, TDLD 10-650} channels, where 240 RBs are allocated for the CORESET. The CORESET duration is 2 OFDM symbols for all aggregation levels.

(a) AL2/1 OFDM Symbol                        (b) AL4/1 OFDM Symbol

(c) AL8/1 OFDM Symbol

Fig 4 – PDCCH performance in terms of BLER as function of SNR using (SCS,CBW) = (480 KHz, 400 MHz) in {TDLA 10-200, TDLD 10-200, TDLA 10-650, TDLD 10-650} channels, where 60 RBs are allocated for the CORESET. The CORESET duration is 1 OFDM symbols for all aggregation levels.

(a) AL2/2 OFDM Symbols                        (b) AL4/2 OFDM Symbols

(c) AL8/2 OFDM Symbols                      (d) AL16/2 OFDM Symbols

Fig 5 – PDCCH performance in terms of BLER as function of SNR using (SCS,CBW) = (480 KHz, 400 MHz) in {TDLA 10-200, TDLD 10-200, TDLA 10-650, TDLD 10-650} channels, where 60 RBs are allocated for the CORESET. The CORESET duration is 2 OFDM symbols for all aggregation levels.

(a) AL2/1 OFDM Symbol                        (b) AL4/1 OFDM Symbol

(c) AL8/1 OFDM Symbol                      (d) AL16/1 OFDM Symbol

Fig 6 – PDCCH performance in terms of BLER as function of SNR using (SCS,CBW) = (480 KHz, 1600 MHz) in {TDLA 10-200, TDLD 10-200, TDLA 10-650, TDLD 10-650} channels, where 240 RBs are allocated for the CORESET. The CORESET duration is 1 OFDM symbols for all aggregation levels.

## A.2	PBCH Performance Results

(a) SS/PBCH Block index unknown                  (b) SS/PBCH Block index Known

Fig 7 – BLER (4 Transmissions) as function of SNR (dB) for PBCH using SCS 120 KHz in {TDLA 10-200, TDLD 10-200, TDLA 10-650, TDLD 10-650} channels.

(a) SS/PBCH Block index unknown                  (b) SS/PBCH Block index Known

Fig 8 – BLER (4 Transmissions) as function of SNR (dB) for PBCH using SCS 480 KHz in {TDLA 10-200, TDLD 10-200, TDLA 10-650, TDLD 10-650} channels.