3GPP TSG-RAN WG4 Meeting #103-e	R4- 2208328

Electronic Meeting, 9th – 20th May 2022

Agenda Item:	9.15.10.2.3

Source:	Ericsson

Title:	SDR requirements in 52.6 – 71 GHz band

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on the SDR requirements for FR2-2, where some related aspects have been gathered in the WF [1] and still need to be discussed. It is worth reminding that the purpose of the SDR test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by the UE capabilities.

Issue 3-1-1: SCS for DL requirements definition Consider the following SCS for DL requirements definition: - 120, 480 kHz - FFS 960 kHz Issue 3-1-2: CBW for DL requirements definition 120 kHz: - 100, 400 MHz 480 MHz: - 400 MHz - FFS on 1600 MHz 960 kHz: - 400 MHz - FFS on 2000 MHz Issue 3-6-1: SDR performance requirements (if introduced pending on outcome of issues Issue 1-2-2 If requirements will be introduced, study MCS configuration applicability for FR2-2 from FR2-1 and SNR values to achieve 85% throughput. Issue 3-3-7: MCS, modulation order for PDSCH requirements Define PDSCH requirements with MCS 4, MCS 13 and MCS [22] - Other MCS values are not precluded Issue 3-3-8: Rank Define PDSCH requirements with: - Rank 1 - FFS FD-OCC is not applied to all the antenna ports for DMRS - FFS Rank 2

# 2	SDR Requirements

The purpose of the SDR requirements is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. According to TS38.101-4, the existing MCS index table has already covered the configuration applicable for UEs such as rank 1/2 and QPSK/16QAM/64QAM. RAN4 only need to evaluate the applicability rule for FR2-2 where rank 1 is supported as shown in Table 3.6-1.

Table 3.6-1	MCS indexes for indicated UE capabilities applicable for FR2-2.

| Received antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 2Rx UE | 1 | 6 | 0.75 | 22 |
|  | 1 | 6 | 0.4 | 14 |
|  | 1 | 4 | 1 | 16 |
|  | 1 | 4 | 0.8 | 16 |
|  | 1 | 4 | 0.75 | 16 |
|  | 1 | 4 | 0.4 | 10 |
|  | 1 | 2 | 1 | 9 |
|  | 1 | 2 | 0.8 | 9 |
|  | 1 | 2 | 0.75 | 9 |
|  | 1 | 2 | 0.4 | 4 |

Proposal 1: Define SDR test for UE in FR2-2 considering 2Rx UE.

# 3	Specification structure

Table 1	Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.5 | Sustained downlink data rate provided by lower layers |  |
| 7.5.1 | FR2 single carrier requirements | Updates on FR2-2 |

# 4	Summary

In this paper we tried to contribute with the following proposal for further discussions:

Proposal 1: Define SDR test for UE in FR2-2 considering 2Rx UE.

# References

- R4-2207223, “WF on demodulation performance requirements definition for 52.6 - 71 GHz”, Intel.

# Appendix	Simulation assumption

## A.1	MCS Table from TS 38.214

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |