3GPP TSG-RAN WG4 Meeting # 103-e 		R4-2208387

Electronic Meeting, May 09 – May 20, 2022

Title:	[Draft] Reply on LS Reply LS on DC location for >2CC

Release:	Rel-17

Work Item:	NR_RF_FR2_req_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Hiromasa Umeda

hiromasa.umeda@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 has further discussed a possible largest offset range elaborated in an LS of R4-2119965 and CC groups elaborated in an LS of R4-2206602. RAN4 concludes the following:

For a possible largest offset range from a default UL DC location, the value can be up to +/- 24 GHz with SCS of 120 KHz. Though the value may bring to the amount of signalling overhead increase, it is RAN4’s understanding that network may not always need the DC locations from all the possible permutations consisting of two edge most frequencies out of any DL and UL frequency components. Hence, the overhead with the larger offset can be mitigated while a specific resolution and its introduction are up to RAN2. For a more concrete example, if an operator A has non-contiguous spectrum blocks and an operator B has contiguous spectrum blocks between those of the operator A, the operator A network may not need UL DC locations to fall into spectrum blocks of the operator A while the operator B network may not need UL DC locations to fall into spectrum blocks of the operator A, or both operators’ network may not need any UL DC locations to fall into outside their own spectrum blocks including guard band between sub-blocks.

For UL DC location reporting framework, although the LS of R4-2206602 stated that “The default DC location framework applies to each CC group”, if a CC group consists of one CC, it’s up to RAN2 whether to apply Rel-15 UL DC location reporting framework to a CC group with one CC and incorporate it into Rel-17 reporting framework since it’s mandatory for all the UE to implement Rel-15 reporting framework.

# 2	Actions

To RAN2

ACTION: RAN4 respectfully asks RAN2 to take the above information into consideration in their work.

# 3	Dates of next TSG RAN WG 4 meetings

RAN4#104	2022-08-22 2022-08-26 		Toulouse, FR