# 3GPP TSG-RAN WG4 Meeting # 103-e										R4-2208406

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | - | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.106: add co-existence requirements for input intermodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters-Core |  |  |  |  |  | Date: |  |  | 2022-4-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The co-existence input IMD requirement is missing in current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The general principle for input IMD is to utilize the same interference level of gNB blocking requirement. Therefore, the legacy interference level of out of band blocking requirement could be applicable for repeater input IMD. The main change is to add the co-existence input IMD requirement with the same interference level as gNB out of band blocking. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Co-existence input IMD requirement is missing and the spec is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.106…CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<First changed section>

### 6.7.3	Co-existence with other systems

#### 6.7.3.1	General

This input intermodulation existence requirement may be applied for the protection of NR repeater receivers when GSM, CDMA, UTRA, E-UTRA, NR BS or repeater operating in a different frequency band co-exist with a NR repeater.

The following requirement applies for interfering signals depending on the repeaters passband.

This requirement applies to the uplink and downlink of the repeater.

#### 6.7.3.2	Minimum requirement

For the parameters specified in table 6.7.3.2-1 and 6.7.3.2-2, the power in the passband shall not increase with more than 10 dB at the output of the repeater as measured with 1MHz measurement bandwidth, compared to the level obtained without interfering signals applied.

The core requirement is applicable for all frequency separation possibilities between the two interfering signals that cause the 3rd order intermodulation product to fall into the passband.

Table 6.7.3.2-1: input intermodulation requirement for NR repeater DL and UL when co-exist with BS/repeater in other non-overlapping frequency bands

| Frequency range of interfering signal | Interfering signal mean power (dBm) | Type of interfering signals | Measurement bandwidth |
| --- | --- | --- | --- |
| Frequency range of co-existence system operating band | -152 | 2 CW carriers | 1MHz |
| NOTE 1: For the operation band except for band n46, n96 and n102, all the interfering signals should be limited into the frequency ranges that are either X MHz higher than FFBWhigh or X MHz lower than FFBWlow, where X equals to 20MHz when passband is not larger than 200MHz, otherwise X equals to 60MHz NOTE 2: For operation band n46, n96 and n102, -15dBm interference signal level applies only when all the interfering signals are limited into the frequency ranges that are either 500 MHz higher than FFBWhigh or 500 MHz lower than FFBWlow. When interference signals are defined in following exception frequency ranges, -35dBm interference signal level applies. The exception frequency ranges include: (FFBWlow -500) to (FFBWlow -Y) , (FFBWhigh +Y) to (FFBWhigh +500), where Y equals to 60MHz for band n46, n96 and 70MHz for band n96. |  |  |  |

<End of changes>