3GPP TSG-RAN WG4 Meeting # 103-e 		R4-2208416

Electronic Meeting, May 09 – May 20, 2022

Agenda Item:	9.11.2.3.1

Source: 	CMCC

Title: 	Discussion on the CRS-IM for NR 30kHz SCS

Document for:	Discussion

- Introduction

In RAN4#102-e meeting, CRS-IM for NR 30kHz SCS feasibility and test setup related issues were discussed, a way forward was agreed in [1]. In this contribution, we further provide our simulation results, views about feasibility, UE capability and so on.

- Discussion

  - CRS-IM feasibility evaluation for 30kHz SCS scenario

Whether to define CRS-IM requirements for 30 kHz SCS scenario

- Only define CRS-IM requirements for scenario with 30 kHz SCS assuming [10%] interference loading, 4 CRS ports and 1+1 DMRS configuration under the condition with enough performance discrimination between CRS-IM on and CRS-IM off i.e. at least 1dB performance difference observed

- Channel BW: 20MHz

- FFS on special slot configuration

- Interested companies can bring results with Rel-15 rate matching (symbol level)

We did the simulation for baseline receiver (MMSE-IRC receiver) and CRS-IM receiver with LLR weighting.  The simulation results are in Table 1. The implementation of LLR weighting is attached in Appendix.

Table 1. Simulation results for 10% interference loading and 1+1 DMRS configuration

| Antenna configuration | SNR at 70% TP (dB) |  | Performance gain |
| --- | --- | --- | --- |
|  | Baseline Receiver（MMSE-IRC） | CRS-IM receiver with LLR weighting algorithm |  |
| 4T2R | 8.7 | 7.1 | 1.6 |
| 4T4R | 6.0 | 3.5 | 2.5 |

Based on the simulation results, the performance gain is larger than 1dB when using CRS-IM receiver with LLR weighting algorithm. Therefore, it is feasible and beneficial to define CRS-IM requirements for serving cell 30kHz SCS scenario.

Observation 1: When using CRS-IM receiver with LLR weighting algorithm, the performance gain is larger than 1dB under all simulation cases comparing with MMSE-IRC receiver.

Proposal 1: Define CRS-IM requirements for scenario with 30 kHz SCS assuming 10% interference loading, 4 CRS ports and 1+1 DMRS configuration.

  - CRS-IM capability signaling for 30kHz SCS scenario

Applicability of UE CRS-IM Capability

- Option 1: FFS until feasibility to support CRS-IM for 30kHz SCS is confirmed

- Option 2: Define separate UE capability for 30kHz SCS CRS-IM, FR1 only, no FDD/TDD difference

In last meeting, the UE capability for CRS-IM has been discussed. The features for CRS-IM in non-DSS and 30 kHz NR SCS scenario have not been approved due to uncertain performance gain. In this meeting, the performance gain can be observed based on our simulation results, we support to approve the features for CRS-IM in non-DSS and 30 kHz NR SCS scenario. The contents of two features proposed in last meeting can be reused [2].

Proposal 2: Approve the features of CRS-IM in non-DSS and 30 kHz NR SCS scenario.

Proposal 3: For the features of CRS-IM in non-DSS and 30 kHz NR SCS scenario, reuse the configuration proposed in last meeting.

| NR_perf_enh2_Demod | X-4 | CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth | Support of neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth |  | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_perf_enh2_Demod | X-5 | CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth | Support of neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth |  | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signaling |

- Conclusion

In this contribution, we provide our simulation results, and discuss our views about feasibility, UE capability. The proposals are:

Observation 1: When using CRS-IM receiver with LLR weighting algorithm, the performance gain is larger than 1dB under all simulation cases comparing with MMSE-IRC receiver.

Proposal 1: Define CRS-IM requirements for scenario with 30 kHz SCS assuming 10% interference loading, 4 CRS ports and 1+1 DMRS configuration.

Proposal 2: Approve the features of CRS-IM in non-DSS and 30 kHz NR SCS scenario.

Proposal 3: For the features of CRS-IM in non-DSS and 30 kHz NR SCS scenario, reuse the configuration proposed in last meeting.

| NR_perf_enh2_Demod | X-4 | CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth | Support of neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth |  | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_perf_enh2_Demod | X-5 | CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth | Support of neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth |  | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signaling |

- Reference

[1] R4-2207240, WF on CRS-IM in scenarios with overlapping spectrum for LTE and NR with 30kHz SCS, CMCC

[2] R4-2207239, WF on general part and 15 kHz NR SCS scenario for CRS-IM receiver, China Telecom.

[3] R4-2203029, WF on CRS-IM in scenarios with overlapping spectrum for LTE and NR with 30kHz SCS, CMCC

[4] R4-2120705, WF on CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR, China Telecom.

[5] RP-213656, Revised WID: Further enhancement on NR demodulation performance, China Telecom.

- Appendix

Implementation of LLR weighting:

- Assume the CRS port number is 4

- Do power estimation for each subcarrier with whole bandwidth granularity the and perform LLR weighting respectively.

- Calculate the actual SINR on these interfered REs

- Scaling the LLRs by post processing SINR value

Simulation assumptions:

Table 2-1. Simulation assumptions for serving cell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| TDD UL-DL pattern |  |  | 7DS2U with S=6D+4G+4U |
| Rank |  |  | 1 |
| MCS |  |  | 13 |
| MCS table |  |  | Table 1 |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | false |
| Propagation channel |  |  | TDLA 30-10 |
| Antenna configuration |  |  | ULA Low 4 x 2 and 4 x 4 |
| Receiver Type |  |  | MMSE-IRC |
| Overhead for TBS determination |  |  | 0 |
| PDCCH configuration | Symbols with PDCCH |  | Symbol# 0 and #1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Position of the first DM-RS for downlink |  | 2 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| Max HARQ re-transmission number |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot with PRB bundling granularity |
| LTE interference cell configuration |  |  | Specified in Table 2-2 |

Table 2-2. Simulation assumptions for interference cells

| Parameter |  | Value | Interference Cell #1 | Interference Cell #2 |
| --- | --- | --- | --- | --- |
| Transmission mode |  |  | TM4 | TM4 |
| Interference power level |  | dB | INR1=10.45dB | INR2=4.6dB |
| TDD UL-DL pattern (Note 1) |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| CRS pattern | LTE carrier centre subcarrier location |  | Same as NR serving carrier centre subcarrier location | Same as NR serving carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 20 | 20 |
|  | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| PDSCH loading level |  |  | 10% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 10% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Modulation order for interference PDSCH when exists |  |  | 16 QAM randomly modulated symbols | 16 QAM randomly modulated symbols |
| Time offset |  | us | 1.5 | -0.5 |
| Frequency offset |  | Hz | 300 | -100 |
| Transmission rank |  |  | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Network-based CRS interference mitigation (CRS muting) |  |  | Disabled | Disabled |
| MBSFN configuration |  |  | No MBSFN is configured | No MBSFN is configured |
| Note 1: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame. Note 2: The channel for interference cells and serving cell are independent. |  |  |  |  |

Simulation results:

| Figure 1. Simulation results for 4T2R | Figure 2. Simulation results for 4T4R |
| --- | --- |