3GPP TSG RAN WG4 Meeting #103-e	R4-2208441

Online,9th-20th May 2022

Agenda item:	8.15.2

Source:	Samsung, Spark

Title:	TP for TR 37.717-11-21: DC_3_n5-n40

Document for:	Approval

1	Introduction

This contribution is a text proposal for TR 37.717-11-21 to include DC_3_n5-n40 according to the request in [1].

2. Reference

[1]   R4-2204510_Revised WID on Rel-17 DC of LTE x band (x=1,2,3,4) DL_1UL and NR 2 band DL_1UL combination

3	Text Proposal

<Start of Text Proposal>

6.X	DC_3_n5-n40

6.X.1	Operating bands for DC

Table 6.X.1-1: EN-DC band combination (three bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_3_n5-n40 | 3 | CA_n5-n40 |

6.X.2	Inter-band DC Configurations

Table 6.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A_n5A-n40A | DC_3A_n5A DC_3A_n40A |

6.X.3	Co-existence studies

Based on the co-existense studies, No IMD of dual Tx falls into the third band Rx.

6.X.4	∆TIB and ∆RIB values

For DC_3_n5-n40, the TIB,c and RIB,c values are given in the tables below, reuse from CA_3-5-40..

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n5-n40 | 3 |  |
|  | n5 |  |
|  | n40 |  |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n5-n40 | 3 |  |
|  | n5 |  |
|  | n40 |  |

6.X.5	MSD

As analysed in 6.X.3, there is no need to specify additional MSD requirement.

<End of Text Proposal>