| 3GPP TR 37.717-21-11 V0.9.0 (2022-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Dual Connectivity of 2 bands LTE inter-band CA and 1 NR band (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

1	Scope	11

2	References	11

3	Definitions of terms, symbols and abbreviations	11

3.1	Terms	11

3.2	Symbols	11

3.3	Abbreviations	12

4	Background	12

4.1	TR Maintenance	12

5	DC of 2 bands LTE inter-band CA and 1 NR band within FR1: Specific Band Combination Part	12

5.x	DC_a-b_nc	12

5.x.1	Configurations for DC	12

5.x.2	Co-existence studies	12

5.x.3	∆TIB and ∆RIB values	12

5.x.4	Reference sensitivity exceptions	13

5.1	DC_3-19_n1	13

5.1.1	Configurations for DC	13

5.1.2	Co-existence studies	13

5.1.3	∆TIB and ∆RIB values	13

5.1.4	Reference sensitivity exceptions	14

5.2	DC_3-21_n1	14

5.2.1	Configurations for DC	14

5.2.2	Co-existence studies	14

5.2.3	∆TIB and ∆RIB values	14

5.2.4	Reference sensitivity exceptions	15

5.3	DC_3-42_n1	15

5.3.1	Configurations for DC	15

5.3.2	Co-existence studies	15

5.3.3	∆TIB and ∆RIB values	15

5.3.4	Reference sensitivity exceptions	16

5.4	DC_19-21_n1	16

5.4.1	Configurations for DC	16

5.4.2	Co-existence studies	16

5.4.3	∆TIB and ∆RIB values	16

5.4.4	Reference sensitivity exceptions	17

5.5	DC_19-42_n1	17

5.5.1	Configurations for DC	17

5.5.2	Co-existence studies	17

5.5.3	∆TIB and ∆RIB values	17

5.5.4	Reference sensitivity exceptions	18

5.6	DC_21-42_n1	18

5.6.1	Configurations for DC	18

5.6.2	Co-existence studies	18

5.6.3	∆TIB and ∆RIB values	18

5.6.4	Reference sensitivity exceptions	19

5.7	DC_1-32_n28	19

5.7.1	Configurations for DC	19

5.7.2	Co-existence studies	20

5.7.3	∆TIB and ∆RIB values	21

5.7.4	Reference sensitivity exceptions	21

5.7.5	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1	21

5.8	DC_7-32_n28	22

5.8.1	Configurations for DC	22

5.8.2	Co-existence studies	22

5.8.3	∆TIB and ∆RIB values	24

5.8.4	Reference sensitivity exceptions	24

5.8.5	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1	24

5.9	DC_7-32_n78	25

5.9.1	Configurations for DC	25

5.9.2	Co-existence studies	25

5.9.3	∆TIB and ∆RIB values	27

5.9.4	Reference sensitivity exceptions	27

5.10	DC_20-32_n28	28

5.10.1	Configurations for DC	28

5.10.2	Co-existence studies	28

5.10.3	∆TIB and ∆RIB values	29

5.10.4	Reference sensitivity exceptions	30

5.10.5	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1	30

5.11	DC_2-48_n5	30

5.11.1	Configurations for DC	30

5.11.2	Co-existence studies	30

5.11.3	∆TIB and ∆RIB values	30

5.11.4	Reference sensitivity exceptions	31

5.12	DC_5-48_n12	31

5.12.1	Configurations for DC	31

5.12.2	Co-existence studies	31

5.12.3	∆TIB and ∆RIB values	31

5.12.4	Reference sensitivity exceptions	32

5.13	DC_5-48_n71	32

5.13.1	Configurations for DC	32

5.13.2	Co-existence studies	32

5.13.3	∆TIB and ∆RIB values	32

5.13.4	Reference sensitivity exceptions	33

5.14	DC_12-48_n5	33

5.14.1	Configurations for DC	33

5.14.2	Co-existence studies	33

5.14.3	∆TIB and ∆RIB values	33

5.14.4	Reference sensitivity exceptions	34

5.15	DC_3_(n)41	34

5.15.1	Configuration for DC	34

5.15.3	∆TIB and ∆RIB values	34

5.15.4	REFSENS requirements	34

5.16	DC_2A-48A_n48A	35

5.16.2	Co-existence studies	35

5.17	DC_48-66A_n25A	35

5.17.2	Co-existence studies	36

5.18	DC_48A-66A_n48A	37

5.18.2	Co-existence studies	37

5.19	DC_3-8_n40	37

5.19.2	Co-existence studies	38

5.20	DC_3-28_n1	38

5.20.2	Co-existence studies	39

5.21	DC_7-8_n40	40

5.21.2	Co-existence studies	40

5.22	DC_7-28_n1	40

5.22.2	Co-existence studies	41

5.23	DC_7-66_n5	42

5.23.1	Configurations for DC	42

5.23.2	Co-existence studies	42

5.23.3	∆TIB and ∆RIB values	44

5.23.4	Reference sensitivity exceptions	44

5.24	DC_2-7_n5	45

5.24.1	Configurations for DC	45

5.24.2	Co-existence studies	45

5.24.3	∆TIB and ∆RIB values	47

5.24.4	Reference sensitivity exceptions	48

5.25	DC_1-40_n78	48

5.26	DC_3-40_n78	50

5.27	DC_7-40_n78	51

5.28	DC_8-40_n78	53

5.29	DC_2-4_n28	54

5.30	DC_2-7_n28	55

5.31	DC_2-66_n28	57

5.32	DC_4-7_n28	58

5.33	DC_5-7_n66	59

5.34	DC_7-66_n28	60

5.35	DC_1-11_n28	61

5.36	DC_3-11_n28	63

5.37	DC_8-11_n28	63

5.38	DC_3-11_n77	64

5.39.2	Configuration for DC	66

5.40.2	Configuration for DC	67

5.41.2	Configuration for DC	68

5.42.2	Configuration for DC	69

5.43.2	Configuration for DC	70

5.44	 DC_7-8_n28	70

5.45	 DC_20-28_n3	74

5.46	 DC_28-66_n66	78

5.47	 DC_7-28_n66	80

5.48	 DC_2-28_n66	84

5.49	 Void	88

5.50	 Void	88

5.51	 DC_8-40_n1	88

5.52	 DC_1-32_n3	92

5.53	 DC_3-32_n1	94

5.54	DC_7-32_n1	97

5.54.1	Configurations for DC	97

5.54.2	Co-existence studies	97

5.54.3	∆TIB and ∆RIB values	98

5.54.4	Reference sensitivity exceptions	99

5.55	DC_2-66_n7	99

5.56	DC_2-5_n7	100

5.57	DC_2-8_n2	101

5.58	DC_5-66_n7	102

5.59	DC_20-32_n1	103

5.60	DC_20-32_n3	104

5.61	DC_1-3_n3	104

5.61.1	Configuration for DC	104

5.61.3	∆TIB and ∆RIB values	105

5.61.4	REFSENS requirements	105

5.62	DC_1-41_n3	105

5.62.1	Configuration for DC	105

5.62.3	∆TIB and ∆RIB values	105

5.62.4	REFSENS requirements	106

5.63	DC_3-18_n3	106

5.63.1	Configuration for DC	106

5.63.3	∆TIB and ∆RIB values	106

5.63.4	REFSENS requirements	107

5.64	DC_3-41_n3	107

5.64.1	Configuration for DC	107

5.64.3	∆TIB and ∆RIB values	107

5.64.4	REFSENS requirements	108

5.65	Void	108

5.66	DC_7-66_n77 and DC_7-7-66_n77	108

5.66.1	Configuration for DC	108

5.66.3	∆TIB and ∆RIB values	109

5.66.4	REFSENS requirements	109

5.67	DC_2-5_n48	109

5.67.1	Configuration for DC	109

5.67.3	∆TIB and ∆RIB values	110

5.67.4	REFSENS requirements	110

5.68	DC_2-13_n48	110

5.68.1	Configuration for DC	110

5.68.3	∆TIB and ∆RIB values	111

5.68.4	REFSENS requirements	111

5.69	Void	111

5.70	DC_5-46_n66	111

5.70.1	Configuration for DC	111

5.70.3	∆TIB and ∆RIB values	112

5.70.4	REFSENS requirements	112

5.71	DC_5-66_n48 and DC_5-66-66_n48	112

5.71.1	Configuration for DC	112

5.71.3	∆TIB and ∆RIB values	113

5.71.4	REFSENS requirements	113

5.72	DC_5-66_n77	113

5.72.1	Configuration for DC	113

5.72.3	∆TIB and ∆RIB values	114

5.72.4	REFSENS requirements	114

5.73	DC_1-42_n3	114

5.74	DC_8-42_n3	115

5.75	DC_3-18_n28	117

5.75.1	Configurations for DC	117

5.75.2	Co-existence studies	117

5.75.3	∆TIB and ∆RIB values	117

5.75.4	Reference sensitivity exceptions	117

5.76	 DC_2-66_n77	118

5.76.4	 Reference sensitivity exceptions	120

5.77	 DC_2-48_n77	121

5.77.4	Reference sensitivity exceptions	123

5.78	 DC_2-13_n77	123

5.78.4	Reference sensitivity exceptions	125

5.79	 DC_2-5_n77	126

5.79.4	Reference sensitivity exceptions	128

5.80	 DC_5-13_n66	129

5.80.4	Reference sensitivity exceptions	131

5.81	 DC_13-66_n77	131

5.82	 DC_13-66_n5	134

5.82.4	Reference sensitivity exceptions	136

5.83	DC_1-18_n28	137

5.83.1	Configurations for DC	137

5.83.2	Co-existence studies	137

5.83.3	∆TIB and ∆RIB values	137

5.83.4	Reference sensitivity exceptions	137

5.84	DC_1-18_n41	138

5.84.1	Configurations for DC	138

5.84.2	Co-existence studies	138

5.84.3	∆TIB and ∆RIB values	138

5.84.4	Reference sensitivity exceptions	138

5.85	DC_5-7_n7	139

5.85.2	Co-existence studies	139

5.86	DC_2-28_n7	140

5.86.2	Co-existence studies	140

5.87	DC_28-66_n7	141

5.87.2	Co-existence studies	141

5.88	DC_7-28_n2	142

5.88.2	Co-existence studies	142

5.89	DC_2-7_n7	143

5.89.2	Co-existence studies	143

5.90	DC_2A-71A_n71A	144

5.90.2	Co-existence studies	144

5.91	DC_66A-71A_n71A	145

5.91.2	Co-existence studies	145

5.92	DC_7-66_n7/ DC_7-66-66_n7	145

5.93	DC_2-7_n77	148

5.93.1	Configurations for DC	148

5.93.2	Co-existence studies	148

5.93.3	∆TIB and ∆RIB values	150

5.93.4	Reference sensitivity exceptions	151

5.94	Void	151

5.95	DC_20-40_n78	151

5.96	DC_3-18_n41	153

5.96.1	Configurations for DC	153

5.96.2	Co-existence studies	153

5.96.3	∆TIB and ∆RIB values	153

5.96.4	Reference sensitivity exceptions	154

5.97	DC_7-25_n77	154

5.97.2	Co-existence studies	154

5.97.3	∆TIB and ∆RIB values	156

5.97.4	Reference sensitivity exceptions	157

5.98	DC_7-25_n78	157

5.98.2	Co-existence studies	157

5.98.3	∆TIB and ∆RIB values	159

5.98.4	Reference sensitivity exceptions	160

5.99	DC_25-66_n77	160

5.99.2	Co-existence studies	160

5.99.3	∆TIB and ∆RIB values	162

5.99.4	Reference sensitivity exceptions	163

5.100	DC_25-66_n78	163

5.100.2	Co-existence studies	163

5.100.3	∆TIB and ∆RIB values	165

5.100.4	Reference sensitivity exceptions	166

5.101	DC_2-29_n78	166

5.102	DC_29-66_n78	168

5.103	DC_1-21_n28	169

5.103.1	Configurations for DC	169

5.103.2	Co-existence studies	170

5.103.3	∆TIB and ∆RIB values	170

5.103.4	Reference sensitivity exceptions	170

5.104	DC_3-21_n28	171

5.104.1	Configurations for DC	171

5.104.2	Co-existence studies	171

5.104.3	∆TIB and ∆RIB values	171

5.104.4	Reference sensitivity exceptions	171

5.105	DC_8-20_n1	172

5.105.1	Configurations for DC	172

5.105.2	Co-existence studies	172

5.105.3	∆TIB and ∆RIB values	175

5.105.4	Reference sensitivity exceptions	175

5.106	DC_8-20_n3	176

5.106.1	Configurations for DC	176

5.106.2	Co-existence studies	176

5.106.3	∆TIB and ∆RIB values	179

5.106.4	Reference sensitivity exceptions	179

5.107	DC_8-32_n1	179

5.107.1	Configurations for DC	179

5.107.2	Co-existence studies	180

5.107.3	∆TIB and ∆RIB values	181

5.107.4	Reference sensitivity exceptions	181

5.108.2	Configuration for DC	182

5.109.2	Configuration for DC	183

5.110.2	Configuration for DC	184

5.111.2	Configuration for DC	185

5.112.2	Configuration for DC	186

5.113.2	Configuration for DC	187

5.114.2	Configuration for DC	188

5.115.2	Configuration for DC	189

5.116.2	Configuration for DC	190

5.117.2	Configuration for DC	191

Annex A (informative): Change history	193

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for Dual Connectivity of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address Dual connectivity (DC) band combinations of 3 different bands DL with 2 different bands UL (2 different LTE bands and 1 NR band) for the Rel-17 band combinations. The actual requirements are added to the corresponding technical specification.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to 21.905 be the first in the list.

3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

4	Background

The present document is a technical report for Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA and 1 NR band under Rel-17 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	DC of 2 bands LTE inter-band CA and 1 NR band within FR1: Specific Band Combination Part

5.x	DC_a-b_nc

5.x.1	Configurations for DC

<Editor’s note: it is required to use the same table format as in TS 38.101-3>

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |

5.x.2	Co-existence studies

<Text will be added.>

5.x.3	∆TIB and ∆RIB values

<Editor’s note: it is required to use the same table format as in TS 38.101-3>

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |

Table 5.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |

5.x.4	Reference sensitivity exceptions

< Editor’s note: text will be added only for reference sensitivity exceptions for intermodulation interference due to dual uplink operation for DC in NR FR1 involving three bands >

5.1	DC_3-19_n1

5.1.1	Configurations for DC

Table 5.1.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-19A_n1A | DC_3A_n1A DC_19A_n1A |

5.1.2	Co-existence studies

Based on co-existence studies of DC_3_n1 and DC_19_n1, own Rx impact of the 3rd band is the followings.

-	IMD generated by DC_3_n1 uplink doesn’t fall into own Rx of band 19.

-	IMD generated by DC_19_n1 uplink doesn’t fall into own Rx of band 3.

5.1.3	∆TIB and ∆RIB values

For DC_3-19_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-19, and are given in the tables below.

Table 5.1.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n1 | 3 | 0.3 |
|  | 19 | 0.3 |
|  | n1 | 0.3 |

Table 5.1.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19_n1 | 3 | 0 |
|  | 19 | 0 |
|  | n1 | 0 |

5.1.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

5.2	DC_3-21_n1

5.2.1	Configurations for DC

Table 5.2.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-21A_n1AX1,X2 | DC_3A_n1A DC_21A_n1A |
| NOTE X1: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE X2: The frequency range in band 3 is restricted for this band combination to 1765 - 1785 MHz for the UL and 1860-1880 MHz for the DL. |  |

5.2.2	Co-existence studies

Based on co-existence studies of DC_3_n1 and DC_21_n1, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_3_n1 uplink may fall into own Rx of band 21.

-	5th order IMD products generated by DC_3_n1 uplink may fall into own Rx of band 21.

-	IMD generated by DC_21_n1 uplink doesn’t fall into own Rx of band 3.

5.2.3	∆TIB and ∆RIB values

For DC_3-21_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-21, and are given in the tables below.

Table 5.2.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n1 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n1 | 0.3 |

Table 5.2.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n1 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n1 | 0 |

5.2.4	Reference sensitivity exceptions

When LTE CA 1-3-21 was introduced, IMD was calculated based on the frequency range that the operator actually owned, which resulted in that IMD3 and IMD5 doesn’t fall into own Rx of band 21. Therefore, there is no need to have MSD added. NOTE X1 and NOTE X2 in Table 5.2.1-1 indicate these frequency ranges.

5.3	DC_3-42_n1

5.3.1	Configurations for DC

Table 5.3.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-42A_n1A | DC_3A_n1A DC_42A_n1A |
| DC_3A-42C_n1A | DC_3A_n1A DC_42A_n1A |

5.3.2	Co-existence studies

Based on co-existence studies of DC_3_n1 and DC_42_n1, own Rx impact of the 3rd band is the followings.

-	4th order IMD products generated by DC_3_n1 uplink may fall into own Rx of band 42.

-	IMD generated by DC_42_n1 uplink doesn’t fall into own Rx of band 3.

5.3.3	∆TIB and ∆RIB values

For DC_3-42_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-42, and are given in the tables below.

Table 5.3.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n1 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |

Table 5.3.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n1 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |

5.3.4	Reference sensitivity exceptions

As mentioned above, IMD4 of B3 and n1 to Band 42 Rx need to be addressed for REFSENS relaxation. For DC_3-42_n1, REFSENS exceptions are reused from the LTE combination CA_1-3-42, and are given in the tables below.

Table 5.3.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-42A_n1A DC_3A-42C_n1A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | 42 | 3425 | 5 | 25 | 3425 | 13.0 | IMD4 |
|  | n1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |

5.4	DC_19-21_n1

5.4.1	Configurations for DC

Table 5.4.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_19A-21A_n1A | DC_19A_n1A DC_21A_n1A |

5.4.2	Co-existence studies

Based on co-existence studies of DC_19_n1 and DC_21_n1, own Rx impact of the 3rd band is the followings.

-	IMD generated by DC_19_n1 uplink doesn’t fall into own Rx of band 21.

-	IMD generated by DC_21_n1 uplink doesn’t fall into own Rx of band 19.

5.4.3	∆TIB and ∆RIB values

For DC_19-21_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-19-21, and are given in the tables below.

Table 5.4.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n1 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n1 | 0.3 |

Table 5.4.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21_n1 | 19 | 0 |
|  | 21 | 0 |
|  | n1 | 0 |

5.4.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

5.5	DC_19-42_n1

5.5.1	Configurations for DC

Table 5.5.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_19A-42A_n1AX1,X3 | DC_19A_n1A DC_42A_n1A |
| DC_19A-42C_n1 AX1,X3 | DC_19A_n1A DC_42A_n1A |
| NOTE X1: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE X3: The frequency range in band 42 is restricted for this band combination to 3440 - 3520 MHz. |  |

5.5.2	Co-existence studies

Based on co-existence studies of DC_19_n1 and DC_42_n1, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_19_n1 uplink may fall into own Rx of band 42.

-	5th order IMD products generated by DC_42_n1 uplink may fall into own Rx of band 19.

5.5.3	∆TIB and ∆RIB values

For DC_19-42_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-19-42, and are given in the tables below.

Table 5.5.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n1 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |

Table 5.5.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-42_n1 | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |

5.5.4	Reference sensitivity exceptions

When LTE CA 1-19-42 was introduced, IMD was calculated based on the frequency range that the operator actually owned, which resulted in that IMD3 doesn’t fall into own Rx of band 42 and IMD5 doesn’t fall into own Rx of band 19. Therefore, there is no need to have MSD added. NOTE X1 and NOTE X3 in Table 5.5.1-1 indicate these frequency ranges.

5.6	DC_21-42_n1

5.6.1	Configurations for DC

Table 5.6.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_21A-42A_n1AX1,X3 | DC_21A_n1A DC_42A_n1A |
| DC_21A-42C_n1 AX1,X3 | DC_21A_n1A DC_42A_n1A |
| NOTE X1: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE X3: The frequency range in band 42 is restricted for this band combination to 3440 - 3520 MHz. |  |

5.6.2	Co-existence studies

Based on co-existence studies of DC_21_n1 and DC_42_n1, own Rx impact of the 3rd band is the followings.

-	2nd order IMD products generated by DC_21_n1 uplink may fall into own Rx of band 42.

-	2nd order IMD products generated by DC_42_n1 uplink may fall into own Rx of band 21.

5.6.3	∆TIB and ∆RIB values

For DC_21-42_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-21-42, and are given in the tables below.

Table 5.6.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n1 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |

Table 5.6.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21-42_n1 | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |

5.6.4	Reference sensitivity exceptions

When LTE CA 1-21-42 was introduced, IMD was calculated based on the frequency range that the operator actually owned, which resulted in that IMD2 doesn’t fall into own Rx of band 42. Therefore, we didn’t specify MSD for band 42. NOTE X1 and NOTE X3 in Table 5.6.1-1 indicate these frequency ranges.

IMD2 of B42 and n1 to Band 21 Rx need to be addressed for REFSENS relaxation. For DC_21-42_n1, REFSENS exceptions are reused from DC_1-11_n78, already defined in TS 38.101-3, due to similarity in frequency range.

Table 5.6.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A-42A_n1A | 21 | 1452 | 5 | 25 | 1500 | 31.4 | IMD2 |
|  | 42 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |

5.7	DC_1-32_n28

5.7.1	Configurations for DC

Table 5.7.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-32A_n28A | DC_1A_n28A |

5.7.2	Co-existence studies

Table 5.7.2-1 lists the Band 1A + Band n28A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.7.2-1: Band 1 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 703 | 748 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 – 3960 |  |  | 1406 – 1496 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5760 – 5940 |  |  | 2109 – 2244 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1172 – 1277 |  |  | 2623 – 2728 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 3092 – 3257 |  |  | 424 – 574 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4543 – 4708 |  |  | 3326 – 3476 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1890 – 2010 |  |  | 683 – 768 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5012 – 5237 |  |  | 129 – 324 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2344 – 2554 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6463 – 6688 |  |  | 4029 – 4224 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 5246 – 5456 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 832 – 1072 |  |  | 6932 – 7217 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 – 1851 |  |  | 4264 – 4534 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 – 4972 |  |  | 8383 – 8668 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 – 6204 |  |  | 7166 – 7436 |  |  |

Based on Table 5.7.2-1,

-	2nd order harmonics may fall into the Rx of bands 11, 21, 32, 45, 46, 47, 75 and 76

-	3rd order harmonics may fall into the Rx frequencies of bands 1, 4, 10, 23, 65, 66 and 77

-	2nd order IMD may fall into Rx frequencies of bands 7 and 41

-	3rd order IMD may fall into Rx frequencies of bands 31, 42 and 77

-	4th order IMD may fall into Rx of bands 30, 40, 41, 46, 46 and 77

-	5th order IMD may fall into Rx frequencies of bands 3, 5, 6, 8, 9, 18, 19, 26, 27 and 35

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.7.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.7.2-2: 2UL Band 1 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD 5 |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic 3, IMD 4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD 4 |
|  | 5470 | - | 5725 | Yes |  | IMD 4 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic 3, IMD 4 |

The requirements for coexistence with protected bands (including band 32) exist for DC_1A_n28A in 38101-3.

5.7.3	∆TIB and ∆RIB values

Table 5.7.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-32A_n28 | 1 | 0.3 |
|  | n28 | 0.7 |

Table 5.7.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-32A_n28 | 1 | 0 |
|  | 32 | 0 |
|  | n28 | 0.2 |

5.7.4	Reference sensitivity exceptions

No additional exceptions for IMD are required.

5.7.5	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

The entries in tables 5.7.5-1 and 5.7.5-2 are to be added to TS38101-3 tables 7.3B.2.3.1-1 and 7.3B.2.3.1-2 respectively.

Table 5.7.5-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n28 | 32 | 28.1 | 25.3 | 24.0 | 22.8 |  |  |  |  |  |  |  |  |

Table 5.7.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n28 | 32 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |

5.8	DC_7-32_n28

5.8.1	Configurations for DC

Table 5.8.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_7A-32A_n28A | DC_7A_n28A |

5.8.2	Co-existence studies

Table 5.8.2-1 lists the Band 7A + Band n28A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.8.2-1: Band 7 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 703 | 748 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 1406 – 1496 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 5940 |  |  | 2109 – 2244 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1752 – 1867 |  |  | 3203 – 3318 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 4252 – 4437 |  |  | 1074 – 1164 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 5703 – 5888 |  |  | 3906 – 4066 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2470 – 2600 |  |  | 683 – 768 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6752 – 7007 |  |  | 256 – 461 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3504 – 3734 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8203 – 8458 |  |  | 4029 – 4224 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 6406 – 6636 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 242 – 492 |  |  | 6932 – 7217 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2756 – 3031 |  |  | 4264 – 4534 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5312 – 5562 |  |  | 8383 – 8668 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7109 – 7384 |  |  | 8906 – 9206 |  |  |

Based on Table 5.8.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 11, 21, 32, 45, 75 and 76

-	3rd order harmonics may fall into Rx frequencies of bands 1, 4, 10, 23, 65 and 66

-	2nd order IMD may fall into Rx frequencies of bands 3, 9, 35 and 77

-	3rd order IMD may fall into Rx frequencies of bands 46, 47 and 77

-	4th order IMD may fall into Rx frequencies of bands 22, 42, 43, 48 and 77

-	5th order IMD may fall into Rx frequencies of bands 31 and 46

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.8.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.8.2-2: 2UL Band 7 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD 5 |
|  | 5470 | - | 5725 | Yes |  | IMD3, IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, IMD5 |

The requirements for coexistence with protected bands, excluding band 32, exist for DC_7A_n28A in 38101-3.

5.8.3	∆TIB and ∆RIB values

Table 5.8.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-32A_n28 | 7 | 0.3 |
|  | n28 | 0.7 |

Table 5.8.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-32A_n28 | 7 | 0 |
|  | 32 | 0 |
|  | n28 | 0.2 |

5.8.4	Reference sensitivity exceptions

The entries in tables 5.8.4-1 and 5.8.4-2 are to be added to TS38101-3 tables 7.3B.2.3.1-1 and 7.3B.2.3.1-2 respectively.

Table 5.8.4-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n28 | 32 | 28.1 | 25.3 | 24.0 | 22.8 |  |  |  |  |  |  |  |  |

Table 5.8.4-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n28 | 32 | 12 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |

5.8.5	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

The entries in tables 5.8.5-1 and 5.8.5-2 are to be added to TS38101-3 tables 7.3B.2.3.1-1 and 7.3B.2.3.1-2 respectively.

Table 5.8.5-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n28 | 32 | 28.1 | 25.3 | 24.0 | 22.8 |  |  |  |  |  |  |  |  |

Table 5.8.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n28 | 32 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |

5.9	DC_7-32_n78

5.9.1	Configurations for DC

Table 5.9.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_7A-32A_n78A | DC_7A_n78A |

5.9.2	Co-existence studies

Table 5.9.2-1 list the Band 7A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.9.2-1: Band 7 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 730 – 1300 |  |  | 5800 – 6370 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1200 – 1840 |  |  | 4030 – 5030 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 8300 – 8940 |  |  | 9100 – 10170 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2400 – 2670 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 – 4410 |  |  | 7330 – 8900 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1460 – 2600 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 – 11510 |  |  | 4029 – 4224 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 11600 – 12740 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630 – 12700 |  |  | 6932 – 7217 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760 – 6400 |  |  | 4264 – 4534 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 – 17770 |  |  | 8383 – 8668 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 – 16540 |  |  | 14100 – 15310 |  |  |

Based on Table 5.9.2-1,

-	2nd order IMD may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 44, 46, 47, 67 and 68

-	3rd order IMD may fall into Rx frequencies of bands 3, 11, 21, 24, 32, 45, 75, 76 and 77

-	4th order IMD may fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45, 65, 66, 69, 70, 75 and 77

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 46, 47, 67 and 68

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.9.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.9.2-2: 2UL Band 7 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3, IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3, IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3, IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3, IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3, IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3, IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD2, IMD5 |

The requirements for coexistence with protected bands (including band 32) exist for DC_7A_n78A in 38101-3.

5.9.3	∆TIB and ∆RIB values

Table 5.9.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-32A_n78 | 7 | 0.5 |
|  | n78 | 0.8 |

Table 5.9.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-32A_n78 | 7 | 0 |
|  | 32 | 0 |
|  | n78 | 0.5 |

5.9.4	Reference sensitivity exceptions

Table 5.9.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-32A_n78A | n78 | 3560.5 | 10 | 50 | 3560.5 | N/A | N/A |
|  | 7 | 2517.5 | 5 | 25 | 2637.5 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1474.5 | 17.6 | IMD3 |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1492 | 4.9 | IMD4 |

5.10	DC_20-32_n28

5.10.1	Configurations for DC

Table 5.10.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_20A-32A_n28A | DC_20A_n28A |

5.10.2	Co-existence studies

Table 5.10.2-1 lists the Band 20A + Band n28A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.10.2-1: Band 20 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 703 | 748 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 1406 – 1496 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 2109 – 2244 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 84 – 159 |  |  | 1535 – 1610 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 916 – 1021 |  |  | 544 – 634 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2367 – 2472 |  |  | 2238 – 2358 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 802 – 892 |  |  | 703 – 748 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1748 – 1883 |  |  | 1247 – 1412 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 168 – 318 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3199 – 3334 |  |  | 4029 – 4224 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 3070 – 3220 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950 – 2160 |  |  | 6932 – 7217 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 385 – 580 |  |  | 4264 – 4534 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3644 – 3854 |  |  | 8383 – 8668 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3773 – 3968 |  |  | 3902 – 4082 |  |  |

Based on Table 5.10.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 11, 21, 32, 38, 41, 45, 69, 75 and 76

-	3rd order harmonics may fall into Rx frequencies of bands 1, 4, 10, 23, 65 and 66

-	2nd order IMD may fall into Rx frequencies of band 24

-	3rd order IMD may fall into Rx frequencies of bands 8, 30 and 40

-	4th order IMD may fall into Rx frequencies of bands 3, 9, 35, 39 and 77

-	5th order IMD may fall into Rx frequencies of bands 1, 2, 4, 7, 10, 25, 31, 34, 36, 38, 41, 43, 48, 65, 66, 69, 70 and 77

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.10.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.10.2-2: 2UL Band 20 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD 2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD 2 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD 2 |
| GPS | 1563 | - | 1587 | Yes |  | IMD 2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD 2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD 2 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

The requirements for coexistence with protected bands (including band 32) exist for DC_20A_n28A in 38101-3.

5.10.3	∆TIB and ∆RIB values

Table 5.10.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-32A_n28 | 20 | 0.5 |
|  | n28 | 0.7 |

Table 5.10.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20A-32A_n28 | 20 | 0 |
|  | 32 | 0 |
|  | n28 | 0.2 |

5.10.4	Reference sensitivity exceptions

The band 32 MSD due to the 2nd harmonic interferene from band n28 can refer to the definition in clause 5.8.4 of this TR.5.10.5	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

The entries in tables 5.10.5-1 and 5.10.5-2 are to be added to TS38101-3 tables 7.3B.2.3.1-1 and 7.3B.2.3.1-2 respectively.

Table 5.10.5-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n28 | 32 | 28.1 | 25.3 | 24.0 | 22.8 |  |  |  |  |  |  |  |  |

Table 5.10.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n28 | 32 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |

5.11	DC_2-48_n5

5.11.1	Configurations for DC

Table 5.11.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-48A_n5A | DC_2A_n5A DC_48A_n5A |

5.11.2	Co-existence studies

When uplink is DC_2A_n5A there is IMD3 interfering band 48 downlink.

When uplink is DC_48A_n5A there is IMD3 interfering band 2 downlink.

5.11.3	∆TIB and ∆RIB values

It is proposed to take the maximum relaxation values from DC_2-n5 and DC_48-n5.

Table 5.11.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n5 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n5 | 0.3 |

Table 5.11.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-48_n5 | 2 | 0.2 |
|  | 48 | 0.5 |
|  | n5 | 0.0 |

5.11.4	Reference sensitivity exceptions

The MSD levels are derived from the text proposal in R4-2010232 and R4-2014135.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-48A_n5A | 2 | 1870 | 5 | 25 | 1950 | 16.9 | IMD3 |
|  | 48 | 3610 | 10 | 50 | 3610 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 2 | 1890 | 5 | 25 | 1970 | N/A | N/A |
|  | 48 | 3570 | 5 | 25 | 3570 | 16.2 | IMD3 |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |

5.12	DC_5-48_n12

5.12.1	Configurations for DC

Table 5.12.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_5A-48A_n12A | DC_5A_n12A DC_48A_n12A |

5.12.2	Co-existence studies

When uplink is DC_5A_n12A there is IMD5 interfering band 48 downlink.

When uplink is DC_48A_n12A there is IMD5 interfering band 5 downlink.

5.12.3	∆TIB and ∆RIB values

It is proposed to re-use relaxation values from E-UTRA CA_5-12-48.

Table 5.12.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-48_n12 | 5 | 0.8 |
|  | 48 | 0.3 |
|  | n12 | 0.4 |

Table 5.12.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-48_n12 | 5 | 0.5 |
|  | 48 | 0 |
|  | n12 | 0.3 |

5.12.4	Reference sensitivity exceptions

It is prosed to re-use the IMD5 MSD values from already specified configuration DC_7A-28A_n5A which is similar to DC_5A-48A_n12A.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-48A_n12A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 48 | 3650 | 5 | 25 | 3650 | 4.4 | IMD5 |
|  | n12 | 705 | 5 | 25 | 735 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | 5.9 | IMD5 |
|  | 48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
|  | n12 | 705 | 5 | 25 | 735 | N/A | N/A |

5.13	DC_5-48_n71

5.13.1	Configurations for DC

Table 5.13.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_5A-48A_n71A | DC_5A_n71A DC_48A_n71A |

5.13.2	Co-existence studies

When uplink is DC_5A_n71A there is IMD5 interfering band 48 downlink.

When uplink is DC_48A_n71A there is IMD5 interfering band 5 downlink.

5.13.3	∆TIB and ∆RIB values

It is proposed to take the maximum relaxation values from DC_5_n71 and DC_48_n71.

Table 5.13.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-48_n71 | 5 | 0.5 |
|  | 48 | 0.3 |
|  | n71 | 0.5 |

Table 5.13.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-48_n71 | 5 | 0 |
|  | 48 | 0 |
|  | n71 | 0 |

5.13.4	Reference sensitivity exceptions

It is prosed to re-use the IMD5 MSD values from already specified configuration DC_7A-28A_n5A which is similar to DC_5A-48A_n71A.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-48A_n71A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 48 | 3590 | 5 | 25 | 3590 | 4.4 | IMD5 |
|  | n71 | 690 | 5 | 25 | 644 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | 5.9 | IMD5 |
|  | 48 | 3600 | 5 | 25 | 3600 | N/A | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | N/A |

5.14	DC_12-48_n5

5.14.1	Configurations for DC

Table 5.14.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_12A-48A_n5A | DC_12A_n5A DC_48A_n5A |

5.14.2	Co-existence studies

When uplink is DC_12A_n5A or DC_48A_n5A there is no IMD interfering 3rd band downlink.

5.14.3	∆TIB and ∆RIB values

It is proposed to re-use relaxation values from E-UTRA CA_5-12-48.

Table 5.14.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-48_n5 | 12 | 0.4 |
|  | 48 | 0.3 |
|  | n5 | 0.8 |

Table 5.14.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-48_n5 | 12 | 0.3 |
|  | 48 | 0 |
|  | n5 | 0.5 |

5.14.4	Reference sensitivity exceptions

There is no need for additional REFSENS requirement.

5.15	DC_3_(n)41

5.15.1	Configuration for DC

Table 5.15.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_3A_(n)41AA | DC_(n)41AA |

### 5.15.2	Co-existence study

Coexistence analysis has been captured into TR 37.716-21-11. There is no harmonic or IMD issues for the third band.

5.15.3	∆TIB and ∆RIB values

For DC_3_(n)41AA, the TIB,c and RIB,c values have been defined in TR 37.716-21-11

5.15.4	REFSENS requirements

There is no additional requirement for this band combination.

5.16	DC_2A-48A_n48A

### 5.16.1	Configurations for DC

Table 5.16.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_2A-48A_n48A | DC_2A_n48A |

Note that DC_48_n48 is not used as uplink configuration.

5.16.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. No further analysis is needed.

### 5.16.3	∆TIB and ∆RIB values

Table 5.16.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-48_n48 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n48 |  |

Table 5.16.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-48_n48 | 2 | 0.2 |
|  | 48 | 0.5 |
|  | n48 |  |

### 5.16.4	Reference sensitivity exceptions

REFSENS exceptions needed for DC_2A-48A_n48A due to band 48 uplink harmonic into band 2 is already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

5.17	DC_48-66A_n25A

### 5.17.1	Configurations for DC

Table 5.17.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | DC_48A_n25A DC_66A_n25A |

5.17.2	Co-existence studies

Co-existence studies have been performed for lower order combinations.

Co-existence analysis for DC_48_n25 UL shows that 4th IMD may fall into NR Band n48 DL. However, since 48 and n48 will be operated as synchronous in same UL/DL configuration there is no IMD problem. Further, a 4th IMD may fall in DL band 66.

Co-existence analysis for DC_66_n25 UL shows that 2nd HAM of band 66 may fall into the lowest 10MHz of NR Band n48 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further 2nd IMD might also fall in n48 DL

### 5.17.3	∆TIB and ∆RIB values

Table 5.17.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_48-66_n25 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n25 | 0.6 |

Table 5.17.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_48-66_n25 | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n25 | 0.2 |

### 5.17.4	Reference sensitivity exceptions

Based on the co-existence studies for DC_48_n25 and DC_66A_n25A it is found that MSD it needed.

Table 5.17.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | 8.3 | IMD4 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |

5.18	DC_48A-66A_n48A

### 5.18.1	Configurations for DC

Table 5.18.2-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_48A-66A_n48A | DC_66A_n48A |

Note that DC_48_n48 is not used as uplink configuration.

5.18.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. No further analysis is needed.

### 5.18.3	∆TIB and ∆RIB values

Table 5.18.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_48-66_n48 | 66 | 0.6 |
|  | 48 | 0.8 |
|  | n48 |  |

Table 5.18.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_48-66_n48 | 66 | 0.2 |
|  | 48 | 0.5 |
|  | n48 |  |

### 5.18.4	Reference sensitivity exceptions

No REFSENS exceptions needed for DC_48A-66A_n48A.

5.19	DC_3-8_n40

### 5.19.1	Configurations for DC

Table 5.19.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_3A-8A_n40A | DC_3A_n40A DC_8A_n40A |

5.19.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. Co-existence analysis for DC_3_n40 UL shows no impact to DL band 8,

Co-existence analysis for DC_8_n40 UL shows that 5th IMD may fall in DL band 3.

### 5.19.3	∆TIB and ∆RIB values

Table 5.19.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n40 | 3 | 0.5 |
|  | 8 | 0.3 |
|  | n40 | 0.5 |

Table 5.19.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n40 | 3 | 0 |
|  | 8 | 0 |
|  | n40 | 0 |

### 5.19.4	Reference sensitivity exceptions

Based on co-existence studies additional MSD is needed.

Table 6. x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-8A_n40A | 3 | 1779 | 5 | 25 | 1874 | 4 | IMD5 |
|  | 8 | 912 | 5 | 25 | 957 | N/A | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | N/A | N/A |

5.20	DC_3-28_n1

### 5.20.1	Configurations for DC

Table 5.20.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_3A-28A_n1A | DC_28A_n1A DC_3A_n1A |

5.20.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. Co-existence analysis for DC_28_n1 UL shows that 3rd HAM might fall into DL band n1 This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further, is shows that 5th IMD may fall in DL band 3.

Co-existence analysis for DC_3_n1 UL shows no impact to DL band 28.

5.20.3	∆TIB and ∆RIB values

Table 5.20.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-28_n1 | 3 | 0.3 |
|  | 28 | 0.6 |
|  | n1 | 0.3 |

Table 5.20.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-28_n1 | 3 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.20.4	Reference sensitivity exceptions

REFSENS exceptions needed due to band 28 uplink harmonic into band n1 is already specified for DC_28A_n1A.

The reference sensitivity exception (MSD) due to IMD5 for DC_3-28_n1 with UL DC_28_n1 is specified as below referring to the MSD for CA_1A-3A-28A from 36.101.

Table 5.20.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-28A_n1A | 3 | 1725 | 5 | 25 | 1820 | 4 | IMD5 |
|  | 28 | 710 | 5 | 25 | 765 | N/A | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |

5.21	DC_7-8_n40

### 5.21.1	Configurations for DC

Table 5.21.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_7A-8A_n40A | DC_7A_n40A DC_8A_n40A |

5.21.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. No further analysis is needed.

### 5.21.3	∆TIB and ∆RIB values

Table 5.21.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_7-8_n40 | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n40 | 0.6 |

Table 5.21.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-8_n40 | 7 | 0 |
|  | 8 | 0.2 |
|  | n40 | 0.5 |

### 5.21.4	Reference sensitivity exceptions

No further REFSENS exceptions needed.

5.22	DC_7-28_n1

### 5.22.1	Configurations for DC

Table 5.22.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_7A-28A_n1A | DC_28A_n1A DC_7A_n1A |

5.22.2	Co-existence studies

Co-existence studies have been performed for lower order combinations.

Co-existence analysis for DC_28_n1 UL shows that 3rd HAM might fall into DL band n1 This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further, is shows that 2th IMD may fall in DL band 7.

Co-existence analysis for DC_7_n1 UL shows that 5th IMD may fall in DL band 28.

### 5.22.3	∆TIB and ∆RIB values

Table 5.22.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_7-28_n1 | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.5 |

Table 5.22.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-28_n1 | 7 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.22.4	Reference sensitivity exceptions

REFSENS exceptions needed due to band 28 uplink harmonic into band n1 is already specified for DC_28A_n1A.

Based on co-existence studies additional MSD is needed. The MSD levels are derived from the text proposal in R4-2011610 and R4-2010889.

Table 5.22.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-28A_n1A | 7 | 2535 | 5 | 25 | 2655 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 4.3 | IMD5 |
|  | n1 | 1950 | 5 | 25 | 2165 | N/A | N/A |
|  | 7 | 2545 | 5 | 25 | 2665 | 29.0 | IMD2 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n1 | 1935 | 5 | 25 | 2125 | N/A | N/A |

5.23	DC_7-66_n5

5.23.1	Configurations for DC

Table 5.23.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | DC_7A_n5A DC_66A_n5A |

5.23.2	Co-existence studies

For UE coexistence study of Band 7 + Band n5, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.23.2-1.

Table 5.23.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 12500 | 12850 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1651 | 1746 | 3324 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 922 | 802 | 4151 | 4316 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4148 | 4268 | 5824 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 98 | 47 | 6651 | 6886 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8534 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3302 | 3492 | 6648 | 6838 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 726 | 896 | 9151 | 9456 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 5796 | 5966 | 10824 | 11129 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 2668 | 2453 | 5802 | 6062 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

For UE coexistence study of Band 66 + Band n5, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.23.2-2

Table 5.23.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 8550 | 8900 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 861 | 956 | 2534 | 2629 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 132 | 12 | 2571 | 2736 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3358 | 3478 | 4244 | 4409 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 692 | 837 | 4281 | 4516 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4182 | 4327 | 5954 | 6164 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1722 | 1912 | 5068 | 5258 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 1516 | 1686 | 5991 | 6296 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 5006 | 5176 | 7664 | 7969 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 1088 | 873 | 3432 | 3692 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 6778 | 7038 | 5892 | 6107 |

Based on co-existence study as presented in the table 5.1.28.4-1 and 5.1.28.4-2, own Rx impact of the 3rd band is shown as the followings.

- 2nd order IMD generated by dual uplink of Band 66 + Band n5 may also fall into own Rx of band 7

- 3rd order IMD generated by dual uplink of Band 66 + Band n5 may also fall into own Rx of band 7

5.23.3	∆TIB and ∆RIB values

Table 5.23.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n5 DC_7-66-66_n5 DC_7-7-66_n5 DC_7-7-66-66_n5 | 7 | 0.3 |
|  | 66 | 0.3 |
|  | n5 | 0.3 |

Table 5.23.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n5 DC_7-66-66_n5 DC_7-7-66_n5 DC_7-7-66-66_n5 | 7 | 0 |
|  | 66 | 0 |
|  | n5 | 0 |

5.23.4	Reference sensitivity exceptions

Band 66 has similar UL frequency ranges as band 3, the MSD values for DC_3-7_n5 can be reused for DC_7-66_n5. Table 5.23.4-1 lists the MSD required for DC_7-66_n5.

Table 5.23.4-1: MSD for the DC configuration due to IMD issue (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | 7 | 2505 | 10 | 50 | 2625 | 30.0 | IMD21 |
|  | 66 | 1775 | 10 | 50 | 2175 | N/A | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

5.24	DC_2-7_n5

5.24.1	Configurations for DC

Table 5.24.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-7A_n5A DC_2A-7C_n5A DC_2A-7A-7A_n5A | DC_2A_n5A DC_7A_n5A |

5.24.2	Co-existence studies

For UE coexistence study of Band 7 + Band n5, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.24.2-1.

Table 5.24.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 12500 | 12850 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1651 | 1746 | 3324 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 922 | 802 | 4151 | 4316 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4148 | 4268 | 5824 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 98 | 47 | 6651 | 6886 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8534 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3302 | 3492 | 6648 | 6838 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 726 | 896 | 9151 | 9456 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 5796 | 5966 | 10824 | 11129 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 2668 | 2453 | 5802 | 6062 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

For UE coexistence study of Band 2 + Band n5, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.24.2-2

Table 5.24.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5550 | 5730 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 7400 | 7640 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 9250 | 9550 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1001 | 1086 | 2674 | 2759 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 262 | 152 | 2851 | 2996 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3498 | 3608 | 4524 | 4669 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 562 | 697 | 4701 | 4906 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4322 | 4457 | 6374 | 6554 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 2002 | 2172 | 5348 | 5518 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 1386 | 1546 | 6551 | 6816 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 5146 | 5306 | 8224 | 8489 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 1348 | 1153 | 3852 | 4082 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 7198 | 7428 | 6172 | 6367 |

Based on co-existence study as presented in the table 5.24.2-1 and 5.24.2-2, own Rx impact of the 3rd band is shown as the followings.

- 2nd order IMD generated by dual uplink of Band 2 + Band n5 may also fall into own Rx of band 7

5.24.3	∆TIB and ∆RIB values

Table 5.24.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n5 DC_2-7-7_n5 | 2 | 0.3 |
|  | 7 | 0.3 |
|  | n5 | 0.3 |

Table 5.24.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n5 DC_2-7-7_n5 | 2 | 0 |
|  | 7 | 0 |
|  | n5 | 0 |

5.24.4	Reference sensitivity exceptions

By adjusting the centre frequency of uplink of Band 2+ Band n5, the MSD requirement due to 2nd order IMD for DC_2-7_n5 can be similar as the requirements of DC_3-7_n5. Table 5.1.28.4-3 lists the MSD required for DC_2-7_n5.

Table 5.24.4-1: MSD for the DC configuration due to IMD issue (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-7A_n5A DC_2A-7C_n5A DC_2A-7A-7A_n5A | 2 | 1855 | 10 | 50 | 1935 | N/A | N/A |
|  | 7 | 2575 | 10 | 50 | 2685 | 30.0 | IMD2 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |

5.25	DC_1-40_n78

### 5.25.1	Operating bands for DC

Table 5.25.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-40_n78 | CA_1-40 | n78 | No |

### 5.25.2	Configurations for DC

Table 5.25.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-40A_n78A DC_1A-40C_n78A | DC_1A_n78A DC_40A_n78A | CA_1A-40A CA_1A-40C | n78 |

### 5.25.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n78 captured in 37.863-01-01, MSD shall be considered since

-	4th order IMD of the two bands may fall into Rx frequencies of band 1 and band 40.

And based on co-existence studies of Band 40 + Band n78 captured in 37.716-11-11, MSD shall be considered since

-	4th order IMD generated by dual uplink of the two bands may fall into own Rx of band 1.

- 	harmonic mixing generated by 2nd harmonic of n78 Tx and 3rd harmonic of band 40 Rx.

### 5.25.4	∆TIB and ∆RIB values

Table 5.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-40-n78 | 1 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.25.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-40-n78 | 1 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.25.5	REFSENS requirements

As stated in 5.25.3, for MSD requirement caused by IMDs is specified below accordingly. To note that the MSD requirements are already defined for DC_1A_n40A-n78A in TS 38.101-3, among which the one for IMD4 on band n40 can be followed here for band 40 IMD4 impact. The requirements apply both to DC_1A-40A_n78A and DC_1A-40C_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-40A_n78A DC_1A-40C_n78A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 40 | 2340 | 5 | 25 | 2340 | 10.6 | IMD4 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
| DC_1A-40A_n78A DC_1A-40C_n78A | 1 | 1950 | 5 | 25 | 2140 | 9.1 | IMD4 |
|  | 40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n78 | 3430 | 10 | 50 | 3430 | N/A | N/A |

Other issues mentioned in 5.25.3 (i.e., harmonic mixing between band 40 Rx and n78 Tx, and MSD for band 1 caused by 4th IMD of band 1 and n78) had already been specified in the corresponding specs.

5.26	DC_3-40_n78

### 5.26.1	Operating bands for DC

Table 5.26.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-40_n78 | CA_3-40 | n78 | DC_3_n78 |

### 5.26.2	Configurations for DC

Table 5.26.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-40A_n78A DC_3A-40C_n78A | DC_3A_n78A DC_40A_n78A | CA_3A-40A CA_3A-40C | n78 |

### 5.26.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n78 captured in 37.863-01-01, MSD shall be considered since

-	2nd 4th and 5th order IMD of the two bands may fall into Rx frequencies of band 3

- 	2nd harmonic of band 3 may fall into Rx frequencies of n78

- 	5th order IMD of the two bands may fall into Rx frequencies of band 40

And based on co-existence studies of Band 40 + Band n78 captured in 37.716-11-11, MSD shall be considered since

-	4th order IMD generated by dual uplink of the two bands may fall into own Rx of band 3.

### 5.26.4	∆TIB and ∆RIB values

Table 5.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-40-n78 | 3 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.26.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-40-n78 | 3 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.26.5	REFSENS requirements

As stated in 5.26.3, for MSD requirement caused by IMDs is specified below accordingly. To note that the MSD requirements are already defined for DC_3A_n40A-n78A in TS 38.101-3, among which the one for IMD4 on band n40 can be followed here for band 40 IMD4 impact. The below requirements apply both to DC_3A-40A_n78A and DC_3A-40C_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-40A_n78A DC_3A-40C_n78A | 3 | 1775 | 5 | 25 | 1870 | 9.1 | IMD4 |
|  | 40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
|  | n78 | 3325 | 10 | 50 | 3325 | N/A | N/A |
| DC_3A-40A_n78A DC_3A-40C_n78A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 40 | 2360 | 5 | 25 | 2360 | 4.4 | IMD5 |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | N/A |

Other issues mentioned in 5.26.3 (i.e., 2nd harmonic from band 3 and MSD for band 3 caused by 2nd, 4th and 5th IMD of band 3 and n78) had already been specified in the corresponding specs. To note that due to the high IMD impact, single UL is allowed between band 3 and band n78.

5.27	DC_7-40_n78

### 5.27.1	Operating bands for DC

Table 5.27.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-40_n78 | CA_7-40 | n78 | No |

### 5.27.2	Configurations for DC

Table 5.27.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_7A-40A_n78A DC_7A-40C_n78A | DC_7A_n78A DC_40A_n78A | CA_7A-40A CA_7A-40C | n78 |

### 5.27.3	Co-existence studies

Based on co-existence studies of Band 7 + Band n78 captured in 37.863-01-01, MSD shall be considered since

-	4th order IMD of the two bands may fall into Rx frequencies of band 40

And based on co-existence studies of Band 40 + Band n78 captured in 37.716-11-11, MSD shall be considered since

-	4th order IMD generated by dual uplink of the two bands may fall into Rx of band 7.

- 	harmonic mixing generated by 2nd harmonic of n78 Tx and 3rd harmonic of band 40 Rx.

### 5.27.4	∆TIB and ∆RIB values

Table 5.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-40-n78 | 7 | 0.5 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-40-n78 | 7 | 0 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.27.5	REFSENS requirements

As stated in 5.27.3, for MSD requirement caused by IMDs is specified below accordingly. The requirements apply both to DC_7A-40A_n78A and DC_7A-40C_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-40A_n78A DC_7A-40C_n78A | 7 | 2510 | 5 | 25 | 2630 | 10.1 | IMD4 |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | N/A |
| DC_7A-40A_n78A DC_7A-40C_n78A | 7 | 2510 | 5 | 25 | 2630 | N/A | N/A |
|  | 40 | 2310 | 5 | 25 | 2310 | 8.7 | IMD4 |
|  | n78 | 3785 | 10 | 50 | 3785 | N/A | N/A |

Other issues mentioned in 5.27.3 had already been specified in the corresponding specs.

5.28	DC_8-40_n78

### 5.28.1	Operating bands for DC

Table 5.28.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8-40_n78 | CA_8-40 | n78 | No |

### 5.28.2	Configurations for DC

Table 5.28.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A-40A_n78A DC_8A-40C_n78A | DC_8A_n78A DC_40A_n78A | CA_8A-40A CA_8A-40C | n78 |

### 5.28.3	Co-existence studies

Based on co-existence studies of Band 8 + Band n78 captured in 37.863-01-01, MSD shall be considered since

-	4th order IMD of the two bands may fall into Rx frequencies of band 8

-	4th harmonic of the band 8 may fall into Rx frequencies of band n78

- 	2nd order IMD of the two bands may fall into Rx frequencies of band 40, under the cases that band 40 is not in sync with band n78 in TDD configuration

And based on co-existence studies of Band 40 + Band n78 captured in 37.716-11-11, MSD shall be considered since

-	2nd and 3rd order IMD generated by dual uplink of the two bands may fall into own Rx of band 8.

- 	harmonic mixing generated by 2nd harmonic of n78 Tx and 3rd harmonic of band 40 Rx.

### 5.28.4	∆TIB and ∆RIB values

Table 5.28.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-40-n78 | 8 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.28.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-40-n78 | 8 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.28.5	REFSENS requirements

As stated in 5.28.3, for MSD requirement caused by IMDs is specified below accordingly. The below requirements apply both to DC_8A-40A_n78A and DC_8A-40C_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-40A_n78A DC_8A-40C_n78A | 8 | 905 | 5 | 25 | 950 | 30.5 | IMD2 |
|  | 40 | 2380 | 5 | 25 | 2380 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_8A-40A_n78ADC_8A-40C_n78A | 8 | 890 | 5 | 25 | 935 | 19.8 | IMD3 |
|  | 40 | 2320 | 5 | 25 | 2320 | N/A | N/A |
|  | n78 | 3705 | 10 | 50 | 3705 | N/A | N/A |
| DC_8A-40A_n78A DC_8A-40C_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 40 | 2395 | 5 | 25 | 2395 | 28 | IMD2 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |

Other issues mentioned in 5.28.3 had already been specified in the corresponding specs.

5.29	DC_2-4_n28

### 5.29.1	Configurations for DC

Table 5.29.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-4A_n28A | DC_2A_n28A DC_4A_n28A |

### 5.29.2	Co-existence studies

Co-existence studies on DC_4_n28 and DC_2_n28 show own Rx impact of the 3rd band due to dual UL as following:

- IMD4 by dual UL of Band 4 and Band n28 may fall into Band 2 DL.

### 5.29.3	∆TIB and ∆RIB values

For DC_2-4_n28, the TIB,c and RIB,c values are reused from the LTE combination CA_2-4-28, and are given in the tables below.

Table 5.29.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-4-n28 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | n28 | 0.8 |

Table 5.29.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-4-n28 | 2 | 0.3 |
|  | 4 | 0.3 |
|  | n28 | 0.5 |

### 5.29.4	REFSENS requirements

The MSD value for the impact on Band 2 DL by dual uplink of Band 4 + Band n28 stated in 5.29.2 is shown in the following table.

Table 5.29.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-4A_n28A | 2 | 1880 | 5 | 25 | 1960 | 11.0 | FDD | IMD4 |
|  | 4 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |

5.30	DC_2-7_n28

### 5.30.1	Configurations for DC

Table 5.30.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-7A_n28A | DC_2A_n28A DC_7A_n28A |

### 5.30.2	Co-existence studies

Co-existence analysis from DC_2_n28 shows that there is IMD2 impact from DC_2_n28 UL to Band 7 DL. And there is no IMD impact from DC_7_n28 UL to Band 2 DL.

### 5.30.3	∆TIB and ∆RIB values

For DC_2-7_n28, the TIB,c and RIB,c values are reused from the LTE combination CA_2-7-28, and are given in the tables below.

Table 5.30.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-n28 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.3 |

Table 5.30.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-n28 | 2 | 0 |
|  | 7 | 0 |
|  | n28 | 0 |

### 5.30.4	REFSENS requirements

The MSD value for the impact on Band 7 DL by dual uplink of Band 2 + Band n28 stated in 5.30.2 is shown in the following table.

Table 5.30.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-7A_n28A | 2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | 7 | 1720 | 5 | 25 | 2120 | 29.0 | FDD | IMD2 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |

5.31	DC_2-66_n28

### 5.31.1	Configurations for DC

Table 5.31.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-66A_n28A | DC_2A_n28A DC_66A_n28A |

### 5.31.2	Co-existence studies

Co-existence studies on DC_28_n66 and DC_2_n28 show the following own Rx impact of the 3rd band:

- IMD4 by dual UL of Band 66 and Band n28 may fall into Band 2 DL.

### 5.31.3	∆TIB and ∆RIB values

For DC_2-66_n28, the TIB,c and RIB,c values are reused from the LTE combination CA_2-66-28, and are given in the tables below.

Table 5.31.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66-n28 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n28 | 0.6 |

Table 5.31.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66-n28 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n28 | 0.2 |

### 5.31.4	REFSENS requirements

The MSD value for the impact on Band 2 DL by dual uplink of Band 66 + Band n28 stated in 5.31.2 is shown in the following table.

Table 5.31.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n28A | 2 | 1880 | 5 | 25 | 1960 | 11.0 | FDD | IMD4 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |

5.32	DC_4-7_n28

### 5.32.1	Configurations for DC

Table 5.32.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_4A-7A_n28A | DC_4A_n28A DC_7A_n28A |

### 5.32.2	Co-existence studies

Based on the co-existence studies of DC_4_n28 and DC_7_n28, own Rx impact of the 3rd band are the following:

- IMD3 by dual UL of Band 4 and Band n28 may fall into Band 7 DL.

### 5.32.3	∆TIB and ∆RIB values

For DC_4-7_n28, the TIB,c and RIB,c values are reused from the LTE combination CA_4-7-28, and are given in the tables below.

Table 5.32.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_4-7-n28 | 4 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.6 |

Table 5.32.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_4-7-n28 | 4 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.2 |

### 5.32.4	REFSENS requirements

The MSD values for own Rx impact of the 3rd band stated in 5.32.2 are shown in the following table.

Table 5.32.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_4A-7A_n28A | 4 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | 18.0 | FDD | IMD3 |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |

5.33	DC_5-7_n66

### 5.33.1	Configurations for DC

Table 5.33.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_5A-7A_n66A DC_5A-7C_n66A DC_5A-7A-7A_n66A | DC_5A_n66A DC_7A_n66A |

### 5.33.2	Co-existence studies

Co-existence studies on DC_7_n66 and DC_5_n66 show own Rx impact on the 3rd band due to dual uplink as follows:

- IMD3 of dual uplink of Band 7 and n66 may fall into Band 5 DL,

- IMD2 and IMD3 of dual uplink of Band 5 and n66 may fall into Band 7 DL.

### 5.33.3	∆TIB and ∆RIB values

For DC_5-7_n66, the TIB,c and RIB,c values are reused from the LTE combination CA_5-7-66, and are given in the tables below.

Table 5.33.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7-n66 | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

Table 5.33.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7-n66 | 5 | 0 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

### 5.33.4	REFSENS requirements

The MSD values for own Rx impact of the 3rd band stated in 5.33.2 are shown in the following table. The MSD levels are derived from the text proposal in R4-2009928 and R4-2014128.

Table 5.33.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-7A_n66A DC_5A-7C_n66A DC_5A-7A-7A_n66A | 5 | 835 | 5 | 25 | 880 | 17.8 | FDD | IMD3 |
|  | 7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | 7 | 2504 | 5 | 25 | 2624 | 29.0 | FDD | IMD21 |
|  | 66 | 1777.5 | 5 | 25 | 2177.5 | N/A | FDD | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.34	DC_7-66_n28

### 5.34.1	Configurations for DC

Table 5.34.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_7A-66A_n28A | DC_7A_n28A DC_66A_n28A |

### 5.34.2	Co-existence studies

Based on the co-existence studies of DC_66_n28 and DC_7_n28, own Rx impact of the 3rd band are the following:

- IMD3 by dual UL of Band 66 and Band n28 may fall into Band 7 DL.

### 5.34.3	∆TIB and ∆RIB values

For DC_7-66_n28, the TIB,c and RIB,c values are reused from the LTE combination CA_4-7-28, and are given in the tables below.

Table 5.34.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66-n28 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n28 | 0.6 |

Table 5.34.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66-n28 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n28 | 0.2 |

### 5.34.4	REFSENS requirements

The MSD values for own Rx impact of the 3rd band stated in 5.34.2 are shown in the following table.

Table 5.34.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-66A_n28A | 7 | 2565 | 5 | 25 | 2685 | 18.0 | FDD | IMD3 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |

5.35	DC_1-11_n28

### 5.35.1	Configurations for DC_1-11_n28

Table 5.35.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n28A | DC_1A_n28A DC_11A_n28A | CA_1A-11A | n28A |

### 5.35.2	Co-existence studies

When Uplink EN-DC configuration is DC_11A_n28A, IMD2 and IMD3 of (B11 - n28) will fall into Rx band of Band 1. But considering the current situation that Band 11 is operated only by a certain operators in Japan, the frequency ranges can be limited as follows.

- Operator X: B11 (UL:1427.9-1437.9 / DL:1475.9-1485.9 MHz), B1 (UL:1960-1980 / DL:2150-2170 MHz), n28 (UL:738-748 / DL:793-803 MHz)

- Opeartor Y: B11 (UL:1437.9-1447.9 / DL:1485.9-1495.9 MHz), B1 (UL:1920-1940 / DL:2110-2130 MHz), n28 (UL:718-728 / DL:773-783 MHz)

Based on the existing frequency restriction in Japan, RAN4 only need to analyze the IMD2 problem as follow

- IMD2 by dual uplink DC_11_n28 will fall into the own Rx band of Band 1

- IMD3 by dual uplink DC_11_n28 is not specified due to the current operator spectrum holding.

### 5.35.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.35.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n28 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | n28 | 0.6 |

Table 5.35.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n28 | 1 | 0 |
|  | 11 | 0 |
|  | n28 | 0.2 |

### 5.35.4	Reference sensitivity exceptions

As mentioned above, IMD2 of B11 and n28 to Band 1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.35.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-11A_n28A | 11 | 1440 | 5 | 25 | 1488 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | N/A | N/A |
|  | 1 | 1960 | 5 | 25 | 2150 | 28.3 | IMD2 |
| DC_1A-11A_n28A | 11 | 1440 | 5 | 5 | 1488 | N/A | N/A |
|  | n28 | 710 | 5 | 5 | 765 | N/A | N/A |
|  | 1 | 1975 | 5 | 5 | 2165 | N/A1 | IMD3 |

5.36	DC_3-11_n28

### 5.36.1	Configurations for DC_3-11_n28

Table 5.36.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-11A_n28A | DC_3A_n28A DC_11A_n28A | CA_3A-11A | n28A |

### 5.36.2	Co-existence studies

There is no additional harmonic and intermodulation impact for the additional band receiver.

### 5.36.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.36.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-11_n28 | 3 | 0.8 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |

Table 5.36.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-11_n28 | 3 | 0.3 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |

### 5.36.4	Reference sensitivity exceptions

As mentioned in 5.36.2, REFSENS exceptions are not expected.

5.37	DC_8-11_n28

### 5.37.1	Configurations for DC_8-11_n28

Table 5.37.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A-11A_n28A | DC_8A_n28A DC_11A_n28A | CA_8A-11A | n28A |

### 5.37.2	Co-existence studies

There is no additional harmonic and intermodulation impact for the additional band receiver.

### 5.37.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.37.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n28 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n28 | 0.6 |

Table 5.37.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n28 | 8 | 0.2 |
|  | 11 | 0 |
|  | n28 | 0.2 |

### 5.37.4	Reference sensitivity exceptions

As mentioned in 5.37.2, REFSENS exceptions are not expected.

5.38	DC_3-11_n77

### 5.38.1	Configurations for DC_3-11_n77

Table 5.38.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-11A_n77A | DC_3A_n77A DC_11A_n77A | CA_3A-11A | n77A |
| DC_3A-11A_n77(2A) | DC_3A_n77A DC_11A_n77A | CA_3A-11A | n77(2A) |

### 5.38.2	Co-existence studies

When Uplink EN-DC configuration is DC_3A_n77A, (1) IMD4 of (B3 - n77) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n77A, (2) IMD2 and (3) IMD5 of (B11 - n77) will fall into Rx band of Band 3.

But considering the current situation that Band 11 is operated only by a certain operators in Japan, the frequency ranges can be limited as follows.

-	Operator X: B11 (UL:1427.9-1437.9 / DL:1475.9-1485.9 MHz), B3 (UL:1750-1765 / DL:1845-1860 MHz), n77 (3400-3440 / 3560-3600 / 3900-4000 MHz)

-	Opeartor Y: B11 (UL:1437.9-1447.9 / DL:1485.9-1495.9 MHz), B3 (UL:1710-1730 / DL:1805-1825 MHz), n77 (3520-3560 / 3700-3800 / 4000-4100 MHz)

Then the own Rx impact can be simplified as below.

- IMD4 of (B3-n77) will fall into Rx band of Band 11.

- IMD5 of (B11-n77) will fall into Rx band of Band 3.

### 5.38.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.38.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-11_n77 | 3 | 0.8 |
|  | 11 | 0.9 |
|  | n77 | 0.8 |

Table 5.38.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-11_n77 | 3 | 0.3 |
|  | 11 | 0.5 |
|  | n77 | 0.5 |

### 5.38.4	Reference sensitivity exceptions

As mentioned above, IMD4 of B3 and n77 to Band11 Rx and IMD5 of B11 and n77 to Band3 Rx need to be addressed for REFSENS relaxation. Based on the exceptions of DC_3-21_n77, the following values are proposed:

Table 5.38.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_3A-11A_n77A DC_3A-11A_n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | 3675 | 10 | 50 | 3675 | N/A | TDD | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 8.8 | FDD | IMD4 |
|  | 11 | 1435.4 | 5 | 25 | 1483.4 | N/A | FDD | N/A |
|  | n77 | 3905 | 10 | 50 | 3905 | N/A | TDD | N/A |
|  | 3 | 1753 | 5 | 25 | 1848 | 3.4 | FDD | IMD5Y |
| Note Y: This band is also subject to IMD2 which is not specified. The frequency range below 3400MHz in n77 is not used for this combination. |  |  |  |  |  |  |  |  |

## 5.39	DC_2-12_n5

### 5.39.1	Operating bands for EN-DC

Table 5.39.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-12_n5 | CA_2-12 | n5 |  |

5.39.2	Configuration for DC

Table 5.39.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A_n5A | DC_2A_n5A DC_12A_n5A | DC_2A-12A | n5A |

### 5.39.3		∆TIB and ∆RIB values

For DC_2-12_n5, the TIB,c and RIB,c values are reused from LTE combination CA_2-5-12 and are given in the tables below.

Table 5.39.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n5 | 2 | 0.3 |
|  | 12 | 0.4 |
|  | n5 | 0.8 |

Table 5.39.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12_n5 | 2 | 0 |
|  | 12 | 0.3 |
|  | n5 | 0.5 |

### 5.39.4	REFSENS requirements

There is IMD5 impact from DC_12_n5 UL affecting band 2 DL. MSD derived from DC_7A-28A_n5A and DC_20A_n7A-n28A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-12A_n5A | 2 | 1900 | 5 | 25 | 1980 | 5.9 | IMD5 |
|  | 12 | 705 | 5 | 25 | 735 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |

## 5.40	DC_2-5_n12

### 5.40.1	Operating bands for EN-DC

Table 5.40.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-5_n12 | CA_2-5 | n12 |  |

5.40.2	Configuration for DC

Table 5.40.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A_n12A | DC_2A_n12A DC_5A_n12A | DC_2A-5A | n12A |

### 5.40.3		∆TIB and ∆RIB values

For DC_2-5_n12, the TIB,c and RIB,c values are reused from LTE combination CA_2-5-12 and are given in the tables below.

Table 5.40.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n12 | 2 | 0.3 |
|  | 5 | 0.8 |
|  | n12 | 0.4 |

Table 5.40.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n12 | 2 | 0 |
|  | 5 | 0.5 |
|  | n12 | 0.3 |

### 5.40.4	REFSENS requirements

There is IMD5 impact from DC_5_n12 UL affecting band 2 DL. MSD derived from DC_7A-28A_n5A and DC_20A_n7A-n28A.

Band 5 is also affected by IMD5 from UL DC_2A_n12A, but MSD value is not specified as there is only partial overlap of IMD5 with DL carrier

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-5A_n12A6 | 2 | 1900 | 5 | 25 | 1980 | 5.9 | IMD5 |
|  | 5 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n12 | 705 | 5 | 25 | 735 | N/A | N/A |
| NOTE 6: Band 5 is also affected by IMD5 from UL DC_2A_n12A, but MSD value is not specified as there is only partial overlap of IMD5 with DL carrier |  |  |  |  |  |  |  |

## 5.41	DC_5-66_n12

### 5.41.1	Operating bands for EN-DC

Table 5.41.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 5-66_n12 | CA_5-66 | n12 |  |

5.41.2	Configuration for DC

Table 5.41.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-66A_n12A | DC_5A_n12A DC_66A_n12A | DC_5A-66A | n12A |

### 5.41.3		∆TIB and ∆RIB values

For DC_5-66_n12, the TIB,c and RIB,c values are reused from LTE combination CA_5-12-66 and are given in the tables below.

Table 5.41.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n12 | 5 | 0.3 |
|  | 66 | 0.8 |
|  | n12 | 0.8 |

Table 5.41.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n12 | 5 | 0 |
|  | 66 | 0.5 |
|  | n12 | 0.5 |

### 5.41.4	REFSENS requirements

There is no need to define MSD.

## 5.42	DC_66_(n)5

### 5.42.1	Operating bands for EN-DC

Table 5.42.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_66-(n)5 | CA_66-5 | n5 |  |

5.42.2	Configuration for DC

Table 5.42.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_66A-(n)5AA | DC_66A_n5A DC_(n)5AA1 | CA_66A-5A | n5A |
| NOTE1: Only single switched UL is supported |  |  |  |

### 5.42.3		∆TIB and ∆RIB values

For DC_66_(n)5 the ΔTIB,c and ΔRIB,c values are derived from DC_66_n5 combination.

Table 5..1.62.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_(n)5 | 5 | 0.3 |
|  | n5 | 0.3 |
|  | 66 | 0.3 |

Table 5.1.116.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_(n)5 | 5 | 0 |
|  | n5 | 0 |
|  | 66 | 0 |

### 5.42.4	REFSENS requirements

No additional REFSENS requirement is needed.

## 5.43	DC_12-66_n5

### 5.43.1	Operating bands for EN-DC

Table 5.43.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 12-66_n5 | CA_12-66 | n5 |  |

5.43.2	Configuration for DC

Table 5.43.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_12A-66A_n5A | DC_12A_n5A DC_66A_n5A | DC_12A-66A | n5A |

### 5.43.3		∆TIB and ∆RIB values

For DC_12-66_n5, the TIB,c and RIB,c values are reused from LTE combination CA_5-12-66 and are given in the tables below.

Table 5.43.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n5 | 12 | 0.8 |
|  | 66 | 0.8 |
|  | n5 | 0.3 |

Table 5.43.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-66_n5 | 12 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0 |

### 5.43.4	REFSENS requirements

There is IMD4 impact from DC_66_n5 UL affecting band 12 DL. MSD derived from DC_3A-28A_n5A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-66A_n5A | 12 | 712 | 5 | 25 | 742 | 9.4 | IMD4 |
|  | 66 | 1745 | 5 | 25 | 2145 | N/A | N/A |
|  | n5 | 829 | 5 | 25 | 874 | N/A | N/A |

5.44		DC_7-8_n28

### 5.44.1	 Operating bands for DC

Table 5.44.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_7-8_n28 | CA_7-8 | n28 |

### 5.44.2 	Configuration for DC

Table 5.44.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-8A_n28A | DC_7A_n28A DC_8A_n28A | CA_7-8 | n28 |

### 5.44.3	 Co-existence studies

For UE coexistence study of Band 8 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.44.3-1.

Table 5.44.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 212 | 132 | 1583 | 1663 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012 | 1127 | 491 | 616 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2463 | 2578 | 2286 | 2411 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1892 | 2042 | 1194 | 1364 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3343 | 3493 | 2989 | 3159 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 264 | 424 | 3166 | 3326 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2112 | 1897 | 2957 | 2772 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 484 | 279 | 1339 | 1144 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692 | 3907 | 4223 | 4408 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3869 | 4074 | 4046 | 4241 |

For UE coexistence study of Band 7 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.44.3-1.

Table 5.44.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1867 | 1752 | 3203 | 3318 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4252 | 4437 | 1164 | 1004 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5703 | 5888 | 3906 | 4066 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6752 | 7007 | 461 | 256 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8203 | 8458 | 4609 | 4814 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3504 | 3734 | 6406 | 6636 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 242 | 9577 | 9252 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2756 | 3031 | 6304 | 6004 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5312 | 5562 | 10703 | 11028 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7109 | 7384 | 8906 | 9206 |

There is no IMD issue for Rx of band 7 with UL DC_8_n28.

There is no IMD issue for Rx of band 8 with UL DC_7_n28.

### 5.44.4	 ∆TIB and ∆RIB values

Table 5.44.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n28 | 7 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |

Table 5.44.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n28 | 7 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.1 |

### 5.44.5	REFSENS requirements

There is no reference sensitivity exception (MSD) for DC_7-8_n28.

5.45		DC_20-28_n3

### 5.45.1	 Operating bands for DC

Table 5.45.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_20-28_n3 | CA_20-28 | n3 |

### 5.45.2 	Configuration for DC

Table 5.45.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_20A-28A_n3A | DC_20A_n3A DC_28A_n3A | CA_20-28 | n3 |

### 5.45.3	 Co-existence studies

For UE coexistence study of Band 20 + Band n3, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.45.3-1.

Table 5.45.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 848 | 953 | 2542 | 2647 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 121 | 14 | 2558 | 2738 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3374 | 3509 | 4252 | 4432 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 711 | 876 | 4268 | 4523 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4206 | 4371 | 5962 | 6217 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1906 | 1696 | 5084 | 5294 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6308 | 5978 | 1738 | 1543 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3691 | 3406 | 834 | 1074 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7672 | 8002 | 5038 | 5233 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6794 | 7079 | 5916 | 6156 |

For UE coexistence study of Band 28 + Band n3, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.45.3-1.

Table 5.45.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1082 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 379 | 214 | 2672 | 2867 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3281 | 4123 | 4318 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 324 | 534 | 4382 | 4652 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4029 | 5833 | 6103 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2164 | 1924 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6437 | 6092 | 1282 | 1027 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3949 | 3634 | 1176 | 1461 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7888 | 4522 | 4777 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6851 | 5529 | 5814 |

IMD4 may fall into Rx of band 28 with UL DC_20_n3.

### 5.45.4	 ∆TIB and ∆RIB values

Table 5.45.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-28_n3 | 2 | 0.5 |
|  | 28 | 0.6 |
|  | n3 | 0.5 |

Table 5.45.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-28_n3 | 2 | 0.3 |
|  | 28 | 0.2 |
|  | n3 | 0.3 |

### 5.45.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD4 for DC_20-28_n3 with UL DC_20_n3 is specified as below referring to the MSD for DC_3A-28A_n5A from 38.101-3.

Table 5.45.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-28A_n3A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | 28 | 730 | 5 | 25 | 785 | 9.4 | IMD4 |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |

5.46		DC_28-66_n66

### 5.46.1	 Operating bands for DC

Table 5.46.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_28-66_n66 | CA_28-66 | n66 |

### 5.46.2 	Configuration for DC

Table 5.46.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_28A-66A_n66A | DC_28A_n66A DC_66A_n66A2 | CA_28-66 | n66 |
| NOTE 2: Only single switched UL is supported |  |  |  |

### 5.46.3	 Co-existence studies

For UE coexistence study of Band 28 + Band n66, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.46.3-1.

Table 5.46.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1077 | 2413 | 2528 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 374 | 214 | 2672 | 2857 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3276 | 4123 | 4308 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 329 | 534 | 4382 | 4637 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4024 | 5833 | 6088 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2154 | 1924 | 4826 | 5056 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6417 | 6092 | 1282 | 1032 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3934 | 3634 | 1176 | 1451 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7868 | 4522 | 4772 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6836 | 5529 | 5804 |

For UL DC_66A_n66A, there is no need to study the IMD since only single switched UL is supported.

IMD4 may fall into Rx of band 66 with UL DC_28_n66.

### 5.46.4	 ∆TIB and ∆RIB values

Table 5.46.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28-66_n66 | 28 | 0.6 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

Table 5.46.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28-66_n66 | 28 | 0.2 |
|  | 66 | 0 |
|  | n66 | 0 |

### 5.46.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD4 for DC_28-66_n66 with UL DC_28_n66 is specified as below referring to the MSD for DC_1A-3A_n28A from 38.101-3.

Table 5.46.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A-66A_n66A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 66 | 1729 | 5 | 25 | 2129 | 11.0 | IMD4 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |

5.47		DC_7-28_n66

### 5.47.1	 Operating bands for DC

Table 5.47.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_7-28_n66 | CA_7-28 | n66 |

### 5.47.2 	Configuration for DC

Table 5.47.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-28A_n66A DC_7C-28A_n66A | DC_7A_n66A DC_28A_n66A | CA_7-28 | n66 |

### 5.47.3	 Co-existence studies

For UE coexistence study of Band 28 + Band n66, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.47.3-1.

Table 5.47.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1077 | 2413 | 2528 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 374 | 214 | 2672 | 2857 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3276 | 4123 | 4308 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 329 | 534 | 4382 | 4637 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4024 | 5833 | 6088 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2154 | 1924 | 4826 | 5056 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6417 | 6092 | 1282 | 1032 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3934 | 3634 | 1176 | 1451 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7868 | 4522 | 4772 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6836 | 5529 | 5804 |

For UE coexistence study of Band 7 + Band n66, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.47.3-1.

Table 5.47.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 860 | 720 | 4210 | 4350 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3220 | 3430 | 850 | 1060 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6710 | 6920 | 5920 | 6130 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5720 | 6000 | 2560 | 2840 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9210 | 9490 | 7630 | 7910 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1440 | 1720 | 8420 | 8700 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4620 | 4270 | 8570 | 8220 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 340 | 10 | 4290 | 3940 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9340 | 9690 | 11710 | 12060 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10130 | 10480 | 10920 | 11270 |

IMD3 may fall into Rx of band 7 with UL DC_28_n66.

IMD2 may fall into Rx of band 28 with UL DC_7_n66.

### 5.47.4	 ∆TIB and ∆RIB values

Table 5.47.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n66 | 7 | 0.5 |
|  | 28 | 0.6 |
|  | n66 | 0.5 |

Table 5.47.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n66 | 7 | 0 |
|  | 28 | 0.2 |
|  | n66 | 0 |

### 5.47.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD3 for DC_7-28_n66 with UL DC_28_n66 is specified as below referring to the MSD for DC_3A-7A_n28A from 38.101-3.

The reference sensitivity exception (MSD) due to IMD2 for DC_7-28_n66 with UL DC_7_n66 is specified as below referring to the MSD for DC_3A_n7A-n28A from 38.101-3.

Table 5.47.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-28A_n66A DC_7C-28A_n66A | 7 | 2562 | 10 | 50 | 2682 | 16.9 | IMD3 |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796 | 20.0 | IMD2 |
|  | n66 | 1747 | 5 | 25 | 2147 | N/A | N/A |

5.48		DC_2-28_n66

### 5.48.1	 Operating bands for DC

Table 5.48.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_2-28_n66 | CA_2-28 | n66 |

### 5.48.2 	Configuration for DC

Table 5.48.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-28A_n66A | DC_2A_n66A DC_28A_n66A | CA_2-28 | n66 |

### 5.48.3	 Co-existence studies

For UE coexistence study of Band 28 + Band n66, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.48.3-1.

Table 5.48.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1077 | 2413 | 2528 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 374 | 214 | 2672 | 2857 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3276 | 4123 | 4308 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 329 | 534 | 4382 | 4637 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4024 | 5833 | 6088 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2154 | 1924 | 4826 | 5056 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6417 | 6092 | 1282 | 1032 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3934 | 3634 | 1176 | 1451 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7868 | 4522 | 4772 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6836 | 5529 | 5804 |

For UE coexistence study of Band 2 + Band n66, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.48.3-1.

Table 5.48.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 200 | 70 | 3560 | 3690 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1920 | 2110 | 1510 | 1710 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5410 | 5600 | 5270 | 5470 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3770 | 4020 | 3220 | 3490 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7260 | 7510 | 6980 | 7250 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 140 | 400 | 7120 | 7380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5270 | 4930 | 5930 | 5620 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1640 | 1310 | 2310 | 1990 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8690 | 9030 | 9110 | 9420 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8830 | 9160 | 8970 | 9290 |

IMD4 may fall into Rx of band 2 with UL DC_28_n66.

There is no IMD issue for Rx of band 28 with UL DC_2_n66.

### 5.48.4	 ∆TIB and ∆RIB values

Table 5.48.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-28_n66 | 2 | 0.5 |
|  | 28 | 0.6 |
|  | n66 | 0.5 |

Table 5.48.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-28_n66 | 2 | 0.3 |
|  | 28 | 0.2 |
|  | n66 | 0.3 |

### 5.48.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD4 for DC_2-28_n66 with UL DC_28_n66 is specified as below referring to the MSD for DC_1A-3A_n28A from 38.101-3.

Table 5.48.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-28A_n66A | 2 | 1900 | 5 | 25 | 1980 | 11 | IMD4 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |

5.49		Void

5.50		Void

5.51		DC_8-40_n1

### 5.51.1	 Operating bands for DC

Table 5.51.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_8-40_n1 | CA_8-40 | n1 |

### 5.51.2 	Configuration for DC

Table 5.51.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_8A-40A_n1A DC_8A-40C_n1A | DC_8A_n1A DC_40A_n1A | CA_8-40 | n1 |

### 5.51.3	 Co-existence studies

For UE coexistence study of Band 8 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.51.3-1.

Table 5.51.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

For UE coexistence study of Band 40 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.51.3-1.

Table 5.51.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 11500 | 12000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 320 | 480 | 4220 | 4380 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1660 | 2620 | 2880 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140 | 6360 | 6520 | 6780 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 3640 | 4920 | 5280 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060 | 8340 | 8820 | 9180 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 960 | 640 | 8440 | 8760 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7680 | 7220 | 5620 | 5280 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 2940 | 1340 | 960 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120 | 11580 | 9980 | 10320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740 | 11160 | 10360 | 10740 |

IMD4 may fall into Rx of band 8 with UL DC_40_n1.

### 5.51.4	 ∆TIB and ∆RIB values

Table 5.51.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-40_n1 | 8 | 0.3 |
|  | 40 | 0.5 |
|  | n1 | 0.3 |

Table 5.51.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-40_n1 | 8 | 0.2 |
|  | 40 | 0.5 |
|  | n1 | 0 |

### 5.51.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD4 for DC_8-40_n1 with UL DC_40_n1 is specified as below referring to the MSD for DC_1A_n8A-n40A from 38.101-3.

Table 5.51.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-40A_n1A DC_8A-40C_n1A | 8 | 885 | 5 | 25 | 930 | 8.0 | IMD4 |
|  | 40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |

5.52		DC_1-32_n3

### 5.52.1	 Operating bands for DC

Table 5.52.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_1-32_n3 | CA_1-32 | n3 |

### 5.52.2 	Configuration for DC

Table 5.52.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-32A_n3A | DC_1A_n3A | CA_1-32 | n3 |

### 5.52.3	 Co-existence studies

For UE coexistence study of Band 1 + Band n3, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.52.3-1.

Table 5.52.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

IMD3 and IMD5 may fall into Rx of band 32 with UL DC_1_n3.

### 5.52.4	 ∆TIB and ∆RIB values

Table 5.52.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-32_n3 | 1 | 0.5 |
|  | n3 | 0.5 |

Table 5.52.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-32_n3 | 1 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

### 5.52.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD3 for DC_1-32_n3 with UL DC_1_n3 is specified as below referring to the MSD for DC_1A-11A_n3A from 38.101-3.

Table 5.52.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-32A_n3A | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1480 | 15.2 | IMD34 |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

5.53		DC_3-32_n1

### 5.53.1	 Operating bands for DC

Table 5.53.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_3-32_n1 | CA_3-32 | n1 |

### 5.53.2 	Configuration for DC

Table 5.53.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-32A_n1A | DC_3A_n1A | CA_3-32 | n1 |

### 5.53.3	 Co-existence studies

For UE coexistence study of Band 3 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.53.3-1.

Table 5.53.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

IMD3 and IMD5 may fall into Rx of band 32 with UL DC_3_n1.

### 5.53.4	 ∆TIB and ∆RIB values

Table 5.53.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-32_n1 | 3 | 0.5 |
|  | n1 | 0.5 |

Table 5.53.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-32_n1 | 3 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

### 5.53.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD3 for DC_3-32_n1 with UL DC_3_n1 is specified as below referring to the MSD for DC_1A-11A_n3A from 38.101-3.

Table 5.53.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-32A_n1A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1480 | 15.2 | IMD34 |
|  | n1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

5.54	DC_7-32_n1

5.54.1	Configurations for DC

Table 5.54.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_7A-32A_n1A | DC_7A_n1A |

5.54.2	Co-existence studies

Table 5.54.2-1 lists the Band 7A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.54.2-1: Band 7 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 520 – 650 |  |  | 4420 – 4550 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 3020 – 3220 |  |  | 1270 – 1460 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 6920 – 7120 |  |  | 6340 – 6530 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2450 – 2620 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5520 – 5790 |  |  | 3190 – 3440 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1040 – 1300 |  |  | 8840-9100 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9420 – 9690 |  |  | 8260 – 8510 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5110 – 5420 |  |  | 8020 – 8360 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 620 – 940 |  |  | 3540 – 3870 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10180 – 10490 |  |  | 11920 – 12260 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10760 – 11080 |  |  | 11340 – 11670 |  |  |

Based on Table 5.54.2-1,

-	2nd order harmonics may fall into Rx frequencies of band 77

-	3rd order harmonics may fall into Rx frequencies of bands 46 and 47

-	2nd order IMD may fall into Rx frequencies of bands 71 and 79

-	3rd order IMD may fall into Rx frequencies of bands 32, 45, 50, 51, 75, 76, 91, 92, 93 and 94

-	4th order IMD may fall into Rx frequencies of bands 42, 46, 52, 77 and 78

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 22, 26, 27, 28, 29, 42, 43, 44, 46, 48, 49, 67, 68, 71, 77, 78 and 85

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.54.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.54.2-2: 2UL Band 7 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H3 n1, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, IMD5 |

The requirements for coexistence with protected bands exist for DC_7A_n1A in 38101-3.

5.54.3	∆TIB and ∆RIB values

Table 5.54.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-32A_n1 | 7 | 0.6 |
|  | n1 | 0.5 |

Table 5.54.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-32A_n1 | 7 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

5.54.4	Reference sensitivity exceptions

Table 5.54.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-32A_n1A | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1454.5 | 15.2 | IMD3 |

5.55	DC_2-66_n7

### 5.55.1	Configurations for DC

Table 5.55.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-66A_n7A DC_2A-66A-66A_n7A | DC_2A_n7A DC_66A_n7A |

### 5.55.2	Co-existence studies

Co-existence studies on DC_2_n7 and DC_66_n7 show that there is no own Rx impact of the 3rd band from dual UL.

### 5.55.3	∆TIB and ∆RIB values

For DC_2-66_n7, the TIB,c and RIB,c values are reused from the LTE combination CA_2-7-66, and are given in the tables below.

Table 5.55.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66-n7 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.55.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66-n7 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.55.4	REFSENS requirements

According to the co-existence studies, no additional MSD requirement is needed.

5.56	DC_2-5_n7

### 5.56.1	Configurations for DC

Table 5.56.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-5A_n7A | DC_2A_n7A DC_5A_n7A |

### 5.56.2	Co-existence studies

Co-existence analysis from DC_2_n7 and DC_7_n5 show that there is no IMD impact from DC_2_n7 UL to band 5 DL or from DC_5_n7 UL to band 2 DL.

### 5.56.3	∆TIB and ∆RIB values

For DC_2-5_n7, the TIB,c and RIB,c values are reused from the LTE combination CA_2-5-7, and are given in the tables below.

Table 5.56.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-n7 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n7 | 0.5 |

Table 5.56.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-n7 | 2 | 0 |
|  | 5 | 0 |
|  | n7 | 0 |

### 5.56.4	REFSENS requirements

According to the co-existence studies, no additional MSD requirement is needed.

5.57	DC_2-8_n2

### 5.57.1	Configurations for DC

Table 5.57.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-8A_n2A | DC_2A_n2A1 DC_8A_n2A |
| NOTE1: Only single switched UL is supported |  |

### 5.57.2	Co-existence studies

Co-existence analysis from DC_8_n2 show that IMD4 of the dual UL of DC_8_n2 may fall into the DL of band 2.

### 5.57.3	∆TIB and ∆RIB values

For DC_2-8_n2, the TIB,c and RIB,c values are reused from the DC_8_n2, and are given in the tables below.

Table 5.57.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-8-n2 | 2 | 0.3 |
|  | 8 | 0.3 |
|  | n2 | 0.3 |

Table 5.57.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-8-n2 | 2 | 0 |
|  | 8 | 0 |
|  | n2 | 0 |

### 5.57.4	REFSENS requirements

Based on the co-existence studies MSD due to IMD4 is required. The MSD values can be reused from DC_8-n2.

Table 6.1.x.6-1: Reference sensitivity exceptions for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_2-8_n2 | 2 | 1860 | 5 | 25 | 1940 | 4 | FDD | IMD4 |
|  | 8 | 910 | 5 | 25 | 955 | N/A |  | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A |  | N/A |

5.58	DC_5-66_n7

### 5.58.1	Configurations for DC

Table 5.58.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_5A-66A_n7A DC_5A-66A-66A_n7A | DC_5A_n7A DC_66A_n7A |

### 5.58.2	Co-existence studies

Co-existence studies from lower order DCs show that the IMD3 of the dual UL of DC_66_n7 may fall into the DL of band 5 while there is no own Rx impact of the 3rd band from dual UL of DC_7_n5.

### 5.58.3	∆TIB and ∆RIB values

For DC_5-66_n7, the TIB,c and RIB,c values are reused from the LTE combination CA_5-7-66, and are given in the tables below.

Table 5.58.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66-n7 | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.58.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66-n7 | 5 | 0 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.58.4	REFSENS requirements

The MSD values for own Rx impact of the 3rd band stated in 5.58.2 are shown in the following table.

Table 5.58.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-66A_n7A DC_5A-66A-66A_n7A | 5 | 835 | 5 | 25 | 880 | 18.0 | FDD | IMD3 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |

5.59	DC_20-32_n1

### 5.59.1	Configurations for DC

Table 5.59.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_20A-32A_n1A | DC_20A_n1A |

### 5.59.2	Co-existence studies

Co-existence analysis from DC_20_n1 shows that IMD5 of the dual UL may fall into the DL of band 32.

### 5.59.3	∆TIB and ∆RIB values

For DC_20-32_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-20-32, and are given in the tables below.

Table 5.59.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-32-n1 | 20 | 0.3 |
|  | n1 | 0.5 |

Table 5.59.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-32-n1 | 20 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

### 5.59.4	REFSENS requirements

Table 5.59.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-32A_n1A | n1 | 1950.5 | 5 | 50 | 2140.5 | N/A | N/A |
|  | 20 | 852.5 | 5 | 25 | 811.5 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1459.5 | 4.0 | IMD5 |

5.60	DC_20-32_n3

### 5.60.1	Configurations for DC

Table 5.60.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_20A-32A_n3A | DC_20A_n3A |

### 5.60.2	Co-existence studies

Co-existence analysis from DC_20_n3 shows that no IMD impact on the DL of band 32 from the dual UL.

### 5.60.3	∆TIB and ∆RIB values

For DC_20-32_n3, the TIB,c and RIB,c values are reused from the LTE combination CA_3-20-32, and are given in the tables below.

Table 5.60.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-32-n3 | 20 | 0.3 |
|  | n3 | 0.5 |

Table 5.60.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-32-n3 | 20 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

### 5.60.4	REFSENS requirements

No additional MSD requirement is needed.

5.61	DC_1-3_n3

5.61.1	Configuration for DC

Table 5.61.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n3A | DC_1A_n3A DC_3A_n3A2 |
| NOTE 2: Only single switched UL is supported |  |

### 5.61.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes. And there is no IMD issue for this DC configuration.

5.61.3	∆TIB and ∆RIB values

For DC_1-3_n3, the TIB,c and RIB,c values are given in the tables below. Numbers come from DC_1_n3.

Table 5.61.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n3 | 0.3 |

Table 5.61.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3 | 1 | 0 |
|  | 3 | 0 |
|  | n3 | 0 |

5.61.4	REFSENS requirements

No additional MSD requirements need to be defined.

5.62	DC_1-41_n3

5.62.1	Configuration for DC

Table 5.62.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_1A-41A_n3A DC_1A-41C_n3A | DC_1A_n3A |

### 5.62.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes. And it can be get:

- IMD5 of band 1 UL and band n3 UL falling to band 41 DL.

The MSD could reuse the value for CA_1A-3A-41A.

5.62.3	∆TIB and ∆RIB values

For DC_1-41_n3, the TIB,c and RIB,c values have been defined in TS 38.101-3..

5.62.4	REFSENS requirements

Table 5.62.4-1 shows the required MSD levels for the DC configuration, its value can reuse the value for CA_1A-3A-41A.

Table 5.62.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-41A_n3A DC_1A-41C_n3A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 ¦3*fB1-2*fn3¦ |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |

5.63	DC_3-18_n3

5.63.1	Configuration for DC

Table 5.63.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_3A-18A_n3A | DC_3A_n3A2 DC_18A_n3A |
| NOTE 2: Only single switched UL is supported |  |

### 5.63.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes. And it can be get:

- IMD4 of band 18 UL and band n3 UL falling to band 3 DL.

The MSD could reuse the value for DC_18A_n3A.

5.63.3	∆TIB and ∆RIB values

For DC_3-18_n3, the TIB,c and RIB,c values are given in the tables below. Numbers come from DC_18_n3.

Table 5.63.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3 | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n3 | 0.3 |

Table 5.63.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-18_n3 | 3 | 0 |
|  | 18 | 0 |
|  | n3 | 0 |

5.63.4	REFSENS requirements

Table 5.63.4-1 shows the required MSD levels for the DC configuration, its value can reuse the value for DC 18A_n3A.

Table 5.63.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-18A_n3A | 3 | 1719 | 5 | 25 | 1814 | 4 | IMD4 ¦2*fn3-2*fB18¦ |
|  | 18 | 823 | 5 | 25 | 868 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |

5.64	DC_3-41_n3

5.64.1	Configuration for DC

Table 5.64.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_3A-41A_n3A DC_3A-41C_n3A | DC_3A_n3A2 DC_41A_n3A DC_41C_n3A |
| NOTE 2: Only single switched UL is supported |  |

### 5.64.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes. And it can be get:

- IMD4 of band 41 UL and band n3 UL falling to band 3 DL.

The MSD could reuse the value for DC_41A_n3A.

5.64.3	∆TIB and ∆RIB values

For DC_3-41_n3, the TIB,c and RIB,c values are given in the tables below. Numbers come from DC_41_n3.

Table 5.64.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3 | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.64.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3 | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

5.64.4	REFSENS requirements

Table 5.64.4-1 shows the required MSD levels for the DC configuration, its value can reuse the value of DC_3A_n41A.

Table 5.64.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-41A_n3A DC_3A-41C_n3A | 3 | 1770 | 5 | 25 | 1865 | 8.2 | IMD4 ¦2*fB41-2*fn3¦ |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |

5.65	Void

5.66	DC_7-66_n77 and DC_7-7-66_n77

5.66.1	Configuration for DC

Table 5.66.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_7A-66A_n77A DC_7A-7A-66A_n77A DC_7A-7A-66A_n77(2A) DC_7A-66A_n77(2A) DC_7C-66A_n77A DC_7C-66A_n77(2A) | DC_7A_n77A DC_66A_n77A |

### 5.66.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes. And it can be get:

- IMD4 of band 7 UL and band n77 UL falling to band 66 DL.

- IMD5 of band 66 UL and band n77 UL falling to band 7 DL.

The MSD could reuse the value for CA_3A-7A-26A.

5.66.3	∆TIB and ∆RIB values

For DC_7-66_n77 and DC_7-7-66_n77, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_5A-7A-66A.

Table 5.66.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n77 DC_7-7-66_n77 | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.66.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n77 DC_7-7-66_n77 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n77 | 0.5 |

5.66.4	REFSENS requirements

Table 5.66.4-1 shows the required MSD levels for the DC configuration.

The MSD levels are derived from the text proposal in R4-2014129 and R4-2015711

Table 5.66.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-66A_n77A DC_7C-66A_n77A DC_7A-7A-66A_n77A DC_7A-66A_n77(2A) DC_7C-66A_n77(2A) DC_7A-7A-66A_n77(2A) | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 8.7 | IMD4 |
|  | n77 | 3625 | 10 | 50 | 3475 | N/A | N/A |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | 7 | 2550 | 5 | 25 | 2670 | 5.2 | IMD5 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | 3.4 | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |

5.67	DC_2-5_n48

5.67.1	Configuration for DC

Table 5.67.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_2A-5A_n48A DC_2A-5A_n48B | DC_2A_n48A DC_5A_n48A |

### 5.67.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be got:

- The IMD3 of UL configuration on DC_5_n48 will impact of the Band 2 DL.

5.67.3	∆TIB and ∆RIB values

For DC_2-5_n48, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_2-5-48.

Table 5.67.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n48 | 2 | 0.6 |
|  | 5 | 0.3 |
|  | n48 | 0.8 |

Table 5.67.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n48 | 2 | 0.2 |
|  | 5 | 0 |
|  | n48 | 0.5 |

5.67.4	REFSENS requirements

Table 5.67.4-1 shows the required MSD levels for the DC configuration.

Table 5.67.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-5A_n48A DC_2A-5A_n48B | 2 | 1882 | 5 | 25 | 1962 | 15.6 | IMD3 ¦ fn48-2*fB5¦ |
|  | 5 | 839 | 5 | 25 | 884 | N/A | N/A |
|  | n48 | 3640 | 5 | 25 | 3640 | N/A | N/A |

5.68	DC_2-13_n48

5.68.1	Configuration for DC

Table 5.68.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_2A-13A_n48A DC_2A-13A_n48B | DC_2A_n48A DC_13A_n48A |

### 5.68.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be got:

- The IMD3 of UL configuration on DC_13_n48 will impact of the Band 2 DL.

5.68.3	∆TIB and ∆RIB values

For DC_2-13_n48, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_2-13-48.

Table 5.68.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n48 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | n48 | 0.8 |

Table 5.68.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n48 | 2 | 0.2 |
|  | 13 | 0 |
|  | n48 | 0.5 |

5.68.4	REFSENS requirements

Table 5.68.4-1 shows the required MSD levels for the DC configuration.

Table 5.68.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-13A_n48A DC_2A-13A_n48B | 2 | 1903.5 | 5 | 25 | 1983.5 | 15.6 | IMD3 ¦ fn48-2*fB13¦ |
|  | 13 | 784.5 | 5 | 25 | 753.5 | N/A | N/A |
|  | n48 | 3552.5 | 5 | 25 | 3552.5 | N/A | N/A |

5.69	Void

5.70	DC_5-46_n66

5.70.1	Configuration for DC

Table 5.70.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_5A-46A_n66A | DC_5A_n66A DC_46A_n66A |

### 5.70.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be got:

- The IMD4 and IMD5 of UL configuration on DC_5_n66 will impact of the Band 46 DL

5.70.3	∆TIB and ∆RIB values

For DC_5-46_n66, the TIB,c and RIB,c values are given in the tables below that reused the values for DC_46-66_n5.

Table 5.70.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-46_n66 | 5 | 0.3 |
|  | 46 | 0 |
|  | n66 | 0.3 |

Table 5.70.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-46_n66 | 5 | 0 |
|  | 46 | 0 |
|  | n66 | 0 |

5.70.4	REFSENS requirements

Table 5.70.4-1 shows the required MSD levels for the DC configuration.

Table 5.70.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-46A_n66A | 5 | 847 | 5 | 25 | 892 | N/A | N/A |
|  | 46 | 5163 | 10 | 50 | 5163 | 9.01 | IMD4 ¦2*fB5+2*fn66¦ |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |

5.71	DC_5-66_n48 and DC_5-66-66_n48

5.71.1	Configuration for DC

Table 5.71.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_5A-66A_n48A DC_5A-66A_n48B DC_5A-66A-66A_n48A DC_5A-66A-66A_n48B | DC_5A_n48A DC_66A_n48A |

### 5.71.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. There is no IMD impact to their own third band RX.

5.71.3	∆TIB and ∆RIB values

For DC_5-66_n48, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_5-66-48.

Table 5.71.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n48 DC_5-66-66_n48 | 5 | 0.3 |
|  | 66 | 0.6 |
|  | n48 | 0.8 |

Table 5.71.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n48 DC_5-66-66_n48 | 5 | 0 |
|  | 66 | 0.2 |
|  | n48 | 0.5 |

5.71.4	REFSENS requirements

There is no additional MSD requirement for this band combination.

5.72	DC_5-66_n77

5.72.1	Configuration for DC

Table 5.72.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_5A-66A_n77A | DC_5A_n77A DC_66A_n77A |

### 5.72.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be got:

- The IMD3 of UL DC_5A_n77A may impact to band 66 Rx.

5.72.3	∆TIB and ∆RIB values

For DC_5-66_n77, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_5-66_n78.

Table 5.72.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n77 | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.72.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n77 | 5 | 0.2 |
|  | 66 | 0.2 |
|  | n77 | 0.5 |

5.72.4	REFSENS requirements

Table 5.72.4-1 shows the required MSD levels for the DC configuration.

Table 5.72.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-66A_n77A | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 66 | 1742 | 5 | 25 | 2142 | 13.2 | IMD3 ¦fn77-2*fB5¦ |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |

5.73	DC_1-42_n3

### 5.73.1	Configurations for DC_1-42_n3

Table 5.73.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-42A_n3A | DC_1A_n3A DC_42A_n3A |
| DC_1A-42C_n3A | DC_1A_n3A DC_42A_n3A DC_42C_n3A |

### 5.73.2	Co-existence studies

When Uplink EN-DC configuration is DC_1A_n3A, IMD4 of (B1 - n3) will fall into Rx band of Band 42.

### 5.73.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.73.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-42_n3 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |

Table 5.73.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-42_n3 | 1 | 0 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |

### 5.73.4	Reference sensitivity exceptions

As mentioned above, IMD4 of B1 and n3 to Band42 Rx needs to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.73.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A-42A_n3A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | FDD | N/A |
|  | 42 | 3425 | 5 | 25 | 3425 | 13.0 | TDD | IMD4 |

5.74	DC_8-42_n3

### 5.74.1	Configurations for DC_8-42_n3

Table 5.74.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-42A_n3A | DC_8A_n3A DC_42A_n3A |
| DC_8A-42C_n3A | DC_8A_n3A DC_42A_n3A DC_42C_n3A |

### 5.74.2	Co-existence studies

When Uplink EN-DC configuration is DC_8A_n3A, IMD3 and IMD5 of (B8 - n3) will fall into Rx band of Band 42.

### 5.74.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.74.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-42_n3 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |

Table 5.74.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-42_n3 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |

### 5.74.4	Reference sensitivity exceptions

As mentioned above, IMD3 of B8 and n3 to Band42 Rx needs to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.74.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-42A_n3A | 8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | FDD | N/A |
|  | 42 | 3540 | 5 | 25 | 3540 | 16.3 | TDD | IMD3 |

5.75	DC_3-18_n28

5.75.1	Configurations for DC

Table 5.75.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-18A_n28A | DC_3A_n28A DC_18A_n28A |

5.75.2	Co-existence studies

Based on co-existence studies of DC_3_n28 and DC_18_n28,  there is no the 3rd band issue.

5.75.3	∆TIB and ∆RIB values

For DC_3-18_n28, the TIB,c and RIB,c values are given in the tables below.

Table 5.75.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28 | 3 | 0.3 |
|  | 18 | 0.5 |
|  | n28 | 0.3 |

Table 5.75.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-18_n28 | 3 | 0 |
|  | 18 | 0 |
|  | n28 | 0 |

5.75.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

5.76		DC_2-66_n77

### 5.76.1	 Operating bands for DC

Table 5.76.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-66A_n77A | DC_2A_n77A DC_66A_n77A |

### 5.76.2	 Co-existence studies

For UE coexistence study of Band 2 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.76.2-1.

Table 5.76.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1390 | 2350 | 5150 | 6110 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | -500 | 520 | 4690 | 6550 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 7000 | 8020 | 8450 | 10310 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1350 | 2430 | 7990 | 10750 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8850 | 9930 | 11750 | 14510 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | -4700 | -2780 | 10300 | 12220 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | -14950 | -11290 | -4340 | -3200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 18710 | 10700 | 11840 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | -8900 | -6080 | 870 | 2850 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 16420 | 12150 | 14130 |

For UE coexistence study of Band 66 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.76.2-2.

Table 5.76.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1520 | 2490 | 5010 | 5980 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 780 | 260 | 4820 | 6690 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6720 | 7760 | 8310 | 10180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 930 | 2040 | 8120 | 10890 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8430 | 9540 | 11610 | 14380 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 4980 | 3040 | 10020 | 11960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 | 11420 | 3820 | 2640 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 18580 | 10140 | 11320 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 | 6340 | 1260 | 3270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 16160 | 11730 | 13740 |

The Rx impacts can be identified as below,

- 2nd, 4th and 5th order IMD products generated by DC_2_n77 uplink may fall into own Rx of band 66

- 2nd, 4th and 5th order IMD products generated by DC_66_n77 uplink may fall into own Rx of band 2

### 5.76.3	 ∆TIB and ∆RIB values

Table 5.76.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n77 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.76.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n77 | 2 | 0.2 |
|  | 66 | 0.2 |
|  | n77 | 0.5 |

5.76.4		Reference sensitivity exceptions

As stated in 5.76.2, for MSD requirement caused by IMDs is specified below accordingly.

Table 5.76.4-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-66A_n77A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | 66 | 1765 | 5 | 25 | 2185 | 29.2 | IMD2 |
|  | n77 | 4040 | 5 | 25 | 4040 | N/A | N/A |
|  | 2 | 1905 | 5 | 25 | 1985 | M/A | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | 10.4 | IMD4 |
|  | n77 | 3595 | 5 | 25 | 3595 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | M/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2195 | 4.0 | IMD5 |
|  | n77 | 3925 | 5 | 25 | 3925 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3700 | 5 | 25 | 3700 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | 9.1 | IMD4 |
|  | 66 | 1775 | 5 | 25 | 2195 | N/A | N/A |
|  | n77 | 3385 | 5 | 25 | 3385 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 4.2 | IMD5 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n77 | 3645 | 5 | 25 | 3645 | N/A | N/A |

5.77		DC_2-48_n77

### 5.77.1	 Operating bands for DC

Table 5.77.1-1: Inter-band DC configuration (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-48A_n77A | DC_2A_n77A DC_48A_n77A |

### 5.77.2	 Co-existence studies

For UE coexistence study of Band 2 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.77.2-1.

Table 5.77.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1390 | 2350 | 5150 | 6110 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 500 | 520 | 4690 | 6550 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 7000 | 8020 | 8450 | 10310 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1350 | 2430 | 7990 | 10750 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8850 | 9930 | 11750 | 14510 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 4700 | 2780 | 10300 | 12220 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14950 | 11290 | 4340 | 3200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 18710 | 10700 | 11840 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 6080 | 870 | 2850 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 16420 | 12150 | 14130 |

For UE coexistence study of Band 48 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.77.2-2.

Table 5.77.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3550 | 3700 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 14200 | 14800 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 17750 | 18500 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 400 | 650 | 6850 | 7900 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2900 | 4100 | 2900 | 4850 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 10400 | 11600 | 10150 | 12100 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 6450 | 7800 | 6200 | 9050 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 13950 | 15300 | 13450 | 16300 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1300 | 800 | 13700 | 15800 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 9500 | 11500 | 10000 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16750 | 20500 | 17500 | 19000 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5500 | 2500 | 4500 | 2250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17000 | 20000 | 17250 | 19500 |

The Rx impacts can be identified as below,

- There is no IMD issue for Rx of band 2 with UL DC_48_n77.

- 5th order harmonics generated by DC_2_n77 uplink may fall into own Rx of band 48.

- As the duplex mode for band 48 and n77 is TDD and both bands shall be synchronized to align uplink and downlink timing across and within the two bands for the U.S. C-band (R4-2008118), thus there is no impact to this TDD band 48 with the UL DC_2_n77.

### 5.77.3	 ∆TIB and ∆RIB values

Table 5.77.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n77 | 2 | 0.3 |
|  | 48 | 0.6 |
|  | n77 | 0.5 |

Table 5.77.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-48_n77 | 2 | 0 |
|  | 48 | 0.2 |
|  | n77 | 0.1 |

5.77.4	Reference sensitivity exceptions

There is no reference sensitivity exception (MSD) for DC_2-48_n77 combination.

5.78		DC_2-13_n77

### 5.78.1	 Operating bands for DC

Table 5.78.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-13A_n77A | DC_2A_n77A DC_13A_n77A |

### 5.78.2	 Co-existence studies

For UE coexistence study of Band 2 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.78.2-1.

Table 5.78.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1390 | 2350 | 5150 | 6110 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 500 | 520 | 4690 | 6550 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 7000 | 8020 | 8450 | 10310 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1350 | 2430 | 7990 | 10750 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8850 | 9930 | 11750 | 14510 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 4700 | 2780 | 10300 | 12220 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14950 | 11290 | 4340 | 3200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 18710 | 10700 | 11840 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 6080 | 870 | 2850 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 16420 | 12150 | 14130 |

For UE coexistence study of Band 13 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.78.2-2.

Table 5.78.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3108 | 3148 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3885 | 3935 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 2513 | 3423 | 4077 | 4987 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2646 | 1726 | 5813 | 7623 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4854 | 5774 | 7377 | 9187 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1869 | 939 | 9113 | 11823 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 5631 | 6561 | 10677 | 13387 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 6846 | 5026 | 8154 | 9974 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16023 | 12413 | 152 | 1092 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13977 | 17587 | 6408 | 7348 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 11046 | 8326 | 4239 | 6069 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11454 | 14174 | 8931 | 10761 |

The Rx impacts can be identified as below,

- There is no IMD issue for Rx of band 13 with UL DC_2_n77.

- 3rd order IMD products generated by DC_13_n77 uplink may fall into own Rx of band 2

### 5.78.3	 ∆TIB and ∆RIB values

Table 5.78.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n77 | 2 | 0.6 |
|  | 13 | 0.5 |
|  | n77 | 0.8 |

Table 5.78.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n77 | 2 | 0.2 |
|  | 13 | 0.2 |
|  | n77 | 0.5 |

5.78.4	Reference sensitivity exceptions

As stated in 5.78.2, for MSD requirement caused by IMDs is specified below accordingly.

Table 5.78.4-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-13A_n77A | 2 | 1864 | 5 | 25 | 1944 | 16.0 | IMD3 |
|  | 13 | 783 | 5 | 25 | 752 | N/A | N/A |
|  | n77 | 3510 | 5 | 25 | 3510 | N/A | N/A |

5.79		DC_2-5_n77

### 5.79.1	 Operating bands for DC

Table 5.79.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-5A_n77A | DC_2A_n77A DC_5A_n77A |

### 5.79.2	 Co-existence studies

For UE coexistence study of Band 2 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.79.2-1.

Table 5.79.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1390 | 2350 | 5150 | 6110 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 500 | 520 | 4690 | 6550 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 7000 | 8020 | 8450 | 10310 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1350 | 2430 | 7990 | 10750 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8850 | 9930 | 11750 | 14510 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 4700 | 2780 | 10300 | 12220 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14950 | 11290 | 4340 | 3200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 18710 | 10700 | 11840 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 6080 | 870 | 2850 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 16420 | 12150 | 14130 |

For UE coexistence study of Band 5 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.79.2-2.

Table 5.79.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 2451 | 3376 | 4124 | 5049 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2552 | 1602 | 5751 | 7576 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4948 | 5898 | 7424 | 9249 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1728 | 753 | 9051 | 11776 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 5772 | 6747 | 10724 | 13449 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 6752 | 4902 | 8248 | 10098 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15976 | 12351 | 96 | 904 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14024 | 17649 | 6596 | 7596 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10952 | 8202 | 4053 | 5928 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11548 | 14298 | 9072 | 10947 |

The Rx impacts can be identified as below..

- 5th order IMD products generated by DC_2_n77 uplink may fall into own Rx of band 5.

- 3rd order IMD products generated by DC_5_n77 uplink may fall into own Rx of band 2.

### 5.79.3	 ∆TIB and ∆RIB values

Table 5.79.3-1: ΔTIB,

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | n77 | 0.8 |

Table 5.79.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n77 | 2 | 0.2 |
|  | 5 | 0.2 |
|  | n77 | 0.5 |

5.79.4	Reference sensitivity exceptions

As stated in 5.79.2, for MSD requirement caused by IMDs is specified below accordingly.

Table 5.79.5-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-5A_n77A | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | 5 | 842.5 | 5 | 25 | 887.5 | 3.8 | IMD5 |
|  | n77 | 3305 | 5 | 25 | 3305 | N/A | N/A |
|  | 2 | 1907 | 5 | 25 | 1987 | 16.5 | IMD3 |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n77 | 3680 | 5 | 25 | 3680 | N/A | N/A |

5.80		DC_5-13_n66

### 5.80.1	 Operating bands for DC

Table 5.80.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_5A-13A_n66A | DC_5A_n66A DC_13A_n66A |

### 5.80.2	 Co-existence studies

For UE coexistence study of Band 5 + Band n66, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.80.2-1.

Table 5.80.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 8550 | 8900 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 861 | 956 | 2534 | 2629 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 132 | 12 | 2571 | 2736 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3358 | 3478 | 4244 | 4409 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 692 | 837 | 4281 | 4516 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4182 | 4327 | 5954 | 6189 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1912 | 1722 | 5068 | 5258 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6296 | 5991 | 1686 | 1516 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 | 7969 | 5006 | 5176 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3692 | 3432 | 873 | 1088 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 | 7038 | 5892 | 6107 |

For UE coexistence study of Band 13 + Band n66, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.80.2-2.

Table 5.80.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3108 | 3148 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3885 | 3935 | 8550 | 8900 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 923 | 1003 | 2487 | 2567 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 226 | 136 | 2633 | 2783 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3264 | 3354 | 4197 | 4347 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 551 | 651 | 4343 | 4563 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4041 | 4141 | 5907 | 6127 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 2006 | 1846 | 4974 | 5134 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6343 | 6053 | 1438 | 1328 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7617 | 7907 | 4818 | 4928 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3786 | 3556 | 1059 | 1229 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6684 | 6914 | 5751 | 5921 |

The Rx impacts can be identified as below..

- 4th order IMD products generated by DC_5_n66 uplink may fall into own Rx of band 13.

- There is no IMD issue for Rx of band 5 with UL DC_13_n66.

### 5.80.3	 ∆TIB and ∆RIB values

Table 5.80.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-13_n66 | 5 | 0.3 |
|  | 13 | 0.3 |
|  | n66 | 0.3 |

Table 5.80.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-13_n66 | 5 | 0 |
|  | 13 | 0 |
|  | n66 | 0 |

5.80.4	Reference sensitivity exceptions

As stated in 5.80.2, the requirement caused by IMDs is specified below accordingly.

Table 5.80.4-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-13A_n66A | 5 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | 13 | 781 | 5 | 25 | 750 | 9.4 | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | N/A |

5.81		DC_13-66_n77

### 5.81.1	 Operating bands for DC

Table 5.81.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_13A-66A_n77A | DC_13A_n77A DC_66A_n77A |

### 5.81.2	 Co-existence studies

For UE coexistence study of Band 13 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.81.2-1.

Table 5.81.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3108 | 3148 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3885 | 3935 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 2513 | 3423 | 4077 | 4987 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2646 | 1726 | 5813 | 7623 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4854 | 5774 | 7377 | 9187 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1869 | 939 | 9113 | 11823 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 5631 | 6561 | 10677 | 13387 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 6846 | 5026 | 8154 | 9974 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16023 | 12413 | 152 | 1092 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13977 | 17587 | 6408 | 7348 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 11046 | 8326 | 4239 | 6069 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11454 | 14174 | 8931 | 10761 |

For UE coexistence study of Band 66 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.81.2-2.

Table 5.81.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1520 | 2490 | 5010 | 5980 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 780 | 260 | 4820 | 6690 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6720 | 7760 | 8310 | 10180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 930 | 2040 | 8120 | 10890 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8430 | 9540 | 11610 | 14380 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 4980 | 3040 | 10020 | 11960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 | 11420 | 3820 | 2640 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 18580 | 10140 | 11320 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 | 6340 | 1260 | 3270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 16160 | 11730 | 13740 |

The Rx impacts can be identified as below..

- 3rd order IMD products generated by DC_13_n77 uplink may fall into own Rx of band 66.

- 3rd order IMD products generated by DC_66_n77 uplink may fall into own Rx of band 13

### 5.81.3	 ∆TIB and ∆RIB values

Table 5.81.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n77 | 13 | 0.5 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.81.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-66_n77 | 13 | 0.3 |
|  | 66 | 0.3 |
|  | n77 | 0.5 |

### 5.81.4	Reference sensitivity exceptions

As stated in 5.81.2, for MSD requirement caused by IMDs is specified below accordingly.

Table 5.81.4-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A-66A_n77A | 13 | 777 | 5 | 25 | 746 | N/A | N/A |
|  | 66 | 1746 | 5 | 25 | 2146 | 17.1 | IMD3 |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | N/A |
|  | 13 | 781 | 5 | 25 | 750 | 15.2 | IMD3 |
|  | 66 | 1710 | 5 | 25 | 2110 | N/A | N/A |
|  | n77 | 4170 | 10 | 50 | 4170 | N/A | N/A |

5.82		DC_13-66_n5

### 5.82.1	 Operating bands for DC

Table 5.82.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_13A-66A_n5A | DC_13A_n5A DC_66A_n5A |

### 5.82.2	 Co-existence studies

For UE coexistence study of Band 13 + Band n5, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.82.2-1.

Table 5.82.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 2472 | 2547 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3108 | 3148 | 3296 | 3396 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3885 | 3935 | 4120 | 4245 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 37 | 72 | 1601 | 1636 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 705 | 750 | 861 | 921 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 2378 | 2423 | 2425 | 2485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1482 | 1537 | 1685 | 1770 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 3155 | 3210 | 3249 | 3334 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 144 | 74 | 3202 | 3272 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2619 | 2509 | 2324 | 2259 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4073 | 4183 | 3932 | 3997 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 993 | 898 | 713 | 633 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4026 | 4121 | 3979 | 4059 |

For UE coexistence study of Band 66 + Band n5, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.82.2-2.

Table 5.82.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 2472 | 2547 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 3296 | 3396 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 4120 | 4245 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 956 | 861 | 2534 | 2629 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2571 | 2736 | 132 | 12 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4244 | 4409 | 3358 | 3478 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 4281 | 4516 | 692 | 837 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 5954 | 6189 | 4182 | 4327 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1722 | 1912 | 5068 | 5258 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1686 | 1516 | 6296 | 5991 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5006 | 5176 | 7664 | 7969 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 873 | 1088 | 3692 | 3432 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5892 | 6107 | 6778 | 7038 |

The Rx impacts can be identified as below..

- There is no IMD issue for Rx of band 66 with UL DC_13_n5.

- 4th order IMD products generated by DC_66_n77 uplink may fall into own Rx of band 13.

### 5.82.3	 ∆TIB and ∆RIB values

Table 5.82.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n5 | 13 | 0.5 |
|  | 66 | 0.3 |
|  | n5 | 0.5 |

Table 5.82.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-66_n5 | 13 | 0 |
|  | 66 | 0 |
|  | n5 | 0 |

5.82.4	Reference sensitivity exceptions

As stated in 5.82.2, the requirement caused by IMDs is specified below accordingly.

Table 5.82.5-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A-66A_n5A | 13 | 781 | 5 | 25 | 750 | 9.4 | IMD4 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |

5.83	DC_1-18_n28

5.83.1	Configurations for DC

Table 5.83.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-18A_n28A | DC_1A_n28A DC_18A_n28A |

5.83.2	Co-existence studies

Based on co-existence studies of DC_1_n28 and DC_18_n28, 5th order IMD generated by dual uplink of Band 18 + Band n28 may fall into own Rx of band 1. The MSD value of this configuration can follow CA_1A-18A-28A with UL CA_18A-28A case. Since band 18 is only used by KDDI, no additional MSD requirement need to be specified for this configuration considering KDDI spectrum (band n28 UL is limited to 718MHz – 728MHz).

5.83.3	∆TIB and ∆RIB values

For DC_1-18_n28, the TIB,c and RIB,c values are given in the tables below.

Table 5.83.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | n28 | 0.5 |

Table 5.83.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-18_n28 | 1 | 0 |
|  | 18 | 0 |
|  | n28 | 0 |

5.83.4	Reference sensitivity exceptions

As stated in 5.83.2, there is no additional MSD requirement for this configuration.

5.84	DC_1-18_n41

5.84.1	Configurations for DC

Table 5.84.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-18A_n41A | DC_1A_n41A DC_18A_n41A |

5.84.2	Co-existence studies

Based on co-existence studies of DC_1_n41 and DC_18_n41, 5th order IMD generated by dual uplink of Band 1 + Band n41 may fall into own Rx of band 18. The MSD value of this configuration can follow CA_1A-18A-41A with UL CA_1A-41A case. Since band 18 is only used by KDDI, no additional MSD requirement need to be specified for this configuration considering KDDI spectrum (band n41 is limited to 2595MHz – 2645MHz).

5.84.3	∆TIB and ∆RIB values

For DC_1-18_n41, the TIB,c and RIB,c values are given in the tables below.

Table 5.84.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n41 | 1 | 0.5 |
|  | 18 | 0.3 |
|  | n41 | 0.5 |

Table 5.84.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-18_n41 | 1 | 0 |
|  | 18 | 0 |
|  | n41 | 0 |

5.84.4	Reference sensitivity exceptions

As stated in 5.84.2, there is no additional MSD requirement for this configuration.

5.85	DC_5-7_n7

### 5.85.1	Configurations for DC

Table 5.85.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_5A-7A_n7A | DC_5A_n7A DC_7A_n7A2 |
| NOTE 2: Only single switched UL is supported |  |

5.85.2	Co-existence studies

Co-existence studies have been performed for lower order combinations.

Co-existence analysis for DC_5_n7 UL shows that 3rd and 5th IMD may fall in DL band 5.

Co-existence analysis for DC_7_n7 UL is not needed since only switched UL is supported.

### 5.85.3	∆TIB and ∆RIB values

Table 5.85.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_5-7_n7 | 5 | 0.5 |
|  | 7 | 0.3 |
|  | n7 | 0.3 |

Table 5.85.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_5-7_n7 | 5 | 0 |
|  | 7 | 0 |
|  | n7 | 0 |

### 5.85.4	Reference sensitivity exceptions

Based on co-existence studies additional MSD is needed defined in Table 7.3B.2.3.5.2-1 of 38.101-3.

Table 6. x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-7A_n7A | 5 | 834 | 5 | 25 | 879 | 12 | IMD34 |
|  | 7 | 2527 | 10 | 50 | 2647 | N/A | N/A |
|  | n7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

5.86	DC_2-28_n7

### 5.86.1	Configurations for DC

Table 5.86.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_2A-28A_n7A | DC_2A_n7A DC_28A_n7A |

5.86.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. of DC_2A_n7A and DC_28A_n7A, where:

- No IMD product caused by DC_2A_n7A fall into own Rx of band 28.

- No IMD product caused by DC_28A_n7A fall into own Rx of band 2.

### 5.86.3	∆TIB and ∆RIB values

Table 5.86.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-28_n7 | 2 | 0.5 |
|  | 28 | 0.3 |
|  | n7 | 0.5 |

Table 5.86.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-28_n7 | 2 | 0 |
|  | 28 | 0 |
|  | n7 | 0 |

### 5.86.4	Reference sensitivity exceptions

No further REFSENS exceptions needed.

5.87	DC_28-66_n7

### 5.87.1	Configurations for DC

Table 5.87.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_28A-66A_n7A | DC_28A_n7A DC_66A_n7A |

5.87.2	Co-existence studies

Co-existence studies have been performed for lower order combinations of DC_28A_n7A and DC_66A_n7A, where:

- No IMD product caused by DC_28A_n7A fall into own Rx of band 66.

- 2nd IMD product caused by DC_66A_n7A may fall into own Rx of band 28.

### 5.87.3	∆TIB and ∆RIB values

### Values are reused from CA including same bands as given in 36.101.

Table 5.87.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_28-66_n7 | 28 | 0.6 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.87.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_28-66_n7 | 28 | 0.2 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.87.4	Reference sensitivity exceptions

Based on co-existence analysis it is found that MSD is needed due to 2nd IMD falling into own Rx band of 28. Therefor additional MSD is needed defined in Table 7.3B.2.3.5.2-1 of 38.101-3.

Table 5. x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A-66A_n7A | 28 | 735 | 5 | 25 | 790 | 27.6 | IMD2 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n7 | 2505 | 5 | 50 | 2625 | N/A | N/A |

5.88	DC_7-28_n2

### 5.88.1	Configurations for DC

Table 5.88.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_7A-28A_n2A | DC_7A_n2A DC_28A_n2A |

5.88.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. of DC_7A_n2A and DC_28A_n2A, where:

- No IMD product caused by DC_7A_n2A fall into own Rx of band 28.

- IMD2 product caused by DC_28A_n2A may fall into own Rx of band 7.

### 5.88.3	∆TIB and ∆RIB values

Table 5.88.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_7-28_n2 | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n2 | 0.5 |

Table 5.88.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-28_n2 | 7 | 0 |
|  | 28 | 0 |
|  | n2 | 0 |

### 5.88.4	Reference sensitivity exceptions

Based on co-existence studies additional MSD is needed defined in Table 7.3B.2.3.5.2-1 of 38.101-3.

Table 5. x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-28A_n2A | 7 | 2510 | 10 | 50 | 2630 | 27.6 | IMD2 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |

5.89	DC_2-7_n7

### 5.89.1	Configurations for DC

Table 5.89.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_2A-7A_n7A | DC_2A_n7A DC_7A_n7A2 |
| NOTE 2: Only single switched UL is supported |  |

5.89.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. No further analysis is needed.

### 5.89.3	∆TIB and ∆RIB values

Table 5.89.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-7_n7 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n7 | 0.5 |

Table 5.89.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-7_n7 | 2 | 0 |
|  | 7 | 0 |
|  | n7 | 0 |

### 5.89.4	Reference sensitivity exceptions

No further REFSENS exceptions needed.

5.90	DC_2A-71A_n71A

### 5.90.1	Configurations for DC

Table 5.90.1-1: Inter-band EN-DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_2A-71A_n71A | DC_2A_n71A |

Note that DC_71_n71 is not used as uplink configuration.

5.90.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. No further analysis is needed.

### 5.90.3	∆TIB and ∆RIB values

Table 5.90.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-71_n71 | 2 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |

Table 5.90.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-71_n71 | 2 | 0 |
|  | 71 | 0 |
|  | n71 |  |

### 5.90.4	Reference sensitivity exceptions

REFSENS exceptions for DC_2A-71A_n71A due to band 71 uplink harmonic into band 2 is already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

Wgap exception have been defined for DC_71A_n71A, no further MSD is needed.

5.91	DC_66A-71A_n71A

### 5.91.1	Configurations for DC

Table 5.91.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_66A-71A_n71A | DC_66A_n71A |

Note that DC_71_n71 is not used as uplink configuration.

5.91.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. No further analysis is needed.

### 5.91.3	∆TIB and ∆RIB values

Table 5.91.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_66-71_n71 | 66 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |

Table 5.91.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_66-71_n71 | 66 | 0 |
|  | 71 | 0 |
|  | n71 |  |

### 5.91.4	Reference sensitivity exceptions

Wgap exception have been defined for DC_71A_n71A, no further MSD is needed.

5.92	DC_7-66_n7/ DC_7-66-66_n7

### 5.92.1	Configurations for DC

Table 5.92.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_7A-66A_n7A DC_7A-66A-66A_n7A | DC_7A_n7A2 DC_66A_n7A |
| NOTE 2: Only single switched UL is supported |  |

### 5.92.2	Co-existence studies

For UE coexistence study of Band 66 + Band n7, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.46.3-1.

Table 5.92.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 860 | 720 | 4210 | 4350 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3220 | 3430 | 850 | 1060 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6710 | 6920 | 5920 | 6130 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5720 | 6000 | 2560 | 2840 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9210 | 9490 | 7630 | 7910 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1440 | 1720 | 8420 | 8700 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4620 | 4270 | 8570 | 8220 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 340 | 10 | 4290 | 3940 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9340 | 9690 | 11710 | 12060 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10130 | 10480 | 10920 | 11270 |

For UL DC_7A_n7A, there is no need to study the IMD since only single switched UL is supported.

IMD4 may fall into Rx of band 7 with UL DC_66_n7.

### 5.92.3	∆TIB and ∆RIB values

For DC_7-66_n7 and DC_7-66-66_n7, the TIB,c and RIB,c values are reused from the LTE combination DC_66_n7, and are given in the tables below.

Table 5.92.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n7 DC_7-66-66_n7 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.92.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n7 DC_7-66-66_n7 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.92.4	Reference sensitivity exceptions

The reference sensitivity exception (MSD) due to IMD4 for DC_7-66_n7/ DC_7-66-66_n7 with UL DC_66_n7 is specified as below referring to the MSD for DC_66A_n7A from 38.101-3.

Table 5.92.x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-66A_n7A DC_7A-66A-66A_n7A | 7 | 2555 | 10 | 50 | 2675 | 15 | IMD4 |
|  | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2515 | 10 | 50 | 2635 | N/A | N/A |

5.93	DC_2-7_n77

5.93.1	Configurations for DC

Table 5.93.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-7A_n77A DC_2A-7C_n77A DC_2A-7A-7A_n77A DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) DC_2A-7A-7A_n77(2A) | DC_2A_n77A DC_7A_n77A |

5.93.2	Co-existence studies

For UE coexistence study of Band 2 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.93.2-1.

Table 5.93.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3300 | 4200 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6600 | 8400 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 9900 | 12600 | 5550 | 5730 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13200 | 16800 | 7400 | 7640 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 16500 | 21000 | 9250 | 9550 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 2350 | 1390 | 5150 | 6110 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 4690 | 6550 | 500 | 520 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 8450 | 10310 | 7000 | 8020 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 7990 | 10750 | 1350 | 2430 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 11750 | 14510 | 8850 | 9030 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 4700 | 2780 | 10300 | 12220 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 11290 | 14950 | 3200 | 4340 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 15050 | 18710 | 10700 | 11840 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 6080 | 8900 | 2850 | 870 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 12150 | 14130 | 13600 | 16420 |

For UE coexistence study of Band 7 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.93.2-2

Table 5.93.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3300 | 4200 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6600 | 8400 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 9900 | 12600 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13200 | 16800 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 16500 | 21000 | 12500 | 12850 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1700 | 730 | 5800 | 6770 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 4030 | 5900 | 800 | 1840 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 9100 | 10970 | 8300 | 9340 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 7330 | 10100 | 3300 | 4410 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 12400 | 15170 | 10800 | 11010 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3400 | 1460 | 11600 | 13540 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 10630 | 14300 | 5800 | 6980 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 15700 | 19370 | 13300 | 14480 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 4760 | 7600 | 900 | 1110 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 14100 | 16110 | 14900 | 17740 |

Based on co-existence study as presented in the table 5.93.2-1 and 5.93.2-2, own Rx impact of the 3rd band is shown as the followings.

- 4th order IMD generated by dual uplink of Band 7 + Band n77 may also fall into own Rx of band 2

- 5th order IMD generated by dual uplink of Band 2 + Band n77 may also fall into own Rx of band 7

5.93.3	∆TIB and ∆RIB values

Table 5.93.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n77 DC_2-7-7_n77 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | n77 | 0.8 |

Table 5.93.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n77 DC_2-7-7_n77 | 2 | 0.2 |
|  | 7 | 0.5 |
|  | n77 | 0.5 |

5.93.4	Reference sensitivity exceptions

The MSD requirement due to 4th order IMD for DC_2-7_n78 can be reused for DC_2-7_n77.

By adjusting the centre frequency of uplink of Band 2 + Band n77, the MSD requirement due to 5th order IMD for DC_2-7_n77 can be similar as the requirements of DC_3-7_n77 since the source of IMD is the same.

Table 5.93.4-1 lists the MSD required for DC_2-7_n77.

Table 5.93.4-1: MSD for the DC configuration due to IMD issue (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-7A_n77A DC_2A-7C_n77A DC_2A-7A-7A_n77A DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) DC_2A-7A-7A_n77(2A) | 2 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3525 | 10 | 50 | 3475 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | 3.4 | IMD5 |
|  | n77 | 4120 | 10 | 50 | 4120 | N/A | N/A |

5.94	Void

5.95	DC_20-40_n78

### 5.95.1	Operating bands for DC

Table 5.95.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_20-40_n78 | CA_20-40 | n78 | No |

### 5.95.2	Configurations for DC

Table 5.95.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_20A-40A_n78A | DC_20A_n78A DC_40A_n78A | CA_20A-40A | n78 |

### 5.95.3	Co-existence studies

Based on co-existence studies of Band 20 + Band n78 captured in 37.863-01-01 there is no IMD interfering band 40

And based on co-existence studies of Band 40 + Band n78 captured in 37.716-11-11, MSD shall be considered since

-	3rd order IMD generated by dual uplink of the two bands may fall into own Rx of band 20.

### 5.95.4	∆TIB and ∆RIB values

It is proposed to re-use relaxation values from DC_8-40-n78 which is very similar.

Table 5.95.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-40-n78 | 20 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.95.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-40-n78 | 20 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.95.5	REFSENS requirements

It is proposed to re-use IMD3 MSD value from DC_8-40-n78 which is very similar.

Table 5.95.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-40A_n78A | 20 | 856 | 5 | 25 | 815 | 19.8 | IMD3 |
|  | 40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |

5.96	DC_3-18_n41

5.96.1	Configurations for DC

Table 5.96.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-18A_n41A | DC_3A_n41A DC_18A_n41A |

5.96.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes. And it can be get:

- IMD2 of band 18 UL and band n41 UL falling to band 3 DL.

- IMD2 and IMD3 of band 3 UL and band n41 UL falling to band 18 DL.

5.96.3	∆TIB and ∆RIB values

For DC_3-18_n41, the TIB,c and RIB,c values are given in the tables below.

Table 5.96.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n41 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n41 | 0.31 |
|  |  | 0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.96.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-18_n41 | 3 | 0 |
|  | 18 | 0 |
|  | n41 | 01 |
|  |  | 0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

5.96.4	Reference sensitivity exceptions

Table 5.96.4-1 shows the required MSD:

Table 5.96.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3-18_n41 | 18 | 820 | 5 | 25 | 865 | 28.9 | IMD2 |
|  | 3 | 1765 | 5 | 25 | 1860 | N/A | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 19.0 | IMD3 |
|  | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | n41 | 2585 | 5 | 25 | 2585 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 28.8 | IMD2 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | MSD | N/A |

5.97	DC_7-25_n77

### 5.97.1	Operating bands for DC

Table 5.97.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_7A-25A_n77A DC_7A-7A-25A_n77A DC_7C-25A_n77A DC_7C-25A-25A_n77A DC_7A-25A-25A_n77A DC_7A-7A-25A-25A_n77A | DC_7A_n77A DC_25A_n77A |

5.97.2	Co-existence studies

For UE coexistence study of Band 7 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.97.2-1.

Table 5.97.2-1: Harmonic and IMD analysis for DC_7_n77

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 2500 | 2570 | 3300 | 4200 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 5000 | 5140 | 6600 | 8400 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 7500 | 7710 | 9900 | 12600 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 730 | 1700 | 5800 | 6770 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 800 | 1840 | 4030 | 5900 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 8300 | 9340 | 9100 | 10970 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 3300 | 4410 | 7330 | 10100 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 10800 | 11910 | 12400 | 15170 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 3400 | 1460 | 11600 | 13540 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 14300 | 10630 | 6980 | 5800 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15700 | 19370 | 13300 | 14480 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 7600 | 4760 | 1110 | 900 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 14900 | 17740 | 14100 | 16110 |

For UE coexistence study of Band 25 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.97.2-2.

Table 5.97.2-2: Harmonic and IMD analysis for DC_25_n77

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1915 | 3300 | 4200 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3830 | 6600 | 8400 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5745 | 9900 | 12600 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1385 | 2350 | 5150 | 6115 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 500 | 530 | 4685 | 6550 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 7000 | 8030 | 8450 | 10315 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1350 | 2445 | 7985 | 10750 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8850 | 9945 | 11750 | 14515 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 4700 | 2770 | 10300 | 12230 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 14950 | 11285 | 4360 | 3200 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15050 | 18715 | 10700 | 11860 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 8900 | 6070 | 855 | 2850 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13600 | 16430 | 12150 | 14145 |

Based on co-existence study as presented in the table 5.97.2-1 and 5.97.2-2, own Rx impact is shown in the following.

- The 4th order IMD generated by dual uplink of Band 7 + Band n77 may fall into own Rx of band 25 and n77

- The 2nd harmonic of Band 25 uplink may fall into own Rx of band n77

- The 2nd, 4th and 5th order IMD generated by dual uplink of Band 25 + Band n77 may fall into own Rx of band 25

- The 5th order IMD generated by dual uplink of Band 25 + Band n77 may fall into own Rx of band 7

- The 4th and 5th order IMD generated by dual uplink of Band 25 + Band n77 may fall into own Rx of Band n77

5.97.3	∆TIB and ∆RIB values

The same relaxation values as DC_2-7_n77 is used.

Table 5.97.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-25_n77 DC_7-7-25_n77 DC_7-25-25_n77 DC_7-7-25-25_n77 | 7 | 0.5 |
|  | 25 | 0.6 |
|  | n77 | 0.8 |

Table 5.97.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-25_n77 DC_7-7-25_n77 DC_7-25-25_n77 DC_7-7-25-25_n77 | 7 | 0.5 |
|  | 25 | 0.2 |
|  | n77 | 0.5 |

5.97.4	Reference sensitivity exceptions

The IMD issues specifc to 3DL/2UL is the 4th order IMD for 25+n77 falling into band 25 and the 5th order IMD falling into band 7. This issue is similar to DC_2-7_n77 and the same MSD is used.

Table 5.97.4-1: MSD for the DC configuration due to IMD issue (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-25A_n77A DC_7A-7A-25A_n77A DC_7C-25A_n77A DC_7C-25A-25A_n77A DC_7A-25A-25A_n77A DC_7A-7A-25A-25A_n77A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | 25 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | 3.4 | IMD5 |
|  | 25 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | n77 | 4120 | 10 | 50 | 4120 | N/A | N/A |

5.98	DC_7-25_n78

### 5.98.1	Operating bands for DC

Table 5.98.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_7A-25A_n78A DC_7A-7A-25A_n78A DC_7C-25A_n78A DC_7A-25A-25A_n78A DC_7A-7A-25A-25A_n78A DC_7C-25A-25A_n78A | DC_7A_n78A DC_25A_n78A |

5.98.2	Co-existence studies

For UE coexistence study of Band 7 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.98.2-1.

Table 5.98.2-1: Harmonic and IMD analysis for DC_7_n78

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 7500 | 7710 | 9900 | 11400 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 730 | 1300 | 5800 | 6370 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1200 | 1840 | 4030 | 5100 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 8300 | 8940 | 9100 | 10170 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 3700 | 4410 | 7330 | 8900 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 10800 | 11510 | 12400 | 13970 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 2600 | 1460 | 11600 | 12740 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 12700 | 10630 | 6980 | 6200 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15700 | 17770 | 13300 | 14080 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 6400 | 4760 | 1110 | 100 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 14900 | 16540 | 14100 | 15310 |

For UE coexistence study of Band 25 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.98.2-2.

Table 5.98.2-2: Harmonic and IMD analysis for DC_25_n78

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1915 | 3300 | 3800 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3830 | 6600 | 7600 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5745 | 9900 | 11400 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1385 | 1950 | 5150 | 5715 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 100 | 530 | 4685 | 5750 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 7000 | 7630 | 8450 | 9515 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1750 | 2445 | 7985 | 9550 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8850 | 9545 | 11750 | 13315 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 3900 | 2770 | 10300 | 11430 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 13350 | 11285 | 4360 | 3600 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15050 | 17115 | 10700 | 11460 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 7700 | 6070 | 855 | 2050 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13600 | 15230 | 12150 | 13345 |

Based on co-existence study as presented in the table 5.98.2-1 and 5.98.2-2, own Rx impact is shown in the following.

- The 4th order IMD generated by dual uplink of Band 7 + Band n78 may fall into own Rx of band 25 and n78

- The 2nd harmonic of Band 25 uplink may fall into own Rx of band n78

- The 2nd, 4th and 5th order IMD generated by dual uplink of Band 25 + Band n78 may fall into own Rx of band 25

- The 4th order IMD generated by dual uplink of Band 25 + Band n78 may fall into own Rx of Band n78

5.98.3	∆TIB and ∆RIB values

The same relaxation values as DC_2-7_n78 is used.

Table 5.98.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-25_n78 DC_7-7-25_n78 DC_7-25-25_n78 DC_7-7-25-25_n78 | 7 | 0.5 |
|  | 25 | 0.6 |
|  | n78 | 0.8 |

Table 5.98.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-25_n78 DC_7-7-25_n78 DC_7-25-25_n78 DC_7-7-25-25_n78 | 7 | 0.5 |
|  | 25 | 0.2 |
|  | n78 | 0.5 |

5.98.4	Reference sensitivity exceptions

The IMD issue specifc to 3DL/2UL is the 4th order IMD for 25+n78 falling into band 25. This issue is similar to DC_2-7_n78 and the same MSD is used.

Table 5.98.4-1: MSD for the DC configuration due to IMD issue (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-25A_n78A DC_7A-7A-25A_n78A DC_7C-25A_n78A DC_7A-25A-25A_n78A DC_7A-7A-25A-25A_n78A DC_7C-25A-25A_n78A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | 25 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | n78 | 3525 | 10 | 50 | 3525 | N/A | N/A |

5.99	DC_25-66_n77

### 5.99.1	Operating bands for DC

Table 5.99.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_25A-66A_n77A DC_25A-25A-66A_n77A | DC_25A_n77A DC_66A_n77A |

5.99.2	Co-existence studies

For UE coexistence study of Band 25 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.99.2-1.

Table 5.99.2-1: Harmonic and IMD analysis for DC_25_n77

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1915 | 3300 | 4200 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3830 | 6600 | 8400 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5745 | 9900 | 12600 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1385 | 2350 | 5150 | 6115 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 500 | 530 | 4685 | 6550 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 7000 | 8030 | 8450 | 10315 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1350 | 2445 | 7985 | 10750 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8850 | 9945 | 11750 | 14515 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 4700 | 2770 | 10300 | 12230 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 14950 | 11285 | 4360 | 3200 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15050 | 18715 | 10700 | 11860 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 8900 | 6070 | 855 | 2850 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13600 | 16430 | 12150 | 14145 |

For UE coexistence study of Band 66 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.99.2-2.

Table 5.99.2-2: Harmonic and IMD analysis for DC_66_n77

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 3300 | 4200 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3420 | 3560 | 6600 | 8400 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5130 | 5340 | 9900 | 12600 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1520 | 2490 | 5010 | 5980 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 780 | 260 | 4820 | 6690 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 6720 | 7760 | 8310 | 10180 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 930 | 2040 | 8120 | 10890 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8430 | 9540 | 11610 | 14380 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -4980 | -3040 | 10020 | 11960 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 15090 | 11420 | 3820 | 2640 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 14910 | 18580 | 10140 | 11320 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 9180 | 6340 | 1260 | 3270 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13320 | 16160 | 11730 | 13740 |

Based on co-existence study as presented in the table 5.99.2-1 and 5.99.2-2, own Rx impact is shown in the following.

- The 2nd harmonic of band 25 may fall into own Rx of band n77

- The 2nd, 4th and 5th order IMD generated by dual uplink of 25+n77 may fall into own Rx of band 25 and 66

- The 4th and 5th order IMD generated by dual uplink of 25+n77 may fall into own Rx of n77

- The 2nd harmonic of band 66 may fall into own Rx of band n77

- The 2nd, 4th and 5th order IMD generated by dual uplink of 66+n77 may fall into own Rx of band 25 and 66

- The 4th and 5th order IMD generated by dual uplink of 66+n77 may fall into own Rx of band n77

5.99.3	∆TIB and ∆RIB values

The same relaxation values as DC_2-66_n77 is used for DC_25-66_n77.

Table 5.99.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_25-66_n77 DC_25-25-66_n77 | 25 | 0.6 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.99.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_25-66_n77 DC_25-25-66_n77 | 25 | 0.2 |
|  | 66 | 0.2 |
|  | n77 | 0.5 |

5.99.4	Reference sensitivity exceptions

The IMD issues specifc to 3DL/2UL is the IMD2/4/5 for 25+n77 falling into band 66 and for 66+n77 falling into band 25. This issues are similar to DC_2-66_n77 and the same MSD is used.

Table 5.99.4-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_25A-66A_n77A DC_25A-25A-66A_n77A | 25 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | 66 | 1765 | 5 | 25 | 2165 | 29.2 | IMD2 |
|  | n77 | 4020 | 10 | 25 | 4020 | N/A | N/A |
|  | 25 | 1905 | 5 | 25 | 1985 | M/A | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | 10.4 | IMD4 |
|  | n77 | 3595 | 10 | 25 | 3595 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | M/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | 4.0 | IMD5 |
|  | n77 | 3915 | 10 | 25 | 3915 | N/A | N/A |
|  | 25 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3700 | 10 | 25 | 3700 | N/A | N/A |
|  | 25 | 1860 | 5 | 25 | 1940 | 9.1 | IMD4 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n77 | 3385 | 10 | 25 | 3385 | N/A | N/A |
|  | 25 | 1900 | 5 | 25 | 1980 | 4.2 | IMD5 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n77 | 3645 | 10 | 25 | 3645 | N/A | N/A |

5.100	DC_25-66_n78

### 5.100.1	Operating bands for DC

Table 5.100.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_25A-66A_n78A DC_25A-25A-66A_n78A | DC_25A_n78A DC_66A_n78A |

5.100.2	Co-existence studies

For UE coexistence study of Band 25 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.100.2-1.

Table 5.100.2-1: Harmonic and IMD analysis for DC_25_n78

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1915 | 3300 | 3800 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3830 | 6600 | 7600 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5745 | 9900 | 11400 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1385 | 1950 | 5150 | 5715 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 100 | 530 | 4685 | 5750 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 7000 | 7630 | 8450 | 9515 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1750 | 2445 | 7985 | 9550 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8850 | 9545 | 11750 | 13315 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 3900 | 2770 | 10300 | 11430 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 13350 | 11285 | 4360 | 3600 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15050 | 17115 | 10700 | 11460 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 7700 | 6070 | 855 | 2050 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13600 | 15230 | 12150 | 13345 |

For UE coexistence study of Band 66 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.100.2-2.

Table 5.100.2-2: Harmonic and IMD analysis for DC_66_n78

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 3300 | 3800 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3420 | 3560 | 6600 | 7600 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5130 | 5340 | 9900 | 11400 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1520 | 2090 | 5010 | 5580 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 380 | 260 | 4820 | 5890 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 6720 | 7360 | 8310 | 9380 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1330 | 2040 | 8120 | 9690 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8430 | 9140 | 11610 | 13180 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 4180 | 3040 | 10020 | 11160 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 13490 | 11420 | 3820 | 3040 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 14910 | 16980 | 10140 | 10920 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 980 | 6340 | 1260 | 2470 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13320 | 14960 | 11730 | 12940 |

Based on co-existence study as presented in the table 5.100.2-1 and 5.100.2-2, own Rx impact is shown in the following.

- The 2nd harmonic of band 25 may fall into own Rx of band n78

- The 2nd, 4th and 5th order IMD generated by dual uplink of 25+n78 may fall into own Rx of band 25

- The 4th order IMD generated by dual uplink of 25+n78 may fall into own Rx of band 66

- The 4th and 5th order IMD generated by dual uplink of 25+n78 may fall into own Rx of n78

- The 2nd harmonic of band 66 may fall into own Rx of band n78

- The 2nd, 4th and 5th order IMD generated by dual uplink of 66+n78 may fall into own Rx of band 25 and 66

- The 4th and 5th order IMD generated by dual uplink of 66+n78 may fall into own Rx of band n78

5.100.3	∆TIB and ∆RIB values

The same relaxation values as DC_2-66_n78 is used for DC_25-66_n78.

Table 5.100.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_25-66_n78 DC_25-25-66_n78 | 25 | 0.6 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |

Table 5.100.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_25-66_n78 DC_25-25-66_n78 | 25 | 0.2 |
|  | 66 | 0.2 |
|  | n78 | 0.5 |

5.100.4	Reference sensitivity exceptions

The IMD issues specifc to 3DL/2UL is the IMD4/5 for 25+n78 falling into band 66 and IMD2/4/5 for 66+n78 falling into band 25. This issues are similar to DC_2-66_n78 and the same MSD is used.

Table 5.100.4-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_25A-66A_n78A DC_25A-25A-66A_n78A | 25 | 1880 | 5 | 25 | 1960 | M/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 10.4 | IMD4 |
|  | n78 | 3480 | 10 | 50 | 3480 | N/A | N/A |
|  | 25 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |
|  | 25 | 1880 | 5 | 25 | 1960 | 9.1 | IMD4 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 25 | 1900 | 5 | 25 | 1980 | 4.2 | IMD5 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n78 | 3645 | 10 | 25 | 3645 | N/A | N/A |

5.101	DC_2-29_n78

### 5.101.1	Operating bands for DC

Table 5.101.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-29_n78 | CA_2-29 | n78 | DC_2_n78 |

### 5.101.2	Configurations for DC

Table 5.101.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-29A_n78A | DC_2A_n78A | CA_2A-29A | n78 |

### 5.101.3	Co-existence studies

Based on co-existence studies of DC_2A-29A_n78A with 2UL, it can get that:

- no IMD of band 2 UL and band n78 UL falling to band 29 DL

### 5.101.4	∆TIB and ∆RIB values

### For DC_2A-29A_n78A, the TIB,c and RIB,c values are reused from the LTE combination CA_2-13-48, and are given in the tables below.

Table 5.101.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29-n78 | 2 | 0.6 |
|  | n78 | 0.8 |

Table 5.101.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-29-n78 | 2 | 0.2 |
|  | n78 | 0.5 |

### 5.101.5	REFSENS requirements

Although DC_29_n78 is not defined, 5th order harmonic mixing is from the band n78 UL and DL on band 29 existed and need be considered here. DC_29_n78’s MSD can refer to DC_28-n78 values. Below table can be merged into 38.101-3 Table 7.3B.2.3.2-1 and Table 7.3B.2.3.2-2 respectively:

Table 5.101.5-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | 292 | 28 | 25 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.101.5-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n78 | 29 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |

5.102	DC_29-66_n78

### 5.102.1	Operating bands for DC

Table 5.102.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_29-66_n78 | CA_29-66 | n78 | DC_66_n78 |

### 5.102.2	Configurations for DC

Table 5.102.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_29A-66A_n78A | DC_66A_n78A | CA_29A-66A | n78 |

### 5.102.3	Co-existence studies

Based on co-existence studies of DC_29A-66A_n78A with 2UL, it can get that:

- no IMD of band 66 UL and band n78 UL falling to band 29 DL

Although DC_29_n78 is not defined, 5th order harmonic mixing is from the band n78 UL and DL on band 29 existed and need be considered here.

### 5.102.4	∆TIB and ∆RIB values

### For DC_29A-66A_n78A, the TIB,c and RIB,c values are reused from the LTE combination CA_13-48-66, and are given in the tables below.

Table 5.102.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_29-66-n78 | 66 | 0.6 |
|  | n78 | 0.8 |

Table 5.102.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_29-66-n78 | 66 | 0.2 |
|  | n78 | 0.5 |

### 5.102.5	REFSENS requirements

Although DC_29_n78 is not defined, 5th order harmonic mixing is from the band n78 UL and DL on band 29 existed and need be considered here. DC_29_n78’s MSD can refer to DC_28-n78 values. Below table can be merged into 38.101-3 Table 7.3B.2.3.2-1 and Table 7.3B.2.3.2-2 respectively:

Table 5.102.5-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | 292 | 28 | 25 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.102.5-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n78 | 29 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |

5.103	DC_1-21_n28

5.103.1	Configurations for DC

The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783-793 MHz for the DL because only a certain operator uses band 21. This restriction is mentioned in TP for DC_21_n28 (R4-2100352).

Table 5.103.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-21A_n28A | DC_1A_n28A DC_21A_n28A |

5.103.2	Co-existence studies

Based on co-existence studies of DC_1_n28 and DC_21_n28, own Rx impact of the 3rd band is the followings.

-	2nd order IMD products generated by DC_21_n28 uplink may fall into own Rx of band 1.

-	3rd order IMD products generated by DC_21_n28 uplink may fall into own Rx of band 1.

5.103.3	∆TIB and ∆RIB values

For DC_1-21_n28, the TIB,c and RIB,c values are reused from the LTE combination CA_1-21-28, and are given in the tables below.

Table 5.103.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n28 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | n28 | 0.6 |

Table 5.103.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21_n28 | 1 | 0 |
|  | 21 | 0 |
|  | n28 | 0.2 |

5.103.4	Reference sensitivity exceptions

-	2nd order IMD products generated by DC_21_n28 uplink may fall into own Rx of band 1.

⇒ IMD was calculated based on the frequency range in band n28 that the operator actually owned, which resulted in that IMD2 doesn’t fall into own Rx of band 1. Therefore, we didn’t specify MSD.

-	3rd order IMD products generated by DC_21_n28 uplink may fall into own Rx of band 1.

⇒ The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

Table 5.103.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-21A_n28AX1 | 1 | 1975.3 | 5 | 25 | 2165.3 | 16.1 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |
| NOTE X1: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2 fall in B1 also which MSD is not specified. |  |  |  |  |  |  |  |

5.104	DC_3-21_n28

5.104.1	Configurations for DC

The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783-793 MHz for the DL because only a certain operator uses band 21. This restriction is mentioned in TP for DC_21_n28 (R4-2100352).

Table 5.104.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-21A_n28A | DC_3A_n28A DC_21A_n28A |

5.104.2	Co-existence studies

Based on co-existence studies of DC_3_n28 and DC_21_n28, own Rx impact of the 3rd band is the followings.

-	IMD generated by DC_3_n28 uplink doesn’t fall into own Rx of band 21.

-	IMD generated by DC_21_n28 uplink doesn’t fall into own Rx of band 3.

5.104.3	∆TIB and ∆RIB values

For DC_3-21_n28, the TIB,c and RIB,c values are reused from the LTE combination CA_3-21-28, and are given in the tables below.

Table 5.104.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n28 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n28 | 0.3 |

Table 5.104.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n28 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n28 | 0 |

5.104.4	Reference sensitivity exceptions

Based on co-existence studies of DC_3-21_n28, there is no need to have MSD added.

5.105	DC_8-20_n1

5.105.1	Configurations for DC

Table 5.105.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-20A_n1A | DC_8A_n1A DC_20A_n1A |

5.105.2	Co-existence studies

Table 5.105.2-1 lists the Band 8A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.105.2-1: Band 8 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2640 – 2745 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1005 – 1100 |  |  | 2800 – 2895 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 90 – 220 |  |  | 2925 – 3080 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3680 – 3810 |  |  | 4720 – 4875 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 830 – 965 |  |  | 1910 – 1990 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 660 – 825 |  |  | 4845 – 5060 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2010 – 2200 |  |  | 5600 – 5790 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4560 – 4725 |  |  | 6640 – 6855 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6765 – 7040 |  |  | 1540 – 1740 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3930 – 4180 |  |  | 1095 – 1320 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8560 – 8835 |  |  | 5440 – 5640 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7520 – 7770 |  |  | 6480 – 6705 |  |  |

Based on Table 5.105.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 7, 41, 77 and 90

-	3rd order harmonics may fall into Rx frequencies of bands 3, 46 and 47

-	3rd order IMD may fall into Rx frequencies of bands 43, 48, 49, 77, 78 and 79

-	4th order IMD may fall into Rx frequencies of bands 1, 4, 10, 12, 13, 14, 17, 20, 23, 28, 29, 34, 44, 46, 65, 66, 67, 68, 70, 79, 79 and 85

-	5th order IMD may fall into Rx frequencies of bands 24, 46 and 77

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.105.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.105.2-2: 2UL Band 8 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | IMD4, IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_8_n1 in 38101-3.

Table 5.105.2-3 lists the Band 20A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.105.2-3: Band 20 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1058 – 1148 |  |  | 2752 – 2842 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 196 – 316 |  |  | 2978 – 3128 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3584 – 3704 |  |  | 4672 – 4822 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 782 – 912 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 516 – 666 |  |  | 4898 – 5108 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2116 – 2296 |  |  | 5504 – 5684 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4416 – 4566 |  |  | 6592 – 6802 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6818 – 7088 |  |  | 1348 – 1528 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4036 – 4276 |  |  | 1254 – 1464 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8512 – 8782 |  |  | 5248 – 5428 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7424 – 7664 |  |  | 6336 – 6546 |  |  |

Based on Table 5.105.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 38, 41, 69, 77 and 90

-	3rd order harmonics may fall into Rx frequencies of bands 46 and 47

-	3rd order IMD may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77, 78 and 79

-	4th order IMD may fall into Rx frequencies of bands 1, 4, 10, 23, 46, 65, 66, 71 and 79

-	5th order IMD may fall into Rx frequencies of bands 11, 21, 24, 32, 45, 46, 50, 51, 74, 75, 76, 77, 91, 92, 93 and 94

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.105.2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.105.2-4: 2UL Band 20 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_20_n1 in 38101-3.

5.105.3	∆TIB and ∆RIB values

Table 5.105.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-20A_n1 | 8 | 0.4 |
|  | 20 | 0.4 |
|  | n1 | 0.3 |

Table 5.105.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A-20A_n1 | 8 | 0 |
|  | 20 | 0 |
|  | n1 | 0 |

5.105.4	Reference sensitivity exceptions

B20 MSD due to IMD4 of 8-n1 is TBD.

Further study required to agree on achievable performance for LB quadplexer.5.106	DC_8-20_n3

5.106.1	Configurations for DC

Table 5.106.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-20A_n3A | DC_8A_n3A DC_20A_n3A |

5.106.2	Co-existence studies

Table 5.106.2-1 lists the Band 8A + Band n3A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.106.2-1: Band 8 and Band n3 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 1710 | 1785 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  |  | 3420 – 3570 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2640 – 2745 |  |  | 5130 – 5355 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 795 – 905 |  |  | 2590 – 2700 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 25 – 120 |  |  | 2505 – 2690 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3470 – 3615 |  |  | 4300 – 4485 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 850 – 945 |  |  | 1700 – 1795 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 855 – 1035 |  |  | 4215 – 4475 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1590 – 1810 |  |  | 5180 – 5400 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4350 – 4530 |  |  | 6010 – 6270 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5925 – 6260 |  |  | 1735 – 1950 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3300 – 3595 |  |  | 675 – 930 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 – 8055 |  |  | 5230 – 5445 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 – 7185 |  |  | 6060 – 6315 |  |  |

Based on Table 5.106.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 7, 22, 41, 42, 48, 49, 77, 78 and 90

-	3rd order harmonics may fall into Rx frequencies of bands 3 and 46

-	2nd order IMD may fall into Rx frequencies of bands 5, 6, 7, 18, 19, 20, 26, 27, 28, 38, 41, 44, 69 and 90

-	3rd order IMD may fall into Rx frequencies of bands 7, 22, 38, 41, 42, 43, 48, 49, 69, 77, 78, 79 and 90

-	4th order IMD may fall into Rx frequencies of bands 3, 5, 6, 8, 18, 19, 26, 27, 46 and 79

-	5th order IMD may fall into Rx frequencies of bands 2, 3, 5, 6, 8, 9, 12, 13, 14, 17, 18, 19, 20, 22, 25, 26, 27, 28, 29, 33, 35, 36, 37, 39, 42, 44, 46, 48, 49, 52, 67, 68, 77, 78 and 85

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.106.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.106.2-2: 2UL Band 8 + Band n3 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 3rd Harmonic, IMD4, IMD5 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_8_n3 in 38101-3.

Table 5.106.2-3 lists the Band 20A + Band n3A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.106.2-3: Band 20 and Band n3 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 1710 | 1785 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 3420 – 3570 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 5130 – 5355 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 848 – 953 |  |  | 2542 – 2647 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 14 – 121 |  |  | 2558 – 2738 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3374 – 3509 |  |  | 4252 – 4432 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 802 – 892 |  |  | 1690 – 1805 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 711 – 876 |  |  | 4268 – 4523 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1696 – 1906 |  |  | 5084 – 5294 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4206 – 4371 |  |  | 5962 – 6217 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5978 – 6308 |  |  | 1543 – 1738 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3406 – 3691 |  |  | 834 – 1074 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7672 – 8002 |  |  | 5038 – 5233 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6794 – 7079 |  |  | 5916 – 6156 |  |  |

Based on Table 5.106.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 22, 38, 41, 42, 48, 49, 69, 77, 78 and 90

-	3rd order harmonics may fall into Rx frequencies of band 46

-	2nd order IMD may fall into Rx frequencies of bands 5, 6, 7, 8, 18, 19, 26, 27, 38, 41, 69 and 90

-	3rd order IMD may fall into Rx frequencies of bands 7, 38, 41, 42, 52, 69, 77, 78, 79 and 90

-	4th order IMD may fall into Rx frequencies of bands 3, 5, 6, 9, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 33, 35, 39, 44, 46, 67, 68, 79 and 85

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 8, 18, 19, 22, 24, 26, 27, 42, 43, 46, 47, 48, 49, 77 and 78

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.106.2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.106.2-4: 2UL Band 20 + Band n3 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 3rd Harmonic, IMD4, IMD5 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_20_n3 in 38101-3.

5.106.3	∆TIB and ∆RIB values

Table 5.106.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-20A_n3 | 8 | 0.4 |
|  | 20 | 0.4 |
|  | n3 | 0.3 |

Table 5.106.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A-20A_n3 | 8 | 0 |
|  | 20 | 0 |
|  | n3 | 0 |

5.106.4	Reference sensitivity exceptions

B20 and B8 MSD for IMD2 of 8-n3 and 20-n3 uplinks respectively is TBD.Further study required to agree on achievable performance of LB quadplexer.

5.107	DC_8-32_n1

5.107.1	Configurations for DC

Table 5.107.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-32A_n1A | DC_8A_n1A |

5.107.2	Co-existence studies

Table 5.107.2-1 lists the Band 8A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.107.2-1: Band 8 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2640 – 2745 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1005 – 1100 |  |  | 2800 – 2895 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 90 – 220 |  |  | 2925 – 3080 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3680 – 3810 |  |  | 4720 – 4875 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 830 – 965 |  |  | 1910 – 1990 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 660 – 825 |  |  | 4845 – 5060 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2010 – 2200 |  |  | 5600 – 5790 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4560 – 4725 |  |  | 6640 – 6855 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6765 – 7040 |  |  | 1540 – 1740 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3930 – 4180 |  |  | 1095 – 1320 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8560 – 8835 |  |  | 5440 – 5640 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7520 – 7770 |  |  | 6480 – 6705 |  |  |

Based on Table 5.107.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 7, 41, 77 and 90

-	3rd order harmonics may fall into Rx frequencies of bands 3, 46 and 47

-	3rd order IMD may fall into Rx frequencies of bands 43, 48, 49, 77, 78 and 79

-	4th order IMD may fall into Rx frequencies of bands 1, 4, 10, 12, 13, 14, 17, 20, 23, 28, 29, 34, 44, 46, 65, 66, 67, 68, 70, 79, 79 and 85

-	5th order IMD may fall into Rx frequencies of bands 24, 46 and 77

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.107.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.107.2-2: 2UL Band 8 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | IMD4, IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_8_n1 in 38101-3.

5.107.3	∆TIB and ∆RIB values

Table 5.107.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-32A_n1 | 8 | 0.3 |
|  | 32 | N/A |
|  | n1 | 0.5 |

Table 5.107.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A-32A_n1 | 8 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

5.107.4	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.5.108	DC_12-66_n41

### 5.108.1	Operating bands for EN-DC

Table 5.108.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 12-66_n41 | CA_12-66 | n41 |  |

5.108.2	Configuration for DC

Table 5.108.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_12A-66A_n41A | DC_12A_n41A DC_66A_n41A | CA_12A-66A | n41A |

### 5.108.3		∆TIB and ∆RIB values

For DC_12-66_n41, the TIB,c and RIB,c values are reused from DC_66_n41-n71 and are given in the tables below.

Table 5.108.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n41 | 12 | 0.6 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

Table 5.108.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n41 | 12 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

### 5.108.4	REFSENS requirements

There is IMD2 impact from UL 66_n41 affecting DL band 12. The MSD value is derived from DC_7A-13A_n66A.

The IMD3 impact from UL 66_n41 affecting DL band 12 is too close to the band edges to allocate test points and can therefore be disregarded.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-66A_n41A | 12 | 712 | 5 | 25 | 742 | 31 | IMD2 |
|  | 66 | 1773 | 5 | 25 | 2173 | N/A | N/A |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | N/A |

5.109	DC_2-12_n41

### 5.109.1	Operating bands for EN-DC

Table 5.109.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-12_n41 | CA_2-12 | n41 |  |

5.109.2	Configuration for DC

Table 5.109.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A_n41A | DC_2A_n41A DC_12A_n41A | CA_2A-12A | n41A |
| DC_2A-2A-12A_n41A | DC_2A_n41A DC_12A_n41A | CA_2A-2A-12A | n41A |

### 5.109.3		∆TIB and ∆RIB values

For DC_2-12_n41, the TIB,c and RIB,c values are reused from CA_2-7-12 and DC_2_n41-n71 and are given in the tables below.

Table 5.109.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n41 DC_2-2-12_n41 | 2 | 0.5 |
|  | 12 | 0.3 |
|  | n41 | 0.5 |

Table 5.109.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n41 DC_2-2-12_n41 | 2 | 0 |
|  | 12 | 0 |
|  | n41 | 0 |

### 5.109.4	REFSENS requirements

There are IMD2 impact from UL 12_n41 affecting DL band 2. The IMD2 MSD value is derived from DC_2A-71A_n38A.

There are IMD2 and IMD5 impact from UL 2_n41 affecting DL band 12. The IMD2 MSD value is derived from DC_2A_n41A-n71A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-12A_n41A DC_2A-2A-12A_n41A | 2 | 1872 | 5 | 25 | 1952 | 26 | IMD2 |
|  | 12 | 708 | 5 | 50 | 738 | N/A | N/A |
|  | n41 | 2660 | 10 | 50 | 2660 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 12 | 708 | 5 | 50 | 738 | 28.7 | IMD24 |
|  | n41 | 2638 | 10 | 50 | 2638 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |

5.110	DC_66-71_n41

### 5.110.1	Operating bands for EN-DC

Table 5.110.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 66-71_n41 | CA_66-71 | n41 |  |

5.110.2	Configuration for DC

Table 5.110.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_66A-71A_n41A | DC_66A_n41A DC_71A_n41A | CA_66A-71A | n41A |

### 5.110.3		∆TIB and ∆RIB values

For DC_66-71_n41, the TIB,c and RIB,c values are reused from DC_66_n41-n71 and are given in the tables below.

Table 5.110.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66-71_n41 | 66 | 0.5 |
|  | 71 | 0.6 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

Table 5.110.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66-71_n41 | 66 | 0.5 |
|  | 71 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

### 5.110.4	REFSENS requirements

There is no need to define MSD.

5.111	DC_2-71_n41

### 5.111.1	Operating bands for EN-DC

Table 5.111.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-71_n41 | CA_2-71 | n41 |  |

5.111.2	Configuration for DC

Table 5.111.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-71A_n41A | DC_2A_n41A DC_71A_n41A | CA_2A-71A | n41A |
| DC_2A-2A-71A_n41A | DC_2A_n41A DC_71A_n41A | CA_2A-2A-71A | n41A |

### 5.111.3		∆TIB and ∆RIB values

For DC_2-71_n41, the TIB,c and RIB,c values are reused from DC_2_n41-n71 and are given in the tables below.

Table 5.111.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-71_n41 DC_2-2-71_n41 | 2 | 0.5 |
|  | 71 | 0.3 |
|  | n41 | 0.5 |

Table 5.111.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-71_n41 DC_2-2-71_n41 | 2 | 0 |
|  | 71 | 0 |
|  | n41 | 0 |

### 5.111.4	REFSENS requirements

There are IMD2 impact from UL 71_n41 affecting DL band 2. The IMD2 MSD value is derived from DC_2A-71A_n38A.

There are IMD2 and IMD5 impact from UL 2_n41 affecting DL band 71. The IMD2 MSD value is derived from DC_2A_n41A-n71A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-71A_n41A DC_2A-2A-71A_n41A | 2 | 1862 | 5 | 25 | 1942 | 26 | IMD2 |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
|  | n41 | 2610 | 10 | 50 | 2610 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 71 | 676 | 5 | 50 | 630 | 28.7 | IMD24 |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |

5.112	DC_7-12_n66

### 5.112.1	Operating bands for EN-DC

Table 5.112.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-12_n66 | CA_7-12 | n66 |  |

5.112.2	Configuration for DC

Table 5.112.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-12A_n66A | DC_7A_n66A DC_12A_n66A | CA_7A-12A | n66A |

### 5.112.3		∆TIB and ∆RIB values

For DC_7-12_n66, the TIB,c and RIB,c values are reused from DC_7-66_n71 and are given in the tables below.

Table 5.112.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-12_n66 | 7 | 0.5 |
|  | 12 | 0.5 |
|  | n66 | 0.5 |

Table 5.112.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-12_n66 | 7 | 0.5 |
|  | 12 | 0.1 |
|  | n66 | 0.5 |

### 5.112.4	REFSENS requirements

There are IMD2 impact from UL 7_n66 affecting DL band 12. The IMD2 MSD value is derived from DC_7A-13A_n66A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-12A_n66A | 7 | 2515 | 5 | 25 | 2635 | N/A | N/A |
|  | 12 | 712 | 5 | 25 | 742 | 31 | IMD2 |
|  | n66 | 1773 | 5 | 25 | 2173 | N/A | N/A |

5.113	DC_7-71_n66

### 5.113.1	Operating bands for EN-DC

Table 5.113.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-71_n66 | CA_7-71 | n66 |  |

5.113.2	Configuration for DC

Table 5.113.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-71A_n66A | DC_7A_n66A DC_71A_n66A | CA_7A-71A | n71A |

### 5.113.3		∆TIB and ∆RIB values

For DC_7-71_n66, the TIB,c and RIB,c values are reused from DC_7-66_n71 and are given in the tables below.

Table 5.113.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-71_n66 | 7 | 0.5 |
|  | 71 | 0.5 |
|  | n66 | 0.5 |

Table 5.113.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-71_n66 | 7 | 0.5 |
|  | 71 | 0.1 |
|  | n66 | 0.5 |

### 5.113.4	REFSENS requirements

There is no need to define MSD.

5.114	DC_7-12_n78

### 5.114.1	Operating bands for EN-DC

Table 5.114.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-12_n78 | CA_7-12 | n78 |  |

5.114.2	Configuration for DC

Table 5.114.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-12A_n78A | DC_7A_n78A DC_12A_n78A | CA_7A-12A | n78A |

### 5.114.3		∆TIB and ∆RIB values

For DC_7-12_n78, the TIB,c and RIB,c values are reused from DC_12_n7-n78 and are given in the tables below.

Table 5.114.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-12_n78 | 7 | 0.5 |
|  | 12 | 0.5 |
|  | n78 | 0.8 |

Table 5.114.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-12_n78 | 7 | 0.2 |
|  | 12 | 0.5 |
|  | n78 | 0.5 |

### 5.114.4	REFSENS requirements

There are IMD2 impact from UL 12_n78 affecting DL band 7. The IMD2 MSD value is derived from DC_12A_n7A-n78A.

There are IMD2 and IMD5 impact from UL 7_n78 affecting DL band 12. The IMD2 MSD value is derived from DC_28A-41A_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-12A_n78A | 7 | 2542 | 5 | 25 | 2662 | 29.6 | IMD2 |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | 12 | 710 | 5 | 25 | 740 | 30.8 | IMD24 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |

5.115	DC_12-66_n78

### 5.115.1	Operating bands for EN-DC

Table 5.115.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 12-66_n78 | CA_12-66 | n78 |  |

5.115.2	Configuration for DC

Table 5.115.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_12A-66A_n78A | DC_12A_n78A DC_66A_n78A | CA_12A-66A | n78A |

### 5.115.3		∆TIB and ∆RIB values

For DC_12-66_n78, the TIB,c and RIB,c values are reused from DC_66-71_n78 and are given in the tables below.

Table 5.115.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n78 | 12 | 0.6 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |

Table 5.115.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-66_n78 | 12 | 0.2 |
|  | 66 | 0.2 |
|  | n78 | 0.5 |

### 5.115.4	REFSENS requirements

There are IMD3 impact from UL 12_n78 affecting DL band 66. The IMD3 MSD value is derived from DC_48A-66A_n12A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-66A_n78A | 12 | 710 | 5 | 25 | 740 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 17.1 | IMD3 |
|  | n78 | 3580 | 5 | 25 | 3580 | N/A | N/A |

5.116	DC_2-12_n78

### 5.116.1	Operating bands for EN-DC

Table 5.116.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-12_n78 | CA_2-12 | n78 |  |

5.116.2	Configuration for DC

Table 5.116.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A_n78A | DC_2A_n78A DC_12A_n78A | CA_2A-12A | n78A |
| DC_2A-2A-12A_78A | DC_2A_n78A DC_12A_n78A | CA_2A-2A-12A | n78A |
| DC_2A-12A_n78(2A) | DC_2A_n78A DC_12A_n78A | CA_2A-12A | CA_n78(2A) |

### 5.116.3		∆TIB and ∆RIB values

For DC_2-12_n78, the TIB,c and RIB,c values are reused from DC_2-71_n78 and are given in the tables below.

Table 5.116.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n78 | 2 | 0.6 |
|  | 12 | 0.6 |
|  | n78 | 0.8 |

Table 5.116.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12_n78 | 2 | 0.2 |
|  | 12 | 0.2 |
|  | n78 | 0.5 |

### 5.116.4	REFSENS requirements

There are IMD3 impact from UL 12_n78 affecting DL band 2. The IMD3 MSD value is derived from DC_2A-71A_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-12A_n78A DC_2A-2A-12A_n78A DC_2A-12A_n78(2A) | 2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD3 |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |

5.117	DC_7-71_n78

### 5.117.1	Operating bands for EN-DC

Table 5.117.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-71_n78 | CA_7-71 | n78 |  |

5.117.2	Configuration for DC

Table 5.117.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-71A_n78A | DC_7A_n78A DC_71A_n78A | CA_7A-71A | n78A |

### 5.117.3		∆TIB and ∆RIB values

For DC_7-71_n78, the TIB,c and RIB,c values are reused from DC_12_n7-n78 and are given in the tables below.

Table 5.117.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-71_n78 | 7 | 0.5 |
|  | 71 | 0.5 |
|  | n78 | 0.8 |

Table 5.117.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-71_n78 | 7 | 0.2 |
|  | 71 | 0.5 |
|  | n78 | 0.5 |

### 5.117.4	REFSENS requirements

There are IMD2 impact from UL 71_n78 affecting DL band 7. The IMD2 MSD value is derived from DC_12A_n7A-n78A.

There are IMD5 impact from UL 7_n78 affecting DL band 71. The IMD2 MSD value is derived from DC_28A-41A_n77A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-71A_n78A | 7 | 2550 | 5 | 25 | 2670 | 29.6 | IMD2 |
|  | 71 | 680 | 5 | 25 | 634 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | 71 | 686 | 5 | 25 | 640 | 3.0 | IMD5 |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | N/A |

5.118	DC_20-40_n1

### 5.118.1	Operating bands for DC

Table 5.118.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_20-40_n1 | CA_20-40 | n1 | No |

### 5.118.2	Configurations for DC

Table 5.118.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_20A-40A_n1A DC_20A-40C_n1A | DC_20A_n1A DC_40A_n1A | CA_20A-40A CA_20A-40C | n1 |

### 5.118.3	Co-existence studies

Based on co-existence studies of Band 20 + Band n1 captured in 37.863-01-01 there is no IMD interfering band 40

And based on co-existence studies of Band 40 + Band n1 captured in 37.863-01-01, MSD shall be considered since

-	4th order IMD generated by dual uplink of the two bands may fall into own Rx of band 20.

### 5.118.4	∆TIB and ∆RIB values

It is proposed to re-use relaxation values from DC_1-20_n38 which is very similar.

Table 5.118.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-40-n1 | 20 | 0.3 |
|  | 40 | 0.5 |
|  | n1 | 0.5 |

Table 5.118.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-40-n1 | 20 | 0 |
|  | 40 | 0 |
|  | n1 | 0 |

### 5.118.5	REFSENS requirements

It is proposed to re-use IMD4 MSD value from DC_1A_n8A-n40A which is very similar.

Table 5.118.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-40A_n1A DC_20A-40C_n1A | 20 | 841 | 5 | 25 | 800 | 8.0 | IMD4 |
|  | 40 | 2330 | 5 | 25 | 2330 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |

5.119	DC_5-30_n2

### 5.119.1	Operating bands for DC

Table 5.119.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-30_n2 | CA_5-30 | n2 | No |

### 5.119.2	Configurations for DC

Table 5.119.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A-30A_n2A | DC_5A_n2A DC_30A_n2A | CA_5A-30A | n2 |

### 5.119.3	Co-existence studies

Based on co-existence studies of Band 5 + Band n2 captured in 37.716-11-11, no MSD is necessary

Based on co-existence studies of Band 30 + Band n2 captured in 37.716-11-11, MSD shall be considered since

-	4th order IMD generated by dual uplink of the two bands may fall into own Rx of band 5.

### 5.119.4	∆TIB and ∆RIB values

Based on E-UTRA CA_2-5-30 configuration, following relaxations are proposed

Table 5.119.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-30_n2 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n2 | 0.5 |

Table 5.119.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-30_n2 | 5 | 0 |
|  | 30 | 0.5 |
|  | n2 | 0.4 |

### 5.119.5	REFSENS requirements

As stated in 5.119.3 MSD requirement caused by 4th order IMD is needed and specified below accordingly which is based on similar case of  DC_1A_n8A-n40A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-30A_n2A | 5 | 835 | 5 | 25 | 880 | 8 | IMD4 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | N/A |

5.120	DC_2-46_n77

### 5.120.1	Operating bands for DC

Table 5.120.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-46A_n77A | DC_2A_n77A |

5.120.2	Co-existence studies

For UE coexistence study of Band 2 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products are presented in Table 5.120.2-1.

Table 5.120.2-1: Harmonic and IMD analysis for DC_2_n77

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1910 | 3300 | 4200 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3820 | 6600 | 8400 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5730 | 9900 | 12600 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1390 | 2350 | 5150 | 6110 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -500 | 520 | 4690 | 6550 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 7000 | 8020 | 8450 | 10310 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1350 | 2430 | 7990 | 10750 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8850 | 9930 | 11750 | 14510 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 4700 | 2780 | 10300 | 12220 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 14950 | 11290 | 4340 | 3200 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15050 | 18710 | 10700 | 11840 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 8900 | 6080 | 870 | 2850 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13600 | 16420 | 12150 | 14130 |

Based on co-existence study as presented in the table 5.120.2-1, own Rx impact is shown in the following.

- The 2nd harmonic of band 2 may fall into own Rx of band n77

- The 3rd harmonic of band 2 may fall into own Rx of band 46

- The 2nd order IMD generated by dual uplink of 2+n77 may fall into own Rx of band 2 and 46

- The 3rd order IMD generated by dual uplink of 2+n77 may fall into own Rx of band 46

- The 4th order IMD generated by dual uplink of 2+n77 may fall into own Rx of band 2 and n77

- The 5th order IMD generated by dual uplink of 2+n77 may fall into own Rx of band 2 and n77

5.120.3	∆TIB and ∆RIB values

The same relaxation values as DC_2_n77 is assumed for DC_2-46_n77.

Table 5.120.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n77 | 2 | 0.6 |
|  | n77 | 0.8 |

Table 5.120.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46_n77 | 2 | 0 |
|  | 46 | 0 |
|  | n77 | 0 |

5.120.4	Reference sensitivity exceptions

The IMD issues specifc to 3DL/2UL is the IMD2/IMD3 of band 2+n77 falling into band 46. The exclusion zone for IMD to band 46 is applied.

Table 5.120.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-46A_n77A5 | 2 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n77A; DC_2A_n77A; fc_2A + fc_n77A; BW_2A + BW_n77A. DC_2A-46A_n77A; DC_2A_n77A; -fc_2A + 2*fc_n77A; -BW_2A + 2*BW_n77A. |

5.121	DC_46-66_n77

### 5.121.1	Operating bands for DC

Table 5.121.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_46A-66A_n77A | DC_66A_n77A |

5.121.2	Co-existence studies

For UE coexistence study of Band 66 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products are presented in Table 5.121.2-1.

Table 5.121.2-1: Harmonic and IMD analysis for DC_66_n77

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 3300 | 4200 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3420 | 3560 | 6600 | 8400 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5130 | 5340 | 9900 | 12600 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1520 | 2490 | 5010 | 5980 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 780 | 260 | 4820 | 6690 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 6720 | 7760 | 8310 | 10180 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 930 | 2040 | 8120 | 10890 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8430 | 9540 | 11610 | 14380 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 4980 | 3040 | 10020 | 11960 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 15090 | 11420 | 3820 | 2640 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 14910 | 18580 | 10140 | 11320 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 9180 | 6340 | 1260 | 3270 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13320 | 16160 | 11730 | 13740 |

Based on co-existence study as presented in the table 5.121.2-1, own Rx impact is shown in the following.

- The 2nd harmonic of band 66 may fall into own Rx of band n77

- The 3rd harmonic of band 66 may fall into own Rx of band 46

- The 2nd order IMD generated by dual uplink of 66+n77 may fall into own Rx of band 46 and 66

- The 3rd order IMD generated by dual uplink of 66+n77 may fall into own Rx of band 46

- The 4th order IMD generated by dual uplink of 66+n77 may fall into own Rx of band n77

- The 5th order IMD generated by dual uplink of 66+n77 may fall into own Rx of band 66 and n77

5.121.3	∆TIB and ∆RIB values

The same relaxation values as DC_66_n77 is assumed for DC_46-66_n77.

Table 5.121.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n77 | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.121.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_46-66_n77 | 46 | 0 |
|  | 66 | 0 |
|  | n77 | 0 |

5.121.4	Reference sensitivity exceptions

The IMD issues specifc to 3DL/2UL is the IMD2/IMD3 of band 66+n77 falling into band 46. The exclusion zone for IMD to band 46 is applied.

Table 5.121.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_46A-66A_n77A5 | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |
|  | 66 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_46A-66A_n77A; DC_66A_n77A; fc_66A + fc_n77A; BW_66A + BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; -fc_66A + 2*fc_n77A; -BW_66A + 2*BW_n77A. |

5.122	DC_13-46_n77

### 5.122.1	Operating bands for DC

Table 5.122.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_13A-46A_n77A | DC_13A_n77A |

5.122.2	Co-existence studies

For UE coexistence study of Band 13 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products are presented in Table 5.122.2-1.

Table 5.122.2-1: Harmonic and IMD analysis for DC_13_n77

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 777 | 787 | 3300 | 4200 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1554 | 1574 | 6600 | 8400 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2331 | 2361 | 9900 | 12600 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 2513 | 3423 | 4077 | 4987 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 2646 | 1726 | 5813 | 7623 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 4854 | 5774 | 7377 | 9187 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1869 | 939 | 9113 | 11823 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 5631 | 6561 | 10677 | 13387 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 6846 | 5026 | 8154 | 9974 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 16023 | 12413 | 152 | 1092 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 13977 | 17587 | 6408 | 7348 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 11046 | 8326 | 4239 | 6069 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 11454 | 14174 | 8931 | 10761 |

Based on co-existence study as presented in the table 5.122.2-1, own Rx impact is shown in the following.

- The 2nd order IMD generated by dual uplink of 13+n77 may fall into own Rx of band n77

- The 3rd order IMD generated by dual uplink of 13+n77 may fall into own Rx of band 46

- The 3rd and 4th order IMD generated by dual uplink of 13+n77 may fall into own Rx of band 46

- The 5th order IMD generated by dual uplink of 13+n77 may fall into own Rx of band 13, 46 and n77

5.122.3	∆TIB and ∆RIB values

The same relaxation values as DC_13_n77 is assumed for DC_13-46_n77.

Table 5.122.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-46_n77 | 13 | 0.5 |
|  | n77 | 0.8 |

Table 5.122.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-46_n77 | 13 | 0 |
|  | 46 | 0 |
|  | n77 | 0 |

5.122.4	Reference sensitivity exceptions

The IMD issues specifc to 3DL/2UL is the IMD3/IMD4/IMD5 of band 13+n77 falling into band 46. The exclusion zone for IMD to band 46 is applied.

Table 5.122.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A-46A_n77A5 | 13 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD3, IMD4, IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_13A-46A_n77A; DC_13A_n77A; 2*fc_13A + fc_n77A; 2*BW_13A + BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 3*fc_13A + fc_n77A; 3*BW_13A + BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; -3*fc_13A + 2*fc_n77A; -3*BW_13A + 2*BW_n77A. |

5.123	DC_1-11_n41

### 5.123.1	Configurations for DC

Table 5.123.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-11A_n41A | DC_1A_n41A DC_11A_n41A |

### 5.123.2	Co-existence studies

Based on co-existence studies of DC_1A-11A_n41A with 2UL, it can get that:

- IMD4 by dual uplink DC_1_n41 will fall into the own Rx band of Band 11

- IMD4 by dual uplink DC_11_n41 will fall into the own Rx band of Band 1

### 5.123.3	∆TIB and ∆RIB values

For DC_1 A-11A_n41A, the TIB,c and RIB,c values are proposed in the tables below.

Table 5.123.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n41 | 1 | 0.5 |
|  | 11 | 0.3 |
|  | n41 | 0.5 |

Table 5.123.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n41 | 1 | 0 |
|  | 11 | 0 |
|  | n41 | 0 |

### 5.123.4	REFSENS requirements

Based the analysis on 5.123.2 above, MSD values due to IMD are shown below:

Table 5.123.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A-11A_n41A | 11 | 1442 | 5 | 25 | 1490 | N/A | FDD | N/A |
|  | n41 | 2520 | 10 | 50 | 2520 | N/A | TDD | N/A |
|  | 1 | 1966 | 5 | 25 | 2156 | 10.2 | FDD | IMD4 |
| DC_1A-11A_n41A | 1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | 11 | 1442 | 5 | 25 | 1490 | 10.6 | FDD | IMD4 |

## 5.124		DC_2-12_n7

### 5.124.1	 Operating bands for DC

Table 5.124.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-12A_n7A DC_2A-12A_n7(2A) | DC_2A_n7A DC_12A_n7A |

### 5.124.2	 Co-existence studies

For UE coexistence study of Band 2 + Band n7, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.124.2-1.

Table 5.124.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 12500 | 12850 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 590 | 720 | 4350 | 4480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | -1130 | -1320 | 3090 | 3290 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6200 | 6390 | 6850 | 7050 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | -2980 | -3230 | 5590 | 5860 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8050 | 8300 | 9350 | 9560 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1180 | 1440 | 8700 | 8960 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 4830 | 5140 | 8090 | 8430 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 9900 | 10210 | 11850 | 12190 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 410 | 730 | 3680 | 4010 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 11200 | 11530 | 10550 | 10870 |

For UE coexistence study of Band 12 + Band n7, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.124.2-2.

Table 5.124.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 12500 | 12850 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1784 | 1871 | 3199 | 3286 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 1172 | 1068 | 4284 | 4441 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3898 | 4002 | 5699 | 5856 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 473 | 352 | 6784 | 7011 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4597 | 4718 | 8199 | 8409 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3568 | 3742 | 6398 | 6572 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 226 | 364 | 9284 | 9581 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 5296 | 5434 | 10699 | 10996 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 3043 | 2852 | 6068 | 6312 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 8898 | 9142 | 7097 | 7288 |

The Rx impacts can be identified as below.

- 5th order IMD products generated by DC_2_n7 uplink may fall into own Rx of band 12.

### 5.124.3	 ∆TIB and ∆RIB values

Table 5.124.3-1: ΔTIB,

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n7 | 2 | 0.5 |
|  | 12 | 0.3 |
|  | n7 | 0.5 |

Table 5.124.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12_n7 | 2 | 0 |
|  | 12 | 0 |
|  | n7 | 0 |

### 5.124.4	Reference sensitivity exceptions

As stated in 5.124.2, for MSD requirement caused by IMDs is specified below accordingly, which is derived from DC_2A-5A_n77A.

Table 5.124.4-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-12A_n7A DC_2A-12A_n7(2A) | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | 12 | 701.5 | 5 | 25 | 731.5 | 4.5 | IMD5 |
|  | n7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |

## 5.125		DC_2-5_n78

### 5.125.1	 Operating bands for DC

Table 5.125.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-5A_n78A DC_2A-5A_n78(2A) | DC_2A_n78A DC_5A_n78A |

### 5.125.2	 Co-existence studies

For UE coexistence study of Band 2 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.125.2-1.

Table 5.125.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 16500 | 19000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1390 | 1950 | 5150 | 5710 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 100 | 520 | 4690 | 5750 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 7000 | 7620 | 8450 | 9510 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1750 | 2430 | 7990 | 9550 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8850 | 9530 | 11750 | 13250 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 2780 | 3900 | 10300 | 11420 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 3600 | 4340 | 11290 | 13350 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 10700 | 11440 | 15050 | 17110 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 2050 | 870 | 6080 | 7700 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 13600 | 15220 | 12150 | 13330 |

For UE coexistence study of Band 5 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.125.2-2.

Table 5.125.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 16500 | 19000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 2451 | 2976 | 4124 | 4649 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2152 | 1602 | 5751 | 6776 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4948 | 5498 | 7424 | 8449 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1328 | 753 | 9051 | 10576 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 5772 | 6347 | 10724 | 12224 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 4902 | 5952 | 8248 | 9298 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 504 | 96 | 12351 | 14376 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 6596 | 7196 | 14024 | 16049 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 5128 | 4053 | 8202 | 9752 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 11548 | 13098 | 9072 | 10147 |

The Rx impacts can be identified as below.

- 5th order IMD products generated by DC_2_n78 uplink may fall into own Rx of band 5.

- 3rd order IMD products generated by DC_5_n78 uplink may fall into own Rx of band 2.

### 5.125.3	 ∆TIB and ∆RIB values

Table 5.125.3-1: ΔTIB,

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n78 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |

Table 5.125.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n78 | 2 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |

### 5.125.4	Reference sensitivity exceptions

As stated in 5.125.2, for MSD requirement caused by IMDs is specified below accordingly, which is reused from DC_2A-5A_n77A.

Table 5.125.5-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-5A_n78A DC_2A-5A_n78(2A) | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | 5 | 842.5 | 5 | 25 | 887.5 | 3.8 | IMD5 |
|  | n78 | 3305 | 5 | 25 | 3305 | N/A | N/A |
|  | 2 | 1907 | 5 | 25 | 1987 | 16.5 | IMD3 |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n78 | 3680 | 5 | 25 | 3680 | N/A | N/A |

## 5.126		DC_7-29_n78

### 5.126.1	 Operating bands for DC

Table 5.126.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_7A-29A_n78A DC_7C-29A_n78A DC_7A-7A-29A_n78A | DC_7A_n78A |

### 5.126.2	 Co-existence studies

For UE coexistence study of Band 7 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.126.2-1.

Table 5.126.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 16500 | 19000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | -1200 | -1840 | 4030 | 5100 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13900 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1460 | 2600 | 11600 | 12740 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 6200 | 6980 | 10630 | 12700 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 13300 | 14080 | 15700 | 17770 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 100 | 1110 | 4760 | 6400 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

The Rx impacts can be identified as below.

- 5th order IMD products generated by DC_7_n78 uplink may fall into own Rx of band 29.

### 5.126.3	 ∆TIB and ∆RIB values

Table 5.126.3-1: ΔTIB,

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-29_n78 | 7 | 0.5 |
|  | n78 | 0.8 |

Table 5.126.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-29_n78 | n78 | 0.5 |

### 5.126.4	Reference sensitivity exceptions

As stated in 5.126.2, for MSD requirement caused by IMDs is specified below accordingly, which is derived from DC_7A-28A_n78A.

Table 5.126.5-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-29A_n78A DC_7C-29A_n78A DC_7A-7A-29A_n78A | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | 29 | N/A | N/A | N/A | 720 | 3.0 | IMD5 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |

5.127	DC_1-38_n28

### 5.127.1	Configurations for DC

Table 5.127.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-38A_n28A | DC_1A_n28A DC_38A_n28A |

### 5.127.2	Co-existence studies

For UE coexistence study of Band 1 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.127.2-1.

Table 5.127.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1277 | 1172 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 | 3257 | 574 | 424 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 | 4708 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5012 | 5237 | 129 | 324 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6463 | 6688 | 4029 | 4224 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2344 | 2554 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1072 | 832 | 7217 | 6932 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 | 1851 | 4534 | 4264 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 | 4972 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 | 6204 | 7166 | 7436 |

For UE coexistence study of Band 38 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.127.2-2.

Table 5.127.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 12850 | 13100 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1822 | 1917 | 3273 | 3368 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1214 | 1074 | 4392 | 4537 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3976 | 4116 | 5843 | 5988 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 511 | 326 | 6962 | 7157 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4679 | 4864 | 8413 | 8608 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3834 | 3644 | 6546 | 6736 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9777 | 9532 | 422 | 192 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6454 | 6214 | 2896 | 3131 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10983 | 11228 | 5382 | 5612 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9116 | 9356 | 7249 | 7484 |

There is no IMD interference for this band combination.

### 5.127.3	∆TIB and ∆RIB values

For DC_1-38_n28, the TIB,c and RIB,c values are reused from the LTE combination DC_66_n7, and are given in the tables below.

Table 5.127.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-38_n28 | 1 | 0.5 |
|  | 38 | 0.5 |
|  | n28 | 0.6 |

Table 5.127.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-38_n28 | 1 | 0 |
|  | 38 | 0 |
|  | n28 | 0.2 |

### 5.127.4	Reference sensitivity exceptions

There is no MSD issue for this band combination.

5.128	DC_3-38_n28

### 5.128.1	Configurations for DC

Table 5.128.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-38A_n28A DC_3C-38A_n28A | DC_3A_n28A DC_38A_n28A |

### 5.128.2	Co-existence studies

For UE coexistence study of Band 38 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.128.2-1.

Table 5.128.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 12850 | 13100 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1822 | 1917 | 3273 | 3368 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1214 | 1074 | 4392 | 4537 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3976 | 4116 | 5843 | 5988 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 511 | 326 | 6962 | 7157 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4679 | 4864 | 8413 | 8608 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3834 | 3644 | 6546 | 6736 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9777 | 9532 | 422 | 192 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6454 | 6214 | 2896 | 3131 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10983 | 11228 | 5382 | 5612 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9116 | 9356 | 7249 | 7484 |

For UE coexistence study of Band 3 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.128.2-2.

Table 5.128.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 962 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 379 | 214 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2164 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1282 | 1027 | 6437 | 6092 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3949 | 3634 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

IMD2 may fall into Rx of band 3 when band 38 and n28 transmit.

### 5.128.3	∆TIB and ∆RIB values

For DC_3-38_n28, the TIB,c and RIB,c values are reused from the LTE combination DC_66_n7, and are given in the tables below.

Table 5.128.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-38_n28 | 3 | 0.5 |
|  | 38 | 0.5 |
|  | n28 | 0.6 |

Table 5.128.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-38_n28 | 3 | 0 |
|  | 38 | 0 |
|  | n28 | 0.2 |

### 5.128.4	Reference sensitivity exceptions

The MSD requirements due to IMD2 from DC_3A-41A_n28A can be reused for DC_3-38_n28. Thus, the requirements can be specified as below.

Table 5.128.4-1 MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-38A_n28A DC_3C-38A_n28A | 38 | 2575 | 5 | 25 | 2575 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 26 | IMD2 |

5.129	DC_14-30_n66

### 5.129.1	Operating bands for EN-DC

Table 5.129.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 14-30_n66 | CA_14-30 | n66 |  |

5.129.2	Configuration for DC

Table 5.129.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-30A_n66A | DC_14A_n66A DC_30A_n66A | CA_14A-30A | n66A |

### 5.129.3		∆TIB and ∆RIB values

For DC_14-30_n66, the TIB,c and RIB,c values are reused from CA_14-30-66 and are given in the tables below.

Table 5.129.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-30_n66 | 14 | 0.3 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |

Table 5.129.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-30_n66 | 14 | 0 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |

### 5.129.4	REFSENS requirements

There are no IMD impact from UL 14_n66 affecting DL band 30, and there are no IMD impact from UL 30_n66 affecting DL band 14. Therefore, MSD is not needed to be defined.

## 5.130	DC_14-30_n2

### 5.130.1	Operating bands for EN-DC

Table 5.130.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 14-30_n2 | CA_14-30 | n2 |  |

5.130.2	Configuration for DC

Table 5.130.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-30A_n2A | DC_14A_n2A DC_30A_n2A | CA_14A-30A | n2A |

### 5.130.3		∆TIB and ∆RIB values

For DC_14-30_n2, the TIB,c and RIB,c values are reused from DC_12-30_n2 and are given in the tables below.

Table 5.130.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-30_n2 | 14 | 0.3 |
|  | 30 | 0.3 |
|  | n2 | 0.5 |

Table 5.130.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-30_n2 | 14 | 0 |
|  | 30 | 0.5 |
|  | n2 | 0.4 |

### 5.130.4	REFSENS requirements

There are no IMD impact from UL 14_n2 affecting DL band 30, and there are no IMD impact from UL 30_n2 affecting DL band 14. Therefore, MSD is not needed to be defined.

## 5.131	DC_2-(n)5

### 5.131.1	Operating bands for EN-DC

Table 5.131.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-(n)5 | CA_2-5 | n5 |  |

5.131.2	Configuration for DC

Table 5.131.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-(n)5AA | DC_2A_n5A DC_(n)5AA1 | CA_2A-5A | n5A |
| NOTE1: Only single switched UL is supported |  |  |  |

### 5.131.3		∆TIB and ∆RIB values

For DC_2-(n)5, the TIB,c and RIB,c values are reused from DC_2_n5 and are given in the tables below.

Table 5.131.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-(n)5 | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n5 | 0.3 |

Table 5.131.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-(n)5 | 2 | 0 |
|  | 5 | 0 |
|  | n5 | 0 |

### 5.131.4	REFSENS requirements

No additional MSD requirements need to be defined.

## 5.132	DC_30-(n)5

### 5.132.1	Operating bands for EN-DC

Table 5.132.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_30-(n)5 | CA_30-5 | n5 |  |

5.132.2	Configuration for DC

Table 5.132.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_30A-(n)5AA | DC_30A_n5A DC_(n)5AA1 | CA_30A-5A | n5A |
| NOTE1: Only single switched UL is supported |  |  |  |

### 5.132.3		∆TIB and ∆RIB values

For DC_30-(n)5, the TIB,c and RIB,c values are reused from DC_30_n5 and are given in the tables below.

Table 5.132.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_30-(n)5 | 30 | 0.3 |
|  | 5 | 0.3 |
|  | n5 | 0.3 |

Table 5.132.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_30-(n)5 | 30 | 0 |
|  | 5 | 0 |
|  | n5 | 0 |

### 5.132.4	REFSENS requirements

No additional MSD requirements need to be defined.

## 5.133	DC_28-40_n78

### 5.133.1	Operating bands for EN-DC

Table 5.133.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 28-40_n78 | CA_28-40 | n78 |  |

5.133.2	Configuration for DC

Table 5.133.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_28A-40A_n78A | DC_28A_n78A DC_40A_n78A | CA_28A-40A | n78A |
| DC_28A-40C_n78A | DC_28A_n78A DC_40A_n78A | CA_28A-40C | n78A |

### 5.133.3		∆TIB and ∆RIB values

For DC_28-40_n78, the TIB,c and RIB,c values are reused from DC_28_n40-n78 and are given in the tables below.

Table 5.133.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28-40_n78 | 28 | 0.5 |
|  | 40 | 0.3 |
|  | n78 | 0.8 |

Table 5.133.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28-40_n78 | 28 | 0.2 |
|  | 40 | 0.4 |
|  | n78 | 0.5 |

### 5.133.4	REFSENS requirements

UL 28_n78 might cause IMD3 into 40 DL. MSD value for 40 is chosen as in DC_28A_n1A-n78A.

UL 40_n78 might cause IMD3 into the sidelobe of 28 DL. MSD value for 28 is chosen as in DC_20A-40A_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A-40A_n78A DC_28A-40C_n78A | 28 | N/A | 5 | 25 | 800.5 | 11 | IMD3 |
|  | 40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | N/A | N/A |
|  | 40 | 2320 | 5 | 25 | 2320 | 15.7 | IMD3 |
|  | n78 | 3750 | 10 | 50 | 3750 | N/A | N/A |

## 5.134	DC_7-13_n25

### 5.134.1	Operating bands for EN-DC

Table 5.134.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-13_n25 | CA_7-13 | n25 |  |

5.134.2	Configuration for DC

Table 5.134.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-13A_n25A | DC_7A_n25A DC_13A_n25A | CA_7A-13A | n25A |
| DC_7A-7A-13A_n25A | DC_7A_n25A DC_13A_n25A | CA_7A-7A-13A | n25A |
| DC_7C-13A_n25A | DC_7A_n25A DC_13A_n25A | CA_7C-13A | n25A |

### 5.134.3		∆TIB and ∆RIB values

For DC_7-13_n25, the TIB,c and RIB,c values are reused from DC_7-28_n2 and are given in the tables below.

Table 5.134.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-13_n25 DC_7-7-13_n25 | 7 | 0.5 |
|  | 13 | 0.3 |
|  | n25 | 0.5 |

Table 5.134.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-13_n25 DC_7-7-13_n25 | 7 | 0 |
|  | 13 | 0 |
|  | n25 | 0 |

### 5.134.4	REFSENS requirements

There are no IMD impact from UL 7_n25 affecting DL band 13.

There are IMD2 impact from UL 13_n25 affecting DL band 7. The MSD value is derived from DC_7A-28A_n2A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-13A_n25A DC_7A-7A-13A_n25A DC_7C-13A_n25A | 7 | 2542 | 10 | 50 | 2662 | 27.6 | IMD2 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |

## 5.135	DC_7-66_n25

### 5.135.1	Operating bands for EN-DC

Table 5.135.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-66_n25 | CA_7-66 | n25 |  |

5.135.2	Configuration for DC

Table 5.135.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-66A_n25A | DC_7A_n25A DC_66A_n25A | CA_7A-66A | n25A |
| DC_7A-7A-66A_n25A | DC_7A_n25A DC_66A_n25A | CA_7A-7A-66A | n25A |
| DC_7C-66A_n25A | DC_7A_n25A DC_66A_n25A | CA_7C-66A | n25A |

### 5.135.3		∆TIB and ∆RIB values

For DC_7-66_n25, the TIB,c and RIB,c values are reused from DC_2-7_n66 and are given in the tables below.

Table 5.135.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n25 DC_7-7-66_n25 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |

Table 5.135.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n25 DC_7-7-66_n25 | 7 | 0.3 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |

### 5.135.4	REFSENS requirements

There are no IMD impact from UL 7_n25 affecting DL band 66.

There are no IMD impact from UL 66_n25 affecting DL band 7.

## 5.136	DC_3-7_n3

### 5.136.1	Operating bands for EN-DC

Table 5.136.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-7_n3 | CA_3-7 | n3 |  |

5.136.2	Configuration for DC

Table 5.136.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n3A | DC_3A_n3A1 DC_7A_n3A | CA_3A-7A | n3A |
| DC_3A-7C_n3A | DC_3A_n3A1 DC_7A_n3A DC_7C_n3A | CA_3A-7C | n3A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.136.3		∆TIB and ∆RIB values

For DC_3-7_n3, the TIB,c and RIB,c values are reused from DC_7_n3 and are given in the tables below.

Table 5.136.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n3 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n3 | 0.5 |

Table 5.136.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n3 | 3 | 0 |
|  | 7 | 0 |
|  | n3 | 0 |

### 5.136.4	REFSENS requirements

No additional MSD requirements need to be defined.

## 5.137	DC_3-28_n3

### 5.137.1	Operating bands for EN-DC

Table 5.137.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-28_n3 | CA_3-28 | n3 |  |

5.137.2	Configuration for DC

Table 5.137.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-28A_n3A | DC_3A_n3A1 DC_28A_n3A | CA_3A-28A | n3A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.137.3		∆TIB and ∆RIB values

For DC_3-28_n3, the TIB,c and RIB,c values are reused from DC_28_n3 and are given in the tables below.

Table 5.137.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n3 | 3 | 0.3 |
|  | 28 | 0.3 |
|  | n3 | 0.3 |

Table 5.137.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n3 | 3 | 0 |
|  | 28 | 0 |
|  | n3 | 0 |

### 5.137.4	REFSENS requirements

No additional MSD requirements need to be defined.

## 5.138	DC_3-3-7_n8, DC_3-7-7_n8, DC_3-3-7-7_n8

### 5.138.1	Configuration for DC

Table 5.138.1-1:  Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-3A-7A_n8A DC_3A-7A-7A_n8A DC_3A-3A-7A-7A_n8A | DC_3A_n8A DC_7A_n8A |

### 5.138.2	Co-existence studies

The co-existence studies can be covered by the fallback combination, DC_3-7_n8, which was specified in Rel.16. Based on the studies in TR 37.716-21-11 section 5.1.124, the IMD2 and IMD3 for Tx band 3 + band n8 may fall into Rx of band 7.

### 5.138.3	∆TIB and ∆RIB values

For DC_3-3_n8, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 5.138.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3-7_n8 DC_3-7-7_n8 DC_3-3-7-7_n8 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n8 | 0.6 |

Table 5.138.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-3-7_n8 DC_3-7-7_n8 DC_3-3-7-7_n8 | 3 | 0 |
|  | 7 | 0 |
|  | n8 | 0.2 |

### 5.138.4	Reference sensitivity exceptions

The needed exceptions can be covered by the fallback combination, DC_3-7_n8, which was specified in Rel.16. No additional exception is foreseen.

5.139	DC_13-46_n66

### 5.139.1	Configurations for DC

Table 5.139.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_13A-46A_n66A3 | DC_13A_n66A |
| NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |

5.139.2	Co-existence studies

Co-existence studies have been performed for lower order combinations of DC_13A_n66, where:

- 4th and 5th IMD product caused by DC_13A_n66A may fall into own Rx of band 46.

### 5.139.3	∆TIB and ∆RIB values

### Values are reused from CA including same bands, CA_13-46-66, as given in 36.101.

Table 5.139.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_13-46_n66 | 13 | 0.3 |
|  | 46 | 0 |
|  | n66 | 0.3 |

Table 5.139.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_13-46_n66 | 13 | 0 |
|  | 46 | 0 |
|  | n66 | 0 |

### 5.139.4	Reference sensitivity exceptions

Based on co-existence analysis it is found that MSD is needed due to 4th and 5th IMD falling into own Rx band of 46. Therefor additional MSD is needed defined in Table 7.3B.2.3.5.2-1 of 38.101-3.

Table 5. x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_13A-46A_n66A5 | 13 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD4, IMD5 |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. DC_13A-46A_n66A; DC_13A_n66A; 3*fc_13A + fc_n66A; BW_13A + 2*BW_n66A. DC_13A-46A_n66A; DC_13A_n66A; 2*fc_13A + 3*fc_n66A; BW_13A + 2*BW_n66A. |

5.140	DC_46-48_n66

### 5.140.1	Configurations for DC

Table 5.140.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_46A-48A_n66A3 DC_46C-48A_n66A3 DC_46D-48A_n66A3 DC_46E-48A_n66A3 | DC_48A_n66A |
| NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |

5.140.2	Co-existence studies

Co-existence studies have been performed for lower order combinations of DC_48A_n66, where:

- 2nd and 3rd IMD product caused by DC_48A_n66A may fall into own Rx of band 46.

### 5.140.3	∆TIB and ∆RIB values

### Values are reused from CA including same bands, CA_46-48-66, as given in 36.101.

Table 5.140.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_46-48_n66 | 46 | 0 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |

Table 5.140.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_46-48_n66 | 46 | 0 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |

### 5.140.4	Reference sensitivity exceptions

Based on co-existence analysis it is found that MSD is needed due to 2nd and 3rd IMD falling into own Rx band of 46. Therefor additional MSD is needed defined in Table 7.3B.2.3.5.2-1 of 38.101-3.

Table 5. x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_46A-48A_n66A5 DC_46C-48A_n66A5 DC_46D-48A_n66A5 DC_46E-48A_n66A5 | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |
|  | 48 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; fc_48A + fc_n66A; BW_48A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; 2*fc_48A + fc_n66A; 2*BW_48A + BW_n66A. |

5.141	DC_2-46_n5

### 5.141.1	Configurations for DC

Table 5.141.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_2A-46A_n5A3 DC_2A-46C_n5A3 DC_2A-46D_n5A3 DC_2A-46E_n5A3 | DC_2A_n5A |
| NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |

5.141.2	Co-existence studies

Co-existence studies have been performed for lower order combinations of DC_2A_n5, where:

- 4th and 5th IMD product caused by DC_2A_n5A may fall into own Rx of band 46.

### 5.141.3	∆TIB and ∆RIB values

### Values are reused from CA including same bands, CA_2-5-46, as given in 36.101.

Table 5.141.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-46_n5 | 2 | 0.3 |
|  | 46 | 0 |
|  | n5 | 0.3 |

Table 5.141.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-46_n5 | 2 | 0 |
|  | 46 | 0 |
|  | n5 | 0 |

### 5.141.4	Reference sensitivity exceptions

Based on co-existence analysis it is found that MSD is needed due to 4th and 5th IMD falling into own Rx band of 46. Therefor additional MSD is needed defined in Table 7.3B.2.3.5.2-1 of 38.101-3.

Table 5. x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-46A_n5A5 DC_2A-46C_n5A5 DC_2A-46D_n5A5 DC_2A-46E_n5A5 | 2 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD4, IMD5 |
|  | n5 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. DC_2A-46_n5A; DC_2A_n5A; 2*fc_2A + 2*fc_n5A; BW_2A + 2*BW_n5A. DC_2A-46_n5A; DC_2A_n5A; fc_2A + 4*fc_n5A; BW_2*2A + BW_n5A. |

5.142	DC_46-48_n5

### 5.142.1	Configurations for DC

Table 5.142.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_46A-48A_n5A3 DC_46C-48A_n5A3 DC_46D-48A_n5A3 DC_46E-48A_n5A3 | DC_48A_n5A |
| NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |

5.142.2	Co-existence studies

Co-existence studies have been performed for lower order combinations of DC_48A_n5, where:

- 3rd and 4th IMD product caused by DC_48A_n5A may fall into own Rx of band 46.

### 5.142.3	∆TIB and ∆RIB values

Values are reused from CA including same bands as given in 36.101.

Table 5.142.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_46-48_n5 | 46 | 0 |
|  | 48 | 0.8 |
|  | n5 | 0.3 |

Table 5.142.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_46-48_n5 | 46 | 0 |
|  | 48 | 0.5 |
|  | n5 | 0 |

### 5.142.4	Reference sensitivity exceptions

Based on co-existence analysis it is found that MSD is needed due to 3rd and 4th IMD falling into own Rx band of 46. Therefor additional MSD is needed defined in Table 7.3B.2.3.5.2-1 of 38.101-3.

Table 5.142.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_46A-48A_n5A5 DC_46C-48A_n5A5 DC_46D-48A_n5A5 DC_46E-48A_n5A5 | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |
|  | 48 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n5 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + fc_n5A; BW_48A + 2*BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + 2*fc_n5A; BW_2*48A + BW_n5A. |

5.143	DC_20-28_n1

### 5.143.1	Configurations for DC

Table 5.143.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_20A-28A_n1A | DC_20A_n1A DC_28A_n1A |

### 5.143.2	Co-existence studies

For UE coexistence study of Band 28 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.143.2-1.

Table 5.143.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1277 | 1172 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 | 3257 | 574 | 424 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 | 4708 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5012 | 5237 | 129 | 324 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6463 | 6688 | 4029 | 4224 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2344 | 2554 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1072 | 832 | 7217 | 6932 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 | 1851 | 4534 | 4264 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 | 4972 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 | 6204 | 7166 | 7436 |

For UE coexistence study of Band 20 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.143.2-2.

Table 5.143.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 832 | 862 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1664 | 1724 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2496 | 2586 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 3328 | 3448 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 4160 | 4310 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 | 1058 | 2752 | 2842 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 | 3128 | 316 | 196 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 | 4822 | 3584 | 3704 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 | 5108 | 516 | 666 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 | 6802 | 4416 | 4566 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2116 | 2296 | 5504 | 5684 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 | 1348 | 7088 | 6818 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 | 1464 | 4276 | 4036 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 | 5428 | 8512 | 8782 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6546 | 7424 | 7664 |

### 5.143.3	∆TIB and ∆RIB values

For DC_20-28_n1, the TIB,c and RIB,c values are reused from DC_1-20_n8, and are given in the tables below.

Table 5.143.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-28_n1 | n1 | 0.5 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |

Table 5.143.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-28_n1 | n1 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |

### 5.143.4	Reference sensitivity exceptions

There is no MSD requirements for this band combination.

5.144	DC_8-20_n1

### 5.144.1	Configurations for DC

Table 5.144.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-20A_n1A | DC_8A_n1A DC_20A_n1A |

### 5.144.2	Co-existence studies

For UE coexistence study of Band 8 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.144.2-1.

Table 5.144.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

For UE coexistence study of Band 20 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.144.2-2.

Table 5.144.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 832 | 862 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1664 | 1724 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2496 | 2586 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 3328 | 3448 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 4160 | 4310 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 | 1058 | 2752 | 2842 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 | 3128 | 316 | 196 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 | 4822 | 3584 | 3704 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 | 5108 | 516 | 666 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 | 6802 | 4416 | 4566 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2116 | 2296 | 5504 | 5684 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 | 1348 | 7088 | 6818 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 | 1464 | 4276 | 4036 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 | 5428 | 8512 | 8782 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6546 | 7424 | 7664 |

IMD4 of Tx band 8 + band n1 may fall into Rx of band 20.

### 5.144.3	∆TIB and ∆RIB values

For DC_8-20_n1, the TIB,c and RIB,c values are reused from DC_1-20_n8, and are given in the tables below.

Table 5.144.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-20_n1 | n1 | 0.3 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |

Table 5.144.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-20_n1 | n1 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |

### 5.144.4	Reference sensitivity exceptions

MSD due to IMD4 for DC_1-20_n8 can be reused for this combination, referring to approved R4-2001051.

Table 5.144.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8-20_n1 | n1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 846 | 5 | 25 | 805 | 11.5 | IMD4 |

5.145	DC_8-20_n3

### 5.145.1	Configurations for DC

Table 5.145.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-20A_n3A | DC_8A_n3A DC_20A_n3A |

### 5.145.2	Co-existence studies

For UE coexistence study of Band 8 + Band n3, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.145.2-1.

Table 5.145.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 905 | 795 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2505 | 2690 | 25 | 120 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4300 | 4485 | 3470 | 3615 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4215 | 4475 | 855 | 1035 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6010 | 6270 | 4350 | 4530 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1590 | 1810 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950 | 1735 | 6260 | 5925 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 675 | 930 | 3595 | 3300 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5230 | 5445 | 7720 | 8055 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6060 | 6315 | 6890 | 7185 |

For UE coexistence study of Band 20 + Band n3, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.145.2-2.

Table 5.145.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 832 | 862 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1664 | 1724 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2496 | 2586 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 3328 | 3448 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 4160 | 4310 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 953 | 848 | 2542 | 2647 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558 | 2738 | 121 | 14 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4252 | 4432 | 3374 | 3509 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4268 | 4523 | 711 | 876 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5962 | 6217 | 4206 | 4371 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1696 | 1906 | 5084 | 5294 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1738 | 1543 | 6308 | 5978 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 834 | 1074 | 3691 | 3406 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5038 | 5233 | 7672 | 8002 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6156 | 6794 | 7079 |

IMD2 and IMD5 of Tx band 8 + band n3 may fall into Rx of band 20.

IMD2 and IMD5 of Tx band 20 + band n3 may fall into Rx of band 8.

### 5.145.3	∆TIB and ∆RIB values

For DC_8-20_n3, the TIB,c and RIB,c values are reused from DC_3-20_n8, and are given in the tables below.

Table 5.145.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-20_n3 | n3 | 0.3 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |

Table 5.145.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-20_n3 | n3 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |

### 5.145.4	Reference sensitivity exceptions

The MSD due to IMD2 for DC_3-20_n8 can be reused for this combination, referring to approved R4-2001053.

Table 5.145.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8-20_n3 | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 27 | IMD24 ¦fn8-fB3¦ |
| DC_8-20_n3 | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | 8 | 890 | 5 | 25 | 930 | 27 | IMD24 ¦fn20-fB3¦ |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

## 5.146	DC_2-13_n25

### 5.146.1	Operating bands for EN-DC

Table 5.146.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-13_n25 | CA_2-13 | n25 |  |

5.146.2	Configuration for DC

Table 5.146.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-13A_n25A13,14 | DC_13A_n25A | CA_2A-13A | n25A |
| NOTE 13: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combinations and for the Band 2 and Band n25 combinations. NOTE 14: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |

### 5.146.3		∆TIB and ∆RIB values

For DC_2-13_n25, the TIB,c and RIB,c values are reused from DC_13_n2 and are given in the tables below.

Table 5.146.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n25 | 2 | 0.3 |
|  | 13 | 0.3 |
|  | n25 | 0.3 |

Table 5.146.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n25 | 2 | 0 |
|  | 13 | 0 |
|  | n25 | 0 |

### 5.146.4	REFSENS requirements

There are no IMD impact from UL 13_n25 affecting DL band 2.

## 5.147	DC_2-66_n25

### 5.147.1	Operating bands for EN-DC

Table 5.147.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-66_n25 | CA_2-66 | n25 | DC_66_n25 |

5.147.2	Configuration for DC

Table 5.147.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A_n25A13,14 | DC_66A_n25A | CA_2A-66A | n25A |
| NOTE 13: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combinations and for the Band 2 and Band n25 combinations. NOTE 14: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |

### 5.147.3		∆TIB and ∆RIB values

For DC_2-66_n25, the TIB,c and RIB,c values are reused from DC_66_n2 and are given in the tables below.

Table 5.147.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n25 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |

Table 5.147.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n25 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |

### 5.147.4	REFSENS requirements

There is IMD3 impact from UL 66_n25 affecting DL band 2. The MSD value is derived from DC_66A_n25A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-66A_n25A | 2 | 1895 | 5 | 25 | 1975 | 20 | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1875 | 5 | 25 | 1955 | N/A | N/A |

5.148	DC_2-12_n77

5.148.1	Configurations for DC

Table 5.148.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-12A_n77A | DC_2A_n77A DC_12A_n77A |

5.148.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	There are no IMD products produced by Band 2 and n77 that might fall in Rx of band 12.

-	IMD3 and IMD4 products are produced by Band 12 and n77 that might fall in Rx of band 2.

5.148.3	∆TIB and ∆RIB values

For DC_2-12_n77, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_n2-n12-n77.

Table 5.148.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n77 | 2 | 0.6 |
|  | 12 | 0.3 |
|  | n77 | 0.8 |

Table 5.148.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12_n77 | 2 | 0.2 |
|  | 12 | 0.2 |
|  | n77 | 0.5 |

5.148.4	Reference sensitivity exceptions

Table 5.148.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from CA-n2A-n12A-n77A.

Table 5.148.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-12A_n77A | 2 | 1880 | 5 | 25 | 1960 | 16.5 | IMD39,11 |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | N/A |
| NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

5.149	DC_2-14_n77

5.149.1	Configurations for DC

Table 5.149.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-14A_n77A | DC_2A_n77A DC_14A_n77A |

5.149.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	There are no IMD products produced by Band 2 and n77 that might fall in Rx of band 14.

-	IMD3 products are produced by Band 14 and n77 that might fall in Rx of band 2.

5.149.3	∆TIB and ∆RIB values

For DC_2-14_n77, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_n2-n14-n77.

Table 5.149.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14_n77 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | n77 | 0.8 |

Table 5.149.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-14_n77 | 2 | 0.2 |
|  | 14 | 0.2 |
|  | n77 | 0.5 |

5.149.4	Reference sensitivity exceptions

Table 5.149.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from CA-n2A-n14A-n77A.

Table 5.149.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-14A_n77A | 2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD3 |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |

5.150	DC_2-29_n77

5.150.1	Configurations for DC

Table 5.150.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-29A_n77A | DC_2A_n77A |

5.150.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback mode, DC_2A_n77A. It can be seen that:

-	There are no IMD products produced by Band 2 and n77 that might fall in Rx of band 29.

Although DC_29_n77 is not defined, 5th order harmonic mixing is produced from band n77 UL that might fall in Rx of band 29.

5.150.3	∆TIB and ∆RIB values

For DC_2-29_n77, the TIB,c and RIB,c values are given in the tables below. The values are proposed to be reused from the values for CA_n2-n12-n77 with the exception that TIB,c would not be applicable for band 29.

Table 5.150.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29_n77 | 2 | 0.6 |
|  | n77 | 0.8 |

Table 5.150.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-29_n77 | 2 | 0.2 |
|  | 29 | 0.2 |
|  | n77 | 0.5 |

5.150.4	Reference sensitivity exceptions

DC_2-29_n77 5th order harmonic mixing impact to band 29 can be derived from DC_12-n77 values.

Table 5.150.4-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77zz | 292 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE ZZ: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.150.4-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 29 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |

5.151	DC_2-30_n77

5.151.1	Configurations for DC

Table 5.151.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-30A_n77A | DC_2A_n77A DC_30A_n77A |

5.151.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	IMD4 and IMD5 products are produced by Band 2 and n77 that might fall in Rx of band 30.

-	IMD4 products are produced by Band 30 and n77 that might fall in Rx of band 2.

5.151.3	∆TIB and ∆RIB values

For DC_2-30_n77, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_n2-n30-n77.

Table 5.151.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-30_n77 | 2 | 0.6 |
|  | 30 | 0.3 |
|  | n77 | 0.8 |

Table 5.151.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-30_n77 | 2 | 0.2 |
|  | 30 | 0 |
|  | n77 | 0.5 |

5.151.4	Reference sensitivity exceptions

Table 5.151.4-1 shows the required MSD levels for the DC configuration. The required MSD values for IMD4 are derived from CA-n2A-n30A-n77A. The required MSD value for IMD5 is derived from DC_2A-7A_n77A.

Table 5.151.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-30A_n77A | 2 | 1906 | 5 | 25 | 1986 | 8.6 | IMD411 |
|  | 30 | 2312 | 5 | 25 | 2357 | N/A | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 2 | 1905 | 5 | 25 | 1985 | N/A | N/A |
|  | 30 | 2309 | 5 | 25 | 2354 | 10.6 | IMD411 |
|  | n77 | 3361 | 10 | 50 | 3361 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | 30 | 2309 | 5 | 25 | 2354 | 3.4 | IMD5 |
|  | n77 | 3967 | 10 | 50 | 3967 | N/A | N/A |
| NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

5.152	DC_5-30_n77

5.152.1	Configurations for DC

Table 5.152.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A-30A_n77A | DC_5A_n77A DC_30A_n77A |

5.152.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	IMD3 products are produced by Band 5 and n77 that might fall in Rx of band 30.

-	IMD3 and IMD5 products are produced by Band 30 and n77 that might fall in Rx of band 5.

5.152.3	∆TIB and ∆RIB values

For DC_5-30_n77, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_n5-n30-n77.

Table 5.152.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-30_n77 | 5 | 0.6 |
|  | 30 | 0.3 |
|  | n77 | 0.8 |

Table 5.152.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-30_n77 | 5 | 0.2 |
|  | 30 | 0 |
|  | n77 | 0.5 |

5.152.4	Reference sensitivity exceptions

Table 5.152.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from CA-n5A-n30A-n77A.

Table 5.152.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-30A_n77A | 5 | 835 | 5 | 25 | 880 | 15.2 | IMD34 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 13.2 | IMD311 |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

5.153	DC_12-30_n77

5.153.1	Configurations for DC

Table 5.153.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_12A-30A_n77A | DC_12A_n77A DC_30A_n77A |

5.153.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	IMD3 products are produced by Band 12 and n77 that might fall in Rx of band 30.

-	IMD3 and IMD5 products are produced by Band 30 and n77 might fall in Rx of band 12.

5.153.3	∆TIB and ∆RIB values

For DC_12-30_n77, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_n12-n30-n77.

Table 5.153.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-30_n77 | 12 | 0.5 |
|  | 30 | 0.3 |
|  | n77 | 0.5 |

Table 5.153.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-30_n77 | 12 | 0.2 |
|  | 30 | 0 |
|  | n77 | 0.5 |

5.153.4	Reference sensitivity exceptions

Table 5.153.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from CA-n12A-n30A-n77A.

Table 5.153.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-30A_n77A | 12 | 710 | 5 | 25 | 740 | 15.2 | IMD34 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | N/A |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 13.2 | IMD3 |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

5.154	DC_12-66_n77

5.154.1	Configurations for DC

Table 5.154.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_12A-66A_n77A | DC_12A_n77A DC_66A_n77A |

5.154.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	IMD3 products are produced by Band 12 and n77 that might fall in Rx of band 66.

-	IMD3 products are produced by Band 66 and n77 that might fall in Rx of band 12.

5.154.3	∆TIB and ∆RIB values

For DC_12-66_n77, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_n12-n66-n77.

Table 5.154.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n77 | 12 | 0.8 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.154.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-66_n77 | 12 | 0.5 |
|  | 66 | 0.5 |
|  | n77 | 0.5 |

5.154.4	Reference sensitivity exceptions

Table 5.154.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from CA-n12A-n66A-n77A.

Table 5.154.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-66A_n77A | 12 | 710 | 5 | 25 | 740 | 15.2 | IMD311 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | N/A |
|  | 12 | 707 | 5 | 25 | 737 | N/A | N/A |
|  | 66 | 1726 | 5 | 25 | 2126 | 13.2 | IMD3 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

5.155	DC_14-30_n77

5.155.1	Configurations for DC

Table 5.155.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_14A-30A_n77A | DC_14A_n77A DC_30A_n77A |

5.155.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	IMD3 products are produced by Band 14 and n77 that might fall in Rx of band 30.

-	IMD3 and IMD5 products are produced by Band 30 and n77 that might fall in Rx of band 14.

5.155.3	∆TIB and ∆RIB values

For DC_14-30_n77, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_n14-n30-n77.

Table 5.155.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-30_n77 | 14 | 0.5 |
|  | 30 | 0.3 |
|  | n77 | 0.8 |

Table 5.155.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-30_n77 | 14 | 0.2 |
|  | 30 | 0 |
|  | n77 | 0.5 |

5.155.4	Reference sensitivity exceptions

Table 5.155.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from CA-n14A-n30A-n77A.

Table 5.155.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A-30A_n77A | 14 | 793 | 5 | 25 | 763 | 15.2 | IMD34 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3857 | 10 | 50 | 3857 | N/A | N/A |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 13.2 | IMD3 |
|  | n77 | 3941 | 10 | 50 | 3941 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

5.156	DC_14-66_n77

5.156.1	Configurations for DC

Table 5.156.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_14A-66A_n77A | DC_14A_n77A DC_66A_n77A |

5.156.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	IMD3 products are produced by Band 14 and n77 that might fall in Rx of band 66.

-	IMD3 products are produced by Band 66 and n77 that might fall in Rx of band 14.

5.156.3	∆TIB and ∆RIB values

For DC_14-66_n77, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_n14-n66-n77.

Table 5.156.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-66_n77 | 14 | 0.6 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.156.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-66_n77 | 14 | 0.2 |
|  | 66 | 0.5 |
|  | n77 | 0.5 |

5.156.4	Reference sensitivity exceptions

Table 5.156.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from CA-n14A-n66A-n77A.

Table 5.156.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A-66A_n77A | 14 | 793 | 5 | 25 | 763 | 15.2 | IMD311 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n77 | 4188 | 10 | 50 | 4188 | N/A | N/A |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1755 | 5 | 25 | 2155 | 13.2 | IMD3 |
|  | n77 | 3741 | 10 | 50 | 3741 | N/A | N/A |
| NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

5.157	DC_29-30_n77

5.157.1	Configurations for DC

Table 5.157.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_29A-30A_n77A | DC_30A_n77A |

5.157.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	IMD3 and IMD5 products are produced by Band 30 and n77 that might fall in Rx of band 29.

Although DC_29_n77 is not defined, 5th order harmonic mixing is produced from band n77 UL that might fall in Rx of band 29.

5.157.3	∆TIB and ∆RIB values

For DC_29-30_n77, the TIB,c and RIB,c values are given in the tables below. The values are proposed to be reused from the values for CA_n12-n30-n77 with the exception that TIB,c would not be applicable for band 29.

Table 5.157.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_29-30_n77 | 30 | 0.3 |
|  | n77 | 0.5 |

Table 5.157.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_29-30_n77 | 29 | 0.2 |
|  | 30 | 0 |
|  | n77 | 0.5 |

5.157.4	Reference sensitivity exceptions

Table 5.157.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from CA-n12A-n30A-n77A.

Table 5.157.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_29A-30A_n77A | 29 | N/A | 5 | N/A | 722 | 15.2 | IMD34 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3898 | 10 | 50 | 3898 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

DC_29-30_n77 5th order harmonic mixing impact to band 29 can be derived from DC_12-n77 values.

Table 5.157.4-2: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77ZZ | 292 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE ZZ: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.157.4-3: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 29 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |

5.158	DC_29-66_n77

5.158.1	Configurations for DC

Table 5.158.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_29A-66A_n77A | DC_66A_n77A |

5.158.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	IMD3 products are produced by Band 66 and n77 that might fall in Rx of band 29.

Although DC_29_n77 is not defined, 5th order harmonic mixing is produced from band n77 UL that might fall in Rx of band 29.

5.158.3	∆TIB and ∆RIB values

For DC_29-66_n77, the TIB,c and RIB,c values are given in the tables below. The values are proposed to be reused from the values for CA_n12-n66-n77 with the exception that TIB,c would not be applicable for band 29.

Table 5.158.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_29-66_n77 | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.158.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_29-66_n77 | 29 | 0.5 |
|  | 66 | 0.5 |
|  | n77 | 0.5 |

5.158.4	Reference sensitivity exceptions

Table 5.158.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from CA-n12A-n66A-n77A.

Table 5.158.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_29A-66A_n77A | 29 | N/A | 5 | N/A | 722 | 15.2 | IMD311 |
|  | 66 | 1734 | 5 | 25 | 2134 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
| NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

DC_29-66_n77 5th order harmonic mixing impact to band 29 can be derived from DC_12-n77 values.

Table 5.158.4-2: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77ZZ | 292 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE ZZ: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.158.4-3: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 29 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |

5.159	DC_30-66_n77

5.159.1	Configurations for DC

Table 5.159.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_30A-66A_n77A | DC_30A_n77A DC_66A_n77A |

5.159.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	IMD4 products are produced by Band 30 and n77 that might fall in Rx of band 66.

-	IMD2 and IMD5 products are produced by Band 66 and n77 that might fall in Rx of band 30.

5.159.3	∆TIB and ∆RIB values

For DC_30-66_n77, the TIB,c and RIB,c values are given in the tables below that reused the values for CA_n30-n66-n77.

Table 5.159.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_30-66_n77 | 30 | 0.3 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.159.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_30-66_n77 | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n77 | 0.5 |

5.159.4	Reference sensitivity exceptions

Table 5.159.4-1 shows the required MSD levels for the DC configuration. The required MSD values for IMD2 and IMD4 are derived from CA-n30A-n66A-n77A. The required MSD value for IMD5 is derived from DC_2A-7A_n77A

Table 5.159.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_30A-66A_n77A | 30 | 2310 | 5 | 25 | 2355 | 29.2 | IMD211 |
|  | 66 | 1745 | 5 | 25 | 2145 | N/A | N/A |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 3.4 | IMD5 |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 8.7 | IMD411 |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

5.160	DC_2-5_n30

### 5.160.1	Operating bands for DC

Table 5.160.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-5A_n30A DC_2A-2A-5A_n30A | DC_2A_n30A DC_5A_n30A |

5.160.2	Co-existence studies

For UE coexistence study of Band 2 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.160.2-1.

Table 5.160.2-1: Harmonic and IMD analysis for DC_2_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1910 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3820 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5730 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 395 | 465 | 4155 | 4225 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1385 | 1515 | 2700 | 2780 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 6005 | 6135 | 6460 | 6540 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 3235 | 3425 | 5005 | 5095 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7855 | 8045 | 8765 | 8855 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 930 | 790 | 8310 | 8450 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 7410 | 7310 | 5335 | 5085 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 11070 | 11170 | 9705 | 9955 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 3245 | 3095 | 1120 | 920 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10615 | 10765 | 10160 | 10360 |

For UE coexistence study of Band 5 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.160.2-2.

Table 5.160.2-2: Harmonic and IMD analysis for DC_5_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 824 | 849 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1648 | 1698 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2472 | 2547 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1456 | 1491 | 3129 | 3164 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 667 | 607 | 3761 | 3806 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3953 | 4013 | 5434 | 5479 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 157 | 242 | 6066 | 6121 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4777 | 4862 | 7739 | 7794 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 2982 | 2912 | 6258 | 6328 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 8436 | 8371 | 1091 | 981 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 10044 | 10109 | 5601 | 5711 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 5297 | 5217 | 2063 | 2158 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 8563 | 8643 | 7082 | 7177 |

Based on co-existence study as presented in the table 5.160.2-1 and 5.160.2-2, the 4th order IMD of dual uplink 2+n30 may falls into the own Rx band of n5.

5.160.3	∆TIB and ∆RIB values

The same relaxation values is reused from E-UTRA CA_2-5-30.

Table 5.160.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n30 DC_2-2-5_n30 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n30 | 0.3 |

Table 5.160.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n30 DC_2-2-5_n30 | 2 | 0.4 |
|  | 5 | 0 |
|  | n30 | 0.5 |

5.160.4	Reference sensitivity exceptions

The IMD issues specifc to 3DL/2UL is the IMD4 for 2+n30 falling into band 5. This issue is similar to CA_n2-n5-n30 and the same MSD is reused.

Table 5.160.4-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-5A_n30A | 2 | 1870 | 5 | 25 | 1959 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | 9.7 | IMD4 |
|  | n30 | 2310 | 10 | 50 | 2355 | N/A | N/A |

5.161	DC_2-12_n30

### 5.161.1	Operating bands for DC

Table 5.161.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-12A_n30A DC_2A-2A-12A_n30A | DC_2A_n30A DC_12A_n30A |

5.161.2	Co-existence studies

For UE coexistence study of Band 2 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.161.2-1.

Table 5.161.2-1: Harmonic and IMD analysis for DC_2_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1910 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3820 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5730 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 395 | 465 | 4155 | 4225 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1385 | 1515 | 2700 | 2780 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 6005 | 6135 | 6460 | 6540 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 3235 | 3425 | 5005 | 5095 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7855 | 8045 | 8765 | 8855 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 930 | 790 | 8310 | 8450 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 7410 | 7310 | 5335 | 5085 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 11070 | 11170 | 9705 | 9955 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 3245 | 3095 | 1120 | 920 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10615 | 10765 | 10160 | 10360 |

For UE coexistence study of Band 12 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.161.2-2.

Table 5.161.2-2: Harmonic and IMD analysis for DC_12_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 699 | 716 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1398 | 1432 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2097 | 2148 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1589 | 1616 | 3004 | 3031 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 917 | 873 | 3894 | 3931 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3703 | 3747 | 5309 | 5346 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 218 | 157 | 6199 | 6246 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4402 | 4463 | 7614 | 7661 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 3232 | 3178 | 6008 | 6062 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 8561 | 8504 | 559 | 481 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 9919 | 9976 | 5101 | 5179 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 5547 | 5483 | 2462 | 2533 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 8313 | 8377 | 6707 | 6778 |

Based on co-existence study as presented in the table 5.161.2-1 and 5.161.2-2, no harmonic or IMD issue is identified.

5.161.3	∆TIB and ∆RIB values

The same relaxation values is reused from E-UTRA CA_2-12-30.

Table 5.161.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n30 DC_2-2-12_n30 | 2 | 0.5 |
|  | 12 | 0.3 |
|  | n30 | 0.3 |

Table 5.161.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12_n30 DC_2-2-12_n30 | 2 | 0.4 |
|  | 12 | 0 |
|  | n30 | 0.5 |

5.161.4	Reference sensitivity exceptions

No Refsens exception is identified.

5.162	DC_2-14_n30

### 5.162.1	Operating bands for DC

Table 5.162.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-14A_n30A DC_2A-2A-14A_n30A | DC_2A_n30A DC_14A_n30A |

5.162.2	Co-existence studies

For UE coexistence study of Band 2 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.162.2-1.

Table 5.162.2-1: Harmonic and IMD analysis for DC_2_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1910 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3820 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5730 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 395 | 465 | 4155 | 4225 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1385 | 1515 | 2700 | 2780 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 6005 | 6135 | 6460 | 6540 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 3235 | 3425 | 5005 | 5095 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7855 | 8045 | 8765 | 8855 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 930 | 790 | 8310 | 8450 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 7410 | 7310 | 5335 | 5085 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 11070 | 11170 | 9705 | 9955 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 3245 | 3095 | 1120 | 920 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10615 | 10765 | 10160 | 10360 |

For UE coexistence study of Band 14 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.162.2-2.

Table 5.162.2-2: Harmonic and IMD analysis for DC_14_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 788 | 798 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1576 | 1596 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2364 | 2394 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1507 | 1527 | 3093 | 3113 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 739 | 709 | 3812 | 3842 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3881 | 3911 | 5398 | 5428 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 49 | 89 | 6117 | 6157 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4669 | 4709 | 7703 | 7743 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 3054 | 3014 | 6186 | 6226 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 8472 | 8422 | 887 | 837 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 10008 | 10058 | 5457 | 5507 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 5369 | 5319 | 2216 | 2266 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 8491 | 8541 | 6974 | 7024 |

Based on co-existence study as presented in the table 5.162.2-1 and 5.162.2-2, own Rx impact is not identified.

5.162.3	∆TIB and ∆RIB values

The same relaxation values is reused from E-UTRA CA_2-14-30.

Table 5.162.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14_n30 DC_2-2-14_n30 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | n30 | 0.5 |

Table 5.162.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-14_n30 DC_2-2-14_n30 | 2 | 0.3 |
|  | 14 | 0 |
|  | n30 | 0.3 |

5.162.4	Reference sensitivity exceptions

There is no exception due to no harmonic or IMD issue.

5.163	DC_2-29_n30

### 5.163.1	Operating bands for DC

Table 5.163.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-29A_n30A DC_2A-2A-29A_n30A | DC_2A_n30A |

5.163.2	Co-existence studies

For UE coexistence study of Band 2 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.163.2-1.

Table 5.163.2-1: Harmonic and IMD analysis for DC_2_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1910 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3820 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5730 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 395 | 465 | 4155 | 4225 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1385 | 1515 | 2700 | 2780 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 6005 | 6135 | 6460 | 6540 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 3235 | 3425 | 5005 | 5095 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7855 | 8045 | 8765 | 8855 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 930 | 790 | 8310 | 8450 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 7410 | 7310 | 5335 | 5085 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 11070 | 11170 | 9705 | 9955 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 3245 | 3095 | 1120 | 920 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10615 | 10765 | 10160 | 10360 |

Based on co-existence study as presented in the table 5.163.2-1 and 5.163.2-2, own Rx impact is not identified.

5.163.3	∆TIB and ∆RIB values

The same relaxation values is reused from E-UTRA CA_2-29-30.

Table 5.163.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29_n30 DC_2-2-29_n30 | 2 | 0.5 |
|  | n30 | 0.3 |

Table 5.163.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-29_n30 DC_2-2-29_n30 | 2 | 0.3 |
|  | 29 | 0 |
|  | n30 | 0.3 |

5.163.4	Reference sensitivity exceptions

There is no exception due to no harmonic or IMD issue.

5.164	DC_2-66_n30

### 5.164.1	Operating bands for DC

Table 5.164.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-66A_n30A DC_2A-2A-66A_n30A DC_2A-66A-66A_n30A DC_2A-2A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |

5.164.2	Co-existence studies

For UE coexistence study of Band 2 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.164.2-1.

Table 5.164.2-1: Harmonic and IMD analysis for DC_2_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1910 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3820 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5730 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 395 | 465 | 4155 | 4225 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1385 | 1515 | 2700 | 2780 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 6005 | 6135 | 6460 | 6540 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 3235 | 3425 | 5005 | 5095 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7855 | 8045 | 8765 | 8855 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 930 | 790 | 8310 | 8450 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 7410 | 7310 | 5335 | 5085 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 11070 | 11170 | 9705 | 9955 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 3245 | 3095 | 1120 | 920 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10615 | 10765 | 10160 | 10360 |

For UE coexistence study of Band 66 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.164.2-2.

Table 5.164.2-2: Harmonic and IMD analysis for DC_66_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3420 | 3560 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5130 | 5340 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 525 | 605 | 4015 | 4095 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1105 | 1255 | 2830 | 2920 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 5725 | 5875 | 6320 | 6410 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 2815 | 3035 | 5135 | 5235 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7435 | 7655 | 8625 | 8725 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 1210 | 1050 | 8030 | 8190 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 7550 | 7440 | 4815 | 4525 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 10930 | 11040 | 9145 | 9435 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 3525 | 3355 | 730 | 500 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10335 | 10505 | 9740 | 9970 |

Based on co-existence study as presented in the table 5.164.2-1 and 5.164.2-2, own Rx impact is not identified.

5.164.3	∆TIB and ∆RIB values

The same relaxation values is reused from E-UTRA CA_2-30-66.

Table 5.164.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n30 DC_2-2-66_n30 DC_2-66-66_n30 DC_2-2-66-66_n30 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n30 | 0.3 |

Table 5.164.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n30 DC_2-2-66_n30 DC_2-66-66_n30 DC_2-2-66-66_n30 | 2 | 0.4 |
|  | 66 | 0.4 |
|  | n30 | 0.5 |

5.164.4	Reference sensitivity exceptions

There is no exception due to no harmonic or IMD issue.

5.165	DC_5-66_n30

### 5.165.1	Operating bands for DC

Table 5.165.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_5A-66A_n30A DC_5A-66A-66A_n30A | DC_5A_n30A DC_66A_n30A |

5.165.2	Co-existence studies

For UE coexistence study of Band 5 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.165.2-1.

Table 5.165.2-1: Harmonic and IMD analysis for DC_5_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 824 | 849 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1648 | 1698 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2472 | 2547 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1456 | 1491 | 3129 | 3164 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 667 | 607 | 3761 | 3806 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3953 | 4013 | 5434 | 5479 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 157 | 242 | 6066 | 6121 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4777 | 4862 | 7739 | 7794 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 2982 | 2912 | 6258 | 6328 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 8436 | 8371 | 1091 | 981 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 10044 | 10109 | 5601 | 5711 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 5297 | 5217 | 2063 | 2158 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 8563 | 8643 | 7082 | 7177 |

For UE coexistence study of Band 66 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.165.2-2.

Table 5.165.2-2: Harmonic and IMD analysis for DC_66_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3420 | 3560 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5130 | 5340 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 525 | 605 | 4015 | 4095 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1105 | 1255 | 2830 | 2920 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 5725 | 5875 | 6320 | 6410 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 2815 | 3035 | 5135 | 5235 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7435 | 7655 | 8625 | 8725 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 1210 | 1050 | 8030 | 8190 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 7550 | 7440 | 4815 | 4525 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 10930 | 11040 | 9145 | 9435 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 3525 | 3355 | 730 | 500 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10335 | 10505 | 9740 | 9970 |

Based on co-existence study as presented in the table 5.165.2-1 and 5.165.2-2, the 5th order IMD of dual uplink 5+n30 may fall into own Rx of band 66.

5.165.3	∆TIB and ∆RIB values

The same relaxation values is reused from E-UTRA CA_5-30-66.

Table 5.165.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n30 DC_5-66-66_n30 | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n30 | 0.3 |

Table 5.165.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n30 DC_5-66-66_n30 | 5 | 0 |
|  | 66 | 0.4 |
|  | n30 | 0.5 |

5.165.4	Reference sensitivity exceptions

The IMD issues specifc to 3DL/2UL is the IMD5 for 5+n30 falling into band 66. This issue is similar to CA_n5-n30-n66 and the same MSD is reused.

Table 5.165.4-1: MSD test points due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-66A_n30A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 66 | 1725 | 5 | 25 | 2125 | 4 | IMD5 |
|  | n30 | 2307.5 | 5 | 50 | 2352.5 | N/A | N/A |

5.166	DC_12-66_n30

### 5.166.1	Operating bands for DC

Table 5.166.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_12A-66A_n30A DC_12A-66A-66A_n30A | DC_12A_n30A DC_66A_n30A |

5.166.2	Co-existence studies

For UE coexistence study of Band 12 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.166.2-1.

Table 5.166.2-1: Harmonic and IMD analysis for DC_12_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 699 | 716 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1398 | 1432 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2097 | 2148 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1589 | 1616 | 3004 | 3031 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 917 | 873 | 3894 | 3931 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3703 | 3747 | 5309 | 5346 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 218 | 157 | 6199 | 6246 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4402 | 4463 | 7614 | 7661 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 3232 | 3178 | 6008 | 6062 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 8561 | 8504 | 559 | 481 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 9919 | 9976 | 5101 | 5179 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 5547 | 5483 | 2462 | 2533 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 8313 | 8377 | 6707 | 6778 |

For UE coexistence study of Band 66 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.166.2-2.

Table 5.166.2-2: Harmonic and IMD analysis for DC_66_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3420 | 3560 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5130 | 5340 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 525 | 605 | 4015 | 4095 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1105 | 1255 | 2830 | 2920 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 5725 | 5875 | 6320 | 6410 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 2815 | 3035 | 5135 | 5235 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7435 | 7655 | 8625 | 8725 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 1210 | 1050 | 8030 | 8190 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 7550 | 7440 | 4815 | 4525 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 10930 | 11040 | 9145 | 9435 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 3525 | 3355 | 730 | 500 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10335 | 10505 | 9740 | 9970 |

Based on co-existence study as presented in the table 5.166.2-1 and 5.166.2-2, the third order harmonic may fall into own Rx band 66.

5.166.3	∆TIB and ∆RIB values

The same relaxation values is reused from E-UTRA CA_12-30-66.

Table 5.166.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n30 DC_12-66-66_n30 | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n30 | 0.3 |

Table 5.166.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-66_n30 DC_12-66-66_n30 | 12 | 0.5 |
|  | 66 | 0.4 |
|  | n30 | 0.5 |

5.166.4	Reference sensitivity exceptions

The MSD due to the harmonic issue is specified in 2DL/1UL CA/DC.

5.167	DC_14-66_n30

### 5.167.1	Operating bands for DC

Table 5.167.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_14A-66A_n30A DC_14A-66A-66A_n30A | DC_14A_n30A DC_66A_n30A |

5.167.2	Co-existence studies

For UE coexistence study of Band 14 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.167.2-1.

Table 5.167.2-1: Harmonic and IMD analysis for DC_14_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 788 | 798 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1576 | 1596 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2364 | 2394 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1507 | 1527 | 3093 | 3113 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 739 | 709 | 3812 | 3842 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3881 | 3911 | 5398 | 5428 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 49 | 89 | 6117 | 6157 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4669 | 4709 | 7703 | 7743 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 3054 | 3014 | 6186 | 6226 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 8472 | 8422 | 887 | 837 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 10008 | 10058 | 5457 | 5507 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 5369 | 5319 | 2216 | 2266 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 8491 | 8541 | 6974 | 7024 |

For UE coexistence study of Band 66 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.167.2-2.

Table 5.167.2-2: Harmonic and IMD analysis for DC_66_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3420 | 3560 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5130 | 5340 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 525 | 605 | 4015 | 4095 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1105 | 1255 | 2830 | 2920 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 5725 | 5875 | 6320 | 6410 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 2815 | 3035 | 5135 | 5235 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7435 | 7655 | 8625 | 8725 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 1210 | 1050 | 8030 | 8190 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 7550 | 7440 | 4815 | 4525 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 10930 | 11040 | 9145 | 9435 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 3525 | 3355 | 730 | 500 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10335 | 10505 | 9740 | 9970 |

Based on co-existence study as presented in the table 5.167.2-1 and 5.167.2-2, no harmonic or IMD issue is identified.

5.167.3	∆TIB and ∆RIB values

The same relaxation values is reused from E-UTRA CA_14-30-66.

Table 5.167.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-66_n30 DC_14-66-66_n30 | 14 | 0.3 |
|  | 66 | 0.5 |
|  | n30 | 0.3 |

Table 5.167.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-66_n30 DC_14-66-66_n30 | 14 | 0 |
|  | 66 | 0.4 |
|  | n30 | 0.5 |

5.167.4	Reference sensitivity exceptions

The MSD due to the harmonic issue is specified in 2DL/1UL CA/DC.

5.168	DC_29-66_n30

### 5.168.1	Operating bands for DC

Table 5.168.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_29A-66A_n30A DC_29A-66A-66A_n30A | DC_66A_n30A |

5.168.2	Co-existence studies

For UE coexistence study of Band 66 + Band n30, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.168.2-1.

Table 5.168.2-1: Harmonic and IMD analysis for DC_66_n30

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3420 | 3560 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5130 | 5340 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 525 | 605 | 4015 | 4095 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1105 | 1255 | 2830 | 2920 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 5725 | 5875 | 6320 | 6410 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 2815 | 3035 | 5135 | 5235 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7435 | 7655 | 8625 | 8725 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 1210 | 1050 | 8030 | 8190 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 7550 | 7440 | 4815 | 4525 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 10930 | 11040 | 9145 | 9435 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 3525 | 3355 | 730 | 500 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10335 | 10505 | 9740 | 9970 |

Based on co-existence study as presented in the table 5.168.2-1 and 5.168.2-2, the 5th order IMD of dual uplink 66+n30 may fall into own Rx band 29.

5.168.3	∆TIB and ∆RIB values

The same relaxation values is reused from E-UTRA CA_29-30-66.

Table 5.168.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_29-66_n30 DC_29-66-66_n30 | 66 | 0.5 |
|  | n30 | 0.3 |

Table 5.168.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_29-66_n30 DC_29-66-66_n30 | 29 | 0 |
|  | 66 | 0.4 |
|  | n30 | 0.5 |

5.168.4	Reference sensitivity exceptions

The IMD issues specifc to 3DL/2UL is the IMD5 for 66+n30 falling into band 29. However, there is only a partial hit of IMD5 for this band combination. The MSDs due to IMD5 of two high bands into a low victim band are generally small (such as 1 dB for band 1+5+7, 3.5dB for band 1+7+26) so we propose that MSD is not specified for this band combination.

5.169	DC_2-46_n2

### 5.169.1	Configurations for DC

Table 5.169.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-46A_n2A3 DC_2A-46C_n2A3 DC_2A-46D_n2A3 DC_2A-46E_n2A3 | DC_2A_n2A2 |
| NOTE 2: Only single switched UL is supported NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |

### 5.169.2	Co-existence studies

For UL DC_2A_n2A, there is no need to study IMD for only single switch UL is supported.

3rd harmonic of n2 UL falls into b46 DL.

### 5.169.3	∆TIB and ∆RIB values

For DC_2-46_n2, the TIB,c and RIB,c values are reused from CA_2-46 shown below.

Table 5.169.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n2 | 2 | 0 |
|  | 46 | 0 |
|  | n2 | 0 |

Table 5.169.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46_n2 | 2 | 0 |
|  | 46 | 0 |
|  | n2 | 0 |

### 5.169.4	REFSENS requirements

Based on 5.169.2 discussion above, REFSENS exceptions are specified below.(A new table below is introduced and could be number as Table 7.3B.2.3.1-6 in TS38.101-3 )

Table 5.169.4-1: Reference sensitivity QPSK PREFSENS (EN-DC with b46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n2 | 461 |  | N/A |  | N/A |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the downlink (victim) transmission bandwidth which falls into the reference sensitivity exclusion region overlapping with UL harmonic zones |  |  |  |  |  |  |  |  |  |  |  |  |

5.170	DC_2-48_n2

### 5.170.1	Configurations for DC

Table 5.170.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | DC_2A_n2A2 |
| NOTE 2: Only single switched UL is supported |  |

### 5.170.2	Co-existence studies

For UL DC_2A_n2A, there is no need to study IMD for only single switch UL is supported.

### 5.170.3	∆TIB and ∆RIB values

For DC_2-48_n2, the TIB,c and RIB,c values are reused from CA_2-48 shown below.

Table 5.170.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n2 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n2 | 0.6 |

Table 5.170.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-48_n2 | 2 | 0.2 |
|  | 48 | 0.5 |
|  | n2 | 0.2 |

### 5.170.4	REFSENS requirements

Based on 5.170.2 discussion above, REFSENS exceptions are not specified.

5.171	DC_13-46_n2

### 5.171.1	Configurations for DC

Table 5.171.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_13A-46A_n2A3 | DC_13A_n2A |
| NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |

### 5.171.2	Co-existence studies

Based on co-existence study of UL DC_13A_n2A, IMD4 of band 13 UL and n2 UL falls into own RX of band46.

3rd harmonic of n2 UL falls into b46 DL.

### 5.171.3	∆TIB and ∆RIB values

For DC_13-46_n2, the TIB,c and RIB,c values reused from CA_2-13-46 shown below.

Table 5.171.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-46_n2 | 13 | 0.3 |
|  | 46 | 0 |
|  | n2 | 0.3 |

Table 5.171.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-46_n2 | 13 | 0 |
|  | 46 | 0 |
|  | n2 | 0 |

### 5.171.4	REFSENS requirements

Based on 5.171.2 discussion above, REFSENS exceptions due to IMD are specified below, REFSENS exceptions due to harmonic are specified in R4-2112446 TP for DC_2-46_n2.

Table 5.171.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_13A-46A_n2A5 | 13 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n2 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. DC_2A-46A_n77A; DC_2A_n77A; fc_2A + fc_n77A; BW_2A + BW_n77A. DC_2A-46A_n77A; DC_2A_n77A; -fc_2A + 2*fc_n77A; -BW_2A + 2*BW_n77A. DC_13A-46A_n2A; DC_13A_n2A; 2*fc_n2A + 2*fc_13A; 2*BW_n2A+2*BW_13A. DC_13A-46A_n77A; DC_13A_n77A; 2*fc_13A + fc_n77A; 2*BW_13A + BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; 3*fc_13A + fc_n77A; 3*BW_13A + BW_n77A. DC_13A-46A_n77A; DC_13A_n77A; -3*fc_13A + 2*fc_n77A; -3*BW_13A + 2*BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; fc_66A + fc_n77A; BW_66A + BW_n77A. DC_46A-66A_n77A; DC_66A_n77A; -fc_66A + 2*fc_n77A; -BW_66A + 2*BW_n77A. DC_13A-46A_n66A; DC_13A_n66A; 3*fc_13A + fc_n66A; BW_13A + 2*BW_n66A. DC_13A-46A_n66A; DC_13A_n66A; 2*fc_13A + 3*fc_n66A; BW_13A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; fc_48A + fc_n66A; BW_48A + 2*BW_n66A. DC_46-48A_n66A; DC_48A_n66A; 2*fc_48A + fc_n66A; 2*BW_48A + BW_n66A. DC_2A-46_n5A; DC_2A_n5A; 2*fc_2A + 2*fc_n5A; BW_2A + 2*BW_n5A. DC_2A-46_n5A; DC_2A_n5A; fc_2A + 4*fc_n5A; BW_2*2A + BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + fc_n5A; BW_48A + 2*BW_n5A. DC_46-48A_n5A; DC_48A_n5A; 2*fc_48A + 2*fc_n5A; BW_2*48A + BW_n5A. |

5.172	DC_13-48_n77

### 5.172.1	Configurations for DC

Table 5.172.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_13A-48A_n77A DC_13A-48A-48A_n77A | DC_13A_n77A |

### 5.172.2	Co-existence studies

DC_48_n77 is not used alone, and no uplink inter-band EN-DC is specified for the configuration since the minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA B48 and NR n77 carriers (please refer to Rev 2 of R4-2112654 for DC_48_n77 this meeting). Thus no co-existence study is needed for TDD Band 48.

### 5.172.3	∆TIB and ∆RIB values

For DC_13-48_n77, the TIB,c and RIB,c values are specified in the tables below, reusing DC_13_n77 values.

Table 5.172.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-48_n77 | 13 | 0.5 |
|  | 48 | 0.8 |
|  | n77 | 0.8 |

Table 5.172.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-48_n77 | 13 | 0.2 |
|  | 48 | 0.5 |
|  | n77 | 0.5 |

### 5.172.4	REFSENS requirements

Based on 5.172.2 discussion above, REFSENS exceptions are not expected.

5.173	DC_48-66_n2

### 5.173.1	Configurations for DC

Table 5.173.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | DC_66A_n2A |

### 5.173.2	Co-existence studies

Based on the co-existence study of UL DC_66A_n2A, IMD2 of b66 UL and n2 UL falls into band 48 RX.

### 5.173.3	∆TIB and ∆RIB values

For DC_48-66_n2, the TIB,c and RIB,c values are reused from DC_2-48_n66 shown below:

Table 5.173.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n2 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |

Table 5.173.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n2 | 48 | 0.5 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.173.4	REFSENS requirements

Based on 5.173.2 discussion above, MSD values due to IMD are reused from DC_2-48_n66 shown below:

Table 5.173.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | 48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |

5.174	DC_48-66_n66

### 5.174.1	Configurations for DC

Table 5.174.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_48A-66A_n66A DC_48C-66A_n66A DC_48D-66A_n66A DC_48E-66A_n66A | DC_66A_n66A2 DC_48A_n66A |
| NOTE 2: Only single switched UL is supported |  |

### 5.174.2	Co-existence studies

For UL DC_66A_n66A, there is no need to study IMD for only single switch UL is supported.

For UL DC_48A_n66A, IMD5 of the 2UL falls into band 66 Rx.

### 5.174.3	∆TIB and ∆RIB values

For DC_48-66_n66, the TIB,c and RIB,c values are specified in the tables below.

Table 5.174.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n66 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |

Table 5.174.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n66 | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n66 | 0.2 |

### 5.174.4	REFSENS requirements

Based on discussion on 5.174.2, Reference exception specifies below:

Table 5.174.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_48A-66A_n66A DC_48C-66A_n66A DC_48D-66A_n66A DC_48E-66A_n66A | 48 | 3660 | 20 | 100 | 3660 | N/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | 4.0 | IMD5 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | N/A |

5.175	DC_48-66_n77

### 5.175.1	Configurations for DC

Table 5.175.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_48A-66A_n77A DC_48A-48A-66A_n77A DC_48C-66A_n77A DC_48D-66A_n77A DC_48E-66A_n77A | DC_66A_n77A |

5.175.2	Co-existence studiesDC_48_n77 is not used alone, and no uplink inter-band EN-DC is specified for the configuration since the minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA B48 and NR n77 carriers (please refer to Rev 2 of R4-2112654 for DC_48_n77 this meeting). Thus no co-existence study is needed for TDD Band 48.

### 5.175.3	∆TIB and ∆RIB values

For DC_48-66_n77, the TIB,c and RIB,c values are specified in the tables below, reusing DC_66_n77 values.

Table 5.175.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n77 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.175.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n77 | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n77 | 0.5 |

### 5.175.4	REFSENS requirements

Based on 5.175.2 discussion above, REFSENS exceptions are not expected.

5.176	DC_1-5_n77

5.176.1	Configurations for DC

Table 5.176.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-5A_n77A | DC_1A_n77A DC_5A_n77A |
| DC_1A-5A_n77(2A) | DC_1A_n77A DC_5A_n77A |

5.176.2	Co-existence studies

Based on co-existence studies of DC_1_n77 and DC_5_n77, own Rx impact of the 3rd band is the followings.

-	5th order IMD products generated by DC_1_n77 uplink may fall into own Rx of Band 5.

-	3rd order IMD products generated by DC_5_n77 uplink may fall into own Rx of Band 1.

-	There was no additional IMD problem for non- contiguous DL CA operation by dual uplink transmission.

5.176.3	∆TIB and ∆RIB values

For DC_1-5_n77, the TIB,c and RIB,c values are reused from DC_1-5_n78, and are given in the tables below.

Table 5.176.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-5_n77 | B1 | 0.3 |
|  | B5 | 0.6 |
|  | n77 | 0.8 |

Table 5.176.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-5_n77 | B1 | 0.2 |
|  | B5 | 0.2 |
|  | n77 | 0.5 |

5.176.4	Reference sensitivity exceptions

For the DC_1A-5A_n77A and DC_1A-5A_n77(2A), RAN4 can reuse the MSD results of the DC_1A-5A_n78A. Hence, the proposed MSD test configuration and MSD requirements as follow.

Table 5.176.4-1: MSD requirements

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-5A_n77A DC_1A-5A_n77(2A) | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n77 | 3405 | 10 | 50 | 3405 | N/A | N/A |

5.177	DC_1-7_n77

5.177.1	Configurations for DC

Table 5.177.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-7A_n77A | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A_n77(2A) | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A-7A-n77A | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A-7A-n77(2A) | DC_1A_n77A DC_7A_n77A |

5.177.2	Co-existence studies

Based on co-existence studies of DC_1_n77 and DC_7_n77, own Rx impact of the 3rd band is the followings.

-	4th & 5th order IMD products generated by DC_1_n77 uplink may fall into own Rx of Band 7.

-	4th order IMD products generated by DC_7_n77 uplink may fall into own Rx of Band 1.

-	There was no additional IMD problem for non- contiguous DL CA operation by dual uplink transmission.

5.177.3	∆TIB and ∆RIB values

For DC_1-7_n77, the TIB,c and RIB,c values are reused from DC_1-7_n78, and are given in the tables below.

Table 5.177.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n77 | B1 | 0.6 |
|  | B7 | 0.6 |
|  | n77 | 0.8 |

Table 5.177.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n77 | B1 | 0.2 |
|  | B7 | 0.2 |
|  | n77 | 0.5 |

5.177.4	Reference sensitivity exceptions

For the DC_1A-7A_n77A, DC_1A-7A_n77(2A), DC_1A-7A-7A_n77A and DC_1A-7A-7A_n77(2A), RAN4 can reuse the MSD results of the DC_1A-7A_n78A. Hence, the proposed MSD test configuration and MSD requirements as follow.

Table 5.177.4-1: MSD requirements

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-7A_n77A DC_1A-7A_n77(2A) DC_1A-7A-7A_n77A DC_1A-7A-7A_n77(2A) | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD41 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| Note 1. The DC combinations is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

5.178	DC_3-5_n77

5.178.1	Configurations for DC

Table 5.178.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-5A_n77A | DC_3A_n77A DC_5A_n77A |
| DC_3A-5A_n77(2A) | DC_3A_n77A DC_5A_n77A |

5.178.2	Co-existence studies

Based on co-existence studies of DC_3_n77 and DC_5_n77, own Rx impact of the 3rd band is the followings.

-	No IMD problems by DC_3_n77 uplink in the own Rx of Band 5.

-	3rd order IMD products generated by DC_5_n77 uplink may fall into own Rx of Band 3.

-	There was no additional IMD problem for non- contiguous DL CA operation by dual uplink transmission.

5.178.3	∆TIB and ∆RIB values

For DC_3-5_n77, the TIB,c and RIB,c values are reused from DC_3-5_n78, and are given in the tables below.

Table 5.178.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-5_n77 | B3 | 0.6 |
|  | B5 | 0.6 |
|  | n77 | 0.8 |

Table 5.178.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-5_n77 | B3 | 0.2 |
|  | B5 | 0.2 |
|  | n77 | 0.5 |

5.178.4	Reference sensitivity exceptions

For the DC_3A-5A_n77A and DC_3A-5A_n77(2A), RAN4 can reuse the MSD results of the DC_3A-20A_n78A. Hence, the proposed MSD test configuration and MSD requirements as follow.

Table 5.178.4-1: MSD requirements

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-5A_n77A DC_3A-5A_n77(2A) | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 5 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n77 | 3510 | 10 | 50 | 3510 | N/A | N/A |

5.179	DC_3-7_n77

5.179.1	Configurations for DC

Table 5.179.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-7A_n77(2A) | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A-7A_n77(2A) | DC_3A_n77A DC_7A_n77A |

5.179.2	Co-existence studies

Based on co-existence studies of DC_3_n77 and DC_7_n77, own Rx impact of the 3rd band is the followings.

-	5th order IMD products generaged by DC_3_n77 uplink may fall into own Rx of Band 7.

-	3rd & 4th order IMD products generated by DC_7_n77 uplink may fall into own Rx of Band 3.

-	There was no additional IMD problem for non- contiguous DL CA operation by dual uplink transmission.

5.179.3	∆TIB and ∆RIB values

For DC_3-7_n77, the TIB,c and RIB,c values are already specified in TS38.101-3 as follow.

Table 5.179.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n77 | B3 | 0.6 |
|  | B7 | 0.6 |
|  | n77 | 0.8 |

Table 5.179.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n77 | B3 | 0.2 |
|  | B7 | 0.2 |
|  | n77 | 0.5 |

5.179.4	Reference sensitivity exceptions

For the DC_3A-7A_n77(2A) and DC_3A-7A-7A_n77(2A), RAN4 can reuse the MSD results of the DC_3A-7A_n77A. Hence, the proposed MSD test configuration and MSD requirements as follow.

Table 5.179.4-1: MSD requirements

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |  |
| DC_3A-7A_n77(2A) DC_3A-7A-7A_n77(2A) | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3475 | 10 | 50 | 3475 | N/A | N/A |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | 7 | 2550 | 5 | 25 | 2670 | 5.2 | IMD5 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | 3.4 | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |

5.180	DC_5-7_n77

5.180.1	Configurations for DC

Table 5.180.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_5A-7A_n77A | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A_n77(2A) | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A-7A-n77A | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A-7A-n77(2A) | DC_5A_n77A DC_7A_n77A |

5.180.2	Co-existence studies

Based on co-existence studies of DC_5_n77 and DC_7_n77, own Rx impact of the 3rd band is the followings.

-	2nd order IMD products generated by DC_5_n77 uplink may fall into own Rx of Band 7.

-	2nd & 3rd order IMD products generated by DC_7_n77 uplink may fall into own Rx of Band 5.

-	There was no additional IMD problem for non- contiguous DL CA operation by dual uplink transmission.

5.180.3	∆TIB and ∆RIB values

For DC_5-7_n77, the TIB,c and RIB,c values are reused from DC_5-7_n78, and are given in the tables below.

Table 5.180.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7_n77 | B5 | 0.6 |
|  | B7 | 0.6 |
|  | n77 | 0.8 |

Table 5.180.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7_n77 | B5 | 0.2 |
|  | B7 | 0.2 |
|  | n77 | 0.5 |

5.180.4	Reference sensitivity exceptions

For the DC_5A-7A_n77A, DC_5A-7A_n77(2A), DC_5A-7A-7A_n77A and DC_5A-7A-7A_n77(2A), RAN4 can reuse the MSD results of the DC_5A-7A_n78A. Hence, the proposed MSD test configuration and MSD requirements as follow.

Table 5.180.4-1: MSD requirements

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-7A_n77A DC_5A-7A_n77(2A) DC_5A-7A-7A_n77A DC_5A-7A-7A_n77(2A) | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | n77 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | IMD21 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n77 | 3429 | 10 | 50 | 3429 | N/A | N/A |
| Note 1. The DC combinations is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

## 5.181	DC_2_(n)66

### 5.181.1	Operating bands for EN-DC

Table 5.181.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_2-(n)66 | CA_2-66 | n66 |

5.181.2	Configuration for DC

Table 5.181.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-(n)66AA | DC_2A_n66A | CA_2A-66A | n66A |
| NOTE1: Only single switched UL is supported |  |  |  |

### 5.181.3	Co-existence studies

Based on co-existence studies of CA_n2-n66, the followings are concluded: .

-	IMD3 generated by UL n66 might affect DL band 2.

-     IMD5 generated by UL band 2 might affect DL n66.

### 5.181.4		∆TIB and ∆RIB values

For DC_2_(n)66 the ΔTIB,c and ΔRIB,c values are derived from CA_n2-n66 combination.

Table 5.181.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_(n)66 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.181.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_(n)66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

### 5.181.5	REFSENS requirements

For DC_2_(n)66, REFSENS are reused from the CA_n2-n66 and are given in the table below.

Table 5.3.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-(n)66AA | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |

  - 5.182			DC_1-7_n3

    - 5.182.1	Configurations for DC

Table 5.182.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-7A_n3A | DC_1A_n3A |

    - 5.182.2	Co-existence studies

The co-existence studies of DC_1_n3 have already been captured in TR 37.716-11-11, where:

- No IMD produce caused by  DC_1_n3 uplink may fall into own Rx of band 7.

    - 5.182.3	∆TIB and ∆RIB values

For DC_1A-7A_n3A, the TIB,c and RIB,c values are given in the tables below.

Table 5.182.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n3 | 1 | 0.3 |
|  | 7 | 0.5 |
|  | n3 | 0.5 |

Table 5.182.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n3 | 1 | 0 |
|  | 7 | 0 |
|  | n3 | 0 |

    - 5.182.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

  - 5.183			DC_1-38_n3

    - 5.183.1	Configurations for DC

Table 5.183.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-38A_n3A | DC_1A_n3A |

    - 5.183.2	Co-existence studies

The co-existence studies of DC_1_n3 have already been captured in TR 37.716-11-11, where:

- No IMD produce caused by  DC_1_n3 uplink may fall into own Rx of band 38.

    - 5.183.3	∆TIB and ∆RIB values

For DC_1A-38A_n3A, same TIB,c and RIB,c values as DC_1A-3A_n38A are reused, and are given in the tables below.

Table 5.183.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-38_n3 | 1 | 0.5 |
|  | 38 | 0.5 |
|  | n3 | 0.5 |

Table 5.183.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-38_n3 | 1 | 0 |
|  | 38 | 0 |
|  | n3 | 0 |

    - 5.183.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

  - 5.184			DC_7-38_n3

    - 5.184.1	Configurations for DC

Table 5.184.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_7A-38A_n3Ax,y | N/A |
| NOTE x: The combination is not used alone as fall back mode of other band combinations. NOTE y: Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |

    - 5.184.2	Co-existence studies

No need to analysis the co-existence studies for this configuration.

    - 5.184.3	∆TIB and ∆RIB values

For DC_7-38_n3, the TIB,c and RIB,c values are given in the tables below.

Table 5.184.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-38_n3 | 7 | 0.5 |
|  | 38 | 0.5 |
|  | n3 | 0.5 |

Table 5.184.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-38_n3 | 7 | 0 |
|  | 38 | 0 |
|  | n3 | 0 |

    - 5.184.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

  - 5.185			DC_20-38_n3

    - 5.185.1	Configurations for DC

Table 5.185.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_20A-38A_n3A | DC_20A_n3A |

    - 5.185.2	Co-existence studies

The co-existence studies of DC_20_n3 have already been captured in TR 37.716-11-11, where:

- IMD2, IMD3 produce caused by  DC_20_n3 uplink may fall into own Rx of band 38.

    - 5.185.3	∆TIB and ∆RIB values

For DC_20A-38A_n3A, the TIB,c and RIB,c values are given in the tables below.

Table 5.185.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-38_n3 | 20 | 0.3 |
|  | 38 | 0.5 |
|  | n3 | 0.5 |

Table 5.185.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_20-38_n3 | 20 | 0 |
|  | 38 | 0 |
|  | n3 | 0 |

    - 5.185.4	Reference sensitivity exceptions

According to co-existence studies, both IMD2 and IMD3 may fall into the Rx of band 38. Here we only define MSD caused by IMD2 for this band combination in table 5.185.4-1. For MSD caused by IMD3, noteX in the table is specfied.

Table 5.185.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-38A_n3A | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | 38 | 2610 | 5 | 25 | 2610 | 28.4 | IMD2X |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
| NOTE X: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

5.186	DC_1-32_n8

5.186.1	Configurations for DC

Table 5.186.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-32A_n8A | DC_1A_n8A |

5.186.2	Co-existence studies

Table 5.186.2-1 lists the Band 1A + Band n8A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.186.2-1: Band 1 and Band n8 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 880 | 915 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 – 3960 |  |  | 1760 – 1830 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5760 – 5940 |  |  | 2640 – 2745 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1005 – 1100 |  |  | 2800 – 2895 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2925 – 3080 |  |  | 90 – 220 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4720 – 4875 |  |  | 3680 – 3810 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1900 – 2000 |  |  | 860 – 935 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 – 5060 |  |  | 660 – 825 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2010 – 2200 |  |  | 5600 – 5790 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 – 6855 |  |  | 4560 – 4725 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1540 – 1740 |  |  | 6765 – 7040 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 – 1320 |  |  | 3930 – 4180 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 – 5640 |  |  | 8560 – 8835 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 – 6705 |  |  | 7520 – 7770 |  |  |

Based on Table 5.186.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 3, 46 and 47

-	3rd order harmonics may fall into Rx frequencies of bands 7, 41, 77 and 90

-	3rd order IMD may fall into Rx frequencies of bands 43, 48, 49, 77, 78 and 79

-	4th order IMD may fall into Rx frequencies of bands 1, 4, 10, 12, 13, 14, 17, 20, 23, 28, 29, 34, 44, 46, 65, 66, 67, 68, 70, 79 and 85

-	5th order IMD may fall into Rx frequencies of bands 24, 46 and 77

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.186.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.186.2-2: 2UL Band 1 + Band n8 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | IMD4, IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_1_n8 in 38101-3.

5.186.3	∆TIB and ∆RIB values

Table 5.186.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-32A_n8 | 1 | 0.5 |
|  | n8 | 0.3 |

Table 5.186.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-32A_n8 | 1 | 0 |
|  | 32 | 0 |
|  | n8 | 0 |

5.186.4	Reference sensitivity exceptions

No additional MSD compared to fallbacks.

5.187	DC_1-38_n8

5.187.1	Configurations for DC

Table 5.187.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-38A_n8A | DC_1A_n8A DC_38A_n8A |

5.187.2	Co-existence studies

Table 5.187.2-1 lists the Band 1A + Band n8A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.187.2-1: Band 1 and Band n8 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 880 | 915 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 – 3960 |  |  | 1760 – 1830 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5760 – 5940 |  |  | 2640 – 2745 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1005 – 1100 |  |  | 2800 – 2895 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2925 – 3080 |  |  | 90 – 220 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4720 – 4875 |  |  | 3680 – 3810 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1900 – 2000 |  |  | 860 – 935 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 – 5060 |  |  | 660 – 825 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2010 – 2200 |  |  | 5600 – 5790 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 – 6855 |  |  | 4560 – 4725 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1540 – 1740 |  |  | 6765 – 7040 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 – 1320 |  |  | 3930 – 4180 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 – 5640 |  |  | 8560 – 8835 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 – 6705 |  |  | 7520 – 7770 |  |  |

Based on Table 5.187.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 3, 46 and 47

-	3rd order harmonics may fall into Rx frequencies of bands 7, 41, 77 and 90

-	3rd order IMD may fall into Rx frequencies of bands 43, 48, 49, 77, 78 and 79

-	4th order IMD may fall into Rx frequencies of bands 1, 4, 10, 12, 13, 14, 17, 20, 23, 28, 29, 34, 44, 46, 65, 66, 67, 68, 70, 79 and 85

-	5th order IMD may fall into Rx frequencies of bands 24, 46 and 77

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.187.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.187.2-2: 2UL Band 1 + Band n8 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe | IMD5 |
|  | 2400 | - | 2494 | No | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | IMD4, IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_1_n8 in 38101-3.

Table 5.187.2-3 lists the Band 38A + Band n8A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.187.2-3: Band 38 and Band n8 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 |  | 880 | 915 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5140 – 5240 |  |  | 1760 – 1830 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7710 – 7860 |  |  | 2640 – 2745 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1655 – 1740 |  |  | 3450 – 3535 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 4225 – 4360 |  |  | 740 – 860 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 6020 – 6155 |  |  | 4330 – 4450 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2550 – 2640 |  |  | 860 – 935 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6795 – 6980 |  |  | 20 – 175 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 3310 – 3480 |  |  | 6900 – 7070 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8590 – 8775 |  |  | 5210 – 5365 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 900 – 1090 |  |  | 9365 – 9600 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2395 – 2600 |  |  | 5880 – 6100 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6090 – 6280 |  |  | 11160 – 11395 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7780 – 7985 |  |  | 9470 – 9690 |  |  |

Based on Table 5.187.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 7, 41, 46 and 90

-	3rd order harmonics may fall into Rx frequencies of band 3

-	2nd order IMD may fall into Rx frequencies of bands 22, 42, 77 and 78

-	3rd order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 26, 27, 28, 44, 67, 68, 79 and 85

-	4th order IMD may fall into Rx frequencies of bands 42, 46, 52, 77 and 78

-	5th order IMD may fall into Rx frequencies of bands 8, 38, 40, 41, 46, 47, 53, 69 and 90

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.187.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.187.2-4: 2UL Band 38 + Band n8 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic, IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, IMD4 |

The requirements for spurious emission band UE coexistence are proposed in a parallel TP for fallback DC_38_n8.

5.187.3	∆TIB and ∆RIB values

Table 5.187.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-38A_n8 | 1 | 0.5 |
|  | 38 | 0.5 |
|  | n8 | 0.3 |

Table 5.187.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-38A_n8 | 1 | 0 |
|  | 38 | 0 |
|  | n8 | 0 |

5.187.4	Reference sensitivity exceptions

No additional MSD required based on fallbacks.

5.188	DC_7-32_n3

5.188.1	Configurations for DC

Table 5.188.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_7A-32A_n3A | DC_7A_n3A |

5.188.2	Co-existence studies

Table 5.188.2-1 lists the Band 7A + Band n3A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.188.2-1: Band 7 and Band n3 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 1710 | 1785 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 3420 – 3570 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 5130 – 5355 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 715 – 860 |  |  | 4210 – 4355 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 3215 – 3430 |  |  | 850 – 1070 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 6710 – 6925 |  |  | 5920 – 6140 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2470 – 2600 |  |  | 1690 – 1805 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5715 – 6000 |  |  | 2560 – 2855 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1430 – 1720 |  |  | 8420 – 8710 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9210 – 9495 |  |  | 7630 – 7925 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4270 – 4640 |  |  | 8215 – 8570 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10 – 355 |  |  | 3930 – 4290 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9340 – 9710 |  |  | 11710 – 12065 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10130 – 10495 |  |  | 10920 – 11280 |  |  |

Based on Table 5.188.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 22, 42, 48, 49, 77 and 78

-	3rd order harmonics may fall into Rx frequencies of band 46

-	2nd order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 26, 27, 28, 29, 44, 67, 68 and 85

-	3rd order IMD may fall into Rx frequencies of bands 5, 6, 8, 18, 19, 26, 27, 42, 46, 47, 52, 77 and 78

-	4th order IMD may fall into Rx frequencies of bands 7, 11, 21, 24, 32, 38, 41, 45, 46, 47, 50, 51, 69, 74, 75, 76, 90, 91, 92, 93 and 94

-	5th order IMD may fall into Rx frequencies of bands 77 and 79

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.188.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.188.2-2: 2UL Band 7 + Band n3 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic, IMD3, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | 3rd Harmonic |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic, IMD4 |

The requirements for spurious emission band UE coexistence exist for DC_7_n3 in 38101-3.

5.188.3	∆TIB and ∆RIB values

Table 5.188.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-32A_n3 | 7 | 0.7 |
|  | n3 | 0.7 |

Table 5.188.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-32A_n3 | 7 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

5.188.4	Reference sensitivity exceptions

Table 5.188.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-32A_n3A | 7 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n3 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | 32 | - | - | - | 1470 | 10.5 | IMD4 |

5.189	DC_3-32_n28

5.189.1	Configurations for DC

Table 5.189.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-32A_n28A DC_3C-32A_n28A | DC_3A_n28A DC_3C_n28A |

5.189.2	Co-existence studies

For UE coexistence study of Band 3 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.189.2-1.

Table 5.189.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 962 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 379 | 214 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2164 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1282 | 1027 | 6437 | 6092 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3949 | 3634 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

The second harmonic interference of band n28 may fall into band 32.

IMD5 may fall into Rx of band 32 when band 3 and n28 transmit.

5.189.3	∆TIB and ∆RIB values

For DC_3-32_n28, the TIB,c and RIB,c values are given in the tables below.

Table 5.189.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-32_n28 | 3 | 0.3 |
|  | n28 | 0.3 |

Table 5.189.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-32_n28 | 3 | 0.5 |
|  | 32 | 0 |
|  | n28 | 0.5 |

5.189.4	Reference sensitivity exceptions

The band 32 MSD requirements due to 2nd order harmonic from band n28 can be specified as below by reusing the MSD requirements for DC_28_n50.

Table 5.189.4-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n28 | 322,13 | 27.8 | 24.6 | 22.8 | 21.6 |  |  |  |  |  |  |  |  |  |
|  | 323 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_42_n3, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50, DC_28_n51, DC_66_n78, DC_25_n77, DC_25_n78. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.189.4-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n28 | 32 | 15 | 12 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |  |

The MSD requirements due to IMD5 from DC_3A-32A_n28A don’t need to be specified, since a test case that the interference hits the victim channel directly can’t be found.

5.190	DC_28-32_n1

5.190.1	Configurations for DC

Table 5.190.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_28A-32A_n1A | DC_28A_n1A |

5.190.2	Co-existence studies

For UE coexistence study of Band n1 + Band 28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.190.2-1.

Table 5.190.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1277 | 1172 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 | 3257 | 574 | 424 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 | 4708 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5012 | 5237 | 129 | 324 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6463 | 6688 | 4029 | 4224 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2344 | 2554 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1072 | 832 | 7217 | 6932 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 | 1851 | 4534 | 4264 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 | 4972 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 | 6204 | 7166 | 7436 |

There is no IMD issue for this band combination.

5.190.3	∆TIB and ∆RIB values

For DC_28-32_n1, the TIB,c and RIB,c values are given in the tables below.

Table 5.190.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28-32_n1 | n1 | 0.5 |
|  | 28 | 0.6 |

Table 5.190.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28-32_n1 | n1 | 0 |
|  | 32 | 0 |
|  | 28 | 0.2 |

5.190.4	Reference sensitivity exceptions

No need to specify the REFSENS exception for this band combination. The REFSENS exception due to harmonic interference between band 28 and 32 can refer to the MSD definition in Table 7.3.1A-0a from TS 36.101.

5.191	DC_28-32_n3

5.191.1	Configurations for DC

Table 5.191.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_28A-32A_n3A | DC_28A_n3A |

5.191.2	Co-existence studies

For UE coexistence study of Band n3 + Band 28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.191.2-1.

Table 5.191.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 962 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 379 | 214 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2164 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1282 | 1027 | 6437 | 6092 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3949 | 3634 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

IMD5 may fall into Rx of band 32 when band n3 and 28 transmit.

5.191.3	∆TIB and ∆RIB values

For DC_28-32_n3, the TIB,c and RIB,c values are given in the tables below.

Table 5.191.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28-32_n3 | n3 | 0.3 |
|  | 28 | 0.3 |

Table 5.191.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28-32_n3 | n3 | 0 |
|  | 32 | 0 |
|  | 28 | 0 |

5.191.4	Reference sensitivity exceptions

The REFSENS exception due to harmonic interference between band 28 and 32 can refer to the MSD definition in Table 7.3.1A-0a from TS 36.101.

The MSD requirements due to IMD5 from DC_28A-32A_n3A don’t need to be specified, since a test case that the interference hits the victim channel directly can’t be found.

5.192	DC_7-32_n8

5.192.1	Configurations for DC

Table 5.192.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_7A-32A_n8A | DC_7A_n8A |

5.192.2	Co-existence studies

Table 5.192.2-1 lists the Band 7A + Band n8A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.192.2-1: Band 7 and Band n8 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 880 | 915 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 1760 – 1830 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 2640 – 2745 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1585 – 1690 |  |  | 3380 – 3485 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 4085 – 4260 |  |  | 670 – 810 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 5880 – 6055 |  |  | 4260 – 4400 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2480 – 2590 |  |  | 860 – 935 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6585 – 6830 |  |  | 70 – 245 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 3170 – 3380 |  |  | 6760 – 6970 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8380 – 8625 |  |  | 5140 – 5315 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 950 – 1160 |  |  | 9085 – 9400 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2255 – 2500 |  |  | 5670 – 5950 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6020 – 6230 |  |  | 10880 – 11195 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7640 – 7885 |  |  | 9260 – 9540 |  |  |

Based on Table 5.192.2-1,

-	2nd order harmonics may fall into Rx frequencies of band 3

-	3rd order harmonics may fall into Rx frequencies of bands 7, 41 and 90

-	2nd order IMD may fall into Rx frequencies of bands 42, 52, 77 and 78

-	3rd order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 28, 29, 44, 46, 47, 67, 68, 77 and 85

-	4th order IMD may fall into Rx frequencies of bands 46, 52, 77 and 78

-	5th order IMD may fall into Rx frequencies of bands 8, 30, 40, 41, 46, 47, 53 and 90

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.192.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.192.2-2: 2UL Band 7 + Band n8 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_7_n8 in 38101-3.

5.192.3	∆TIB and ∆RIB values

Table 5.192.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-32A_n8 | 7 | 0.7 |
|  | n8 | 0.6 |

Table 5.192.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-32A_n8 | 7 | 0 |
|  | 32 | 0 |
|  | n8 | 0.2 |

5.192.4	Reference sensitivity exceptions

No additional MSD compared to fallbacks.

5.193	DC_20-32_n8

5.193.1	Configurations for DC

Table 5.193.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_20A-32A_n8A | DC_20A_n8A |

5.193.2	Co-existence studies

Table 5.193.2-1 lists the Band 20A + Band n8A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.193.2-1: Band 20 and Band n8 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 880 | 915 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 1760 – 1830 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 2640 – 2745 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 18 – 83 |  |  | 1712 – 1777 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 749 – 844 |  |  | 898 – 998 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2544 – 2639 |  |  | 2592 – 2692 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 812 – 882 |  |  | 860 – 935 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1581 – 1706 |  |  | 1778 – 1913 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 36 – 166 |  |  | 3424 – 3554 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 – 3501 |  |  | 3472 – 3607 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2658 – 2828 |  |  | 2413 – 2568 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 916 – 1081 |  |  | 666 – 826 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4352 – 4522 |  |  | 4208 – 4363 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4304 – 4469 |  |  | 4256 – 4416 |  |  |

Based on Table 5.193.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 3, 38, 41, 69 and 90

-	3rd order harmonics may fall into Rx frequencies of bands 7, 41 and 90

-	3rd order IMD may fall into Rx frequencies of bands 7, 8, 13, 14, 20, 28, 38, 41, 44, 67, 68, 69 and 90

-	4th order IMD may fall into Rx frequencies of bands 3, 9, 22, 33, 35, 37, 39, 42, 43, 48, 49, 52, 77 and 78

-	5th order IMD may fall into Rx frequencies of bands 7, 8, 12, 13, 14, 17, 20, 28, 29, 41, 44, 53, 67, 68, 79, 85 and 90

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.193.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.193.2-2: 2UL Band 20 + Band n8 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

The requirements for spurious emission band UE coexistence exist for DC_20_n8 in 38101-3.

5.193.3	∆TIB and ∆RIB values

Table 5.193.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-32A_n8 | 20 | 0.4 |
|  | n8 | 0.4 |

Table 5.193.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20A-32A_n8 | 20 | 0 |
|  | 32 | 0 |
|  | n8 | 0 |

5.193.4	Reference sensitivity exceptions

No additional MSD compared to fallbacks.

5.194	DC_20-38_n1

5.194.1	Configurations for DC

Table 5.194.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_20A-38A_n1A | DC_20A_n1A DC_38A_n1A |

5.194.2	Co-existence studies

Table 5.194.2-1 lists the Band 20A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.194.2-1: Band 20 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1058 – 1148 |  |  | 2752 – 2842 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 196 – 316 |  |  | 2978 – 3128 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3584 – 3704 |  |  | 4672 – 4822 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 782 – 912 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 516 – 666 |  |  | 4898 – 5108 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2116 – 2296 |  |  | 5504 – 5684 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4416 – 4566 |  |  | 6592 – 6802 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6818 – 7088 |  |  | 1348 – 1528 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4036 – 4276 |  |  | 1254 – 1464 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8512 – 8782 |  |  | 5248 – 5428 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7424 – 7664 |  |  | 6336 – 6546 |  |  |

Based on Table 5.194.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 46 and 47

-	3rd order harmonics may fall into Rx frequencies of bands 38, 41, 69, 77 and 90

-	3rd order IMD may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77, 78 and 79

-	4th order IMD may fall into Rx frequencies of bands 1, 4, 10, 23, 46, 65, 66, 71 and 79

-	5th order IMD may fall into Rx frequencies of bands 11, 21, 24, 32, 45, 46, 50, 51, 74, 75,  76, 77,  91, 92, 93 and 94

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.194.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.194.2-2: 2UL Band 20 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_20_n1 in 38101-3.

Table 5.194.2-3 lists the Band 38A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.194.2-3: Band 38 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5140 – 5240 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7710 – 7860 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 590 – 700 |  |  | 4490 – 4600 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 3160 – 3320 |  |  | 1220 – 1390 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 7060 – 7220 |  |  | 6410 – 6580 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2520 – 2670 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5730 – 5940 |  |  | 3140 – 3370 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1180 – 1400 |  |  | 8980 – 9200 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9630 – 9840 |  |  | 8330 – 8560 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5060 – 5350 |  |  | 8300 – 8560 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 520 – 800 |  |  | 3750 – 4020 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10250 – 10540 |  |  | 12200 – 12460 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10900 – 11180 |  |  | 11550 – 11820 |  |  |

Based on Table 5.194.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 46 and 47

-	3rd order harmonics may fall into Rx frequencies of band 77

-	2nd order IMD may fall into Rx frequencies of bands 71 and 79

-	3rd order IMD may fall into Rx frequencies of bands 52, 77 and 78

-	4th order IMD may fall into Rx frequencies of bands 46, 47, 52, 77 and 78

-	5th order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 28, 29, 43, 44, 46, 67, 68, 71, 77, 78 and 85

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.194.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.194.2-4: 2UL Band 38 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic, IMD5 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence are proposed in a parallel TP for fallback DC_38_n1 mirroring the values used for DC_1_n38.

5.194.3	∆TIB and ∆RIB values

Table 5.194.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-38A_n1 | 20 | 0.5 |
|  | 38 | 0.3 |
|  | n1 | 0.5 |

Table 5.194.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20A-38A_n1 | 20 | 0.2 |
|  | 38 | 0 |
|  | n1 | 0 |

5.194.4	Reference sensitivity exceptions

No additional MSD required for band 20 3rd harmonic hit in band 38 compared to CA_1A-20A-38A in 36101.

The following can be re-used from DC_1A-20A_n38A to cover the DC_38A_n1A IMD5 hit in band 20:

Table 5.194.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration |  | EUTRA / NR band |  | UL Fc (MHz) |  | UL/DL BW (MHz) |  | UL LCRB |  | DL Fc (MHz) |  | MSD (dB) |  | IMD order |  |
| DC_20A-38A_n1A |  | n1 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  | 20 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD5 |  |
|  |  | 38 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |

5.195	DC_32-38_n1

5.195.1	Configurations for DC

Table 5.195.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_32A-38A_n1A | DC_38A_n1A |

5.195.2	Co-existence studies

Table 5.195.2-1 lists the Band 38A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.195.2-1: Band 38 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5140 – 5240 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7710 – 7860 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 590 – 700 |  |  | 4490 – 4600 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 3160 – 3320 |  |  | 1220 – 1390 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 7060 – 7220 |  |  | 6410 – 6580 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2520 – 2670 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5730 – 5940 |  |  | 3140 – 3370 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1180 – 1400 |  |  | 8980 – 9200 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9630 – 9840 |  |  | 8330 – 8560 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5060 – 5350 |  |  | 8300 – 8560 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 520 – 800 |  |  | 3750 – 4020 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10250 – 10540 |  |  | 12200 – 12460 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10900 – 11180 |  |  | 11550 – 11820 |  |  |

Based on Table 5.195.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 46 and 47

-	3rd order harmonics may fall into Rx frequencies of band 77

-	2nd order IMD may fall into Rx frequencies of bands 71 and 79

-	3rd order IMD may fall into Rx frequencies of bands 52, 77 and 78

-	4th order IMD may fall into Rx frequencies of bands 46, 47, 52, 77 and 78

-	5th order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 28, 29, 43, 44, 46, 67, 68, 71, 77, 78 and 85

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.195.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.195.2-2: 2UL Band 38 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic, IMD5 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence are proposed in a parallel TP for fallback DC_38_n1 mirroring the values used for DC_1_n38.

5.195.3	∆TIB and ∆RIB values

Table 5.195.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_32A-38A_n1 | 38 | 0.5 |
|  | n1 | 0.5 |

Table 5.195.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_32A-38A_n1 | 32 | 0 |
|  | 38 | 0 |
|  | n1 | 0 |

5.195.4	Reference sensitivity exceptions

No additional MSD exceptions required.

5.196	DC_7-7-28_n1

### 5.196.1	Configurations for DC

Table 5.196.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_7A-7A-28A_n1A | DC_28A_n1A DC_7A_n1A |

5.196.2	Co-existence studies

Co-existence studies have been performed for DC_7-28_n1 as well as for lower order combinations.

### 5.196.3	∆TIB and ∆RIB values

Table 5.196.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_7-7-28_n1 | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.5 |

Table 5.196.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-7-28_n1 | 7 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.196.4	Reference sensitivity exceptions

The MSD requirements can reuse the same requirements for DC_7A-28A_n1A.

5.197	DC_8-20_n28

5.197.1	Configurations for DC

Table 5.197.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-20A_n28A6,X | DC_8A_n28A DC_20A_n28A |
| NOTE 6: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758 – 788 MHz for the DL. NOTE X: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. |  |

5.197.2	Co-existence studies

For UE coexistence study of Band 8 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.197.2-1.

Table 5.197.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 132 | 212 | 1583 | 1663 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 491 | 616 | 1012 | 1127 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2286 | 2411 | 2463 | 2578 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1194 | 1364 | 1892 | 2042 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2989 | 3159 | 3343 | 3493 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 424 | 264 | 3166 | 3326 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2957 | 2772 | 2112 | 1897 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1339 | 1144 | 484 | 279 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4223 | 4408 | 3692 | 3907 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4046 | 4241 | 3869 | 4074 |

For UE coexistence study of Band 20 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.197.2-2.

Table 5.197.2-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 832 | 862 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 1664 | 1724 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 2496 | 2586 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 3328 | 3448 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 4160 | 4310 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 84 | 159 | 1535 | 1610 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 544 | 664 | 916 | 1021 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2238 | 2358 | 2367 | 2472 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1247 | 1412 | 1748 | 1883 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2941 | 3106 | 3199 | 3334 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 318 | 168 | 3070 | 3220 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2745 | 2580 | 2160 | 1950 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1180 | 1000 | 580 | 385 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4031 | 4196 | 3644 | 3854 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3902 | 4082 | 3773 | 3968 |

IMD3 of Tx band 20 + band n28 may fall into Rx of band 8.

5.197.3	∆TIB and ∆RIB values

For DC_8-20_n28, the TIB,c and RIB,c values are reused from DC_20_n28 and DC_8_n28, and are given in the tables below.

Table 5.197.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-20_n28 | 8 | 0.6 |
|  | 20 | 0.5 |
|  | n28 | 0.5 |

Table 5.197.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-20_n28 | 8 | 0.2 |
|  | 20 | 0 |
|  | n28 | 0.1 |

5.197.4	Reference sensitivity exceptions

MSD due to IMD3 of Tx band 20 + band n28 can be specified as below. Currently, the RF architecture with three antennas for low band are assumed. The implementation with three antennas for low band need a larger form factor than implementation with two antennas.

Table 5.197.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A-20A_n28A | 8 | 901 | 5 | 25 | 946 | [23.5] | IMD3 |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 773 | N/A | N/A |

5.198	DC_8_(n)3

### 5.198.1		Operating bands for EN-DC

Table 5.198.1.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8_(n)3 | CA_8-3 | n3 | DC_(n)3AA1 |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.198.2	Configurations for EN-DC

Table 5.198.1.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A_(n)3AA | DC_(n)3AA DC_8A_n3A | CA_8A-3A | n3A |

### 5.198.3	∆TIB and ∆RIB values

∆TIB, ∆RIB values for DC_8_n3 are specified as follows:

Table 5.198.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_(n)3 | 8 | 0.3 |
|  | 3 | 0.3 |
|  | n3 | 0.3 |

Table 5.198.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_(n)3 | 8 | 0 |
|  | 3 | 0 |
|  | n3 | 0 |

### 5.198.4	MSD Analysis and requirements

For DC_8A_n3A the harmonics and intermodulation product intervals are calculated as below:

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

- 2nd order harmonic of band 8 could fall on n3 DL frequency range.

- IMD4 and IMD5 could fall on n3 DL frequency range.

The MSD values are similar to DC_3_n8, which can be found on Table 7.3B.2.3.5.1-1 in TS 38.101-3.

For DC_(n)3AA the harmonics and intermodulation product intervals are calculated as below:

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 75 | 75 | 3420 | 3570 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1635 | 1860 | 1635 | 1860 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5130 | 5355 | 5130 | 5355 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3345 | 3645 | 3345 | 3645 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6840 | 7140 | 6840 | 7140 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 150 | 150 | 6840 | 7140 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5430 | 5055 | 5430 | 5055 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1935 | 1560 | 1935 | 1560 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8550 | 8925 | 8550 | 8925 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8550 | 8925 | 8550 | 8925 |

No harmonic or intermodulation product of DC_(n)3AA interfers with the band 8 DL frequency range.

For MSD values for n3 DL, refer to DC_(n)3AA configuration in TR 37.717-11-11

5.199	DC_1-11_n79

### 5.199.1	Configurations for DC_1-11_n79

Table 5.199.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-11A_n79A5 | DC_1A_n79A DC_11A_n79A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

### 5.199.2	Co-existence studies

When Uplink EN-DC configuration is DC_1A_n79A, (1) 4th IMD of (B1 - n79) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n79A, (2) 3rd IMD of (B11 - n79) will fall into Rx band of Band 1.

### 5.199.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.199.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n79 | 1 | 0.3 |
|  | 11 | 0.3 |
|  | n79 | 0 |

Table 5.199.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n79 | 1 | 0 |
|  | 11 | 0 |
|  | n79 | 0 |

### 5.199.4	Reference sensitivity exceptions

As mentioned above, IMD4 of B1 and n79 to Band11 Rx and IMD3 of B11 and n79 to Band1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.199.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A-11A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n79 | 4427 | 40 | 216 | 4427 | N/A | TDD | N/A |
|  | 11 | 1435 | 5 | 25 | 1483 | 10.2 | FDD | IMD4 |
| DC_1A-11A_n79A | 11 | 1431 | 5 | 25 | 1479 | N/A | FDD | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | TDD | N/A |
|  | 1 | 1928 | 5 | 25 | 2118 | 15.6 | FDD | IMD3 |

5.200	DC_8-11_n79

### 5.200.1	Configurations for DC_8-11_n79

Table 5.200.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-11A_n79A5 | DC_8A_n79A DC_11A_n79A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

### 5.200.2	Co-existence studies

When Uplink EN-DC configuration is DC_8A_n79A, (1) IMD5 of (B8 - n79) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n79A, (2) IMD5 of (B11 - n79) will fall into Rx band of Band 8.

### 5.200.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.200.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n79 | 8 | 0.3 |
|  | 11 | 0.4 |
|  | n79 | 0 |

Table 5.200.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n79 | 8 | 0 |
|  | 11 | 0 |
|  | n79 | 0 |

### 5.200.4	Reference sensitivity exceptions

As mentioned above, IMD5 of B8 and n79 to Band11 Rx and IMD5 of B11 and n79 to Band8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.200.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-11A_n79A | 8 | 882.5 | 5 | 25 | 927.5 | N/A | FDD | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | TDD | N/A |
|  | 11 | 1430.4 | 5 | 25 | 1478.4 | 1.2 | FDD | IMD5 |
| DC_8A-11A_n79A | 11 | 1435 | 5 | 25 | 1483 | N/A | FDD | N/A |
|  | n79 | 4810 | 40 | 216 | 4810 | N/A | TDD | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 2.8 | FDD | IMD5 |

5.201	DC_8-41_n3

### 5.201.1	Configurations for DC_8-41_n3

Table 5.201.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-41A_n3A5 DC_8A-41C_n3A5 | DC_8A_n3A DC_41A_n3A DC_41C_n3A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

### 5.201.2	Co-existence studies

When Uplink EN-DC configuration is DC_8A_n3A, (1) IMD2/3 of (B8 - n3) will fall into Rx band of Band 41. When Uplink EN-DC configuration is DC_41A_n3A, (2) IMD2/3 of (B41 - n3) will fall into Rx band of Band 8.

5.201.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.201.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n3 | 8 | 0.3 |
|  | 41 | 0.33/0.84 |
|  | n3 | 0.5 |
| NOTE 3: Applicable for the frequency range of 2515 - 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. |  |  |

Table 5.201.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n3 | 8 | 0 |
|  | 41 | 03/0.54 |
|  | n3 | 0 |
| NOTE 3: Applicable for the frequency range of 2515 - 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. |  |  |

### 5.201.4	Reference sensitivity exceptions

As mentioned above, IMD2/3 of B8 and n3 to Band41 Rx and IMD2/3 of B41 and n3 to Band8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.201.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-41A_n3A DC_8A-41C_n3A | 8 | 885 | 5 | 25 | 930 | N/A | FDD | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | 41 | 2665 | 5 | 25 | 2665 | 27.4 | TDD | IMD21 |
| DC_8A-41A_n3A DC_8A-41C_n3A | 41 | 2665 | 5 | 25 | 2665 | N/A | TDD | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | 8 | 905 | 5 | 25 | 950 | 28.9 | FDD | IMD21 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.202	DC_8-42_n1

### 5.202.1	Configurations for DC_8-42_n1

Table 5.202.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-42A_n1A5 DC_8A-42C_n1A5 | DC_8A_n1A DC_42A_n1A DC_42C_n1A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

### 5.202.2	Co-existence studies

When Uplink EN-DC configuration is DC_42A_n1A, (2) IMD5 of (B42 - n1) will fall into Rx band of Band 8. 5.202.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.202.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-42_n1 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |

Table 5.202.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-42_n1 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n1 | 0 |

### 5.202.4	Reference sensitivity exceptions

As mentioned above, IMD5 of B42 and n1 to Band8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.202.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-42A_n1A DC_8A-42C_n1A | 42 | 3405 | 10 | 50 | 3405 | N/A | FDD | N/A |
|  | n1 | 1955 | 5 | 25 | 2145 | N/A | TDD | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 3.3 | FDD | IMD5 |

5.203	DC_8-41_n77

### 5.203.1	Configurations for DC_8-41_n77

Table 5.203.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-41A_n77A DC_8A-41C_n77A | DC_8A_n77A DC_41A_n77A DC_41C_n77A |

### 5.203.2	Co-existence studies

When Uplink EN-DC configuration is DC_8A_n77A, (1) IMD2 of (B8 - n77) will fall into Rx band of Band 41. When Uplink EN-DC configuration is DC_41A_n77A, (2) IMD2/3/5 of (B41 - n77) will fall into Rx band of Band 8. But the synchronous operation is considered to DC_41A_n77A, (1) do not need to be considered. 5.203.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.203.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n77 | 8 | 0.6 |
|  | 41 | 0.3 |
|  | n77 | 0.8 |

Table 5.203.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n77 | 8 | 0.2 |
|  | 41 | 0 |
|  | n77 | 0.5 |

### 5.203.4	Reference sensitivity exceptions

As mentioned above, IMD2/3/5 of B41 and n77 to Band8 Rx and IMD2 of B8 and n77 to Band41 need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.203.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-41A_n77A DC_8A-41C_n77A | 41 | 2630 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
|  | 8 | 905 | 5 | 25 | 950 | 29.1 | FDD | IMD21, 4 |
| DC_8A-41A_n77A DC_8A-41C_n77A | 8 | 895 | 5 | 25 | 940 | N/A | FDD | N/A |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | TDD | N/A |
|  | 41 | 2650 | 5 | 25 | 2650 | 28.0 | TDD | IMD2 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.204	DC_5-13_n77

5.204.1	Configuration for DC

Table 5.204.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A-13A_n77A DC_5A-13A_n77C | DC_5A_ n77A DC_13A_ n77A |

5.204.2	Co-existence studies

According to the studies in 37.717-11-11, the Rx impacts are identified as below,

- For UL DC_5A_n77A, IMD5 of the 2UL may fall into band n13 Rx (For IMD4, although in calculation there is 3MHz overlap with n13 Rx, but considering there is no way to allocate a minimum channel bandwidth on either transmitter side, IMD4 shall not be pursued)

- For UL DC_13A_n77A, IMD5 of the 2UL may fall into band n5 Rx

Thus additional MSD should be considered to mitigate the impact of the interference for DC_5A-13A_n77A combination.

5.204.3	∆TIB and ∆RIB values

For DC_5-13_n77, the TIB,c and RIB,c values are given in the tables below, reused from DC_13_n5-n77.

Table 5.204.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-13_n77 | 5 | 0.6 |
|  | 13 | 0.5 |
|  | n77 | 0.8 |

Table 5.204.3-2 ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-13_n77 | 5 | 0.2 |
|  | 13 | 0.2 |
|  | n77 | 0.5 |

5.204.4	REFSENS requirements

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 produced by Band 13 and NR band n77 that impact the reference sensitivity of NR band n5.

The required MSD level and test configuration are specified in Table 5.204.4-1 for this dual connectivity configuration.

Table 5.204.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-13A_n77A11 DC_5A-13A_n77C11 | 5 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n77 | 4110 | 10 | 50 | 4110 | N/A | N/A |
|  | 13 | 781 | 5 | 20 | 750 | 4.4 | IMD5 |
|  | 13 | 782 | 5 | 20 | 751 | N/A | N/A |
|  | n77 | 4013 | 10 | 50 | 4013 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 4.5 | IMD5 |
| NOTE 11:The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

5.205	DC_5-48_n77

### 5.205.1	Configurations for DC

Table 5.205.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A-48A_n77A DC_5A-48C_n77A DC_5A-48D_n77A DC_5A-48A_n77C DC_5A-48C_n77C DC_5A-48D_n77C | DC_5A_n77A |

### 5.205.2	Co-existence studies

DC_48_n77 is fully limited for downlink only, and there is no simultaneous Tx/Rx operation between E-UTRA B48 and NR n77 carriers. Thus there is no additional co-existence study is needed for TDD Band 48.

### 5.205.3	∆TIB and ∆RIB values

For DC_5-48_n77, the ΔTIB,c and RIB,c values are specified in the tables below, reusing DC_5_n77 values.

Table 5.205.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-48_n77 | 5 | 0.6 |
|  | 48 | 0.8 |
|  | n77 | 0.8 |

Table 5.205.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-48_n77 | 5 | 0.2 |
|  | 48 | 0.5 |
|  | n77 | 0.5 |

### 5.205.4	REFSENS requirements

Based on 5.205.2 discussion above, REFSENS exceptions are not expected.

5.206			DC_1-7_n38

5.206.1	Configurations for DC

Table 5.206.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_1A-7A_n38A17,18 | N/A | 1A | N/A |
| NOTE 17: The combination is not used alone as fall back mode of other band combinations. NOTE 18: Power imbalance between downlink carriers on Band 7 and Band 38 or band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

To avoid duplicated NOTEs in the TS38.101-3 specification, it is proposed to add ‘or band n38’ on top of the existing NOTE 18 in table 5.5B.4.2 in the TS38.101-3, shown as above.

5.206.2	Co-existence studies

No need to analysis the co-existence studies for this configuration.

5.206.3	∆TIB and ∆RIB values

For DC_1-7_n38, same TIB,c and RIB,c values of LTE CA 1-7-38 are reused which are given in the tables below.

Table 5.206.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n38 | 1 | 0.5 |

Table 5.206.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n38 | 1 | 0 |
|  | 7 | 0 |
|  | n38 | 0.2 |

5.206.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

5.207			DC_3-7_n38

5.207.1	Configurations for DC

Table 5.207.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_3A-7A_n38A17,18 | N/A | 3A | N/A |
| NOTE 17: The combination is not used alone as fall back mode of other band combinations. NOTE 18: Power imbalance between downlink carriers on Band 7 and Band 38 or band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

To avoid duplicated NOTEs in the TS38.101-3 specification, it is proposed to add ‘or band n38’ on top of the existing NOTE 18 in table 5.5B.4.2 in the TS38.101-3, shown as above.

5.207.2	Co-existence studies

No need to analysis the co-existence studies for this configuration.

5.207.3	∆TIB and ∆RIB values

For DC_3-7_n38, the TIB,c and RIB,c values are given in the tables below.

Table 5.207.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n38 | 3 | 0.5 |

Table 5.207.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n38 | 3 | 0 |
|  | 7 | 0 |
|  | n38 | 0.2 |

5.207.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

5.208			DC_7-20_n38

5.208.1	Configurations for DC

Table 5.208.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_7A-20A_n38A17,18 | N/A | 20A | N/A |
| NOTE 17: The combination is not used alone as fall back mode of other band combinations. NOTE 18: Power imbalance between downlink carriers on Band 7 and Band 38 or band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

To avoid duplicated NOTEs in the TS38.101-3 specification, it is proposed to add ‘or band n38’ on top of the existing NOTE 18 in table 5.5B.4.2 in the TS38.101-3, shown as above.

5.208.2	Co-existence studies

No need to analysis the co-existence studies for this configuration.

5.208.3	∆TIB and ∆RIB values

For DC_7-20_n38, the TIB,c and RIB,c values are given in the tables below.

Table 5.208.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20_n38 | 20 | 0.3 |

Table 5.208.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-20_n38 | 7 | 0 |
|  | 20 | 0 |
|  | n38 | 0.2 |

5.208.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

5.209	DC_8-41_n1

5.209.1	Configurations for DC_8-41_n1

Table 5.209.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-41A_n1A DC_8A-41C_n1A | DC_8A_n1A DC_41A_n1A |

5.209.2	Co-existence studies

When Uplink EN-DC configuration is DC_41A_n1A, IMD5 of (B41 - n1) will fall into Rx band of Band 8. Thus, it is proposed to evaluate.

5.209.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.209.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n1 | 8 | 0.3 |
|  | 41 | 0.3 |
|  | n1 | 0.3 |

Table 5.209.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n1 | 8 | 0 |
|  | 41 | 0 |
|  | n1 | 0 |

5.209.4	Reference sensitivity exceptions

As mentioned above, IMD of B8 and n1 to Band41 Rx and IMD of B41 and n1 to Band8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.209.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-41A_n1A DC_8A-41C_n1A | 41 | 2500 | 5 | 25 | 2500 | N/A | TDD | N/A |
|  | n1 | 1977 | 5 | 25 | 2167 | N/A | FDD | N/A |
|  | 8 | 886 | 5 | 25 | 931 | 4.5 | FDD | IMD5 |

5.210	DC_n3_1-8

5.210.1	Configuration for DC

Table 5.210.1-1: Inter-band NE-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_n3A_1A-8A | DC_n3A_1A DC_n3A-8A |

### 5.210.2	Co-existence study

Similar to DC_1A-8A_n3A, which can be found in TR 37.716-21-11 section 5.1.59.3

5.210.3	∆TIB and ∆RIB values

Similar to DC_1A-8A_n3A, which can be found in TR 37.716-21-11 section 5.1.59.4

5.210.4	REFSENS requirements

Similar to DC_1A-8A_n3A, which can be found in TR 37.716-21-11 section 5.1.59.4

5.211	DC_n77_1-8

5.211.1	Configuration for DC

Table 5.211.1-1: Inter-band NE-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_n77A_1A-8A DC_n77(2A)_1A-8A | DC_n77A_1A DC_n77A_8A |

### 5.211.2	Co-existence study

Similar to DC_1A-8A_n77A, which can be found in TR 37.716-21-11 section 5.1.15.3

5.211.3	∆TIB and ∆RIB values

Similar to DC_1A-8A_n77A, which can be found in TR 37.716-21-11 section 5.1.15.4

5.211.4	REFSENS requirements

Similar to DC_1A-8A_n77A, which can be found in TR 37.716-21-11 section 5.1.15.5

5.212	DC_n77_3-1

5.212.1	Configuration for DC

Table 5.212.1-1: Inter-band NE-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_n77A_3A-1A | DC_n77A_1A DC_n77A_3A |

### 5.212.2	Co-existence study

Similar to DC_1A_n3A-n77A. Which can be found in TR 37.717-01-21 section 6.13.3

5.212.3	∆TIB and ∆RIB values

Similar to DC_1A_n3A-n77A. Which can be found in TR 37.717-01-21 section 6.13.4

5.212.4	REFSENS requirements

Similar to DC_1A_n3A-n77A. Which can be found in TR 37.717-01-21 section 6.13.5

5.213	DC_n77_3-8

5.213.1	Configuration for DC

Table 5.213.1-1: Inter-band NE-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_n77A_3A-8A DC_n77(2A)_3A-8A | DC_n77A_3A DC_n77A_8A |

### 5.213.2	Co-existence study

Similar to DC_3A-8A_n77A, which can be found in TR 37.716-21-11 section 5.1.10.3

5.213.3	∆TIB and ∆RIB values

Similar to DC_3A-8A_n77A, which can be found in TR 37.716-21-11 section 5.1.10.4

5.213.4	REFSENS requirements

Similar to DC_3A-8A_n77A, which can be found in TR 37.716-21-11 section 5.1.1.4

5.214			DC_1-38_n78

5.214.1	Configurations for DC

Table 5.214.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_1A-38A_n78A | DC_1A_n78A | CA_1A-38A | n78A |

5.214.2	Co-existence studies

Co-existence analysis for DC_1_n78 UL in TR 37.863-01-01 shows IMD4 produce may fall into LTE Band 38 DL.

5.214.3	∆TIB and ∆RIB values

For DC_1-38_n78, same TIB,c and RIB,c values of DC_1_n38-n78 are reused which are given in the tables below.

Table 5.214.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-38_n78 | 1 | 0.5 |
|  | 38 | 0.5 |
|  | n78 | 0.8 |

Table 5.214.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-38_n78 | 1 | 0 |
|  | 38 | 0 |
|  | n78 | 0.5 |

5.214.4	Reference sensitivity exceptions

Based on co-existence studies additional MSD is needed to be defined. The MSD value is given in table 6.x.5-1, where same MSD values as DC_1_n38-n78 with UL configuration of DC_1_n78 are reused .

Table 6.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-38A_n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 38 | 2590 | 5 | 25 | 2590 | 12.7 | IMD4 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |

5.215			DC_7-38_n78

5.215.1	Configurations for DC

Table 5.215.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_7A-38A_n78A17,18 | N/A | N/A | n78A |
| NOTE 17: The combination is not used alone as fall back mode of other band combinations. NOTE 18: Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

5.215.2	Co-existence studies

No need to analysis the co-existence studies for this configuration.

5.215.3	∆TIB and ∆RIB values

For DC_7-38_n78, same TIB,c and RIB,c values are given in the tables below.

Table 5.215.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-38_n78 | n78 | 0.8 |

Table 5.215.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-38_n78 | 7 | 0 |
|  | 38 | 0 |
|  | n78 | 0.5 |

5.215.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.216	DC_2-38_n78

5.216.1	Configuration for DC

Table 5.216.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-38A_n78A | DC_2A_n78A DC_38A_n78A | CA_2A-38A | n78A |

### 5.216.2	 Co-existence studies

For UE coexistence study of Band 2 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.216.2-1.

Table 5.216.2-1: Harmonic and IMD analysis

|  | Fx low | Fx high | Fy low | Fy high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1850 | 1910 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 10300 | 11850 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 15450 | 17775 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 20600 | 23700 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 25750 | 29625 | 16500 | 19000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1390 | 1950 | 5150 | 5710 |
| Two tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 100 | 520 | 4690 | 5750 |
| Two tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7000 | 7620 | 8450 | 9510 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1750 | 2430 | 7990 | 9550 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3900 | 2780 | 10300 | 11420 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8850 | 9530 | 11750 | 13310 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13350 | 11290 | 4340 | 3600 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7700 | 6080 | 870 | 2050 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 17110 | 10700 | 11440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 15220 | 12150 | 13330 |

For UE coexistence study of Band 38 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.216.2-2.

Table 5.216.2-2: Harmonic and IMD analysis

|  | Fx low | Fx high | Fy low | Fy high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2570 | 2620 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 10300 | 11850 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 15450 | 17775 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 20600 | 23700 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 25750 | 29625 | 16500 | 19000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 680 | 1230 | 5870 | 6420 |
| Two tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1340 | 1940 | 3980 | 5030 |
| Two tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8440 | 9040 | 9170 | 10220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3910 | 4560 | 7280 | 8830 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2460 | 1360 | 11740 | 12840 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11010 | 11660 | 12470 | 14020 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12630 | 10580 | 7180 | 6480 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 4660 | 1260 | 110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15770 | 17820 | 13580 | 14280 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15040 | 16640 | 14310 | 15460 |

The Rx impacts can be identified as below.

- No IMD products generated by DC_2_n78 uplink fall into own Rx of band 38.

- 3rd and 4th order IMD products generated by DC_38_n78 uplink may fall into own Rx of band 2.

### 5.216.3		∆TIB and ∆RIB values

For DC_2A-38A_n78A, the TIB,c and RIB,c values are reused from DC_2_n38-n78 and are given in the tables below.

Table 5.216.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-38_n78 | 2 | 0.6 |
|  | 38 | 0.9 |
|  | n78 | 0.8 |

Table 5.216.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-38_n78 | 2 | 0.5 |
|  | 38 | 0.5 |
|  | n78 | 0.5 |

### 5.216.4	Reference sensitivity exceptions

UL band 38_n78 might cause IMD3 and IMD4 into band 2 DL. MSD value is derived from similar combination.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-38A_n78A | 2 | 1852.5 | 5 | 25 | 1932.5 | 16 | IMD39 |
|  | 38 | 2617.5 | 5 | 25 | 2617.5 | N/A | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
| NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

## 5.217	DC_2-28_n78

5.217.1	Configuration for DC

Table 5.217.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-28A_n78A | DC_2A_n78A DC_28A_n78A | CA_2A-28A | n78A |

### 5.217.2	 Co-existence studies

For UE coexistence study of Band 2 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.217.2-1.

Table 5.217.2-1: Harmonic and IMD analysis

|  | Fx low | Fx high | Fy low | Fy high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1850 | 1910 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 10300 | 11850 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 15450 | 17775 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 20600 | 23700 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 25750 | 29625 | 16500 | 19000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1390 | 1950 | 5150 | 5710 |
| Two tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 100 | 520 | 4690 | 5750 |
| Two tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7000 | 7620 | 8450 | 9510 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1750 | 2430 | 7990 | 9550 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3900 | 2780 | 10300 | 11420 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8850 | 9530 | 11750 | 13310 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13350 | 11290 | 4340 | 3600 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7700 | 6080 | 870 | 2050 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 17110 | 10700 | 11440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 15220 | 12150 | 13330 |

For UE coexistence study of Band 28 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.217.2-2.

Table 5.217.2-2: Harmonic and IMD analysis

|  | Fx low | Fx high | Fy low | Fy high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 703 | 748 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 10300 | 11850 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 15450 | 17775 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 20600 | 23700 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 25750 | 29625 | 16500 | 19000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 3097 | 4003 | 4548 |
| Two tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2394 | 703 | 5852 | 6897 |
| Two tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4706 | 5296 | 7303 | 8348 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1691 | 1056 | 9152 | 10697 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6194 | 5104 | 8006 | 9096 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6044 | 10603 | 12148 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14497 | 12452 | 308 | 988 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9994 | 8404 | 4356 | 5491 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 15948 | 6112 | 6792 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 12896 | 8709 | 9844 |

The Rx impacts can be identified as below.

- No IMD products generated by DC_2_n78 uplink fall into own Rx of band 28.

- 3rd order IMD products generated by DC_28_n78 uplink may fall into own Rx of band 2.

### 5.217.3		∆TIB and ∆RIB values

For DC_2A-28A_n78A, the TIB,c and RIB,c values are reused from DC_28_n78 and DC_2_n78 and are given in the tables below.

Table 5.217.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-28_n78 | 2 | 0.6 |
|  | 28 | 0.5 |
|  | n78 | 0.8 |

Table 5.217.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-28_n78 | 2 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

### 5.217.4	Reference sensitivity exceptions

UL band 28_n78 might cause IMD3 into band 2 DL. MSD value is derived from similar combination.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-28A_n78A | 2 | 1904 | 5 | 25 | 1984 | 16.5 | IMD3 |
|  | 28 | 708 | 5 | 25 | 763 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |

5.218	DC_8-32_n3

5.218.1	Configurations for DC

Table 5.218.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-32A_n3A | DC_8A_n3A |

5.218.2	Co-existence studies

Table 5.218.2-1 lists the Band 8A + Band n3A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.218.2-1: Band 8 and Band n3 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 1710 | 1785 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  |  | 3420 – 3570 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2640 – 2745 |  |  | 5130 – 5355 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 795 – 905 |  |  | 2590 – 2700 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 25 – 120 |  |  | 2505 – 2690 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3470 – 3615 |  |  | 4300 – 4485 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 850 – 945 |  |  | 1700 – 1795 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 855 – 1035 |  |  | 4215 – 4475 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1590 – 1810 |  |  | 5180 – 5400 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4350 – 4530 |  |  | 6010 – 6270 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5925 – 6260 |  |  | 1735 – 1950 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3300 – 3595 |  |  | 675 – 930 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 – 8055 |  |  | 5230 – 5445 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 – 7185 |  |  | 6060 – 6315 |  |  |

Based on Table 5.218.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 3 and 46.

-	3rd order harmonics may fall into Rx frequencies of band 7, 22, 41, 42, 48, 49, 77, 78 and 90

-	2nd order IMD may fall into Rx frequencies of bands 5, 6, 7, 18, 19, 20, 26, 27, 28, 38, 41, 44, 69 and 90

-	3rd order IMD may fall into Rx frequencies of bands 7, 22, 38, 41, 42, 43, 48, 49, 69, 77, 78, 79 and 90

-	4th order IMD may fall into Rx frequencies of bands 3, 5, 6, 8, 18, 19, 26, 27, 46 and 79

-	5th order IMD may fall into Rx frequencies of bands 2, 3, 5, 6, 8, 9, 12, 13, 14, 17, 18, 19, 20, 22, 25, 26, 27, 28, 29, 33, 35, 36, 37, 39, 42, 44, 46, 48, 49, 52, 67, 68, 77, 78 and 85

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.218.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.218.2-2: 2UL Band 8 + Band n3 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd Harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_8_n3 in 38.101-3.

5.218.3	∆TIB and ∆RIB values

Table 5.218.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-32A_n3 | 8 | 0.3 |
|  | n3 | 0.8 |

Table 5.218.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A-32A_n3 | 8 | 0 |
|  | 32 | 0.5 |
|  | n3 | 0.3 |

5.218.4	Reference sensitivity exceptions

The following MSD (obtained from values used for n8 and 3 in 38.101-3) is proposed for the band 8 2nd harmonic hit in band 3:

Table 5.218.4-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 8 | n314 | N/A | N/A | N/A | N/A |  |  |  |  |  |  |  |  |  |
| NOTE 14: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 from TS 36.101-1 apply unless otherwise specified). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

5.219	DC_8-38_n1

5.219.1	Configurations for DC

Table 5.219.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-38A_n1A | DC_8A_n1A DC_38A_n1A |

5.219.2	Co-existence studies

Table 5.219.2-1 lists the Band 8A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.219.2-1: Band 8 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2640 – 2745 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1005 – 1100 |  |  | 2800 – 2895 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 90 – 220 |  |  | 2925 – 3080 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3680 – 3810 |  |  | 4720 – 4875 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 830 – 965 |  |  | 1910 – 1990 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 660 – 825 |  |  | 4845 – 5060 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2010 – 2200 |  |  | 5600 – 5790 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4560 – 4725 |  |  | 6640 – 6855 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6765 – 7040 |  |  | 1540 – 1740 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3930 – 4180 |  |  | 1095 – 1320 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8560 – 8835 |  |  | 5440 – 5640 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7520 – 7770 |  |  | 6480 – 6705 |  |  |

Based on Table 5.219.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 3, 46 and 47.

-	3rd order harmonics may fall into Rx frequencies of band 7, 41, 77 and 90.

-	3rd order IMD may fall into Rx frequencies of bands 43, 48, 49, 77, 78 and 79.

-	4th order IMD may fall into Rx frequencies of bands 1, 4, 10, 12, 13, 14, 17, 20, 23, 28, 29, 34, 44, 46, 65, 66, 67, 68, 70, 79 and 85.

-	5th order IMD may fall into Rx frequencies of bands 24, 46 and 77.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.219.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.219.2-2: 2UL Band 8 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | IMD4, IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_8_n1 in 38.101-3.

Table 5.219.2-3 lists the Band 38A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.219.2-3: Band 38 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5140 – 5240 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7710 – 7860 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 590 – 700 |  |  | 4490 – 4600 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 3160 – 3320 |  |  | 1220 – 1390 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 7060 – 7220 |  |  | 6410 – 6580 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2520 – 2670 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5730 – 5940 |  |  | 3140 – 3370 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1180 – 1400 |  |  | 8980 – 9200 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9630 – 9840 |  |  | 8330 – 8560 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5060 – 5350 |  |  | 8300 – 8560 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 520 – 800 |  |  | 3750 – 4020 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10250 – 10540 |  |  | 12200 – 12460 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10900 – 11180 |  |  | 11550 – 11820 |  |  |

Based on Table 5.219.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 46 and 47.

-	3rd order harmonics may fall into Rx frequencies of band 77.

-	2nd order IMD may fall into Rx frequencies of bands 71 and 79.

-	3rd order IMD may fall into Rx frequencies of bands 52, 77 and 78.

-	4th order IMD may fall into Rx frequencies of bands 46, 47, 52, 77 and 78.

-	5th order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 28, 29, 43, 44, 46, 67, 68, 71, 77, 78 and 85

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.219.2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.219.2-4: 2UL Band 38 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd Harmonic, IMD5 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_38_n1 in 38.101-3.

5.219.3	∆TIB and ∆RIB values

Table 5.219.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-38A_n1 | 8 | 0.3 |
|  | 38 | 0.5 |
|  | n1 | 0.5 |

Table 5.219.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A-38A_n1 | 8 | 0 |
|  | 38 | 0 |
|  | n1 | 0 |

5.219.4	Reference sensitivity exceptions

No additional exceptions compared to fallbacks.

5.220	DC_28-38_n1

5.220.1	Configurations for DC

Table 5.220.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_28A-38A_n1A | DC_28A_n1A DC_38A_n1A |

5.220.2	Co-existence studies

Table 5.220.2-1 lists the Band 28A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.220.2-1: Band 28 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1496 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2244 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1172 – 1277 |  |  | 2623 – 2728 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 424 – 574 |  |  | 3092 – 3257 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3326 – 3476 |  |  | 4543 – 4708 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 653 – 798 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 129 – 324 |  |  | 5012 – 5237 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2344 – 2554 |  |  | 5246 – 5456 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4029 – 4224 |  |  | 6463 – 6688 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6932 – 7217 |  |  | 832 – 1072 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4264 – 4534 |  |  | 1596 – 1851 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8383 – 8668 |  |  | 4732 – 4972 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7166 – 7436 |  |  | 5949 – 6204 |  |  |

Based on Table 5.220.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 11, 21, 32, 45, 46, 47, 50, 51, 74, 75, 76, 91, 92, 93 and 94.

-	3rd order harmonics may fall into Rx frequencies of band 1, 4, 10, 23, 65, 66 and 77.

-	2nd order IMD may fall into Rx frequencies of bands 7, 41 and 90

-	3rd order IMD may fall into Rx frequencies of bands 31, 42, 52, 72, 73, 77, 78, 79, 87 and 88.

-	4th order IMD may fall into Rx frequencies of bands 30, 40, 41, 46, 53, 77 and 90.

-	5th order IMD may fall into Rx frequencies of bands 3, 5, 6, 8, 9, 18, 19, 26, 27, 35 and 79.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.220.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.220.2-2: 2UL Band 28 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD4 |

The requirements for spurious emission band UE coexistence exist for DC_28_n1 in 38.101-3.

Table 5.220.2-3 lists the Band 38A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.220.2-3: Band 38 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5140 – 5240 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7710 – 7860 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 590 – 700 |  |  | 4490 – 4600 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 3160 – 3320 |  |  | 1220 – 1390 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 7060 – 7220 |  |  | 6410 – 6580 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2520 – 2670 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5730 – 5940 |  |  | 3140 – 3370 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1180 – 1400 |  |  | 8980 – 9200 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9630 – 9840 |  |  | 8330 – 8560 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5060 – 5350 |  |  | 8300 – 8560 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 520 – 800 |  |  | 3750 – 4020 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10250 – 10540 |  |  | 12200 – 12460 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10900 – 11180 |  |  | 11550 – 11820 |  |  |

Based on Table 5.220.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 46 and 47.

-	3rd order harmonics may fall into Rx frequencies of band 77.

-	2nd order IMD may fall into Rx frequencies of bands 71 and 79.

-	3rd order IMD may fall into Rx frequencies of bands 52, 77 and 78.

-	4th order IMD may fall into Rx frequencies of bands 46, 47, 52, 77 and 78.

-	5th order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 28, 29, 43, 44, 46, 67, 68, 71, 77, 78 and 85

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.220.2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.220.2-4: 2UL Band 38 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd Harmonic, IMD5 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_38_n1 in 38.101-3.

5.220.3	∆TIB and ∆RIB values

Table 5.220.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A-38A_n1 | 28 | 0.6 |
|  | 38 | 0.5 |
|  | n1 | 0.5 |

Table 5.220.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28A-38A_n1 | 28 | 0.2 |
|  | 38 | 0 |
|  | n1 | 0 |

5.220.4	Reference sensitivity exceptions

The following MSD test point (based on DC_1A_n28A-n41A) is proposed for the band 20 IMD5 hit from 38A_n1A uplink:Table 5.220.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A-38A_n1A | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 28 | 720 | 5 | 25 | 775 | 4.5 | IMD5 |
|  | 38 | 2575 | 5 | 25 | 2575 | N/A | N/A |

5.221	DC_8-11_n1

5.221.1	Configurations for DC_8-11_n1

Table 5.221.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-11A_n1A | DC_8A_n1A DC_11A_n1A |

5.221.2	Co-existence studies

5.221.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.221.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n1 | 8 | 0.3 |
|  | 11 | 0.4 |
|  | n1 | 0.3 |

Table 5.221.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n1 | 8 | 0 |
|  | 11 | 0 |
|  | n1 | 0 |

5.221.4	Reference sensitivity exceptions

As mentioned above, IMD of B11 and n1 to Band8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.221.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-11A_n1A | 11 | 1435 | 5 | 25 | 1483 | N/A | FDD | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 16.6 | FDD | IMD35 |
| NOTE 5: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.222	DC_2-14_n5

5.222.1	Configurations for DC

Table 5.222.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-14A_n5A | DC_2A_n5A DC_14A_n5A |
| DC_2A-2A-14A_n5A | DC_2A_n5A DC_14A_n5A |

5.222.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	There are no IMD products produced by Band 2 and n5 that might fall in Rx of band 14.

-	There are no IMD products produced by Band 14 and n5 that might fall in Rx of band 2.

5.222.3	∆TIB and ∆RIB values

For DC_2-14_n5, the TIB,c and RIB,c values are given in the tables below and are derived from the values for DC_2-12_n5.

Table 5.222.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14_n5 DC_2-2-14_n5 | 2 | 0.3 |
|  | 14 | 0.4 |
|  | n5 | 0.8 |

Table 5.222.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-14_n5 DC_2-2-14_n5 | 2 | 0 |
|  | 14 | 0.3 |
|  | n5 | 0.5 |

5.222.4	Reference sensitivity exceptions

There is no IMD impact from UL 2_n5 affecting DL band 14 and there is no IMD impact from UL 14_n5 affecting DL band 2. Therefore, MSD is not needed to be defined.

5.223	DC_14-66_n5

5.223.1	Configurations for DC

Table 5.223.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_14A-66A_n5A | DC_14A_n5A DC_66A_n5A |
| DC_14A-66A-66A_n5A | DC_14A_n5A DC_66A_n5A |

5.223.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	There are no IMD products produced by Band 14 and n5 that might fall in Rx of band 66

-	IMD4 products are produced by Band 66 and n5 that might fall in Rx of band 14.

5.223.3	∆TIB and ∆RIB values

For DC_14-66_n5, the TIB,c and RIB,c values are given in the tables below and are derived from the values for DC_12-66_n5.

Table 5.223.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-66_n5 DC_14-66-66_n5 | 14 | 0.8 |
|  | 66 | 0.8 |
|  | n5 | 0.3 |

Table 5.223.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-66_n5 DC_14-66-66_n5 | 14 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0 |

5.223.4	Reference sensitivity exceptions

Table 5.223.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from CA_n3-n18-n28.

Table 5.223.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A-66A_n5A | 14 | 792 | 5 | 25 | 762 | 9.4 | IMD4 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | N/A | N/A |

5.224	DC_1-20_n7

5.224.1	Configurations for DC

Table 5.224.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-20A_n7A | DC_1A_n7A DC_20A_n7A |

5.224.2	Co-existence studies

Table 5.224.2-1 lists the Band 1A + Band n7A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.224.2-1: Band 1 and Band n7 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 2500 | 2570 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 – 3960 |  |  | 5000 – 5140 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5760 – 5940 |  |  | 7500 – 7710 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 520 – 650 |  |  | 4420 – 4550 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1270 – 1460 |  |  | 3020 – 3220 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 6340 – 6530 |  |  | 6920 – 7120 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1870 – 2030 |  |  | 2480 – 2590 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3190 – 3440 |  |  | 5520 – 5790 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1040 – 1300 |  |  | 8840 – 9100 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8260 – 8510 |  |  | 9420 – 9690 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8020 – 8360 |  |  | 5110 – 5420 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3540 – 3870 |  |  | 620 – 940 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 – 12260 |  |  | 10180 – 10490 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 – 11670 |  |  | 10760 – 11080 |  |  |

Based on Table 5.224.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 46 and 47.

-	3rd order harmonics may fall into Rx frequencies of band 77.

-	2nd order IMD may fall into Rx frequencies of bands 71 and 79.

-	3rd order IMD may fall into Rx frequencies of bands 32, 45, 50, 51, 75, 76, 91, 92, 93 and 94.

-	4th order IMD may fall into Rx frequencies of bands 42, 46, 52, 77 and 78.

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 22, 26, 27, 28, 29, 42, 43, 44, 46, 48, 49, 67, 68, 71, 77, 78 and 85.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.224.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.224.2-2: 2UL Band 1 + Band n7 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_1_n7 in 38.101-3.

Table 5.224.2-3 lists the Band 20A + Band n7A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.224.2-3: Band 20 and Band n7 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 2500 | 2570 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 5000 – 5140 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 7500 – 7710 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1638 – 1738 |  |  | 3332 – 3432 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 776 – 906 |  |  | 4138 – 4308 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4164 – 4294 |  |  | 5832 – 6002 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 782 – 912 |  |  | 2480 – 2590 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 – 86 |  |  | 6638 – 6878 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 3276 – 3476 |  |  | 6664 – 6864 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4996 – 5156 |  |  | 8332 – 8572 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9138 – 9448 |  |  | 758 – 948 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5776 – 6046 |  |  | 2414 – 2644 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10832 – 11142 |  |  | 5828 – 6018 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9164 – 9434 |  |  | 7496 – 7726 |  |  |

Based on Table 5.224.2-3,

-	2nd order harmonics may fall into Rx frequencies of bands 38, 41, 69 and 90.

-	2nd order IMD may fall into Rx frequencies of bands 42, 52, 77 and 78.

-	3rd order IMD may fall into Rx frequencies of bands 5, 6, 18, 19, 20, 26, 27, 28, 44, 46, 47, 68 and 77.

-	4th order IMD may fall into Rx frequencies of bands 42, 46, 52, 77, 78 and 79.

-	5th order IMD may fall into Rx frequencies of bands 5, 6, 7, 8, 14, 18, 19, 20, 26, 27, 28, 38, 41, 44, 46, 47, 53, 68, 69 and 90

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.224.2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.224.2-4: 2UL Band 20 + Band n7 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, IMD5 |

The requirements for spurious emission band UE coexistence exist for DC_20_n7 in 38.101-3.

5.224.3	∆TIB and ∆RIB values

Table 5.224.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n7 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n7 | 0.6 |

Table 5.224.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20_n7 | 1 | 0 |
|  | 20 | 0 |
|  | n7 | 0 |

5.224.4	Reference sensitivity exceptions

The following MSD is proposed for the IMD5 hit in B20 from the DC_1A_n7A UL:

| Table 5.2244-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-20A_n7A | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |

5.225	DC_48-66_n2

### 5.225.1	Configurations for DC

Table 5.225.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | DC_48A_n2A |

### 5.225.2	Co-existence studies

Coexistence studies have been performed for combination of DC_2A-66A_n48A, where,

- For UL DC_48A_n2 configuration, IMD4 products fall into the band 66 Rx

### 5.225.3	∆TIB and ∆RIB values

For DC_48-66_n2, the TIB,c and RIB,c values are shown below:

Table 5.225.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n2 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |

Table 5.225.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n2 | 48 | 0.5 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.225.4	REFSENS requirements

Based on 5.225.2 discussion above, MSD values due to 4th IMD are reused from DC_2A-66A_n48A shown below:

Based on co-existence analysis, it is found that MSD is needed due to 4th IMD falling into own Rx band of 66. Therefor additional MSD is needed defined in Table 7.3B.2.3.5.2-1 of 38.101-3 along with the requirement of table 5.225.4-1

Table 5.225.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | 48 | 3560 | 5 | 25 | 3560 | N/A | N/A |
|  | 66 | 1755 | 5 | 25 | 2155 | 12.1 | IMD4 |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | N/A |

5.226	DC_2-48_n2

5.226.1	Configurations for DC

Table 5.226.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | DC_48A_n2Ax |
| Note x: The downlink DC_2_n2 RESSENS requirements only apply when the band n2 downlink carrier is configured closer to the uplink operating band than the E-UTRA Band 2 downlink carrier. |  |

5.226.2	Co-existence studies

Coexistence studies have been performed for the lower order combination of DC_2A_n48A, where,

- For UL DC_48A_n2 configuration, IMD4 products fall into the band 2 Rx

5.226.3	∆TIB and ∆RIB values

Table 5.226.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n2 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n2 | 0.6 |

Table 5.226.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-48_n2 | 2 | 0.2 |
|  | 48 | 0.5 |
|  | n2 | 0.2 |

### 5.226.4	Reference sensitivity exceptions

Based on co-existence analysis, it is found that MSD is needed due to 4th IMD falling into own Rx band of 2. Therefor additional MSD is needed defined in Table 7.3B.2.3.5.2-1 of 38.101-3 along with the requirement of table 5.226.4-1.

Table 5. x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | n2 | 1853 | 5 | 25 | 1933 | N/A | N/A |
|  | 48 | 3590 | 20 | 100 | 3590 | N/A | N/A |
|  | 2 | 1889 | 5 | 25 | 1969 | 12 | IMD4 |

5.227	DC_32A-38A_n28A

5.227.1	Configurations for DC

Table 5.227.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_32A-38A_n28A | DC_38A_n28A |

5.227.2	Co-existence studies

Based on co-existence studies of Band 38 + Band n28, own Rx impact of the 3rd band is the followings.

-	2nd order harmonics generated by Band n28 uplink may fall into own Rx of band 32.

5.227.3	∆TIB and ∆RIB values

For DC_32-38_n28, the TIB,c and RIB,c values are reused from the LTE combination DC_7-32_n8, and are given in the tables below.

Table 5.227.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_32-38_n28 | 38 | 0.7 |
|  | n28 | 0.6 |

Table 5.127.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_32-38_n28 | 32 | 0 |
|  | 38 | 0 |
|  | n28 | 0.2 |

5.227.4	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

No additional MSD requirement needs to be defined for this dual connectivity configuration.

5.228	DC_3-38_n78

5.228.1	Configurations for DC

Table 5.228.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-38A_n78A | DC_38A_n78A |
| DC_3C-38A_n78A | DC_3A_n78A DC_3C_n78A DC_38A_n78A |

5.228.2	Co-existence studies

For UE coexistence study of Band 38 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.228.2-1.

Table 5.228.2-1: Harmonic and IMD analysis between Band 38 and Band n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5140 | 5240 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7710 | 7860 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10280 | 10480 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12850 | 13100 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 680 | 1230 | 5870 | 6420 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1340 | 1940 | 3980 | 5030 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8440 | 9040 | 9170 | 10220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3910 | 4560 | 7280 | 8830 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11010 | 11660 | 12470 | 14020 |
| Two-tone 4th order IMD products | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low –2* fy_high¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1360 | 2360 | 11740 | 12840 |
| Two-tone 5th order IMD products | ¦fx_high – 4*fy_low¦ | ¦fx_low – 4*fy_high¦ | ¦fy_high – 4*fx_low¦ | ¦fy_low – 4*fx_high¦ |
| IMD frequency limits (MHz) | 10580 | 12630 | 6480 | 7180 |
| Two-tone 5th order IMD products | ¦2*fx_high - 3*fy_low¦ | ¦2*fx_low - 3*fy_high¦ | ¦2*fy_high -3*fx_low¦ | ¦2*fy_low - 3*fx_high¦ |
| IMD frequency limits (MHz) | 4660 | 6260 | 110 | 1260 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15770 | 17820 | 13580 | 14280 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15040 | 16640 | 14310 | 15460 |

As we can see from the above table:

- the 3rd IMD may fall into DL reception frequency of Band 3.

- the 4th IMD may fall into DL reception frequency of Band 3.

For UE coexistence study of Band 3 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.228.2-2.

Table 5.228.2-2: Harmonic and IMD analysis between Band 3 and Band n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 | 2090 | 5010 | 5585 |
| Two-tone 3rd order IMD products | ¦2*fx_high – fy_low¦ | ¦2*fx_low – fy_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 270 | 380 | 4815 | 5890 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7370 | 8310 | 9385 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 2055 | 8115 | 9690 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9155 | 11610 | 13185 |
| Two-tone 4th order IMD products | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low –2* fy_high¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3030 | 4030 | 10020 | 11170 |
| Two-tone 5th order IMD products | ¦fx_high – 4*fy_low¦ | ¦fx_low – 4*fy_high¦ | ¦fy_high – 4*fx_low¦ | ¦fy_low – 4*fx_high¦ |
| IMD frequency limits (MHz) | 11415 | 13490 | 3040 | 3840 |
| Two-tone 5th order IMD products | ¦2*fx_high - 3*fy_low¦ | ¦2*fx_low - 3*fy_high¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330 | 7980 | 1245 | 2470 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16985 | 10140 | 10940 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14970 | 11730 | 12955 |

As we can see from the above table:

- the 2nd harmonic caused by UL transmission on Band 3 may fall into DL reception frequency of Band n78.

- the 2nd IMD may fall into DL reception frequency of Band 3.

- the 4th IMD may fall into DL reception frequency of Band 3.

- the 5th IMD may fall into DL reception frequency of Band 3.

- NO impacts to the DL reception frequency of Band 38.

5.228.3	∆TIB and ∆RIB values

For DC_3-38_n78, the TIB,c and RIB,c values are given in the following table.

Table 5.228.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-38_n78 | 3 | 0.5 |
|  | 38 | 0.5 |
|  | n78 | 0.8 |

Table 5.228.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-38_n78 | 3 | 0 |
|  | 38 | 0 |
|  | n78 | 0.5 |

5.228.4	Reference sensitivity exceptions

UL band 38_n78 might cause IMD3 and IMD4 into band 3 DL. MSD value is derived from similar combination listed in Table 7.3B.2.3.5.2-1.

Table 5.228.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-38A_n78A, DC_3C-38A_n78A | 3 | 1735 | 5 | 25 | 1830 | 16.4 | IMD39 |
|  | 38 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
| NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

5.229	DC_8-32_n28

5.229.1	Configurations for DC

Table 5.229.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-32A_n28A | DC_8A_n28A |

5.229.2	Co-existence studies

For UE coexistence study of Band 8 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.229.2-1.

Table 5.229.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 132 | 212 | 1583 | 1663 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 491 | 616 | 1012 | 1127 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2286 | 2411 | 2463 | 2578 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1194 | 1364 | 1892 | 2042 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2989 | 3159 | 3343 | 3493 |
| Two-tone 4th order IMD products | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low –2* fy_high¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 264 | 334 | 3166 | 3326 |
| Two-tone 5th order IMD products | ¦fx_high – 4*fy_low¦ | ¦fx_low – 4*fy_high¦ | ¦fy_high – 4*fx_low¦ | ¦fy_low – 4*fx_high¦ |
| IMD frequency limits (MHz) | 2772 | 2957 | 1897 | 2112 |
| Two-tone 5th order IMD products | ¦2*fx_high - 3*fy_low¦ | ¦2*fx_low - 3*fy_high¦ | ¦2*fy_high -3*fx_low¦ | ¦2*fy_low - 3*fx_high¦ |
| IMD frequency limits (MHz) | 1144 | 1339 | 279 | 484 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4223 | 4408 | 3692 | 3907 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4046 | 4241 | 3869 | 4074 |

As we can see from the above table, the 2nd harmonic generated by Band n28 may fall into DL reception frequency of Band 32.

5.229.3	∆TIB and ∆RIB values

For DC_8-32_n28, the TIB,c and RIB,c values are given in the following table.

Table 5.229.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-32_n28 | 8 | 0.5 |
|  | n28 | 0.5 |

Table 5.229.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-32_n28 | 8 | 0 |
|  | 32 | 0 |
|  | n28 | 0 |

5.229.4	Reference sensitivity exceptions

Since there are MSD requirements specified for band 32 due to UL harmonic caused by band n28, they can be referred to the definition in clause 5.8.4 of this TR.

## 5.230	DC_2-7_n25

### 5.230.1	Operating bands for EN-DC

Table 5.230.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| 2-7_n25 | CA_2-7 | n25 |

5.230.2	Configuration for DC

Table 5.230.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A_n25A15, 16 | DC_7A_n25A | CA_2A-7A | n25A |
| DC_2A-7A-7A_n25A15, 16 | DC_7A_n25A | CA_2A-7A-7A | n25A |
| DC_2A-7C_n25A15, 16 | DC_7A_n25A | CA_2A-7C | n25A |
| NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combinations and for the Band 2 and Band n25 combinations. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |

### 5.230.3		∆TIB and ∆RIB values

For DC_2-7_n25, the TIB,c and RIB,c values are reused from DC_2_n7 and are given in the tables below.

Table 5.230.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n25 DC_2-7-7_n25 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n25 | 0.5 |

Table 5.230.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n25 DC_2-7-7_n25 | 2 | 0 |
|  | 7 | 0 |
|  | n25 | 0 |

### 5.230.4	REFSENS requirements

There are no IMD impact from UL 7_n25 affecting DL band 2.

5.231	DC_1-38_n7

5.231.1	Configurations for DCTable 5.231.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_1A-38A_n7A17,18 | N/A | 1A | N/A |
| NOTE 17: The combination is not used alone as fall back mode of other band combinations. NOTE 18: Power imbalance between downlink carriers on Band 7 and Band 38 or band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

### 5.231.3	Co-existence studies

No need to analysis the co-existence studies for this configuration.

### 5.231.3	∆TIB and ∆RIB values

For DC_1-38_n7, same TIB,c and RIB,c values of  DC_1-7_n38 are reused which are given in the tables as below.

Table 5.231.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-38_n7 | 1 | 0.5 |
|  | 38 | 0 |
|  | n7 | 0 |

Table 5.231.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-38_n7 | 1 | 0 |
|  | 38 | 0.2 |
|  | n7 | 0 |

### 5.231.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

5.232	DC_3-38_n7

5.232.1	Configurations for DCTable 5.232.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_3A-38A_n7A17,18 | N/A | 3A | N/A |
| NOTE 17: The combination is not used alone as fall back mode of other band combinations. NOTE 18: Power imbalance between downlink carriers on Band 7 and Band 38 or band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

### 5.232.3	Co-existence studies

No need to analysis the co-existence studies for this configuration.

### 5.232.3	∆TIB and ∆RIB values

For DC_3-38_n7, same TIB,c and RIB,c values of  DC_3-7_n38 are reused which are given in the tables as below.

Table 5.232.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-38_n7 | 3 | 0.5 |
|  | 38 | 0 |
|  | n7 | 0 |

Table 5.232.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-38_n7 | 3 | 0 |
|  | 38 | 0.2 |
|  | n7 | 0 |

### 5.232.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.233	DC_40-42_n77

5.233.1	Configuration for DC

Table 5.233.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_40A-42A_n77A | DC_40A_n77A | CA_40A-42A | n77A |

### 5.233.2	 Co-existence studies

For UE coexistence study of Band 40 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.233.2-1.

Table 5.233.2-1: Harmonic and IMD analysis

|  | Fx low | Fx high | Fy low | Fy high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2300 | 2400 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 16500 | 21000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 900 | 1900 | 5600 | 6600 |
| Two tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 400 | 703 | 4200 | 6100 |
| Two tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7900 | 9000 | 8900 | 10800 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2700 | 3900 | 7500 | 10300 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3800 | 1800 | 11200 | 13200 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10200 | 11400 | 12200 | 15000 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14500 | 10800 | 6300 | 5000 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8000 | 5100 | 600 | 1500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15500 | 19200 | 12500 | 13800 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14500 | 17400 | 13500 | 15600 |

Additional UE coexistence study of Band 42 + Band n77, there is no need since it is not used for UL due to overlapped bands.

The Rx impacts can be identified as below.

- 5th Harmonic generated and 4th order IMD products generated by DC_40_n77 uplink fall into Rx of band 42.

### 5.233.3		∆TIB and ∆RIB values

For DC_40A-42A_n77A, the TIB,c and RIB,c values are reused from CA_40-42, DC_40_n77 and DC_42_n78 as given in the tables below.

Table 5.233.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_40-42_n77 | 40 | 0.45 |
|  | 42 | 0.55 |
|  | n77 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.233.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_40-42_n77 | 40 | 0.46 |
|  | 42 | 0.56 |
|  | n77 | 0.56 |
| NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.233.4	Reference sensitivity exceptions

MSD test points are not needed since DC_42_n77 is specified as non-simultaneous Tx/Rx, and no dual uplink transmissions. Meaning neither B42 MSD nor B40 MSD may occur in that combination.

## 5.234	DC_40-42_n78

5.234.1	Configuration for DC

Table 5.234.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_40A-42A_n78A | DC_40A_n78A | CA_40A-42A | n78A |

### 5.234.2	 Co-existence studies

For UE coexistence study of Band 40 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.234.2-1.

Table 5.234.2-1: Harmonic and IMD analysis

|  | Fx low | Fx high | Fy low | Fy high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2300 | 2400 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 16500 | 21000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 900 | 1500 | 5600 | 6200 |
| Two tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 | 703 | 4200 | 5300 |
| Two tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7900 | 8600 | 8900 | 10000 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3100 | 3900 | 7500 | 9100 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3000 | 1800 | 11200 | 12400 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10200 | 11000 | 12200 | 13800 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12900 | 10800 | 6300 | 5400 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6800 | 5100 | 600 | 700 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15500 | 17600 | 12500 | 13400 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14500 | 16200 | 13500 | 14800 |

Additional UE coexistence study of Band 42 + Band n78, there is no need since it is not used for UL due to overlapped bands.

The Rx impacts can be identified as below.

- 5th Harmonic generated and 4th order IMD products generated by DC_40_n78 uplink fall into Rx of band 42.

### 5.234.3		∆TIB and ∆RIB values

For DC_40A-42A_n78A, the TIB,c and RIB,c values are reused from CA_40-42, DC_40_n78 and DC_42_n78 as given in the tables below.

Table 5.234.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_40-42_n78 | 40 | 0.45 |
|  | 42 | 0.55 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.234.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_40-42_n78 | 40 | 0.46 |
|  | 42 | 0.56 |
|  | n78 | 0.56 |
| NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.234.4	Reference sensitivity exceptions

MSD test points are not needed since DC_42_n77 is specified as non-simultaneous Tx/Rx, and no dual uplink transmissions. Meaning neither B42 MSD nor B40 MSD may occur in that combination.

5.235	DC_n77_1-3

5.235.1	Configuration for DC

Table 5.211.1-1: Inter-band NE-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_n77A_1A-3A DC_n77(2A)_1A-3A | DC_n77A_1A DC_n77A_3A |

### 5.235.2	Co-existence study

Similar to DC_1A_n3A-n77A. Which can be found in TR 37.717-01-21 section 6.13.3

5.235.3	∆TIB and ∆RIB values

Similar to DC_1A_n3A-n77A. Which can be found in TR 37.717-01-21 section 6.13.4

5.235.4	REFSENS requirements

Similar to DC_1A_n3A-n77A. Which can be found in TR 37.717-01-21 section 6.13.5

5.236	DC_1_(n)3

### 5.236.1		Operating bands for EN-DC

Table 5.236.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_(n)3 | CA_1-3 | n3 | DC_1_n3 |

### 5.236.2	Configurations for EN-DC

Table 5.236.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_(n)3AA | DC_1A_n3A | CA_1A-3A | n3A |

### 5.236.3	∆TIB and ∆RIB values

Table 5.236.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_(n)3 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n3 | 0.3 |

Table 5.236.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_(n)3 | 1 | 0 |
|  | 3 | 0 |
|  | n3 | 0 |

### 5.236.4	MSD Analysis and requirements

The harmonics and intermodulation products of DC_1A_n3A are listed below:

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

IMD3 and IMD5 of DC_1A_n3A may fall on B1 DL frequency range.

No IMD product falls on DL bands of B3, hence no MSD consideration is needed.

MSD test points of DC_1_n3 is already available in TS38.101-3 Table 7.3B.2.3.5.1-1:

5.237	DC_12-30_n5

### 5.237.1	Configurations for DC

Table 5.237.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_12A-30A_n5A | DC_12A_n5A DC_30A_n5A |

### 5.237.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	IMD3 products are produced by Band 12 and n5 that might fall in Rx of band 30

-	There are no IMD products produced by Band 30 and n5 that might fall in Rx of band 12.

### 5.237.3	∆TIB and ∆RIB values

For DC_12-30_n5, the TIB,c and RIB,c values are given in the tables below and are derived from the values for DC_12-66_n5.

Table 5.237.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-30_n5 | 12 | 0.8 |
|  | 30 | 0.8 |
|  | n5 | 0.3 |

Table 5.237.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-30_n5 | 12 | 0.5 |
|  | 30 | 0.5 |
|  | n5 | 0 |

### 5.237.4	Reference sensitivity exceptions

Table 5.237.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from DC_7A-20A_n8A.

Table 5.237.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-30A_n5A | 12 | 702 | 5 | 25 | 732 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 18.8 | IMD3 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |

5.238	DC_14-30_n5

### 5.238.1	Configurations for DC

Table 5.238.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_14A-30A_n5A | DC_14A_n5A DC_30A_n5A |

### 5.238.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be seen that:

-	IMD5 products are produced by Band 14 and n5 that might fall in Rx of band 30

-	There are no IMD products produced by Band 30 and n5 that might fall in Rx of band 14.

### 5.238.3	∆TIB and ∆RIB values

For DC_14-30_n5, the TIB,c and RIB,c values are given in the tables below and are derived from the values for DC_12-66_n5.

Table 5.238.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-30_n5 | 14 | 0.8 |
|  | 30 | 0.8 |
|  | n5 | 0.3 |

Table 5.238.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-30_n5 | 14 | 0.5 |
|  | 30 | 0.5 |
|  | n5 | 0 |

### 5.238.4	Reference sensitivity exceptions

Table 5.238.4-1 shows the required MSD levels for the DC configuration. The required MSD values are derived from DC_7A-28A_n5A.

Table 5.238.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A-30A_n5A | 14 | 795 | 5 | 25 | 765 | N/A | N/A |
|  | 30 | 2308 | 5 | 25 | 2353 | 5.9 | IMD5 |
|  | n5 | 827 | 5 | 25 | 872 | N/A | N/A |

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-8 | 3GPP RAN4#96-e |  |  |  |  | TR skeleton | 0.0.1 |
| 2020-8 | 3GPP RAN4#96-e | R4-2011889 |  |  |  | The following approved TPs at RAN4#96-e have been implemented: R4-2011584 TP for TR 37.717-21-11: DC_1-32_n28 R4-2011585 TP for TR 37.717-21-11: DC_7-32_n28 R4-2009716 TP for TR 37.717-21-11: DC_7-32_n78 R4-2011586 TP for TR 37.717-21-11: DC_20-32_n28 R4-2010232 TP for TR 37.717.21-11: DC_2A-48A_n5A R4-2010233 TP for TR 37.717.21-11: DC_5A-48A_n12A R4-2010234 TP for TR 37.717.21-11: DC_5A-48A_n71A R4-2010235 TP for TR 37.717.21-11: DC_12A-48A_n5A R4-2010245 TP for TR 37.717-21-11 DC_3A_(n)41AA R4-2011605 TP for 37.717-21-11 to introduce DC_2A-48A_n48A R4-2011607 TP for 37.717-21-11 to introduce DC_48-66A_n25A R4-2010410 TP for 37.717-21-11 to introduce DC_48A-66A_n48A R4-2011608 TP for 37.717-21-11 to introduce DC_3A-8A_n40A R4-2011609 TP for 37.717-21-11 to introduce DC_3A-28A_n1A R4-2010421 TP for 37.717-21-11 to introduce DC_7A-8A_n40A R4-2011610 TP for 37.717-21-11 to introduce DC_7A-28A_n1A R4-2010439 TP for DC_3-19_n1 for TR 37.717-21-11 R4-2011612 TP for DC_3-21_n1 for TR 37.717-21-11 R4-2010453 TP for DC_3-42_n1 for TR 37.717-21-11 R4-2010454 TP for DC_19-21_n1 for TR 37.717-21-11 R4-2011613 TP for DC_19-42_n1 for TR 37.717-21-11 R4-2011614 TP for DC_21-42_n1 for TR 37.717-21-11 R4-2011616 TP for TR 37.717-21-11: DC_7-66_n5 R4-2010513 TP for TR 37.717-21-11: DC_2-7_n5 R4-2010556 TP to TR 37.717-21-11 DC_1A-40A_n78A R4-2010557 TP to TR 37.717-21-11 DC_3A-40A_n78A R4-2010558 TP to TR 37.717-21-11 DC_7A-40A_n78A R4-2011618 TP to TR 37.717-21-11 DC_8A-40A_n78A R4-2011620 TP for TR 37.717-21-11 to include DC_2A-12A_n5A R4-2011621 TP for TR 37.717-21-11 to include DC_2A-5A_n12A R4-2010699 TP for TR 37.717-21-11 to include DC_5A-66A_n12A R4-2010700 TP for TR 37.717-21-11 to include DC_66A_(n)5AA R4-2010701 TP for TR 37.717-21-11 to include DC_12A-66A_n5A R4-2010883 TP for TR 37.717-21-11: DC_7A-8A_n28A R4-2010884 TP for TR 37.717-21-11: DC_20A-28A_n3A R4-2010885 TP for TR 37.717-21-11: DC_28A-66A_n66A R4-2010886 TP for TR 37.717-21-11: DC_7A-28A_n66A / DC_7C-28A_n66A R4-2010887 TP for TR 37.717-21-11: DC_2A-28A_n66A R4-2010888 TP for TR 37.717-21-11: DC_3A-28A_n1A R4-2010889 TP for TR 37.717-21-11: DC_7A-28A_n1A R4-2010890 TP for TR 37.717-21-11: DC_8A-40A_n1A / DC_8A-40C_n1A R4-2010894 TP for TR 37.717-21-11: DC_1A-32A_n3A R4-2010895 TP for TR 37.717-21-11: DC_3A-32A_n1A R4-2009924 TP for TR 37.717-21-11 for DC_2A-4A_n28A R4-2009925 TP for TR 37.717-21-11 for DC_2A-7A_n28A R4-2009926 TP for TR 37.717-21-11 for DC_2A-66A_n28A R4-2009927 TP for TR 37.717-21-11 for DC_4A-7A_n28A R4-2009928 TP for TR 37.717-21-11 for DC_5-7_n66 R4-2009929 TP for TR 37.717-21-11 for DC_7A-66A_n28A R4-2009992 TP for TR 37.717-21-11: EN-DC_1-11_n28 R4-2009993 TP for TR 37.716-21-11: EN-DC_3-11_n28 R4-2009994 TP for TR 37.717-21-11: EN-DC_8-11_n28 R4-2011593 TP for TR 37.717-21-11: EN-DC_3-11_n77 | 0.1.0 |
| 2020-11 | 3GPP RAN4#97-e | R4-2015704 |  |  |  | The following approved TPs at RAN4#97-e have been implemented: R4-2014056 TP for TR 37.717-21-11: DC_7-32_n78 R4-2014057 TP for TR 37.717-21-11: DC_7-32_n1 R4-2014031 TP for 37.717-21-11 for DC_2-66_n7 R4-2014032 TP for 37.717-21-11 for DC_2-5_n7 R4-2014033 TP for 37.717-21-11 for DC_2-8_n2 R4-2014034 TP for 37.717-21-11 for DC_5-66_n7 R4-2016656 TP for 37.717-21-11 for DC_20-32_n1 R4-2014036 TP for 37.717-21-11 for DC_20-32_n3 R4-2014103 TP for TR 37.717-21-11 DC_1-3_n3 R4-2014104 TP for TR 37.717-21-11 DC_1-41_n3 R4-2014105 TP for TR 37.717-21-11 DC_3-18_n3 R4-2014106 TP for TR 37.717-21-11 DC_3-41_n3 R4-2014128 TP for TR 37.717-21-11 DC_5A-7A_n66A R4-2014129 TP for TR 37.717-21-11 DC_7-66_n77 R4-2014132 TP for TR 37.717-21-11 DC_2-5_n48 R4-2014133 TP for TR 37.717-21-11 DC_2-13_n48 R4-2014135 TP for TR 37.717-21-11 DC_2-48_n5 R4-2014136 TP for TR 37.717-21-11 DC_5-46_n66 R4-2014137 TP for TR 37.717-21-11 DC_5-66_n48 R4-2014138 TP for TR 37.717-21-11 DC_5-66_n77 R4-2014612 TP for TR 37.717-21-11: EN-DC_1-42_n3 R4-2014613 TP for TR 37.717-21-11: EN-DC_8-42_n3 R4-2016662 TP update for TR 37.717-21-11: EN-DC_1-11_n28 R4-2014811 TP for DC_3-18_n28 R4-2014852 TP for TR 37.717-21-11: CA_2-66_n77 R4-2014854 TP for TR 37.717-21-11: CA_2-48_n77 R4-2014856 TP for TR 37.717-21-11: CA_2-13_n77 R4-2014857 TP for TR 37.717-21-11: CA_2-5_n77 R4-2014858 TP for TR 37.717-21-11: CA_5-13_n66 R4-2014860 TP for TR 37.717-21-11: CA_13-66_n77 R4-2014862 TP for TR 37.717-21-11: CA_13-66_n5 R4-2016664 TP for DC_1-18_n28 R4-2016665 TP for DC_1-18_n41 R4-2014982 TP for DC_3-42_n1 for TR 37.717-21-11 R4-2016668 TP for 37.717-21-11 to introduce DC_5A-7A_n7A R4-2015226 TP for 37.717-21-11 to introduce DC_2A-28A_n7A R4-2016669 TP for 37.717-21-11 to introduce DC_28A-66A_n7A R4-2015228 TP for 37.717-21-11 to introduce DC_7A-28A_n2A R4-2015229 TP for 37.717-21-11 to introduce DC_2A-7A_n7A R4-2015246 TP for 37.717-21-11 to introduce DC_2A-71A_n71A and DC_66A-71A_n71A R4-2015268 TP to TR 37.717-21-11 DC_1A-40C_n78A R4-2015269 TP to TR 37.717-21-11 DC_3A-40C_n78A R4-2015270 TP to TR 37.717-21-11 DC_7A-40C_n78A R4-2015271 TP to TR 37.717-21-11 DC_8A-40C_n78A R4-2015404 TP for TR 37.717-21-11: DC_7A-66A_n7A/DC_7A-66A-66A_n7A R4-2015710 TP for TR 37.717-21-11: DC_2-7_n77 R4-2015711 TP for TR 37.717-21-11: DC_7-66_n77 | 0.2.0 |
| 2021-02 | 3GPP RAN4#98-e | R4-2101508 |  |  |  | The following approved TPs at RAN4#98-e have been implemented: R4-2103006 TP to TR 37.717-21-11: DC_20-40_n78 R4-2103010 TP for TR 37.717-21-11: DC_3-18_n41 R4-2103011 TP update for TR 37.717-21-11: EN-DC_1-11_n28 R4-2103014 TP to TR 37.717-21-11 DC_7-25_n77 R4-2103015 TP to TR 37.717-21-11 DC_7-25_n78 R4-2103016 TP to TR 37.717-21-11 DC_25-66_n77 R4-2103017 TP to TR 37.717-21-11 DC_25-66_n78 R4-2103018 TP for TR 37.717-21-11: DC_2-29_n78 R4-2103019 TP for TR 37.717-21-11: DC_29-66_n78 R4-2103022 TP for DC_1-21_n28 for TR 37.717-21-11 R4-2101231 TP for DC_3-21_n28 for TR 37.717-21-11 R4-2101511 TP for 37.717-21-11: correction of duplicated TPS for some combinations R4-2103023 TP for TR 37.717-21-11: DC_8-20_n1 R4-2103024 TP for TR 37.717-21-11: DC_8-20_n3 R4-2101548 TP for TR 37.717-21-11: DC_8-32_n1 R4-2101913 TP for TR 37.717-21-11 to include 12A-66A_n41A R4-2101914 TP for TR 37.717-21-11 to include 2A-12A_n41A, 2A-2A-12A_n41A R4-2101915 TP for TR 37.717-21-11 to include 66A-71A_n41A R4-2101916 TP for TR 37.717-21-11 to include 2A-71A_n41A, 2A-2A-71A_n41A R4-2101917 TP for TR 37.717-21-11 to include 7A-12A_n66A R4-2101918 TP for TR 37.717-21-11 to include 7A-71A_n66A R4-2101919 TP for TR 37.717-21-11 to include 7A-12A_n78A R4-2101920 TP for TR 37.717-21-11 to include 12A-66A_n78A R4-2101921 TP for TR 37.717-21-11 to include 2A-12A_78A, 2A-2A-12A_78A R4-2102050 TP for TR 37.717-21-11 to include 7A-71A_n78A | 0.3.0 |
| 2021-04 | 3GPP RAN4#98-bis-e | R4-2106490 |  |  |  | The following approved TPs at RAN4#98-bis-e have been implemented: R4-2104406 TP to TR 37.717-21-11: DC_20-40_n1 R4-2104411 TP to TR 37.717-21-11: DC_5A-30A_n2A R4-2105293 TP to TR 37.717-21-11 DC_2-46_n77 R4-2105294 TP to TR 37.717-21-11 DC_13-46_n77 R4-2105295 TP to TR 37.717-21-11 DC_46-66_n77 R4-2105327 TP for TR 37.717-21-22: DC_1-11_n41 R4-2105306 TP for TR 37.717-21-11: DC_2-12_n7 R4-2106499 TP for TR 37.717-21-11: DC_5A-7A-7A_n66a R4-2106500 TP for TR 37.717-21-11: DC_2-5_n78 R4-2106501 TP for TR 37.717-21-11: DC_2A-12A_n78(2A) R4-2106502 TP for TR 37.717-21-11: DC_7-29_n78 R4-2106641 TP for TR 37.717-21-11: DC_1A-38A_n28A R4-2106642 TP for TR 37.717-21-11: DC_3A-38A_n28A/ DC_3C-38A_n28A R4-2105312 TP for TR 37.717-21-11 to include 14-30_n66 R4-2106712 TP for TR 37.717-21-11 to include 14-30_n2 R4-2106713 TP for TR 37.717-21-11 to include 2-(n)5 R4-2106714 TP for TR 37.717-21-11 to include 30-(n)5 R4-2105315 TP for TR 37.717-21-11 to include 28-40_n78 R4-2106739 TP for TR 37.717-21-11 to include DC_7-13_n25 R4-2106740 TP for TR 37.717-21-11 to include DC_7-66_n25 R4-2106761 TP for TR 37.717-21-11 to include 3-7_n3 R4-2106762 TP for TR 37.717-21-11 to include 3-28_n3 R4-2106925 TP for TR 37.717-21-11: support of DC_3-3-7_n8, DC_3-7-7_n8, DC_3-3-7-7_n8 R4-2105319 TP to TR 37.717-21-11 Addition of DC_13-46_n66 R4-2105321 TP to TR 37.717-21-11 Addition of DC_2-46_n5 R4-2105322 TP to TR 37.717-21-11 Addition of DC_46-48_n5 | 0.4.0 |
| 2021-04 | 3GPP RAN4#99-e | R4-2110665 |  |  |  | The following approved TPs at RAN4#99-e have been implemented: R4-2109463 TP update for TR 37.717-21-11: EN-DC_1-11_n41 R4-2110240 TP for TR 37.717-21-11: DC_20A-28A_n1A R4-2110241 TP for TR 37.717-21-11: DC_8A-20A_n1A R4-2107688 TP for TR 37.717-21-11: DC_8A-20A_n3A The following approved TPs at RAN4#98-bis-e which are missing in v 0.4.0 have been implemented: R4-2106737 TP for TR 37.717-21-11 to include 2-13_n25 R4-2106738 TP for TR 37.717-21-11 to include 2-66_n25 | 0.5.0 |
| 2021-08 | 3GPP RAN4#100-e | R4-2113051 |  |  |  | R4-2114794 TP for TR 37.717-21-11 Addition of DC_2-12_n77 R4-2114795 TP for TR 37.717-21-11 Addition of DC_2-14_n77 R4-2114796 TP for TR 37.717-21-11 Addition of DC_2-29_n77 R4-2114797 TP for TR 37.717-21-11 Addition of DC_2-30_n77 R4-2114798 TP for TR 37.717-21-11 Addition of DC_5-30_n77 R4-2111755 TP for TR 37.717-21-11 Addition of DC_12-30_n77 R4-2114799 TP for TR 37.717-21-11 Addition of DC_12-66_n77 R4-2114800 TP for TR 37.717-21-11 Addition of DC_14-30_n77 R4-2114801 TP for TR 37.717-21-11 Addition of DC_14-66_n77 R4-2114802 TP for TR 37.717-21-11 Addition of DC_29-30_n77 R4-2114803 TP for TR 37.717-21-11 Addition of DC_29-66_n77 R4-2114804 TP for TR 37.717-21-11 Addition of DC_30-66_n77 R4-2111784 TP to TR 37.717-21-11 DC_2-5_n30 R4-2111785 TP to TR 37.717-21-11 DC_2-12_n30 R4-2111786 TP to TR 37.717-21-11 DC_2-14_n30 R4-2111787 TP to TR 37.717-21-11 DC_2-29_n30 R4-2111788 TP to TR 37.717-21-11 DC_2-66_n30 R4-2111789 TP to TR 37.717-21-11 DC_5-66_n30 R4-2111790 TP to TR 37.717-21-11 DC_12-66_n30 R4-2111791 TP to TR 37.717-21-11 DC_14-66_n30 R4-2111792 TP to TR 37.717-21-11 DC_29-66_n30 R4-2114806 TP for TR 37.717-21-11 DC_2-46_n2 R4-2112447 TP for TR 37.717-21-11 DC_2-48_n2 R4-2114807 TP for TR 37.717-21-11 DC_13-46_n2 R4-2114808 TP for TR 37.717-21-11 DC_13-48_n77 R4-2112450 TP for TR 37.717-21-11 DC_48-66_n2 R4-2112451 TP for TR 37.717-21-11 DC_48-66_n66 R4-2114809 TP for TR 37.717-21-11 DC_48-66_n77 R4-2114811 TP for TR 37.717-21-11 DC_1-5_n77, DC_1-7_n77, DC_3-5_n77, DC_3-7_n77 and DC_5-7_n77 R4-2112929 TP for TR 37.717-21-11_DC_1A-7A_n3A R4-2112759 TP for TR 37.717-21-11 to include DC_2-(n)66 R4-2112930 TP for TR 37.717-21-11_DC_1A-38A_n3A R4-2112931 TP for TR 37.717-21-11_DC_7A-38A_n3A R4-2112932 TP for TR 37.717-21-11_DC_20A-38A_n3A R4-2113347 TP for TR 37.717-21-11: DC_1-32_n8 R4-2113348 TP for TR 37.717-21-11: DC_1-38_n8 R4-2113371 TP for TR 37.717-21-11: DC_7-32_n3 R4-2114820 TP for TR 37.717-21-11: DC_3A-32A_n28A and DC_3C-32A_n28A R4-2114821 TP for TR 37.717-21-11: DC_28A-32A_n1A R4-2114822 TP for TR 37.717-21-11: DC_28A-32A_n3A R4-2113393 TP for TR 37.717-21-11: DC_7-32_n8 R4-2113395 TP for TR 37.717-21-11: DC_20-32_n8 R4-2113396 TP for TR 37.717-21-11: DC_20-38_n1 R4-2113482 TP for TR 37.717-21-11: DC_32-38_n1 R4-2114151 TP to TR 37.717-21-11 for DC_7-7-28-n1 R4-2114901 TP for TR 37.717-21-11: DC_8A-20A_n28A |  |
| 2021-08 | 3GPP RAN4#101-e | R4-2117897 |  |  |  | The following approved TPs at RAN4#101-e have been implemented: R4-2119807 TP for TR 37.717-21-11: DC_8A_(n)3AA R4-2117654 TP for TR 37.717-21-11: EN-DC_1-11_n79 R4-2117657 TP for TR 37.717-21-11: EN-DC_8-11_n79 R4-2119812 TP for TR 37.717-21-11: EN-DC_8-41_n3 R4-2119813 TP for TR 37.717-21-11: EN-DC_8-42_n1 R4-2119816 TP for TR 37.717-21-11: EN-DC_8-41_n77 R4-2117917 PC3 TP for TR 37.717-21-11 DC_5-13_n77 R4-2117918 PC3 TP for TR 37.717-21-11 DC_5-48_n77 R4-2118195 TP for TR 37.717-21-11: DC_1A-7A_n38A R4-2118196 TP for TR 37.717-21-11: DC_3A-7A_n38A R4-2118197 TP for TR 37.717-21-11: DC_7A-20A_n38A | 0.7.0 |
| 2022-03 | 3GPP RAN4#102-e | R4-2205165 |  |  |  | The following approved TPs at RAN4#101bis-e have been implemented: R4-2200367 TP for TR 37.717-21-11 DC_n3A_1A-8A R4-2200368 TP for TR 37.717-21-11: DC_n77A_1A-8A and DC_n77(2A)_1A-8A R4-2200369 TP for TR 37.717-21-11 DC_n77A_3A_1A R4-2200370 TP for TR 37.717-21-11 DC_n77A_3A-8A and DC_n77(2A)_3A-8A R4-2201356 TP for TR 37.717-21-11_DC_1A-38A_n78A R4-2201357 TP for TR 37.717-21-11_DC_7A-38A_n78A R4-2202005 TP update for TR 37.717-21-11: EN-DC_1-11_n79 R4-2202007 TP update for TR 37.717-21-11: EN-DC_8-11_n79 R4-2202166 TP for TR 37.717-21-11: EN-DC_8-41_n1 R4-2202168 TP to TR 37.717-21-11 Addition of DC_2A-38A_n78A R4-2202169 TP to TR 37.717-21-11 Addition of DC_2A-28A_n78A The following approved TPs at RAN4#102-e have been implemented: R4-2203628 TP for TR 37.717-21-11: DC_8-32_n3 R4-2203629 TP for TR 37.717-21-11: DC_8-38_n1 R4-2206261 TP for TR 37.717-21-11: DC_28-38_n1 R4-2204554 TP for TR 37.717-21-11: EN-DC_8-11_n1 R4-2206269 Updated TP for TR 37.717-21-11: add MSD due to harmonic interference between band n28 and 32 | 0.8.0 |
| 2022-06 | 3GPP RAN4#103-e | R4-2208558 |  |  |  | The following approved TPs at RAN4#102-e have been implemented: R4-2207715 TP for TR 37.717-21-11 Addition of DC_2-14_n5 R4-2210747 TP for TR 37.717-21-11 Addition of DC_14-66_n5 R4-2207765 TP for TR 37.717-21-11: DC_1-20_n7 R4-2207932 TP for TR 37.717-21-11: DC_48-66_n2 R4-2210368 TP for TR 37.717-21-11: DC_2-48_n2 R4-2209020 TP for TR 37.717-21-11: DC_32A-38A_n28A R4-2210370 TP for TR 37.717-21-11 DC_3A-38A_n78A and DC_3C-38A_n78A R4-2209171 TP for TR 37.717-21-11 DC_8A-32A_n28A R4-2210371 TP for TR 37.717-21-11 to include DC_2-7_n25 R4-2209633 TP for TR 37.717-21-11_DC_1A-38A_n7A R4-2209634 TP for TR 37.717-21-11_DC_3A-38A_n7A R4-2210372 TP to TR 37.717-21-11 Addition of DC_40A-42A_n77A R4-2210373 TP to TR 37.717-21-11 Addition of DC_40A-42A_n78A R4-2209993 TP for TR 37.717-21-11: DC_n77A_1A-3A, DC_n77(2A)_1A-3A R4-2211128 Draft TP to TR on DC_1A_(n)3AA R4-2207716 TP for TR 37.717-21-11 Addition of DC_12-30_n5 R4-2207717 TP for TR 37.717-21-11 Addition of DC_14-30_n5 | 0.9.0 |