3GPP TSG RAN WG4 Meeting #103e	R4-2208578

Online, 9th – 20th May 2022

Source:		Rohde & Schwarz

Title:	Discussion on scheduling conflicts in UE performance RMCs

Agenda Item:		4.1.6.1

Document for:	Approval

# Introduction

During implementation of NR SA performance Test Cases an issue in the corresponding RMCs for UE testing has been observed with respect to collisions between SIB1 and PDSCH scheduling, which leads to undefined UE behaviour and devices potentially failing the test cases.

# Discussion

The RMCs for performance are defined in Annex A of TS 38.101-4 [1]. An example for FDD RMCs from [1] is shown below, for TDD the same issue exists as well:

Table A.3.2.1.1-1: PDSCH Reference Channel for FDD (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-1.1 FDD | R.PDSCH.1-1.2 FDD | R.PDSCH.1-1.3 FDD | R.PDSCH.1-1.4 FDD |  |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |  |
| Number of allocated resource blocks | PRBs | 52 | 6 | 52 | 52 |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 7 | 12 |  |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAMLowSE |  |
| MCS index |  | 4 | 4 | 4 | 14 |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |  |
| Target Coding Rate |  | 0.30 | 0.30 | 0.30 | 0.59 |  |
| Number of MIMO layers |  | 1 | 1 | 1 | 1 |  |
| Number of DMRS REs |  | 18 | 12 | 12 | 12 |  |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 3904 | 480 | 2280 | 8064 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 24 | 16 | 16 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | CBs | 1 | 1 | 1 | 1 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 10, 11 | Bits | 12480 | 1512 | 6864 | 13104 |  |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 13104 | 1584 | 7488 | 13728 |  |
| Max. Throughput averaged over 2 frames | Mbps | 3.709 | 0.456 | 2.166 | 7.661 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

As can be seen from the RMC no DL data is scheduled in slot 0 to avoid collisions in between PDSCH and SSB/PBCH. However no exception has been made to accommodate SIB1 transmission, which leads to collisions between SIB1 and the PDSCH scheduled according to the RMC.

Observation 1: All slots in the performance RMCs are filled with PDSCH, except for the ones with SSB/PBCH.

For the performance TCs in general TS 38.508-1 [2] chapter 5.4.2 defines the message contents for all TCs. Here the search space is configured as follows:

In our example above this leads to a SIB1 transmission in slot 5, however according to the RMC this slot is already filled with PDSCH transmission.

Observation 2: SIB1 transmission collides with PDSCH for all performance TCs, since all DL slots are filled with PDSCH, except for the slots with SSB/PBCH.

In TS 38.508-1 [2] there is also general section in chapter 4.6, which is used for all other test cases, for example RF test cases. Here the configuration is such that the SIB1 transmission happens in slot 1. This has been considered in the RMCs for Tx/Rx in TS 38.101-1 [3] and here there is no PDSCH scheduled in the corresponding slot, as can be seen in the example below.

Observation 3: For Tx/Rx TCs the issue does not exist, since here no DL transmission is scheduled for slots where SIB1 is expected.

To solve this problem, we propose to add some general text to the beginning of the Annex for the performance RMCs in TS 38.101-4 [1] to clarify the SIB1/PDSCH scheduling conflict. Since this issue is only existing in the case of SA (for NSA no SIB1 is transmitted) and the RMCs are used in both cases, this way we prevent having to create separate RMCs for SA and NSA. Additionally, it is a future proof solution in case RAN5 discovers the need to make some changes to SIB1 scheduling parameters.

An example for the wording is provided in the Annex of this paper.

Proposal: Update the the Annex A.3.1 of TS 38.101-4 to resolve the scheduling conflict between SIB1 and PDSCH.

# Proposals

In this paper we provide some further analysis for the FR1 UL MIMO EVM measurements.

Observation 1: All slots in the Performance RMCs are filled with PDSCH, except for the ones with SSB/PBCH.

Observation 2: SIB1 transmission collides with PDSCH for all performance TCs, since all DL slots are filled with PDSCH, except for the slots with SSB/PBCH.

Observation 3: For Tx/Rx TCs the issue does not exist, since here no DL transmission is scheduled for slots where SIB1 is expected.

Proposal: Update the the Annex A.3.1 of TS 38.101-4 to resolve the scheduling conflict between SIB1 and PDSCH.

# References

- TS 38.101-4

- TS 38.508-1

- TS 38.101-1

# Annex: Potential wording for TS 38.101-4

# A.3	DL reference measurement channels

## A.3.1	General

The transport block size (TBS) determination procedure is described in clause 5.1.3.2 of TS 38.214 [12].

Unless otherwise stated, no user data is scheduled on slot #0 within 20 ms in order to avoid SSB and PDSCH transmissions in one slot and simplify test configuration.

No user data is transmitted in slots where SIB1 transmissions are scheduled.