3GPP TSG-RAN WG4 Meeting # 103-e 	R4-2208649

# Electronic Meeting, May 09 – May 20, 2022

Source:	LG Electronics

Title:	60GHz channel and synchronization raster

Agenda item:	9.15.2	Operation bands and system parameters (channelization, raster, CBW, etc)

Document for:	Discussion and Approval

# Introduction

In RAN4#102bis-e meeting following agreement [1] was made on channel and sync raster arrangement for un-licensed operation within FR2-2.

Agreement: Use the fixed RF channel raster with the step size of 1680 (100.8 MHz) as baseline to define the channel raster for the unlicensed band, and accordingly provide the channel raster numbers

- To Check if the above solution can support CA with different bandwidth combinations

  - If there is issue identified, then the above agreement can be revisited

In this document we provide our views and proposal for channel and synchronization raster for 57-71GHz frequency range. We make two proposals where

- First one is aligned with proposal in previous meeting R4-2205233 [3] and utilizes the floating 960kHz ARFCN raster and ~100MHz (combination of 99.86/100.8MHz) SSB raster for 120kHz SCS. SSB raster locations for 480k SCS are down selected from 120kHz SCS SSB raster. This proposal is re-submitted as it utilizes the full 14 000MHz of spectrum and provides a lot of freedom for selecting the channel frequencies and flexibility for intra-band CA combinations.

- Second one uses the 100.8MHz fixed ARFCN and SSB raster. SSB raster locations for 480k SCS are down selected from 120kHz SCS SSB raster. RF channels are selected in a way that they target maximization of the spectrum usage and enable flexibility for difference CA combinations. Additional channel locations are added for alignment with 802.11 channels.

Proposals are described in more detail in sections 2 and 3 of this contribution.

# Channel and SSB raster proposals for 57-71GHz frequency range

## Proposal #1

### Proposal #1 ARFCN raster

In this proposal channel raster step size of 960kHz is used, which is the same as the largest supported SCS. Use of 960kHz is also aligning with tentative agreement on 100.8MHz based channel raster made in last meeting [1] as 105 x 0.96= 100.8MHz but also supports other alternatives that enable more options for positioning of the channels and flexibility for intra-band CA configurations.

More detailed analysis and the start/end points for frequency range 57-71GHz (n263) is shown below. Nref= 2 563 339, which corresponds to CF= 57 050.40MHz, has been selected as lowest channel location and all other channel locations are having offset that is multiple of <16> from this one (16x60kHz= 960kHz).

Last channel location has Nref= 2 794 987, which corresponds to CF= 70 949,28MHz and the number of channel locations within FR2-2 band is (2 563 339 - 2 490 011) / 16 + 1 = 14 479.

For 480k SCS and 960kHz SCS the lowest and highest channel center frequency is 200MHz away from the band edge, which is half of the 400MHz minimum CBW.

All the proposed channel raster entries have frequency offset to GSCN grid, which is a multiple of the 960kHz, and hence the NRU type of wide-band operation is possible.

|  | Min CBW MHz | N Low | Fc Low MHz | N High | Fc High MHz | n |
| --- | --- | --- | --- | --- | --- | --- |
| 120k SCS | 100 | 2 563 339 | 57050.4 | 2 794 987 | 70949.28 | 14479 |
| 480k SCS | 400 | 2 565 835 | 57200.16 | 2 792 491 | 70799.52 | 14167 |
| 960k SCS | 400 | 2 565 835 | 57200.16 | 2 792 491 | 70799.52 | 14167 |

This proposal can also be presented using the similar approach as in 38.101-2 table 5.4.2.3-1 as shown below.

Table 5.2-1: NR operating bands in FR2

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |
| … |  |  |  |  |  |  |  |
| n263 | 57000 MHz | – | 71000 MHz | 57000 MHz | – | 71000 MHz | TDD |

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n257 | 60 | 2054166 – <1> – 2104165 |
|  | 120 | 2054167 – <2> – 2104165 |
| n258 | 60 | 2016667 – <1> – 2070832 |
|  | 120 | 2016667 – <2> – 2070831 |
| n260 | 60 | 2229166 – <1> – 2279165 |
|  | 120 | 2229167 – <2> – 2279165 |
| n261 | 60 | 2070833 – <1> – 2084999 |
|  | 120 | 2070833 – <2> – 2084999 |
| … |  |  |
| n263 | 120 | 2563339– <16> – 2794987 |
|  | 480 | 2565835 – <16> – 2792491 |
|  | 960 | 2565835 – <16> – 2792491 |

### Proposal #1 SSB raster

A global synchronization raster defined in 38.101-2 [2] can be used as starting point also for FR2-2. The specified GSCN frequency range already covers the 57 to 71GHz frequency range and 17.28MHz step size is suitable for 100 and 400MHz minimum channel bandwidths agreed for 120kHz and 480kHz SCS respectively.

In order to limit the number of the SSB locations and simplify the cell search complexity one 120k SCS SSB location is defined for each 100MHz of spectrum. This leads to 140 SSB locations with 120k SCS.

For 480k SCS and each 400MHz of spectrum two SSB locations are defined. SSB locations for 400MHz are down selected from 120kHz SSB raster as shown in Figure 1.

Figure 1. High level description on selection of GSCN locations for 100MHz channels and further down-selection for 400MHz.

The down-selection of the global SSB raster for 100MHz channels can be done with step size vector with length of 19 and values of < 6 6 6 6 5 6 6 6 6 5 6 6 6 5 6 6 6 6 5 >. In order to align the GSCN raster with ARFCN raster to support wide-band operation as in NRU a starting point of 24153 is selected. For 480k SCS with further down-selected entries (from 100MHz locations) the 70 locations can be listed in the specification and are shown in Table 2 below.

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2)

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n263 57-71GHz | 120kHz | Case D | 24153 < NOTE2 > 24958 |
|  | 480kHz | Case F | 24153 < NOTE2 > 24958 |
|  | 960kHz | Case G | 24153 < NOTE2 > 24958 |
| NOTE1: SS Block pattern is defined in sub clause 4.1 in TS 38.213 [10] NOTE2: Step size vector with length of 19: < 6 6 6 6 5 6 6 6 6 5 6 6 6 5 6 6 6 6 5 > NOTE3: The following GSCN are allowed for 480kHz and 960kHz Pcell and Scell operation in band n263: for 480 kHz, GSCN = { 24159, 24165, 24182, 24188, 24206, 24211, 24229, 24234, 24252, 24258, 24275, 24281, 24298, 24304, 24321, 24327, 24344, 24350, 24368, 24373, 24391, 24397, 24414, 24420, 24437, 24443, 24460, 24466, 24483, 24489, 24507, 24512, 24530, 24536, 24553, 24559, 24576, 24582, 24599, 24605, 24622, 24628, 24646, 24651, 24669, 24674, 24692, 24698, 24715, 24721, 24738, 24744, 24761, 24767, 24784, 24790, 24808, 24813, 24831, 24837, 24854, 24860, 24877, 24883, 24900, 24906, 24923, 24929, 24947, 24952} for 960 kHz, no applicable SS raster entries exist for PCell and PScell. |  |  |  |

With this approach the total number of SS raster entries is 210 as shown in Table 3.

Table 2. Number of SS raster entries

|  | Channels | Locations per channel | Total |
| --- | --- | --- | --- |
| 120k SCS | 140 | 1 | 140 |
| 480k SCS | 35 | 2 | 70 |
| TOTAL |  |  | 210 |

Figure 2 below show how the SSBs falls inside the 100 and 400MHz channels when defined as in Table 3. The width of the upper and lower guard-band (gap between SSB and channel edge) are always larger than the minimum guard-bands currently defined in 38.101-2, which are 2.42MHz for 100MHz and 9.86MHz for 400MHz CBW.

Figure 2. SSB center frequency locations within each 100MHz and 400MHz of spectrum.

List of SSB locations for 120kHz and 480kHz SCS for Proposal 1 are shown in Annex 1.

## Proposal #2

### Proposal #2 ARFCN raster

In this proposal fixed channel raster step size of 100.8MHz is used, which is aligned with tentative agreement made in last meeting [1].

ARFCN= 2 564 083, which corresponds to CF= 57095.04 MHz, has been selected as lowest channel location and all other 100.8MHz channel locations are having offset that is multiple of <1680> from this one (1680x60kHz= 100.8MHz). Last channel location has ARFCN = 2 794 243, which corresponds to CF= 70 904.64 MHz and the number of channel locations within FR2-2 band is 138. These 100MHz channel locations are the same as in R4-2205998 [4].

Wider channels are proposed to be located as shown in Figure 3 below. First 34+33+16+12 channels (with 403.2, 806.4, 1612.8 and 2016 raster) are placed with intent to utilize as much spectrum as possible and enable flexible support for intra-band CA. In addition, 20+16+6+6 channels (with 403.2, 806.4, 1612.8 and 2016MHz raster) are added for alignment with 802.11. These additional channels are needed to align with 802.11ad channels 1, 3, 4 and 6 while baseline channels can be used for alignment with 802.11ad channels 2 and 5. The total number of channels 233+48= 281 becomes altogether.

Table 3. Number of channels for each (CBW, SCS) combination

| SCS | CBW |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 100.8 MHz | 403.2 MHz | 806.4 MHz | 1612.8 MHz | 2016 MHz |
| 120 kHz | 138 | 34+20=54 | N/A | N/A | N/A |
| 480 kHz | N/A |  | 33+16=49 | 16+6=22 | N/A |
| 960 kHz | N/A |  |  |  | 12+6=18 |

Figure 3. Proposal #2 channels

This proposal can also be presented using the similar approach as in 38.101-2 table 5.3.2.3-1 as shown below.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n257 | 60 | 2054166 – <1> – 2104165 |
|  | 120 | 2054167 – <2> – 2104165 |
| n258 | 60 | 2016667 – <1> – 2070832 |
|  | 120 | 2016667 – <2> – 2070831 |
| n260 | 60 | 2229166 – <1> – 2279165 |
|  | 120 | 2229167 – <2> – 2279165 |
| n261 | 60 | 2070833 – <1> – 2084999 |
|  | 120 | 2070833 – <2> – 2084999 |
| … |  |  |
| n263 | 120 | 2564083– <1680> – 2794243 |
|  | 480 | 2566603– <1680> – 2788363 |
|  | 960 | 2566603– <1680> – 2788363 |
| NOTE 1: Applicable NR-ARFCN for band n263 for 400 MHz channel bandwidth, NREF = { 2566603, 2573323, 2580043, 2586763, 2593483, 2600203, 2606923, 2613643, 2620363, 2627083, 2633803, 2640523, 2647243, 2653963, 2660683, 2667403, 2674123, 2680843, 2687563, 2694283, 2701003, 2707723, 2714443, 2721163, 2727883, 2734603, 2741323, 2748043, 2754763, 2761483, 2768203, 2774923, 2781643, 2788363, 2571643, 2578363, 2585083, 2591803, 2598523, 2642203, 2648923, 2655643, 2662363, 2669083, 2679163, 2685883, 2692603, 2699323, 2706043, 2751403, 2758123, 2764843, 2771563, 2778283} for 800 MHz channel bandwidth, NREF = { 2569963, 2576683, 2583403, 2590123, 2596843, 2603563, 2610283, 2617003, 2623723, 2630443, 2637163, 2643883, 2650603, 2657323, 2664043, 2670763, 2677483, 2684203, 2690923, 2697643, 2704363, 2711083, 2717803, 2724523, 2731243, 2737963, 2744683, 2751403, 2758123, 2764843, 2771563, 2778283, 2785003, 2575003, 2581723, 2588443, 2595163, 2645563, 2652283, 2659003, 2665723, 2682523, 2689243, 2695963, 2702683, 2754763, 2761483, 2768203, 2774923} for 1600 MHz channel bandwidth, NREF = { 2576683, 2590123, 2603563, 2617003, 2630443, 2643883, 2657323, 2670763, 2684203, 2697643, 2711083, 2724523, 2737963, 2751403, 2764843, 2778283, 2581723, 2623723, 2652283, 2695963, 2724523, 2768203} for 2000 MHz channel bandwidth, NREF = { 2576683, 2603563, 2610283, 2637163, 2643883, 2670763, 2677483, 2704363, 2711083, 2737963, 2744683, 2771563, 2585083, 2620363, 2655643, 2692603, 2727883, 2764843} |  |  |

### Proposal #2 SSB raster

In order to limit the number of the SSB locations and simplify the cell search complexity one 120k SCS SSB location is defined for each 100.8MHz channel. This leads to 138 SSB locations with 120k SCS.

For 480k SCS and 403.2MHz channels one SSB location is also defined. SSB locations for 403.2MHz channels are down selected from 120kHz SSBs in a way that raster location of the 2nd 100.8MHz channel inside each 403.2MHz is reused as shown in Figure 4. 806.4, 1612.8 and 2016MHz channels can reuse the GSCN locations of the 403.2MHz channels.

Figure 4. GSCN down-selection principle for 480kHz SSBs.

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2)

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n263 57-71GHz | 120kHz | Case D | 24153 < NOTE3 > 24956 |
|  | 480kHz | Case F | 24153 < NOTE3 > 24956 |
|  | 960kHz | Case G | 24153 < NOTE3 > 24956 |
| NOTE1: SS Block pattern is defined in sub clause 4.1 in TS 38.213 [10] NOTE2: Step size vector with length of 6: < 6 5 6 6 6 6 > NOTE3: The following GSCN are allowed for 480kHz and 960kHz Pcell and Scell operation in band n263: for 480 kHz, GSCN = { 24163, 24180, 24186, 24203, 24209, 24227, 24233, 24250, 24256, 24273, 24279, 24303, 24326, 24349, 24373, 24396, 24419, 24425, 24443, 24448, 24466, 24472, 24489, 24495, 24513, 24518, 24536, 24553, 24559, 24577, 24583, 24600, 24606, 24623, 24629, 24647, 24653, 24676, 24699, 24723, 24746, 24769, 24793, 24804, 24816, 24828, 24839, 24851, 24863, 24874, 24886, 24898, 24909, 24933}. for 960 kHz, no applicable SS raster entries exist for PCell and PScell. |  |  |  |

With this approach the total number of SS raster entries is 210 as shown in Table 3.

Table 2. Number of SS raster entries

|  | Channels | Locations per channel | Total |
| --- | --- | --- | --- |
| 120k SCS | 138 | 1 | 138 |
| 480k SCS | 54 | 1 | 54 |
| TOTAL |  |  | 192 |

Figure 5 below show how the SSBs falls inside the 100.8MHz channels when defined as in Table 3. The width of the upper and lower guard-band (gap between SSB and channel edge). With 100.8MHz channel raster and the 6 long step size vector < 6 5 6 6 6 6 > the SSB remains all the time in the center part of the channel as 35 x 17.28MHz= 6 x 100.8MHz.

Figure 5. SSB center frequency locations within each 100.8MHz and 403.2MHz channels.

# Conclusion

In this document we provide our views and proposal for channel and synchronization raster for 57-71GHz frequency range. We make two proposals where

- First one is aligned with proposal in previous meeting R4-2205233 [3] and utilizes the floating 960kHz ARFCN raster and ~100MHz (combination of 99.86/100.8MHz) SSB raster for 120kHz SCS. SSB raster locations for 480k SCS are down selected from 120kHz SCS SSB raster. This proposal is re-submitted as it utilizes the full 14 000MHz of spectrum and provides a lot of freedom for selecting the channel frequencies and flexibility for intra-band CA combinations.

- Second one uses the 100.8MHz fixed ARFCN and SSB raster. SSB raster locations for 480k SCS are down selected from 120kHz SCS SSB raster. RF channels are selected in a way that they target maximization of the spectrum usage and enable flexibility for difference CA combinations. Additional channel locations are added for alignment with 802.11 channels.

Proposals are described in more detail in sections 2 and 3 of this contribution.

The key difference and similarities between these proposals are shown in table below.

|  | Proposal #1 | Proposal #2 |
| --- | --- | --- |
| Channel raster type | Floating | Fixed |
| Channel raster | 960kHz | 100.8MHz for 100MHz CBW Raster steps for wider CBWs are multiples of 100.8MHz |
| Flexibility to support different intra-band CA combinations | Very high | High |
| 120kHz SCS SSB raster | Fixed One SSB per 100MHz 140 locations | Fixed One SSB per 100.8MHz 138 locations |
| 480kHz SCS SSB raster | Fixed Two SSB per 400MHz 70 locations | Fixed One SSB per 400MHz 54 locations |
| Alignment with 802.11 possible | YES | YES |

List of SSB locations for Proposal #1 and Proposal #2 are listed in Annex 1 and Annex 2 respectively.

# References

[1] R4-2206535, WF on system parameters of FR2-2, RAN4#102bis-e, March 2022

[2] 3GPP TS 38.101-2, User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone

[3] R4-2205233, 60GHz channel and synchronization raster, March 2022

[4] R4-2205998, Views on FR2-2 channelization, March 2022

ANNEX 1:

SSB locations for n263 as per Proposal#1

ARFCN from 960k floating raster for the table below has been selected in a way that delta between the channel center frequency and the center frequency of each 100MHz spectrum block is minimized.

| 100MHz CH# | ARFCN | Channel low | Channel High | GSCN 120k SCS | 400MHz CH# | GSCN 480k SCS |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 563 339 | 57 000.40 | 57 100.40 | 24 153 |  |  |
| 2 | 2 565 003 | 57 100.24 | 57 200.24 | 24 159 | 1 | 24 159 |
| 3 | 2 566 667 | 57 200.08 | 57 300.08 | 24 165 | 1 | 24 165 |
| 4 | 2 568 331 | 57 299.92 | 57 399.92 | 24 171 |  |  |
| 5 | 2 569 995 | 57 399.76 | 57 499.76 | 24 177 |  |  |
| 6 | 2 571 659 | 57 499.60 | 57 599.60 | 24 182 | 2 | 24 182 |
| 7 | 2 573 339 | 57 600.40 | 57 700.40 | 24 188 | 2 | 24 188 |
| 8 | 2 575 003 | 57 700.24 | 57 800.24 | 24 194 |  |  |
| 9 | 2 576 667 | 57 800.08 | 57 900.08 | 24 200 |  |  |
| 10 | 2 578 331 | 57 899.92 | 57 999.92 | 24 206 | 3 | 24 206 |
| 11 | 2 579 995 | 57 999.76 | 58 099.76 | 24 211 | 3 | 24 211 |
| 12 | 2 581 659 | 58 099.60 | 58 199.60 | 24 217 |  |  |
| 13 | 2 583 339 | 58 200.40 | 58 300.40 | 24 223 |  |  |
| 14 | 2 585 003 | 58 300.24 | 58 400.24 | 24 229 | 4 | 24 229 |
| 15 | 2 586 667 | 58 400.08 | 58 500.08 | 24 234 | 4 | 24 234 |
| 16 | 2 588 331 | 58 499.92 | 58 599.92 | 24 240 |  |  |
| 17 | 2 589 995 | 58 599.76 | 58 699.76 | 24 246 |  |  |
| 18 | 2 591 659 | 58 699.60 | 58 799.60 | 24 252 | 5 | 24 252 |
| 19 | 2 593 339 | 58 800.40 | 58 900.40 | 24 258 | 5 | 24 258 |
| 20 | 2 595 003 | 58 900.24 | 59 000.24 | 24 263 |  |  |
| 21 | 2 596 667 | 59 000.08 | 59 100.08 | 24 269 |  |  |
| 22 | 2 598 331 | 59 099.92 | 59 199.92 | 24 275 | 6 | 24 275 |
| 23 | 2 599 995 | 59 199.76 | 59 299.76 | 24 281 | 6 | 24 281 |
| 24 | 2 601 659 | 59 299.60 | 59 399.60 | 24 287 |  |  |
| 25 | 2 603 339 | 59 400.40 | 59 500.40 | 24 292 |  |  |
| 26 | 2 605 003 | 59 500.24 | 59 600.24 | 24 298 | 7 | 24 298 |
| 27 | 2 606 667 | 59 600.08 | 59 700.08 | 24 304 | 7 | 24 304 |
| 28 | 2 608 331 | 59 699.92 | 59 799.92 | 24 310 |  |  |
| 29 | 2 609 995 | 59 799.76 | 59 899.76 | 24 316 |  |  |
| 30 | 2 611 659 | 59 899.60 | 59 999.60 | 24 321 | 8 | 24 321 |
| 31 | 2 613 339 | 60 000.40 | 60 100.40 | 24 327 | 8 | 24 327 |
| 32 | 2 615 003 | 60 100.24 | 60 200.24 | 24 333 |  |  |
| 33 | 2 616 667 | 60 200.08 | 60 300.08 | 24 339 |  |  |
| 34 | 2 618 331 | 60 299.92 | 60 399.92 | 24 344 | 9 | 24 344 |
| 35 | 2 619 995 | 60 399.76 | 60 499.76 | 24 350 | 9 | 24 350 |
| 36 | 2 621 659 | 60 499.60 | 60 599.60 | 24 356 |  |  |
| 37 | 2 623 339 | 60 600.40 | 60 700.40 | 24 362 |  |  |
| 38 | 2 625 003 | 60 700.24 | 60 800.24 | 24 368 | 10 | 24 368 |
| 39 | 2 626 667 | 60 800.08 | 60 900.08 | 24 373 | 10 | 24 373 |
| 40 | 2 628 331 | 60 899.92 | 60 999.92 | 24 379 |  |  |
| 41 | 2 629 995 | 60 999.76 | 61 099.76 | 24 385 |  |  |
| 42 | 2 631 659 | 61 099.60 | 61 199.60 | 24 391 | 11 | 24 391 |
| 43 | 2 633 339 | 61 200.40 | 61 300.40 | 24 397 | 11 | 24 397 |
| 44 | 2 635 003 | 61 300.24 | 61 400.24 | 24 402 |  |  |
| 45 | 2 636 667 | 61 400.08 | 61 500.08 | 24 408 |  |  |
| 46 | 2 638 331 | 61 499.92 | 61 599.92 | 24 414 | 12 | 24 414 |
| 47 | 2 639 995 | 61 599.76 | 61 699.76 | 24 420 | 12 | 24 420 |
| 48 | 2 641 659 | 61 699.60 | 61 799.60 | 24 426 |  |  |
| 49 | 2 643 339 | 61 800.40 | 61 900.40 | 24 431 |  |  |
| 50 | 2 645 003 | 61 900.24 | 62 000.24 | 24 437 | 13 | 24 437 |
| 51 | 2 646 667 | 62 000.08 | 62 100.08 | 24 443 | 13 | 24 443 |
| 52 | 2 648 331 | 62 099.92 | 62 199.92 | 24 449 |  |  |
| 53 | 2 649 995 | 62 199.76 | 62 299.76 | 24 454 |  |  |
| 54 | 2 651 659 | 62 299.60 | 62 399.60 | 24 460 | 14 | 24 460 |
| 55 | 2 653 339 | 62 400.40 | 62 500.40 | 24 466 | 14 | 24 466 |
| 56 | 2 655 003 | 62 500.24 | 62 600.24 | 24 472 |  |  |
| 57 | 2 656 667 | 62 600.08 | 62 700.08 | 24 478 |  |  |
| 58 | 2 658 331 | 62 699.92 | 62 799.92 | 24 483 | 15 | 24 483 |
| 59 | 2 659 995 | 62 799.76 | 62 899.76 | 24 489 | 15 | 24 489 |
| 60 | 2 661 659 | 62 899.60 | 62 999.60 | 24 495 |  |  |
| 61 | 2 663 339 | 63 000.40 | 63 100.40 | 24 501 |  |  |
| 62 | 2 665 003 | 63 100.24 | 63 200.24 | 24 507 | 16 | 24 507 |
| 63 | 2 666 667 | 63 200.08 | 63 300.08 | 24 512 | 16 | 24 512 |
| 64 | 2 668 331 | 63 299.92 | 63 399.92 | 24 518 |  |  |
| 65 | 2 669 995 | 63 399.76 | 63 499.76 | 24 524 |  |  |
| 66 | 2 671 659 | 63 499.60 | 63 599.60 | 24 530 | 17 | 24 530 |
| 67 | 2 673 339 | 63 600.40 | 63 700.40 | 24 536 | 17 | 24 536 |
| 68 | 2 675 003 | 63 700.24 | 63 800.24 | 24 541 |  |  |
| 69 | 2 676 667 | 63 800.08 | 63 900.08 | 24 547 |  |  |
| 70 | 2 678 331 | 63 899.92 | 63 999.92 | 24 553 | 18 | 24 553 |
| 71 | 2 679 995 | 63 999.76 | 64 099.76 | 24 559 | 18 | 24 559 |
| 72 | 2 681 659 | 64 099.60 | 64 199.60 | 24 564 |  |  |
| 73 | 2 683 339 | 64 200.40 | 64 300.40 | 24 570 |  |  |
| 74 | 2 685 003 | 64 300.24 | 64 400.24 | 24 576 | 19 | 24 576 |
| 75 | 2 686 667 | 64 400.08 | 64 500.08 | 24 582 | 19 | 24 582 |
| 76 | 2 688 331 | 64 499.92 | 64 599.92 | 24 588 |  |  |
| 77 | 2 689 995 | 64 599.76 | 64 699.76 | 24 593 |  |  |
| 78 | 2 691 659 | 64 699.60 | 64 799.60 | 24 599 | 20 | 24 599 |
| 79 | 2 693 339 | 64 800.40 | 64 900.40 | 24 605 | 20 | 24 605 |
| 80 | 2 695 003 | 64 900.24 | 65 000.24 | 24 611 |  |  |
| 81 | 2 696 667 | 65 000.08 | 65 100.08 | 24 617 |  |  |
| 82 | 2 698 331 | 65 099.92 | 65 199.92 | 24 622 | 21 | 24 622 |
| 83 | 2 699 995 | 65 199.76 | 65 299.76 | 24 628 | 21 | 24 628 |
| 84 | 2 701 659 | 65 299.60 | 65 399.60 | 24 634 |  |  |
| 85 | 2 703 339 | 65 400.40 | 65 500.40 | 24 640 |  |  |
| 86 | 2 705 003 | 65 500.24 | 65 600.24 | 24 646 | 22 | 24 646 |
| 87 | 2 706 667 | 65 600.08 | 65 700.08 | 24 651 | 22 | 24 651 |
| 88 | 2 708 331 | 65 699.92 | 65 799.92 | 24 657 |  |  |
| 89 | 2 709 995 | 65 799.76 | 65 899.76 | 24 663 |  |  |
| 90 | 2 711 659 | 65 899.60 | 65 999.60 | 24 669 | 23 | 24 669 |
| 91 | 2 713 339 | 66 000.40 | 66 100.40 | 24 674 | 23 | 24 674 |
| 92 | 2 715 003 | 66 100.24 | 66 200.24 | 24 680 |  |  |
| 93 | 2 716 667 | 66 200.08 | 66 300.08 | 24 686 |  |  |
| 94 | 2 718 331 | 66 299.92 | 66 399.92 | 24 692 | 24 | 24 692 |
| 95 | 2 719 995 | 66 399.76 | 66 499.76 | 24 698 | 24 | 24 698 |
| 96 | 2 721 659 | 66 499.60 | 66 599.60 | 24 703 |  |  |
| 97 | 2 723 339 | 66 600.40 | 66 700.40 | 24 709 |  |  |
| 98 | 2 725 003 | 66 700.24 | 66 800.24 | 24 715 | 25 | 24 715 |
| 99 | 2 726 667 | 66 800.08 | 66 900.08 | 24 721 | 25 | 24 721 |
| 100 | 2 728 331 | 66 899.92 | 66 999.92 | 24 727 |  |  |
| 101 | 2 729 995 | 66 999.76 | 67 099.76 | 24 732 |  |  |
| 102 | 2 731 659 | 67 099.60 | 67 199.60 | 24 738 | 26 | 24 738 |
| 103 | 2 733 339 | 67 200.40 | 67 300.40 | 24 744 | 26 | 24 744 |
| 104 | 2 735 003 | 67 300.24 | 67 400.24 | 24 750 |  |  |
| 105 | 2 736 667 | 67 400.08 | 67 500.08 | 24 756 |  |  |
| 106 | 2 738 331 | 67 499.92 | 67 599.92 | 24 761 | 27 | 24 761 |
| 107 | 2 739 995 | 67 599.76 | 67 699.76 | 24 767 | 27 | 24 767 |
| 108 | 2 741 659 | 67 699.60 | 67 799.60 | 24 773 |  |  |
| 109 | 2 743 339 | 67 800.40 | 67 900.40 | 24 779 |  |  |
| 110 | 2 745 003 | 67 900.24 | 68 000.24 | 24 784 | 28 | 24 784 |
| 111 | 2 746 667 | 68 000.08 | 68 100.08 | 24 790 | 28 | 24 790 |
| 112 | 2 748 331 | 68 099.92 | 68 199.92 | 24 796 |  |  |
| 113 | 2 749 995 | 68 199.76 | 68 299.76 | 24 802 |  |  |
| 114 | 2 751 659 | 68 299.60 | 68 399.60 | 24 808 | 29 | 24 808 |
| 115 | 2 753 339 | 68 400.40 | 68 500.40 | 24 813 | 29 | 24 813 |
| 116 | 2 755 003 | 68 500.24 | 68 600.24 | 24 819 |  |  |
| 117 | 2 756 667 | 68 600.08 | 68 700.08 | 24 825 |  |  |
| 118 | 2 758 331 | 68 699.92 | 68 799.92 | 24 831 | 30 | 24 831 |
| 119 | 2 759 995 | 68 799.76 | 68 899.76 | 24 837 | 30 | 24 837 |
| 120 | 2 761 659 | 68 899.60 | 68 999.60 | 24 842 |  |  |
| 121 | 2 763 339 | 69 000.40 | 69 100.40 | 24 848 |  |  |
| 122 | 2 765 003 | 69 100.24 | 69 200.24 | 24 854 | 31 | 24 854 |
| 123 | 2 766 667 | 69 200.08 | 69 300.08 | 24 860 | 31 | 24 860 |
| 124 | 2 768 331 | 69 299.92 | 69 399.92 | 24 866 |  |  |
| 125 | 2 769 995 | 69 399.76 | 69 499.76 | 24 871 |  |  |
| 126 | 2 771 659 | 69 499.60 | 69 599.60 | 24 877 | 32 | 24 877 |
| 127 | 2 773 339 | 69 600.40 | 69 700.40 | 24 883 | 32 | 24 883 |
| 128 | 2 775 003 | 69 700.24 | 69 800.24 | 24 889 |  |  |
| 129 | 2 776 667 | 69 800.08 | 69 900.08 | 24 894 |  |  |
| 130 | 2 778 331 | 69 899.92 | 69 999.92 | 24 900 | 33 | 24 900 |
| 131 | 2 779 995 | 69 999.76 | 70 099.76 | 24 906 | 33 | 24 906 |
| 132 | 2 781 659 | 70 099.60 | 70 199.60 | 24 912 |  |  |
| 133 | 2 783 339 | 70 200.40 | 70 300.40 | 24 918 |  |  |
| 134 | 2 785 003 | 70 300.24 | 70 400.24 | 24 923 | 34 | 24 923 |
| 135 | 2 786 667 | 70 400.08 | 70 500.08 | 24 929 | 34 | 24 929 |
| 136 | 2 788 331 | 70 499.92 | 70 599.92 | 24 935 |  |  |
| 137 | 2 789 995 | 70 599.76 | 70 699.76 | 24 941 |  |  |
| 138 | 2 791 659 | 70 699.60 | 70 799.60 | 24 947 | 35 | 24 947 |
| 139 | 2 793 339 | 70 800.40 | 70 900.40 | 24 952 | 35 | 24 952 |
| 140 | 2 794 987 | 70 899.28 | 70 999.28 | 24 958 |  |  |

ANNEX 2:

SSB locations for n263 as per Proposal#2

100.8MHz CH# and 403.2MHzCH# are using same channel index numbering as Figure 3.

| 100.8MHz CH# | ARFCN | Channel low | Channel High | GSCN 120k SCS | 403.2MHz CH# | GSCN 480k SCS | Added to align with 802.11 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 564 083 | 57 044.64 | 57 145.44 | 24 157 |  |  |  |
| 2 | 2 565 763 | 57 145.44 | 57 246.24 | 24 163 | 1 | 24 163 |  |
| 3 | 2 567 443 | 57 246.24 | 57 347.04 | 24 168 |  |  |  |
| 4 | 2 569 123 | 57 347.04 | 57 447.84 | 24 174 |  |  |  |
| 5 | 2 570 803 | 57 447.84 | 57 548.64 | 24 180 | 35 | 24 180 | X |
| 6 | 2 572 483 | 57 548.64 | 57 649.44 | 24 186 | 3 | 24 186 |  |
| 7 | 2 574 163 | 57 649.44 | 57 750.24 | 24 192 |  |  |  |
| 8 | 2 575 843 | 57 750.24 | 57 851.04 | 24 198 |  |  |  |
| 9 | 2 577 523 | 57 851.04 | 57 951.84 | 24 203 | 36 | 24 203 | X |
| 10 | 2 579 203 | 57 951.84 | 58 052.64 | 24 209 | 3 | 24 209 |  |
| 11 | 2 580 883 | 58 052.64 | 58 153.44 | 24 215 |  |  |  |
| 12 | 2 582 563 | 58 153.44 | 58 254.24 | 24 221 |  |  |  |
| 13 | 2 584 243 | 58 254.24 | 58 355.04 | 24 227 | 37 | 24 227 | X |
| 14 | 2 585 923 | 58 355.04 | 58 455.84 | 24 233 | 4 | 24 233 |  |
| 15 | 2 587 603 | 58 455.84 | 58 556.64 | 24 238 |  |  |  |
| 16 | 2 589 283 | 58 556.64 | 58 657.44 | 24 244 |  |  |  |
| 17 | 2 590 963 | 58 657.44 | 58 758.24 | 24 250 | 38 | 24 250 | X |
| 18 | 2 592 643 | 58 758.24 | 58 859.04 | 24 256 | 5 | 24 256 |  |
| 19 | 2 594 323 | 58 859.04 | 58 959.84 | 24 262 |  |  |  |
| 20 | 2 596 003 | 58 959.84 | 59 060.64 | 24 268 |  |  |  |
| 21 | 2 597 683 | 59 060.64 | 59 161.44 | 24 273 | 39 | 24 273 | X |
| 22 | 2 599 363 | 59 161.44 | 59 262.24 | 24 279 | 6 | 24 279 |  |
| 23 | 2 601 043 | 59 262.24 | 59 363.04 | 24 285 |  |  |  |
| 24 | 2 602 723 | 59 363.04 | 59 463.84 | 24 291 |  |  |  |
| 25 | 2 604 403 | 59 463.84 | 59 564.64 | 24 297 |  |  |  |
| 26 | 2 606 083 | 59 564.64 | 59 665.44 | 24 303 | 7 | 24 303 |  |
| 27 | 2 607 763 | 59 665.44 | 59 766.24 | 24 308 |  |  |  |
| 28 | 2 609 443 | 59 766.24 | 59 867.04 | 24 314 |  |  |  |
| 29 | 2 611 123 | 59 867.04 | 59 967.84 | 24 320 |  |  |  |
| 30 | 2 612 803 | 59 967.84 | 60 068.64 | 24 326 | 8 | 24 326 |  |
| 31 | 2 614 483 | 60 068.64 | 60 169.44 | 24 332 |  |  |  |
| 32 | 2 616 163 | 60 169.44 | 60 270.24 | 24 338 |  |  |  |
| 33 | 2 617 843 | 60 270.24 | 60 371.04 | 24 343 |  |  |  |
| 34 | 2 619 523 | 60 371.04 | 60 471.84 | 24 349 | 9 | 24 349 |  |
| 35 | 2 621 203 | 60 471.84 | 60 572.64 | 24 355 |  |  |  |
| 36 | 2 622 883 | 60 572.64 | 60 673.44 | 24 361 |  |  |  |
| 37 | 2 624 563 | 60 673.44 | 60 774.24 | 24 367 |  |  |  |
| 38 | 2 626 243 | 60 774.24 | 60 875.04 | 24 373 | 10 | 24 373 |  |
| 39 | 2 627 923 | 60 875.04 | 60 975.84 | 24 378 |  |  |  |
| 40 | 2 629 603 | 60 975.84 | 61 076.64 | 24 384 |  |  |  |
| 41 | 2 631 283 | 61 076.64 | 61 177.44 | 24 390 |  |  |  |
| 42 | 2 632 963 | 61 177.44 | 61 278.24 | 24 396 | 11 | 24 396 |  |
| 43 | 2 634 643 | 61 278.24 | 61 379.04 | 24 402 |  |  |  |
| 44 | 2 636 323 | 61 379.04 | 61 479.84 | 24 408 |  |  |  |
| 45 | 2 638 003 | 61 479.84 | 61 580.64 | 24 413 |  |  |  |
| 46 | 2 639 683 | 61 580.64 | 61 681.44 | 24 419 | 12 | 24 419 |  |
| 47 | 2 641 363 | 61 681.44 | 61 782.24 | 24 425 | 40 | 24 425 | X |
| 48 | 2 643 043 | 61 782.24 | 61 883.04 | 24 431 |  |  |  |
| 49 | 2 644 723 | 61 883.04 | 61 983.84 | 24 437 |  |  |  |
| 50 | 2 646 403 | 61 983.84 | 62 084.64 | 24 443 | 13 | 24 443 |  |
| 51 | 2 648 083 | 62 084.64 | 62 185.44 | 24 448 | 41 | 24 448 | X |
| 52 | 2 649 763 | 62 185.44 | 62 286.24 | 24 454 |  |  |  |
| 53 | 2 651 443 | 62 286.24 | 62 387.04 | 24 460 |  |  |  |
| 54 | 2 653 123 | 62 387.04 | 62 487.84 | 24 466 | 14 | 24 466 |  |
| 55 | 2 654 803 | 62 487.84 | 62 588.64 | 24 472 | 42 | 24 472 | X |
| 56 | 2 656 483 | 62 588.64 | 62 689.44 | 24 478 |  |  |  |
| 57 | 2 658 163 | 62 689.44 | 62 790.24 | 24 483 |  |  |  |
| 58 | 2 659 843 | 62 790.24 | 62 891.04 | 24 489 | 15 | 24 489 |  |
| 59 | 2 661 523 | 62 891.04 | 62 991.84 | 24 495 | 43 | 24 495 | X |
| 60 | 2 663 203 | 62 991.84 | 63 092.64 | 24 501 |  |  |  |
| 61 | 2 664 883 | 63 092.64 | 63 193.44 | 24 507 |  |  |  |
| 62 | 2 666 563 | 63 193.44 | 63 294.24 | 24 513 | 16 | 24 513 |  |
| 63 | 2 668 243 | 63 294.24 | 63 395.04 | 24 518 | 44 | 24 518 | X |
| 64 | 2 669 923 | 63 395.04 | 63 495.84 | 24 524 |  |  |  |
| 65 | 2 671 603 | 63 495.84 | 63 596.64 | 24 530 |  |  |  |
| 66 | 2 673 283 | 63 596.64 | 63 697.44 | 24 536 | 17 | 24 536 |  |
| 67 | 2 674 963 | 63 697.44 | 63 798.24 | 24 542 |  |  |  |
| 68 | 2 676 643 | 63 798.24 | 63 899.04 | 24 548 |  |  |  |
| 69 | 2 678 323 | 63 899.04 | 63 999.84 | 24 553 | 45 | 24 553 | X |
| 70 | 2 680 003 | 63 999.84 | 64 100.64 | 24 559 | 18 | 24 559 |  |
| 71 | 2 681 683 | 64 100.64 | 64 201.44 | 24 565 |  |  |  |
| 72 | 2 683 363 | 64 201.44 | 64 302.24 | 24 571 |  |  |  |
| 73 | 2 685 043 | 64 302.24 | 64 403.04 | 24 577 | 46 | 24 577 | X |
| 74 | 2 686 723 | 64 403.04 | 64 503.84 | 24 583 | 19 | 24 583 |  |
| 75 | 2 688 403 | 64 503.84 | 64 604.64 | 24 588 |  |  |  |
| 76 | 2 690 083 | 64 604.64 | 64 705.44 | 24 594 |  |  |  |
| 77 | 2 691 763 | 64 705.44 | 64 806.24 | 24 600 | 47 | 24 600 | X |
| 78 | 2 693 443 | 64 806.24 | 64 907.04 | 24 606 | 20 | 24 606 |  |
| 79 | 2 695 123 | 64 907.04 | 65 007.84 | 24 612 |  |  |  |
| 80 | 2 696 803 | 65 007.84 | 65 108.64 | 24 618 |  |  |  |
| 81 | 2 698 483 | 65 108.64 | 65 209.44 | 24 623 | 48 | 24 623 | X |
| 82 | 2 700 163 | 65 209.44 | 65 310.24 | 24 629 | 21 | 24 629 |  |
| 83 | 2 701 843 | 65 310.24 | 65 411.04 | 24 635 |  |  |  |
| 84 | 2 703 523 | 65 411.04 | 65 511.84 | 24 641 |  |  |  |
| 85 | 2 705 203 | 65 511.84 | 65 612.64 | 24 647 | 49 | 24 647 | X |
| 86 | 2 706 883 | 65 612.64 | 65 713.44 | 24 653 | 22 | 24 653 |  |
| 87 | 2 708 563 | 65 713.44 | 65 814.24 | 24 658 |  |  |  |
| 88 | 2 710 243 | 65 814.24 | 65 915.04 | 24 664 |  |  |  |
| 89 | 2 711 923 | 65 915.04 | 66 015.84 | 24 670 |  |  |  |
| 90 | 2 713 603 | 66 015.84 | 66 116.64 | 24 676 | 23 | 24 676 |  |
| 91 | 2 715 283 | 66 116.64 | 66 217.44 | 24 682 |  |  |  |
| 92 | 2 716 963 | 66 217.44 | 66 318.24 | 24 688 |  |  |  |
| 93 | 2 718 643 | 66 318.24 | 66 419.04 | 24 693 |  |  |  |
| 94 | 2 720 323 | 66 419.04 | 66 519.84 | 24 699 | 24 | 24 699 |  |
| 95 | 2 722 003 | 66 519.84 | 66 620.64 | 24 705 |  |  |  |
| 96 | 2 723 683 | 66 620.64 | 66 721.44 | 24 711 |  |  |  |
| 97 | 2 725 363 | 66 721.44 | 66 822.24 | 24 717 |  |  |  |
| 98 | 2 727 043 | 66 822.24 | 66 923.04 | 24 723 | 25 | 24 723 |  |
| 99 | 2 728 723 | 66 923.04 | 67 023.84 | 24 728 |  |  |  |
| 100 | 2 730 403 | 67 023.84 | 67 124.64 | 24 734 |  |  |  |
| 101 | 2 732 083 | 67 124.64 | 67 225.44 | 24 740 |  |  |  |
| 102 | 2 733 763 | 67 225.44 | 67 326.24 | 24 746 | 26 | 24 746 |  |
| 103 | 2 735 443 | 67 326.24 | 67 427.04 | 24 752 |  |  |  |
| 104 | 2 737 123 | 67 427.04 | 67 527.84 | 24 758 |  |  |  |
| 105 | 2 738 803 | 67 527.84 | 67 628.64 | 24 763 |  |  |  |
| 106 | 2 740 483 | 67 628.64 | 67 729.44 | 24 769 | 27 | 24 769 |  |
| 107 | 2 742 163 | 67 729.44 | 67 830.24 | 24 775 |  |  |  |
| 108 | 2 743 843 | 67 830.24 | 67 931.04 | 24 781 |  |  |  |
| 109 | 2 745 523 | 67 931.04 | 68 031.84 | 24 787 |  |  |  |
| 110 | 2 747 203 | 68 031.84 | 68 132.64 | 24 793 | 28 | 24 793 |  |
| 111 | 2 748 883 | 68 132.64 | 68 233.44 | 24 798 |  |  |  |
| 112 | 2 750 563 | 68 233.44 | 68 334.24 | 24 804 | 50 | 24 804 | X |
| 113 | 2 752 243 | 68 334.24 | 68 435.04 | 24 810 |  |  |  |
| 114 | 2 753 923 | 68 435.04 | 68 535.84 | 24 816 | 29 | 24 816 |  |
| 115 | 2 755 603 | 68 535.84 | 68 636.64 | 24 822 |  |  |  |
| 116 | 2 757 283 | 68 636.64 | 68 737.44 | 24 828 | 51 | 24 828 | X |
| 117 | 2 758 963 | 68 737.44 | 68 838.24 | 24 833 |  |  |  |
| 118 | 2 760 643 | 68 838.24 | 68 939.04 | 24 839 | 30 | 24 839 |  |
| 119 | 2 762 323 | 68 939.04 | 69 039.84 | 24 845 |  |  |  |
| 120 | 2 764 003 | 69 039.84 | 69 140.64 | 24 851 | 52 | 24 851 | X |
| 121 | 2 765 683 | 69 140.64 | 69 241.44 | 24 857 |  |  |  |
| 122 | 2 767 363 | 69 241.44 | 69 342.24 | 24 863 | 31 | 24 863 |  |
| 123 | 2 769 043 | 69 342.24 | 69 443.04 | 24 868 |  |  |  |
| 124 | 2 770 723 | 69 443.04 | 69 543.84 | 24 874 | 53 | 24 874 | X |
| 125 | 2 772 403 | 69 543.84 | 69 644.64 | 24 880 |  |  |  |
| 126 | 2 774 083 | 69 644.64 | 69 745.44 | 24 886 | 32 | 24 886 |  |
| 127 | 2 775 763 | 69 745.44 | 69 846.24 | 24 892 |  |  |  |
| 128 | 2 777 443 | 69 846.24 | 69 947.04 | 24 898 | 54 | 24 898 | X |
| 129 | 2 779 123 | 69 947.04 | 70 047.84 | 24 903 |  |  |  |
| 130 | 2 780 803 | 70 047.84 | 70 148.64 | 24 909 | 33 | 24 909 |  |
| 131 | 2 782 483 | 70 148.64 | 70 249.44 | 24 915 |  |  |  |
| 132 | 2 784 163 | 70 249.44 | 70 350.24 | 24 921 |  |  |  |
| 133 | 2 785 843 | 70 350.24 | 70 451.04 | 24 927 |  |  |  |
| 134 | 2 787 523 | 70 451.04 | 70 551.84 | 24 933 | 34 | 24 933 |  |
| 135 | 2 789 203 | 70 551.84 | 70 652.64 | 24 938 |  |  |  |
| 136 | 2 790 883 | 70 652.64 | 70 753.44 | 24 944 |  |  |  |
| 137 | 2 792 563 | 70 753.44 | 70 854.24 | 24 950 |  |  |  |
| 138 | 2 794 243 | 70 854.24 | 70 955.04 | 24 956 |  |  |  |