# 3GPP TSG-RAN WG4 Meeting # 103-e                                                        R4-2208696

# Electronic Meeting, May. 9-May. 20, 2022

Source:	ZTE Corporation

Title:	TP for TR 37.717-11-21: DC_3A-8A_n40A-n79

Agenda Item:	8.6.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC_3A-8A_n40A-n79A for TR 37.717-11-21[1].

- Reference

- TR 37.717-11-21 Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) v0.8.0

- Text Proposal

----- Start of TP -----

# 7.x	DC_3-8_n40-n79

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n40-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

    - 7.x.2	Configuration for DC

Table 7.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | Uplink NR band CA configuration |
| --- | --- | --- | --- |
| DC_3A-8A_n40A-n79A | DC_3A_n40A DC_3A_n79A DC_8A_n40A DC_8A_n79A | CA_3A-8A | CA_n40A-n79A |

### 7.x.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

# 7.x.4	∆TIB and ∆RIB values

For DC_3-8_n40-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n40-n79 | 3 | 0.5 |
|  | 8 | 0.3 |
|  | n40 | 0.5 |
|  | n79 | 0 |

Table 7.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n40-n79 | 3 | 0 |
|  | 8 | 0 |
|  | n40 | 0 |
|  | n79 | 0 |

### 7.x.5	MSD

All MSD requirements for DC_3-8_n40-n79 are already covered by constituent fallback modes. No additional MSD requirement is needed.

----- End of TP -----