3GPP TSG-RAN WG4 Meeting #103-e	R4-2208782

Electronic Meeting, May 09 – May 20, 2022

Agenda item:	4.1.1.3

Source:            	Google Inc., Comcast, Federated Wireless, CableLabs

Title:                	Discussion on Intra-Band EN-DC support

Document for:	Approval

# Introduction

In RAN4#102-e meeting, intra-band EN-DC support definition has been discussed but there is still no agreement which can be reached. Four cases are clarified in the approved WF [1] after several rounds discussion. It is agreed that there is no confused configurations about Case 1 which all CCs are contiguous in DL and UL and Case2 which all CCs are non-contiguous to each other. However, for Case 3 which all CCs are contiguous in DL with non-contiguous UL and Case 4 which LTE and NR adjacent carriers are contiguous but carriers in LTE or NR are non-contiguous, there are still some ambiguous issues need to be solved.

# Background:

For contiguous and non-contiguous intra-band ENDC combination more than 2 carriers, there are four kinds of band combinations in current R-16 and R-17 Spec

- Case 1: All CCs are contiguous in both of UL and DL, i.e., DL DC_(n)48CA with UL DC_(n)48AA

- Case2: Neither carrier is contiguous to each other in both UL and DL, i.e., DL DC_48A-48A_n48A with UL DC_48A_n48A

- Case 3: All CCs are contiguous in DL but  neither carrier is contiguous to each other in UL, i.e., DL DC_(n)48CA with UL DC_48A_n48A

- Case 4: LTE and NR adjacent carriers are contiguous but carriers in LTE or NR are non-contiguous, it will has two kinds of UL ENDC configurations, i.e., DL DC_48A_(n)48AA with UL DC_(n)48AA, DL DC_48A_(n)48AA with UL DC_48A_n48A

IE intraBandENDC-Support in UE capability can indicate three states{contiguous, non-contiguous, both} per band combination, it can’t indicate per band combination per UL or Per DL:

| intraBandENDC-Support Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3 [4]. If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Therefore, there is no any confusion for

- Case 1 All CCs are contiguous in both of UL and DL, indicate by intraBandENDC-Support= contiguous

- Case2: Neither carrier is contiguous to each other in both UL and DL, indicate by intraBandENDC-Support= non-contiguous

But for case 3 and case 4, no matter what the IE intraBandENDC-Support indicates will cause some confusion for the UE actual configuration. How to indicate case 3 and case 4 need further study.

In order to converge the discussion about Case 3 and Case 4, there are some alternatives proposed in the agreed WF. For Case 3, the point would be whether to remove the ambiguous combination for intra-band contiguous DL EN-DC configuration with non-contiguous UL EN-DC configuration. For Case 4, the issue would be how to configure IE intraBandENDC-Support for DL DC_48A_(n)48AA with UL DC_(n)48AA and DL DC_48A_(n)48AA with UL DC_48A_n48A. In this paper, we would like to share our view in the following.

# WF

- Case 3:

  - Alternative 1: remove below combinations from  R-16 and R-17 Spec, according to current fall-back rules in TS 38.101-3 and TS 38.306,

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)41AB5 DC_(n)41CA5 DC_(n)41DA5 | DC_41A_n41A |
| DC_(n)48CA5 | DC_48A_n48A6 |
| DC_(n)48DA5 | DC_48A_n48A6 |

  - Alternative 2: the UE should report the additional band combination DC_48A_n48A or DC_41A_n41A to support these combinations or introduce new UE capability:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)41AB5 DC_(n)41CA5 DC_(n)41DA5 | DC_41A_n41A |
| DC_(n)48CA5 | DC_48A_n48A6 |
| DC_(n)48DA5 | DC_48A_n48A6 |

- Case 4:

  - Alternative 1: it is mandatory to define both  kinds of UL ENDC configurations for the combinations simultaneously, i.e., DL DC_48A_(n)48AA with both of UL DC_(n)48AA and DC_48A_n48A,  indicate by intraBandENDC-Support= both

  - Alternative 2: Move the following combinations to intra-band contiguous EN-DC combination, indicate by intraBandENDC-Support= contiguous

    - DL DC_48A_(n)48AA with UL DC_(n)48AA

    - DL DC_48A_(n)48AA with UL DC_48A_n48A

  - Alternative 3: No specification change to the following intra-band EN-DC combinations, indicated by intraBandENDC-Support= non-contiguous

    - DL DC_48A_(n)48AA with UL DC_(n)48AA

    - DL DC_48A_(n)48AA with UL DC_48A_n48A

# Discussion

The IE intraBandENDC-Support in the UE capability signaling is designed for the network to know whether the UE supports contiguous or non-contiguous spectrum for an intra-band EN-DC band combination. At the beginning in Rel-15, the discussion of intra-band EN-DC combination is limited to one LTE carrier and one NR carrier. Thus, it is easy to distinguish an intra-band contiguous or non-contiguous EN-DC combination by indicating intraBandENDC-Support=not reported(contiguous) or intraBandENDC-Support=non-contiguous. For example, if the UE reports LTE 48A and NR n48A to the network, the network can know that the UE can support DC_(n)48AA if intraBandENDC-Support=not reported(contiguous) or the UE can support DC_48A_n48A if intraBandENDC-Support=non-contiguous. At the same time, there is also no problem for the network to know about DL EN-DC and UL EN-DC configuration in DC_(n)48AA or DC_48A_n48A when total EN-DC carrier is limited to two.

With the combinations growing up, the definition for IE intraBandENDC-Support become ambiguous to DL EN-DC and UL EN-DC configuration in some scenarios if the total intra-band EN-DC carriers is above two. Take one possible definition for intraBandENDC-Support as an example in the following, if the UE reports LTE CA_48C and NR n48A with intraBandENDC-Support=not reported(contiguous) to the network, it means that the UE can support intra-band contiguous DL DC_(n)48CA with contiguous UL DC_(n)48AA but it is not clear about whether non-contiguous UL DC_48A_n48A can be supported or not. On the other hand, if the UE reports LTE CA_48C and NR n48A with intraBandENDC-Support=non-contiguous, it means that the UE can support the intra-band non-contiguous DL DC_48C_n48A and only one possible non-contiguous UL DC_48A_n48A configuration. Furthermore, if the UE reports LTE CA_48C and NR n48A with intraBandENDC-Support=both, it means that the UE can support both intra-band contiguous DC_(n)48CA and non-contiguous DC_48C_n48A.

However, for the WF Case 3 scenario such as DL DC_(n)48CA with UL DC_48A_n48A, if the UE reports LTE CA_48C and NR n48A with intraBandENDC-Support=not reported(contiguous), the network can only know that DL DC_(n)48CA with UL DC_(n)48AA is supported by the UE. If the UE reports LTE CA_48C and NR n48A with intraBandENDC-Support=non-contiguous, the network can know DL DC_48C_n48A with UL DC_48A_n48A is supported by the UE. Neither intraBandENDC-Support=not reported(contiguous) or intraBandENDC-Support=non-contiguous can represent the intra-band EN-DC configuration for DL DC_(n)48CA with UL DC_48A_n48A.

In the other way, for the WF Case 4 scenario such as DL DC_48A_(n)48AA with UL DC_(n)48AA and UL DC_48A_n48A, the controversial point would be how to indicate intraBandENDC-Support to support both UL EN-DC configurations. If intraBandENDC-Support=both is indicated, both UL EN-DC configurations can be supported but it forces the UE to support both intra-band non-contiguous and contiguous combination. If intraBandENDC-Support=not reported(contiguous) is indicated, the configuration for DL DC_48A_(n)48AA with UL DC_(n)48AA and UL DC_48A_n48A needs to move from intra-band non-contiguous band combination to intra-band contiguous band combination. If intraBandENDC-Support=non-contiguous, the configuration DL DC_48A_(n)48AA with UL DC_(n)48AA can be also supported by the UE according to the fallback band combination definition in TS38.306 clause 3.1, where an intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination. In other words, an intra-band contiguous band combination is considered to be a fallback band combination of an intra-band non-contiguous band combination.

In the following, we summarize the relation between the UE capability and DL/UL EN-DC configuration in accordance with the different definition of intraBandENDC-Support in three tables. The UE capability IE intraBandENDC-Support definition in Table 1 is to differentiate DC_48A_(n)48AA from DC_48A-48A_n48A by intraBandENDC-Support=not reported(contiguous) for DC_48A_(n)48AA and by intraBandENDC-Support=non-contiguous for DC_48A-48A_n48A, which is different from TS38.101-3 definition. In this case, the configuration for DL DC_(n)48CA with UL DC_48A_n48A, DL DC_(n)48DA with UL DC_48A_n48A and DL DC_48A_(n)48AA with UL DC_48A_n48A can be supported by reporting an additional band combination or introducing a new UE capability to the network. Hence, the first cost for this intraBandENDC-Support definition is to move DC_48A_(n)48AA from intra-band non-contiguous combination to intra-band contiguous EN-DC combination in TS38.101-3. Moreover, in order to follow TS38.306 fallback definition, the second cost would be reporting an additional band combination or introducing a new UE capability signaling if the UE would like to support more intra-band UL EN-DC configurations.

| Band Combination | UE Capability |  |  |  |  | DL/UL EN-DC Configuration |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LTE-DL | LTE-UL | NR-DL | NR-UL | intraBandENDC-Support | DL EN-DC Configuration | UL EN-DC Configuration |  |
| DC_(n)48CA | 48C | 48A | n48A | n48A | contiguous | DC_(n)48CA | DC_(n)48AA |  |
| DC_(n)48CA | 48C | 48A | n48A | n48A | contiguous | DC_(n)48CA | DC_48A_n48A | Report an additional band combination DC_48A_n48A to the network or introduce a new UE capability to indicate UL EN-DC configuration |
| DC_48C_n48A | 48C | 48A | n48A | n48A | non-contiguous | DC_48C_n48A | DC_48A_n48A |  |
| DC_(n)48DA | 48D | 48A | n48A | n48A | contiguous | DC_(n)48DA | DC_(n)48AA |  |
| DC_(n)48DA | 48D | 48A | n48A | n48A | contiguous | DC_(n)48DA | DC_48A_n48A | Report an additional band combination DC_48A_n48A to the network or introduce a new UE capability to indicate UL EN-DC configuration |
| DC_48D_n48A | 48D | 48A | n48A | n48A | non-contiguous | DC_48D_n48A | DC_48A_n48A |  |
| DC_48A_(n)48AA | 48A-48A | 48A | n48A | n48A | contiguous | DC_48A_(n)48AA | DC_(n)48AA |  |
| DC_48A_(n)48AA | 48A-48A | 48A | n48A | n48A | contiguous | DC_48A_(n)48AA | DC_48A_n48A | Report an additional band combination DC_48A_n48A to the network or introduce a new UE capability to indicate UL EN-DC configuration |
| DC_48A-48A_n48A | 48A-48A | 48A | n48A | n48A | non-contiguous | DC_48A-48A_n48A | DC_48A_n48A |  |

Table 1. intraBandENDC-Support definition to differentiate DC_48A_(n)48AA from DC_48A-48A_n48A

In Table 2, the UE capability IE intraBandENDC-Support definition is to use intra-band UL EN-DC configuration between primary cells in each group to differentiate intraBandENDC-Support=not reported(contiguous) from intraBandENDC-Support=non-contiguous, which is different from TS38.101-3 definition. For example, the UE needs to report intraBandENDC-Support=non-contiguous for the configuration DL DC_(n)48CA with UL DC_48A_n48A. In this case, the major cost is that DL DC_(n)48CA with UL DC_48A_n48A and DL DC_(n)48DA with UL DC_48A_n48A needs to be moved to from intra-band contiguous EN-DC combination to intra-band non-contiguous EN-DC combination, and DL DC_48A_(n)48AA with UL DC_(n)48AA needs to be moved from intra-band non-contiguous EN-DC combination to intra-band contiguous EN-DC combination. In addition, the extra cost for this intraBandENDC-Support definition would be to support more configurations with the same reported UE capability signaling as follows.

- If the UE reports LTE 48C and NR n48A with the intraBandENDC-Support=non-contiguous, the UE needs to support two configurations DL DC_48C_n48A with UL DC_48A_n48A and DL DC_(n)48CA with UL DC_48A_n48A

- If the UE reports LTE 48D and NR n48A with the intraBandENDC-Support=non-contiguous, the UE needs to support two configurations DL DC_48D_n48A with UL DC_48A_n48A and DL DC_(n)48DA with UL DC_48A_n48A

- If the UE reports LTE 48A-48A and NR n48A with the intraBandENDC-Support=non-contiguous, the UE needs to support two configurations DL DC_48A-48A_n48A with UL DC_48A_n48A and DL DC_48A_(n)48AA with UL DC_48A_n48A.

From the implementation perspective, it is reasonable to assume that the UE supporting the configuration in non-contiguous spectrum can also support the configuration in contiguous spectrum with the same reported UE capability signaling to the network. Besides, if the UE would like to support both uplink EN-DC configurations for the band combination such as DC_48A_(n)48AA, it needs to report LTE 48A-48A and NR n48A with the intraBandENDC-Support=both to the network, which also means that the UE needs to support DC_48A-48A_n48A as well.

| Band Combination | UE Capability |  |  |  |  | DL/UL EN-DC Configuration |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LTE-DL | LTE-UL | NR- DL | NR-UL | intraBandENDC-Support | DL EN-DC Configuration | UL EN-DC Configuration |  |
| DC_(n)48CA | 48C | 48A | n48A | n48A | contiguous | DC_(n)48CA | DC_(n)48AA |  |
| DC_(n)48CA | 48C | 48A | n48A | n48A | non-contiguous | DC_(n)48CA | DC_48A_n48A | Move to the intra-band non-contiguous EN-DC combination |
| DC_48C_n48A | 48C | 48A | n48A | n48A | non-contiguous | DC_48C_n48A | DC_48A_n48A |  |
| DC_(n)48DA | 48D | 48A | n48A | n48A | contiguous | DC_(n)48DA | DC_(n)48AA |  |
| DC_(n)48DA | 48D | 48A | n48A | n48A | non-contiguous | DC_(n)48DA | DC_48A_n48A | Move to the intra-band non-contiguous EN-DC combination |
| DC_48D_n48A | 48D | 48A | n48A | n48A | non-contiguous | DC_48D_n48A | DC_48A_n48A |  |
| DC_48A_(n)48AA | 48A-48A | 48A | n48A | n48A | contiguous | DC_48A_(n)48AA | DC_(n)48AA | Move to the intra-band contiguous EN-DC combination |
| DC_48A_(n)48AA | 48A-48A | 48A | n48A | n48A | non-contiguous | DC_48A_(n)48AA | DC_48A_n48A |  |
| DC_48A-48A_n48A | 48A-48A | 48A | n48A | n48A | non-contiguous | DC_48A-48A_n48A | DC_48A_n48A |  |

Table 2. intraBandENDC-Support definition to follow UL EN-DC configuration between primary cell in each cell group

In Table 3, the UE capability IE intraBandENDC-Support definition is to follow TS38.101-3 Table 5.3B.1.2-1 and Table 5.3B.1.3-1 to differentiate intraBandENDC-Support=not reported(contiguous) from intraBandENDC-Support=non-contiguous. In this case, there is no specification change for intra-band EN-DC band combination in TS38.101-3. The major cost for this definition is that the configuration for DL DC_(n)48CA with UL DC_48A_n48A and DL DC_(n)48DA with UL DC_48A_n48A can be supported by reporting an additional band combination or introducing a new UE capability. In addition, if the UE reports LTE 48A-48A and NR n48A with the intraBandENDC-Support=non-contiguous to the network, the additional cost would be that the UE needs to support three configurations in two combinations such as DL DC_48A_(n)48AA with UL DC_(n)48AA, DL DC_48A_(n)48AA with UL DC_48A_n48A and DC_48A-48A_n48A with UL DC_48A_n48A.

As for the configuration DL DC_48A_(n)48AA with UL DC_(n)48AA, it can be supported with intraBandENDC-Support=non-contiguous by following TS38.306 fallback definition. It is a common understanding that the UE can support intra-band contiguous EN-DC band combination if the UE can support intra-band non-contiguous band combination. From the implementation perspective, it is also reasonable to assume that the UE supporting the configuration in non-contiguous spectrum can also support the configuration in contiguous spectrum under the same reported UE capability signaling to the network.

| Band Combination | UE Capability |  |  |  |  | DL/UL EN-DC Configuration |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LTE-DL | LTE-UL | NR-DL | NR-UL | intraBandENDC-Support | DL EN-DC Configuration | UL EN-DC Configuration |  |
| DC_(n)48CA | 48C | 48A | n48A | n48A | contiguous | DC_(n)48CA | DC_(n)48AA |  |
| DC_(n)48CA | 48C | 48A | n48A | n48A | contiguous | DC_(n)48CA | DC_48A_n48A | Report an additional band combination DC_48A_n48A to the network or introduce a new UE capability to indicate UL EN-DC configuration |
| DC_48C_n48A | 48C | 48A | n48A | n48A | non-contiguous | DC_48C_n48A | DC_48A_n48A |  |
| DC_(n)48DA | 48D | 48A | n48A | n48A | contiguous | DC_(n)48DA | DC_(n)48AA |  |
| DC_(n)48DA | 48D | 48A | n48A | n48A | contiguous | DC_(n)48DA | DC_48A_n48A | Report an additional band combination DC_48A_n48A to the network or introduce a new UE capability to indicate UL EN-DC configuration |
| DC_48D_n48A | 48D | 48A | n48A | n48A | non-contiguous | DC_48D_n48A | DC_48A_n48A |  |
| DC_48A_(n)48AA | 48A-48A | 48A | n48A | n48A | non-contiguous | DC_48A_(n)48AA | DC_(n)48AA | non-contiguous combination supports fallback to contiguous combination and DC_(n)48AA can be supported |
| DC_48A_(n)48AA | 48A-48A | 48A | n48A | n48A | non-contiguous | DC_48A_(n)48AA | DC_48A_n48A |  |
| DC_48A-48A_n48A | 48A-48A | 48A | n48A | n48A | non-contiguous | DC_48A-48A_n48A | DC_48A_n48A |  |

Table 3. intraBandENDC-Support definition to follow 38.101-3 Table 5.3B.1.2-1 and Table 5.3B.1.3-1

As stated above, we summarize all different intraBandENDC-Support definitions for comparison in Table 4. It is shown that Table 1 may have TS38.101-3 specification impacts and report an additional band combination or possible UE capability signaling introduction. In Table 2, there are only TS38.101-3 specification changes without introducing UE capability signaling but the UE may need to support more band combinations with the same reported UE capability. In Table 3, there is no change in TS38.101-3 but the UE may need to report an additional band combination or possible signaling change. In addition, the UE also needs to support more configurations and band combinations with the same reported UE capability.

From our viewpoint, we would like to have more flexible configuration of spectrum and it is not preferred to remove any existing intra-band EN-DC configurations in TS38.101-3. In Table 4, we think that the intraBandENDC-Support definition in Table 3 following the 38.101-3 Table 5.3B.1.2-1 and Table 5.3B.1.3-1 may have the minimum impact to the current specification. For the intra-band contiguous DL EN-DC configuration with the non-contiguous UL EN-DC configuration, compared to the new UE capability signaling introduction, we think it may have lower cost for the UE by reporting an additional band combination to the network. Furthermore, we also think that it is feasible from implementation perspective to report an additional band combination to let the network know which UL configuration is supported by the UE. By this way, the network can understand the UE supports intra-band contiguous DL EN-DC configuration with the non-contiguous UL EN-DC configuration by checking the additional reported band combination from the UE. For the intra-band non-contiguous DL EN-DC configuration with the contiguous UL EN-DC configuration, it is acceptable from implementation perspective to assume that the UE supporting the configuration in non-contiguous spectrum can also support the configuration in contiguous spectrum. However, the additional cost for the UE would be to support three configurations within two combinations.

Based on the analysis above, we propose:

Proposal 1: The intraBandENDC-Support definition should follow TS38.101-3 Table 5.3B.1.2-1 and Table 5.3B.1.3-1, and the UE should report the additional band combination DC_48A_n48A to support the following configurations.

- DL DC_(n)48CA with UL DC_48A_n48A

- DL DC_(n)48DA with UL DC_48A_n48A

Proposal 2: The intraBandENDC-Support definition should follow TS38.101-3 Table 5.3B.1.2-1 and Table 5.3B.1.3-1, and the UE should report intraBandENDC-Support=non-contiguous to support the following configurations.

- DL DC_48A_(n)48AA with UL DC_(n)48AA

- DL DC_48A_(n)48AA with UL DC_48A_n48A

- DL DC_48A-48A_n48A with UL DC_48A_n48A

| intraBandENDC-Support definition | Specification Change in TS38.101-3 | Additional Cost |
| --- | --- | --- |
| Table1. Differentiate DC_48A_(n)48AA from DC_48A-48A_n48A | Move the following combinations to intra-band contiguous EN-DC combination - DL DC_48A_(n)48AA with UL DC_(n)48AA - DL DC_48A_(n)48AA with UL DC_48A_n48A | Report an additional band combination DC_48A_n48A to the network or introduce a new UE capability to indicate UL EN-DC configuration for the following combinations - DL DC_(n)48CA with UL DC_48A_n48A - DL DC_(n)48DA with UL DC_48A_n48A - DL DC_48A_(n)48AA with UL DC_48A_n48A |
| Table 2. Follow UL EN-DC configuration between primary cell in each cell group | Move the following combinations to intra-band non-contiguous EN-DC combination - DL DC_(n)48CA with UL DC_48A_n48A - DL DC_(n)48DA with UL DC_48A_n48A Move the following combinations to intra-band contiguous EN-DC combination - DL DC_48A_(n)48AA with UL DC_(n)48AA | If the UE reports LTE 48C and NR n48A with intraBandENDC-Support=non-contiguous, the UE needs to support two configurations - DL DC_(n)48CA with UL DC_48A_n48A - DL DC_48C_n48A with UL DC_48A_n48A If the UE reports LTE 48D and NR n48A with intraBandENDC-Support=non-contiguous, the UE needs to support two configurations - DL DC_(n)48DA with UL DC_48A_n48A - DL DC_48D_n48A with UL DC_48A_n48A If the UE reports LTE 48A-48A and NR n48A with intraBandENDC-Support=non-contiguous, the UE needs to support two configurations - DL DC_48A_(n)48AA with UL DC_48A_n48A - DL DC_48A-48A_n48A with UL DC_48A_n48A |
| Table 3. Follow 38.101-3 Table 5.3B.1.2-1 and Table 5.3B.1.3-1 | No change for intra-band EN-DC band combination | Report an additional band combination DC_48A_n48A to the network or introduce a new UE capability to indicate UL EN-DC configuration for the following combinations - DL DC_(n)48CA with UL DC_48A_n48A - DL DC_(n)48DA with UL DC_48A_n48A If the UE reports LTE 48A-48A and NR n48A with intraBandENDC-Support=non-contiguous, the UE needs to support two combinations DC_48A_(n)48AA and DC_48A-48A_n48A with the following three configurations - DL DC_48A_(n)48AA with UL DC_(n)48AA - DL DC_48A_(n)48AA with UL DC_48A_n48A - DL DC_48A-48A_n48A with UL DC_48A_n48A |

Table 4. Comparison of different intraBandENDC-Support definitions

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: The intraBandENDC-Support definition should follow TS38.101-3 Table 5.3B.1.2-1 and Table 5.3B.1.3-1, and the UE should report the additional band combination DC_48A_n48A to support the following configurations.

- DL DC_(n)48CA with UL DC_48A_n48A

- DL DC_(n)48DA with UL DC_48A_n48A

Proposal 2: The intraBandENDC-Support definition should follow TS38.101-3 Table 5.3B.1.2-1 and Table 5.3B.1.3-1, and the UE should report intraBandENDC-Support=non-contiguous to support the following configurations.

- DL DC_48A_(n)48AA with UL DC_(n)48AA

- DL DC_48A_(n)48AA with UL DC_48A_n48A

- DL DC_48A-48A_n48A with UL DC_48A_n48A

# Reference

- R4-2206344, ”WF on IntrabandENDC-Support”, Xiaomi

- R4-2205879, ”Discussion on Intra-Band EN-DC support”, Google