3GPP TSG-RAN4 WG4 Meeting #103-e	R4-2208802

# Electronic meeting, May 9 – May 20, 2022

Agenda item:			9.20.2.2

Source:	vivo

Title:	Discussion on performance requirements for latency reduction of positioning measurements

Document for:	Discussion

- Introduction

In the last meeting, the core part of R17 positioning measurements has been completed. In this contribution, we will provide our views on the following issues on performance requirements for latency reduction of positioning measurements.

- Performance requirements for reduced number of samples

- Performance requirements without measurement gaps

- Performance requirements for reduced number of samples

For the impact of reduced number of samples, the following three factors need to be considered.

- SINR side condition

- Applicable propagation channel

- Applicable PRS BW

2.1 SINR Side Condition

In the RAN4 #101-bis-e meeting, the associated conditions related to PRS measurement for reduced number of samples has been agreed as follows [1]:

Issue 1-1-1: Number of samples (M1) excluding sample(s) for AGC and associated conditions Agreements: Parameters; Value. No of samples w/o AGC (M1); 1. PRS Ês/Iot (dB); ≥ -6. Propagation conditions; LOS. PRS BW (RBs); ≥ [48]. Accuracy; R16. Repetition; R16.

Considering low latency NR positioning measurements for R17 need to meet the existing R16 positioning accuracy requirements, the SINR side condition must be larger than -6dB. In addition, according to the agreed simulation assumptions in [2], for RSTD, -3dB for reference TRP and -6dB for neighbour TRP should be defined for latency reduction of positioning measurements.

For PRS RSRP, the existing side condition for serving cell is -3dB and for neighbour cell is -13dB for R16 positioning. For latency reduction of positioning measurements, we suggest to define the SINR side condition of -3 dB and -6dB for PRS RSRP measurements.

For UE Rx-Tx time difference measurement, same as PRS RSRP, the SINR side condition of -3dB and -6dB should be clarified.

Proposal 1: For reduced number of samples of positioning accuracy measurement, the SINR side condition should be defined as follows:

For RSTD measurement, -3dB for reference cell and -6dB for neighbour cell.

For PRS RSRP measurement, -3dB for serving cell and -6dB for neighbour cell.

For UE Rx-Tx time difference measurement, -3dB for serving cell and -6dB for neighbour cell.

2.2 Applicable propagation channels

Based on the conclusion from [1], considering low latency NR positioning measurements for R17 need to meet the existing R16 positioning accuracy requirements, the LOS channel model need to be clarified in the requirements. In the existing positioning requirements, the fading channel is TDL-A (30ns delay spread, 5Hz) and TDL-C (60ns delay spread, 300Hz) for FR1 and FR2 respectively. In the agreed simulation assumptions in [2], TDL-D (30ns delay spread, 5Hz) channel model is assumed for FR1 and FR2. The details for TDL-D from 38.901 were duplicated as below:

Table 7.7.2-4. TDL-D. Tap #; Normalized delay; Power in [dB]; Fading distribution. 1; 0; -0.2; LOS path. 0; -13.5; Rayleigh. 2; 0.035; -18.8; Rayleigh. 3; 0.612; -21; Rayleigh. 4; 1.363; -22.8; Rayleigh. 5; 1.405; -17.9; Rayleigh. 6; 1.804; -20.1; Rayleigh. 7; 2.596; -21.9; Rayleigh. 8; 1.775; -22.9; Rayleigh. 9; 4.042; -27.8; Rayleigh. 10; 7.937; -23.6; Rayleigh. 11; 9.424; -24.8; Rayleigh. 12; 9.708; -30.0; Rayleigh. 13; 12.525; -27.7; Rayleigh. NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 13.3 dB and a mean power of 0dB..

From the above table, the power of LOS path is very high compared with other paths for TDL-D. Therefore, in our understanding, there exist very little difference for a sample between TDL-D channel model and AWGN channel model for positioning measurements which can be observed based on our simulation results in [3]. So it may be feasible to only define a channel model (i.e., AWGN or TDL-D) for latency positioning measurements including RSTD measurement, PRS RSRP measurement and UE Rx-Tx time difference measurement. Due to the test case for positioning measurement is based on AWGN channel, we propose to consider AWGN channel model.

Observation 1: The power of LOS path is very high compared with other paths for TDL-D and there exist very little difference for a sample between TDL-D channel model and AWGN channel model for positioning accuracy measurements.

Proposal 2: For reduced number of samples of positioning accuracy measurement including RSTD measurement, PRS-RSRP measurement and UE Rx-Tx time difference measurement, the AWGN channel model should be considered.

2.3 Applicable PRS BW

Based on the conclusion from [1], considering low latency NR positioning measurements for R17 need to meet the existing R16 positioning accuracy requirements, the 	PRS BWs should be more than 48. Based on our simulation results in [3], apart from the combination of 24RB and 15kHz, PRS BWs in the existing requirements should be defined for latency reduction of positioning accuracy measurement.

Proposal 3: PRS BWs in the existing requirements apart from the combination of 24RB and 15kHz should be defined for reduced number of samples of positioning accuracy measurement.

- Performance requirements without measurement gaps

In R16, the positioning measurement requirements with measurement gaps were discussed and the following test cases were introduced in 38.133:

| Section | Test case description | FR/MR-DC mode | Note |
| --- | --- | --- | --- |
| A.6.6.12 | RSTD measurement | FR1 SA | Measurement procedure |
| A.6.6.13 | PRS-RSRP measurement | FR1 SA | Measurement procedure |
| A.6.6.14 | UE Rx-Tx time difference measurement | FR1 SA | Measurement procedure |
| A.6.7.13 | RSTD measurement | FR1 SA | Measurement performance |
| A.6.7.14 | PRS-RSRP measurement | FR1 SA | Measurement performance |
| A.6.7.15 | UE Rx-Tx time difference measurement | FR1 SA | Measurement performance |
| A.7.6.9 | RSTD measurement | FR2 SA | Measurement procedure |
| A.7.6.10 | PRS-RSRP measurement | FR2 SA | Measurement procedure |
| A.7.6.11 | UE Rx-Tx time difference measurement | FR2 SA | Measurement procedure |
| A.7.7.10 | RSTD measurement | FR2 SA | Measurement performance |
| A.7.7.11 | PRS-RSRP measurement | FR2 SA | Measurement performance |
| A.7.7.12 | UE Rx-Tx time difference measurement | FR2 SA | Measurement performance |

Based on the design from R16, for PRS measurement test cases without measurement gaps, the following test cases need to be defined. In addition, the measurement gap configuration in the existing requirements should be replaced with the PPW.

| TC index | Test case description | FR/MR-DC mode | Note |
| --- | --- | --- | --- |
| 1 | RSTD measurement without measurement gaps | FR1 SA | Measurement procedure |
| 2 | PRS-RSRP measurement without measurement gaps | FR1 SA | Measurement procedure |
| 3 | UE Rx-Tx time difference measurement without measurement gaps | FR1 SA | Measurement procedure |
| 4 | RSTD measurement without measurement gaps | FR1 SA | Measurement performance |
| 5 | PRS-RSRP measurement without measurement gaps | FR1 SA | Measurement performance |
| 6 | UE Rx-Tx time difference measurement without measurement gaps | FR1 SA | Measurement performance |
| 7 | RSTD measurement without measurement gaps | FR2 SA | Measurement procedure |
| 8 | PRS-RSRP measurement without measurement gaps | FR2 SA | Measurement procedure |
| 9 | UE Rx-Tx time difference measurement without measurement gaps | FR2 SA | Measurement procedure |
| 10 | RSTD measurement without measurement gaps | FR2 SA | Measurement performance |
| 11 | PRS-RSRP measurement without measurement gaps | FR2 SA | Measurement performance |
| 12 | UE Rx-Tx time difference measurement without measurement gaps | FR2 SA | Measurement performance |

Proposal 4: For PRS measurement test cases without measurement gaps, the following test cases need to be defined. And the measurement gap configuration in the existing requirements should be replaced with the PPW.

| TC index | Test case description | FR/MR-DC mode | Note |
| --- | --- | --- | --- |
| 1 | RSTD measurement without measurement gaps | FR1 SA | Measurement procedure |
| 2 | PRS-RSRP measurement without measurement gaps | FR1 SA | Measurement procedure |
| 3 | UE Rx-Tx time difference measurement without measurement gaps | FR1 SA | Measurement procedure |
| 4 | RSTD measurement without measurement gaps | FR1 SA | Measurement performance |
| 5 | PRS-RSRP measurement without measurement gaps | FR1 SA | Measurement performance |
| 6 | UE Rx-Tx time difference measurement without measurement gaps | FR1 SA | Measurement performance |
| 7 | RSTD measurement without measurement gaps | FR2 SA | Measurement procedure |
| 8 | PRS-RSRP measurement without measurement gaps | FR2 SA | Measurement procedure |
| 9 | UE Rx-Tx time difference measurement without measurement gaps | FR2 SA | Measurement procedure |
| 10 | RSTD measurement without measurement gaps | FR2 SA | Measurement performance |
| 11 | PRS-RSRP measurement without measurement gaps | FR2 SA | Measurement performance |
| 12 | UE Rx-Tx time difference measurement without measurement gaps | FR2 SA | Measurement performance |

- Summary

Observation 1: The power of LOS path is very high compared with other paths for TDL-D and there exist very little difference for a sample between TDL-D channel model and AWGN channel model for positioning accuracy measurements.

Proposal 1: For reduced number of samples of positioning accuracy measurement, the SINR side condition should be defined as follows:

For RSTD measurement, -3dB for reference cell and -6dB for neighbour cell.

For PRS RSRP measurement, -3dB for serving cell and -6dB for neighbour cell.

For UE Rx-Tx time difference measurement, -3dB for serving cell and -6dB for neighbour cell.

Proposal 2: For reduced number of samples of positioning accuracy measurement including RSTD measurement, PRS-RSRP measurement and UE Rx-Tx time difference measurement, the AWGN channel model should be considered.

Proposal 3: PRS BWs in the existing requirements apart from the combination of 24RB and 15kHz should be defined for reduced number of samples of positioning accuracy measurement.

Proposal 4: For PRS measurement test cases without measurement gaps, the following test cases need to be defined. And the measurement gap configuration in the existing requirements should be replaced with the PPW.

| TC index | Test case description | FR/MR-DC mode | Note |
| --- | --- | --- | --- |
| 1 | RSTD measurement without measurement gaps | FR1 SA | Measurement procedure |
| 2 | PRS-RSRP measurement without measurement gaps | FR1 SA | Measurement procedure |
| 3 | UE Rx-Tx time difference measurement without measurement gaps | FR1 SA | Measurement procedure |
| 4 | RSTD measurement without measurement gaps | FR1 SA | Measurement performance |
| 5 | PRS-RSRP measurement without measurement gaps | FR1 SA | Measurement performance |
| 6 | UE Rx-Tx time difference measurement without measurement gaps | FR1 SA | Measurement performance |
| 7 | RSTD measurement without measurement gaps | FR2 SA | Measurement procedure |
| 8 | PRS-RSRP measurement without measurement gaps | FR2 SA | Measurement procedure |
| 9 | UE Rx-Tx time difference measurement without measurement gaps | FR2 SA | Measurement procedure |
| 10 | RSTD measurement without measurement gaps | FR2 SA | Measurement performance |
| 11 | PRS-RSRP measurement without measurement gaps | FR2 SA | Measurement performance |
| 12 | UE Rx-Tx time difference measurement without measurement gaps | FR2 SA | Measurement performance |

- References

- R4-2202776 WF on NR Positioning Enhancements (Part 1)

- R4-2120330 Link level simulation assumptions for NR PRS measurements with reduced latency

- R4-2208808 Further simulation results for PRS measurement with reduced number of samples