3GPP TSG RAN WG4 Meeting #103-e	R4-2208821

Electronic Meeting, May 9 - May 20, 2022

Agenda item:			9.22.1.1

Source:	vivo

Title:	Updated simulation results for TRS based PDC

Document for:	Discussion

- Introduction

It was agreed in the last RAN4 meeting, as captured in the WF [1] that for UE Rx-Tx time difference measurement requirements for TRS,

  - RAN4 will define UE Rx-Tx time difference measurement requirement for TRS requirements for FR2

  - RAN4 agree to define UE Rx-Tx time difference measurement requirement for TRS requirements for 60KHz

  - Channel model simulation assumptions:

    - AWGN

    - TDL-A

  - 4 TRS resources over 2 slots

In this contribution, we provide simulation results for RTT-based PDC with TRS with updated simulation assumptions provided in the WF [2].

- Discussion

Simulation results for TRS based PDC are provided in Table 1 and Table 2 for FR1 and FR2, respectively.

Table 1. simulation results for TRS based UE Rx-Tx time difference for FR1 (Tc)

| SCS(kHz) | BW(RB) | Sample rate (Tc) | AWGN |  | TDL-A |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 sample | 4 samples | 1 sample | 4 samples |
| 15kHz | 24 | 256 | 132.58 | 128.02 | 269.49 | 201.40 |
|  | 52 | 128 | 63.52 | 62.30 | 191.71 | 119.40 |
|  | 104 | 64 | 30.83 | 30.38 | 126.91 | 101.37 |
|  | 268 | 32 | 14.84 | 14.73 | 108.75 | 48.53 |
| 30kHz | 24 | 128 | 66.21 | 62.29 | 194.83 | 108.99 |
|  | 52 | 64 | 31.75 | 30.54 | 127.12 | 90.57 |
|  | 132 | 32 | 15.07 | 14.80 | 107.85 | 48.09 |
|  | 272 | 16 | 7.41 | 7.32 | 42.04 | 38.06 |
| 60kHz | 24 | 64 | 33.14 | 30.41 | 128.97 | 78.66 |
|  | 64 | 32 | 15.44 | 14.78 | 108.19 | 47.99 |
|  | 132 | 16 | 7.53 | 7.33 | 42.52 | 36.17 |

Table 1. simulation results for TRS based UE Rx-Tx time difference for FR2 (Tc)

| SCS(kHz) | BW(RB) | Sample rate (Tc) | AWGN |  | TDL-A |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 sample | 4 samples | 1 sample | 4 samples |
| 60kHz | 24 | 64 | 33.14 | 30.41 | 122.44 | 56.84 |
|  | 64 | 32 | 15.44 | 14.78 | 53.78 | 47.36 |
|  | 132 | 16 | 7.53 | 7.33 | 41.65 | 32.69 |
| 120kHz | 32 | 32 | 15.84 | 14.56 | 101.48 | 46.63 |
|  | 64 | 16 | 7.72 | 7.26 | 42.91 | 28.96 |
|  | 128 | 8 | 3.78 | 3.62 | 37.03 | 29.25 |

For RTT-based PDC with TRS, it can be seen from simulation results in Table 1 and Table 2 that measurement accuracy with 4 samples and 1 sample are almost the same under AWGN channel. Improvement on measurement accuracy can be seen under TDL-A fading channel.

Observation 1: Measurement accuracy with 4 samples and 1 sample are almost the same under AWGN channel.

Observation 2: Measurement accuracy can be improved with 4 samples compared to 1 sample measurement under TDL-A fading channel.

- Summary

Following observations are made based on simulation results.

Observation 1: Measurement accuracy with 4 samples and 1 sample are almost the same under AWGN channel.

Observation 2: Measurement accuracy can be improved with 4 samples compared to 1 sample measurement under TDL-A fading channel.

- Appendix: Simulation assumptions

Updated simulation assumptions in WF [1] are provided as below.

Table 1 General parameters

| Parameter | Value |
| --- | --- |
| Cell layout | 1 serving cell |
| Duplex modes | FDD and TDD |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) |
| Data and CCH load in non-TRS symbols | - 50% utilization in time - 100% RE utilization |
| Cyclic prefix | Normal |
| DRX | OFF |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 5 MHz, 10 MHz, 20 MHz - 15 kHz - FDD, TDD - 4 GHz - 10 MHz, 20 MHz, 50 MHz - 30 kHz, 60kHz - FDD, TDD - 40 GHz - 50 MHz, 100 MHz, 200 MHz - 60kHz, 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | AWGN, [TDL-A (30 ns delay spread, 5Hz)] for FR1 TDL-A (30 ns delay spread, 300Hz) for FR2 |
| Es/Iot [dB] | -3 |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |
| UE measurement bandwidth | Full carrier bandwidth |

Table 2 CSI-RS for tracking transmission configuration parameters

| Parameter | Value |
| --- | --- |
| Number of transmit TRS antennas | 1 |
| Cell ID, TRP ID, CSI-RS Resource Set ID | 1 |
| Number with CSI-RS Resource sets with trs-Info | 1 |
| TRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz), 268 (50MHz) - 30 kHz: 24(5MHz), 52 (20MHz),132 (50MHz), 272 (100MHz) |
| Sample rate (Tc) | - 15 kHz: 256(5MHz), 128(10MHz), 64 (20MHz) - 30 kHz: 128(5MHz), 64 (20MHz), 32 (50MHz) |
| TRS periodicity | 40ms |
| TRS resources number | 4 |
| TRS symbol location | {4,8} |
| TRS frequency density | 3 |
| TRS samples | 1, 4 |
| Evaluation period | 40 ms, 200 ms |

At least the following performance characteristics are to be provided for TUE-RX:

- 90%-ile of the TUE-RX errors for each cell

and,

- TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge).

- References

- R4-2207020	WF on NR_IIOT_URLLC_enh RRM requirements, Nokia, Nokia Shanghai Bell

- R4-2202782	WF on NR_IIOT_URLLC_enh RRM requirements, Nokia, Nokia Shanghai Bell