3GPP TSG-RAN WG4 Meeting #103-e                                                            													R4-2208843

Electronic Meeting, May 09th – May 20th, 2022

Source:	Samsung

Title:	Further Discussion on capability and feature list for FR2 HST UE

Agenda item:			9.8.2.1

Document for:	Discussion

# 1. Introduction

In RAN Plenary #89-e, the RAN4-led work item of NR support for high speed train scenario in FR2 has been approved [1, RP-202118] (which has been further revised to [2, RP-210800]).  In last meeting (RAN4#102-e), some progress and agreement were achieved for FR2 HST UE’s capability and feature list in the approved WF [3], as follows:

- WF on Topic#1: General - Issue 1-2-1: Type definition for HST FR2 RRM features GtW Agreement: - Agreement: - The following UE feature list description for feature “x-1 Support of FR2 HST operation” is endorsed in the RRM session. Further confirmation in the RAN4 Main and Demod session is required. Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not ignalin by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. x-1; Support of FR2 HST operation; 1) Support of FR2 UE PC6 2) Support of enhanced RRM requirements for FR2 HST (except the requirement for one shot large UL timing adjustment) 3) Support of demodulation processing for FR2 HST; [R15 RAN4 feature group: Support of FR2 UE power class 6]; Yes; No; UE does not meet FR2 high speed train scenario; Per Band; No; Applicable to FR2 only; N/A; FR2 UE power class PC6 signalling is used to indicate support of feature group; Optional with capability signaling. Agreement: Remove pre-requisite feature groups “[R15 RAN4 feature group: Support of FR2 UE power class 6]”. - Issue 1-2-2: Capability for one shot large UL timing adjustment Agreement: - Introduce feature group x-2 “Support of one shot large UL timing adjustment” with prerequisite feature group (x-1, “Support of FR2 HST operation”) Way forward: Companies are encouraged to discuss further the following two options: - Option 1: Define feature as mandatory with capability signaling. - Option 2: Define feature as optional with capability signaling.. - Issue 1-2-3: Indication of HST FR2 RRM feature support and Applicability of enhanced RRM requirements (PC 6) Agreement: - No enhanced requirement should be applied to other than PC6 UEs even when HST FR2 flags are configured. - PC6 shall be used to identify the feature support of HST FR2 operation..

Furthermore, contained in the LS to RAN2 [4, R4-2206572], the feature list related to NR FR2 HST is provided:

- NR_HST_FR2 Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 22. NR_HST_FR2; 22-1; Support of FR2 HST operation; 1) Support of FR2 UE PC6 2) Support of enhanced RRM requirements for FR2 HST (except the requirement for one shot large UL timing adjustment) 3) Support of demodulation processing for FR2 HST; Yes; No; UE does not meet FR2 high speed train scenario; Per Band; NO; FR2 only; N/A; FR2 UE power class PC6 signalling is used to indicate support of feature group; Optional with capability signalling.

In this contribution, we would like to further discuss on the remaining issues on UE capability and feature list for FR2 HST UE.

# 2. Discussion

For the UL timing related feature, i.e, one shot large UL timing adjustment mechanism, RAN4 agree to introduce this mechanism for FR2 HST scenarios with UE allowed to adjust uplink timing during TCI switching, beyond gradual timing adjustment (i.e., one step size restricted by Tq). Particularly, in the last meeting, it is agreed that

Agreement: - Introduce feature group x-2 “Support of one shot large UL timing adjustment” with prerequisite feature group (x-1, “Support of FR2 HST operation”)

So it is straightforward to introduce the corresponding feature group for ” Support of one shot large UL timing adjustment”. For one shot large UL timing adjustment, the feature is needed to make sure gNB to receive UE’s uplink signals during inter-RRH TCI switching. If the feature is not supported, RA-based UL timing adjustment is needed, which is not preferred for the FR2 HST scenario. Therefore we propose the feature is mandatorily supported with capability signaling.

Proposal 1: For the feature group “Support of one shot large UL timing adjustment”, it is proposed that the feature is mandatorily supported with capability signaling.

Proposal 2: The feature list for FR2 HST UE is updated by adding the feature group “Support of one shot large UL timing adjustment”

Table 1. The feature list update for FR2 HST UE

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22. NR_HST_FR2 | 22-1 | Support of FR2 HST operation | 1) Support of FR2 UE PC6 2) Support of enhanced RRM requirements for FR2 HST (except the requirement for one shot large UL timing adjustment) 3) Support of demodulation processing for FR2 HST |  | Yes | No | UE does not meet FR2 high speed train scenario | Per Band | NO | FR2 only | N/A | FR2 UE power class PC6 signalling is used to indicate support of feature group | Optional with capability signalling |
| 22. NR_HST_FR2 | 22-2 | Support of one shot large UL timing adjustment | 1) Support of one shot large UL timing adjustment | 22-1 Support of FR2 HST operation | Yes | No | UE does not support one shot large UL timing adjustment | Per Band | NO | FR2 only | N/A |  | Mandatory with capability signaling |

# 4. Conclusion

In this contribution, we provided the discussion on UE capability and feature list for FR2 HST, with the following observation and proposals obtained:

Proposal 1: For the feature group “Support of one shot large UL timing adjustment”, it is proposed that the feature is mandatorily supported with capability signaling.

Proposal 2: The feature list for FR2 HST UE is updated by adding the feature group “Support of one shot large UL timing adjustment”

Table 1. The feature list update for FR2 HST UE

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22. NR_HST_FR2 | 22-1 | Support of FR2 HST operation | 1) Support of FR2 UE PC6 2) Support of enhanced RRM requirements for FR2 HST (except the requirement for one shot large UL timing adjustment) 3) Support of demodulation processing for FR2 HST |  | Yes | No | UE does not meet FR2 high speed train scenario | Per Band | NO | FR2 only | N/A | FR2 UE power class PC6 signalling is used to indicate support of feature group | Optional with capability signalling |
| 22. NR_HST_FR2 | 22-2 | Support of one shot large UL timing adjustment | 1) Support of one shot large UL timing adjustment | 22-1 Support of FR2 HST operation | Yes | No | UE does not support one shot large UL timing adjustment | Per Band | NO | FR2 only | N/A |  | Mandatory with capability signaling |

# 5. Reference

[1] RP-202118, “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia.

[2] RP-210800, “Revised WID on NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[3] R4-2206848, “WF on FR2 HST RRM (part 1)”, Nokia.

[4] R4-2206572, “LS on Rel-17 RAN4 UE feature list for NR”, CMCC.