3GPP TSG-RAN WG4 Meeting #103-e	  R4-2208853

# Electronic Meeting, 9th May to 20th May, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38101-1-h50 Further note for SUL combos general part |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SUL_combos_R17-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The UL/DL frequency separation shall be kept consistent for FDD bands and its corresponding SUL band, such as n1_SUL_n84, n8_SUL_n81. The DL center frequency shall keep same TX/RX separatoin as correspoinding FDD band or MSD need to be characterized due to the change on T/R separation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the text “Unless otherwise stated, the SUL uplink carrier centre frequency and its corresponding FDD band downlink carrier centre frequency shall follow the FDD band TX–RX frequency separation in Table 5.4.4-1.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sensitivity would be degraded if T/R separation is not consistent with SUL band’s corresponding FDD band or MSD need to be characterized due to the change on T/R separation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

5.2C	Operating band combination for SUL

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1, Table 5.2C-2, Table 5.2C-3 and Table 5.2C-4, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a band combination, the mandatory simultaneous Rx/Tx capability also applies for the band combination when the applicable band combination is a subset of a higher order band combination.

Unless otherwise stated, the SUL uplink carrier centre frequency and its corresponding FDD band downlink carrier centre frequency shall follow the FDD band TX–RX frequency separation in Table 5.4.4-1.

<<< End of changed sections >>>