3GPP TSG- RAN4 Meeting # 103-e	R4-2208975

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on applicability rule of requirements for Redcap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2022-4-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Redcap has no impact on some requirements, e.g., Cell phase synchronization accuracy, deriveSSB-IndexFromCell tolerance, Active TCI state switching delay, etc. It means that the legacy requirements can be reused. To avoid duplicating the legacy requirements and avoid the redundancy of specification, it is perfered to specify an applicability rule, rather than creating multiple clauses with suffix for Redcap in many places. It is suggested to add a cluase named “applicability of requirements” in chapter 3. Moreover from future proof perspective, to efficiently develop a feature on top of RedCap (e.g., UE supports RedCap+ feature X), if RedCap has no impact on this feature, it is expected to cite the clause No. of this feature’ requirements in “applicablity of requirements”, rather than adding new section(s) for “redcap + feature X” and copying the legacy requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Adding applicability of requirements; -Removing some placeholder sections without content. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support of RedCap |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

3.6.11.4	Applicability of requirements

Besides explicitly defined requirements for RedCap, the requirements in section 7.4, 7.7, 8.4, 8.6.2, 8.6.3, 8.13 are applicable to RedCap.

<End of Change 1>

<Start of Change 2>

Unchanged sections omitted

## 8.10B Active TCI state switching delay for RedCap

Big CR Editor’s Note: Placeholder for TCI state switching delay requirements.

<End of Change 2>

<Start of Change 3>

.

<End of Change 3>