3GPP TSG-RAN WG4 Meeting # 103-e                                				R4-2209084

Electronic Meeting, 9– 20 May, 2022

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.717-11-21: DC_n8A-n77A_1A, DC_n8A-n77(2A)_1A

Agenda Item:	8.15.2

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of NR bands n8, n77 and LTE band B1 for TR 37.717-11-21[1]. Since the EN-DC counterpart (DC_1A_n8A-n77A and DC_1A_n8A-n77(2A)) is introduced in the current meeting (R4-2208838), identical coexistence studies, ∆TIB and ∆RIB values and MSD values are proposed.

# Reference

- TR 37.717-11-21, 	Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL), v0.8.0

# Text Proposal

----- Start of TP -----

## 6.x	DC_n8-n77_1

### 6.x.1		Operating bands for DC

Table 6.x.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_n8-n77_1 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |

### 6.x.2		Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC inter-band NR 2DL/1UL + one LTE band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_n8A-n77A_1A | n8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n28 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
| DC_n8A-n77(2A)_1A | n8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n28 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |

### 6.x.3		Co-existence studies

- Similar to DC_1_n8-n77 in TR 37.717-11-21

### 6.x.4	∆TIB and ∆RIB values

Similar to DC_1_n8-n77 in TR 37.717-11-21

### 6.x.5	MSD

Similar to DC_1_n8-n77 in TR 37.717-11-21

----- End of TP -----