3GPP TSG-RAN WG4 Meeting #103-e	R4-2209101

Electronic Meeting, 09th May – 20th May 2022

Source: 	Ericsson

Title:  	General requirements for NTN

Agenda Item:	9.12.6.1

Document for:	Discussion

# Introduction

In RAN4# 102 a WF on NTN RRM measurement requirements was approved [1]. In this contribution we develop topics on general RRM measurement part in the WF.

# Discussion

## Issue 4-1-0: View collection for NTN UE Capability

Agreement: - The following NTN UE capabilities will be defined as UE optional capability, and the details are subject to be further discussion in the next RAN4 meeting: - Parallel measurement and normal operation - Parallel measurement of LEO in one SMTC - Support enhanced (e.g. TN HST) Idle/Inactive mode cell reselection requirements for LEO - Perform measurements on more than one neighbor cells belonging to different satellites in parallel without scaling. - Measurement for more than 2 LEO satellites per carrier (Up to [4 or 6] satellites) - Support performing measurements on different numbers of target cells within multiple SMTCs on a single carrier - Support 2 parallel measurement gaps - Support performing measurements on cells belonging to different satellite as the serving cell at the same time with normal operations in serving cell

The capability to support enhanced (e.g. TN HST) Idle/Inactive mode cell reselection requirements for LEO is unclear. Following the wordings in below table, it may be interpreted that UE can choose enhanced (HST) or normal measurement requirements for LEO, which isn’t the expectation. In other words, there is only one set of measurement requirements for LEO, not two sets, e.g. normal requirements and enhanced requirements. As follows, we'd want to clarify our understanding:

- If UE has the capability, enhanced measurement requirements for LEO is valid.

- If UE hasn’t the capability, no measurement requirements for LEO.

| X-Y-4 | Enhanced RRM requirements for measurement in Idle and Inactive mode | Support of enhanced RRM requirements for measurement in Idle and Inactive mode as specified in TS 38.133 |  | no | no | UE does not support of enhanced RRM requirements for measurement in Idle and Inactive mode | [Per UE] | FDD only | FR1 only | NA |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1: The capability to support enhanced (e.g. TN HST) Idle/Inactive mode cell reselection requirements for LEO shall be interpreted as:

- If UE has the capability, measurement requirements for LEO is valid and follow TN HST requirements.

- If UE hasn’t the capability, no measurement requirements for LEO.

# Summary

Proposal 1: The capability to support enhanced (e.g. TN HST) Idle/Inactive mode cell reselection requirements for LEO shall be interpreted as:

- If UE has the capability, measurement requirements for LEO is valid and follow TN HST requirements.

- If UE hasn’t the capability, no measurement requirements for LEO.

# References

- R4-2207114	WF on RRM requirements for NTN measurement and mobility	Qualcomm