3GPP TSG-RAN WG4 Meeting #103-e	R4-2209147

Electronic Meeting, 09 May–20 May, 2022

Source:	ZTE Corporation

Title:	CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR

Agenda Item:	9.11.2.3

Document for:	Approval

- Introduction

- In RAN4#102-e meeting, test setup for 15 kHz SCS scenario was discussed, and a way forward was agreed [1]. Regarding necessity of test setup for 15 kHz SCS scenario for CRS-IM receiver, there are still issue opens, we would like to share our views on the following issue for further discussion:

- Test setup for scenario 2

- Discussion

Discussion in last meeting from [1] is summarized below:

Test setup for scenario 2

Test setup for scenario 2 - Define one set of test setup with the new NWA signalling on LTE CBW configured. - FFS whether to define the other set of test setup with only inter-RAT MO configured: –FFS whether the same test requirements for CRS-IM can be applied in the two sets of test setup, considering that: - Whether or not to assume no error in LTE CBW detection: 1) based on PBCH decoding and/or power detection for the two interferers with different power level, or 2) based on PBCH decoding and/or power detection for the first dominant interferer. - TE does not start PDSCH scheduling of serving cell until UE acquires LTE channel bandwidth, further discuss the time needed for UE to acquire LTE channel bandwidth: - Option A: N x inter-RAT measurement period where N is the number of inter-RAT measurement configuration. One candidate value for N is 4, and other values are not precluded. FFS for the inter-RAT measurement period. - Other options are not precluded Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test

Based on our analysis, NR UE capable of performing CRS-IM in scenario 2 with 15 kHz SCS has two different signallings configuration, one is without new network assistant signalling and another is with Rel-17 new network assistant signalling on LTE channel bandwidth. So it has different ways to acquire LTE channel bandwidth. It is necessary to define the other set of test set up with only inter-RAT MO configured. Firstly, We think there is no error on LTE CBW detection based on PBCH decoding or power detection. The minimum performance of PBCH is about -6dB for LTE decoding. Based on this value, we believe that the minimum performance of PBCH is lower than this value for NR UE. So, it is reasonable to assume no error for PBCH decoding.

For the time need for UE to acquire LTE channel bandwidth, we think Option A is reasonable. In the previous discussion, PBCH can be decoded during the measurement gap. And the maximum number of PBCH detection is four. Based on this reason, Option A is reasonable.

The inter-RAT MO can be configured during the beginning of the test to acquire LTE cell information. If the UE acquire LTE cell information at the beginning, it can be always used until test is end.

Proposal 1: It is necessary to define the other set of test set up with only inter-RAT MO configured.

Proposal 2: N x inter-RAT measurement period where N is the number of inter-RAT measurement configuration and N is 4.

Proposal 3: The inter-RAT MO is only configured during the beginning of the test.

- Conclusion

In this contribution, we have the following proposals for :

Proposal 1: It is necessary to define the other set of test set up with only inter-RAT MO configured.

Proposal 2: N x inter-RAT measurement period where N is the number of inter-RAT measurement configuration and N is 4.

Proposal 3: The inter-RAT MO is only configured during the beginning of the test.

- Reference

R4-2207239,”WF on general part and 15 kHz NR SCS scenario for CRS-IM receiver”. China Telecom