3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209176

# Electronic Meeting, May 9 – May 20, 2022

Source: 	Qualcomm Incorporated

Title: 	CQI requirements for 1024QAM FR1 UE Demod Tests

Agenda item:	9.6.4.3

Document for:	Discussion

- Introduction

In the previous meetings, we have reached agreements over most of the simulation assumptions for the definition of CQI Requirements for 1024QAM in FR1. This contribution addresses the remaining item in the discussion [1].

- CQI Requirements

Based on our simulations results for CQI reporting, we tend to support Option 2 for both 2 and 4 RX requirements.

However, we would also like to highlight that the 29/30dB SNR requirement for 2 RX could be close respect to the floor level given by the considered Tx EVM, and we would like to further discuss during the meeting.

Proposal 1: Targeting CQI Index 14, support Option 2 for both 2Rx (29/30dB) and 4 Rx UEs (26/27) dB.

Observation 1: Proposed CQI Requirement for 2 RX, which goes up to 30dB SNR, might very close to the considered Tx EVM floor level. RAN4 should discuss whether this can have an impact on the test setup.

- Conclusions

Proposal 1: Targeting CQI Index 14, support Option 2 for both 2Rx (29/30dB) and 4 Rx UEs (26/27)dB.

Observation 1: Proposed CQI Requirement for 2 RX, which goes up to 30dB SNR, might very close to the considered Tx EVM floor level. RAN4 should discuss whether this can have an impact on the test setup.

- References

[1] R4-2207254, 3GPP TSG-RAN WG4 Meeting #102-e, “Way forward for NR DL1024QAM demodulation and CQI reporting requirements”, Electronic Meeting, 21 February – 3 March 2022