3GPP TSG-RAN WG4 Meeting #103-e	R4-2209193

Electronic Meeting, 9 – 20 May, 2022

Title: 	Discussion on remaining issues in PRS measurement period

Source: 	Huawei, HiSilicon

Agenda item:	4.1.5.1

Document for:	Discussion

Introduction

Remaining issues in core requirements for PRS measurement were discussed in RAN4#102-e, and the outcomes are captured in the WF [1]. Although all the remaining issues in [1] are closed, we would like to discuss one additional issue, which is related to the start point of PRS measurement period.

In this paper we will provide our views on the start point of PRS measurement period.

Discussion

In the current requirements, the start point of the PRS measurement period is defined as follows.

The time TRSTD,Total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after both the NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP [34].

It can be seen that the assumption in the requirements is that UE shall immediately start PRS measurement after receiving the assistance data and the measurement request from LMF. In our view, this assumption is not valid for all types of positioning requests.

Different types of location requests are defined in clause 4.1a of 23.273, and one particular type is deferred MT-LR. The description for this request type is copied below.

### 4.1a.5 Deferred Location Request With a deferred location request, an LCS client or AF sends a location request to a PLMN for a target UE (or group of target UEs) and expects to receive a response containing the indication of event occurrence and location information if requested for the target UE (or group of target UEs) at some future time (or times), which may be associated with specific events associated with the target UE (or group of target UEs). In this version of the specification, only deferred location requests for an MT-LR are supported. #### 4.1a.5.1 Types of event The following types of event are defined for a deferred location request. a) UE availability: Any event in which the 5GCN has established a contact with the UE. This event is considered to be applicable when the UE is temporarily unavailable due to inaction by the user, or for temporarily loss of radio connectivity or IMSI detach and so on. The UE Available event only requires one response to an LCS client/AF and after this response, the UE Available event is concluded. b) Area: An event where the UE enters, leaves or remains within a pre-defined geographical area. At least one type of area event can be defined (i.e. entering, leaving or remaining within the area). The LCS client or AF may define the target area as a geographical area or as a geopolitical name of an area. The PLMN may translate and define the target area as the identities of one or more radio cells or tracking areas. The area event may be reported one time only, or multiple times. The area event report shall contain an indication of the event occurrence. The location estimate may be included in the report. If an area event is detected by the UE but an event report cannot be sent (e.g. because the UE cannot access the network or due to a minimum reporting interval), a report shall be sent later when possible irrespective of whether the area event still applies for the current UE location. Area event reporting is controlled by a minimum and a maximum reporting time. The minimum reporting time defines the minimum allowed time between successive area events. The maximum reporting time defines the maximum time between successive reports. When a UE sends a report due to expiration of the maximum reporting time, the UE indicates expiration of the maximum reporting time as the trigger event. The maximum reporting time enables the AF, LCS client and HGMLC to remain aware of continuing support by the UE for the area event (e.g. to detect if area event reporting may have been aborted due to UE power off). c) Periodic Location: An event where a defined periodic timer expires in the UE and activates a location report. If a periodic event is detected by the UE but an event report cannot be sent (e.g. because the UE cannot access the network temporarily), a report shall be sent later when possible and the periodic timer for the next event shall then be started. The reporting duration for periodic location shall equal the requested number of reports multiplied by the periodic interval even when reports are delayed. d) Motion: An event where the UE moves by more than some predefined straight line distance from a previous location. The motion event may be reported one time only, or multiple times. The motion event report shall contain an indication of the event occurrence. A location estimate may be included in the report if requested by the LCS client or AF. For successive motion event reports, motion is determined relative to the UE location corresponding to the immediately preceding event report (including an event report triggered by expiration of the maximum reporting time). If a motion event is detected by the UE but an event report is deferred (e.g. because the UE cannot access the network temporarily), a report shall be sent later when possible irrespective of whether the motion event still applies to the current UE location. Motion reporting is controlled by a minimum and a maximum reporting time. The minimum reporting time defines the minimum allowed time between successive event reports. The maximum reporting time defines the maximum time between successive reports. When a UE sends a report due to expiration of the maximum reporting time, the UE indicates expiration of the maximum reporting time as the trigger event. The maximum reporting time enables the AF, LCS client and HGMLC to remain aware of continuing support by the UE for the motion event (e.g. to detect if motion event reporting may have been aborted due to UE power off).

It can be seen that for this type of location requests, the LMF expects response containing location information, which includes measurement report from UE, at future times which may be associated with specific events. In clause 4.1a.5.1, 4 types of events are defined. This means for deferred MT-LR, UE is not supposed to perform PRS measurement immediately after receiving the assistance data and the measurement request from LMF, but should do so when the events occur.

In Rel-17 ePOS, it was agreed [2] to update the start point of PRS measurement period for scheduled location. In our view, the issue is same as deferred MT-LR. Since deferred MT-LR is already supported in Rel-16, we suggest to update the start point of PRS measurement period in Rel-16 requirements to account for the deferred MT-LR.

Proposal: Update the start point of PRS measurement period to account for the deferred MT-LR.

Conclusions

In this paper we provided our views on the start point of PRS measurement period.

Proposal: Update the start point of PRS measurement period to account for the deferred MT-LR.

Reference

- R4-2206821, WF on maintenance to R16 POS requirements, Intel Corporation

- R4-2206979, WF on NR Positioning Enhancements (Part 1), Ericsson