3GPP TSG-RAN WG4 Meeting #103-e	R4-2209237

Electronic Meeting, 9 – 20 May, 2022

Title: 	Simulation results for TRS based PDC

Source: 	Huawei, HiSilicon

Agenda item:	9.22.2

Document for:	Discussion

Introduction

RRM requirements for PDC enhancement were discussed in RAN4#102-e, and the outcomes are captured in the WF [1]. One important issue to be addressed in the Perf part is the accuracy for the TRS based PDC, for which the simulation assumption was agreed in [1].

In this paper we will provide our simulation results for TRS based PDC.

Discussion

Simulation assumption

The simulation are conducted based on agreed simulation assumption in [1], which are copied below for convenience. Some differences:

- For fading channel, we are using TDL-C (60 ns delay spread, 300 Hz) as used in PRS requirements.

- Since it was agreed in [1] to derive requirements based on 2-slot TRS, we only simulated TRS resource number 4.

- Only 4-sample are simulated.

Table 1: General parameters (TRS)

| Parameter | Value |
| --- | --- |
| Cell layout | 1 serving cell |
| Duplex modes | FDD and TDD |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) |
| Data and CCH load in non-TRS symbols | - 50% utilization in time - 100% RE utilization |
| Cyclic prefix | Normal |
| DRX | OFF |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 5 MHz, 10 MHz, 20 MHz - 15 kHz - FDD, TDD - 4 GHz - 10 MHz, 20 MHz, 50 MHz - 30 kHz - FDD, TDD - [4] GHz - 20 MHz, 40 MHz, 100 MHz - 60 kHz - FDD, TDD - [30] GHz - 40 MHz, 80 MHz, 200 MHz - 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | AWGN, [TDL-A (30 ns delay spread, 5Hz)] for FR1 [TDL-C (60 ns delay spread, 300Hz)] for FR2 |
| Es/Iot [dB] | -3 |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |
| UE measurement bandwidth | Full carrier bandwidth |

Table 2: CSI-RS for tracking transmission configuration parameters (TRS)

| Parameter | Value |
| --- | --- |
| Number of transmit TRS antennas | 1 |
| Cell ID, TRP ID, CSI-RS Resource Set ID | 1 |
| Number with CSI-RS Resource sets with trs-Info | 1 |
| TRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz) - 30 kHz: 24(10MHz), 52 (20MHz),132 (50MHz) - 60 kHz: 24(20MHz), 64 (50MHz),132 (100MHz) - 120 kHz: 32(50MHz), 64 (100MHz), 128 (200MHz) |
| Sample rate (Tc) | - 15 kHz: 256(5MHz), 128(10MHz), 64 (20MHz) - 30 kHz: 128(10MHz), 64 (20MHz), 32 (50MHz) - 60 kHz: 128(20MHz), 64 (40MHz), 32 (100MHz) - 60 kHz: 128(40MHz), 64 (80MHz), 32 (200MHz) |
| TRS periodicity | 40ms |
| TRS resources number | 2, 4 |
| TRS symbol location | {4,8} |
| TRS frequency density | 3 |
| TRS samples | 1, 4 |
| Evaluation period | 40 ms, 200 ms |

Simulation results

The simulation results for AWGN and fading channel are shown in Table 3 and Table 4, respectively.

Table 3: UE Rx-Tx accuracy for AWGN

| FR | SCS (kHz) | RB | Accuracy (Tc) |
| --- | --- | --- | --- |
| FR1 | 15 | 24 | 88 |
|  |  | 52 | 56 |
|  |  | 104 | 24 |
|  | 30 | 24 | 40 |
|  |  | 48 | 28 |
|  |  | 132 | 16 |
| FR2 | 60 | 24 | 24 |
|  |  | 64 | 14 |
|  |  | 132 | 8 |
|  | 120 | 32 | 16 |
|  |  | 64 | 7 |
|  |  | 128 | 4 |

Table 4: UE Rx-Tx accuracy for fading channel

| FR | SCS (kHz) | RB | Accuracy (Tc) |
| --- | --- | --- | --- |
| FR1 | 15 | 24 | 106 |
|  |  | 52 | 86 |
|  |  | 104 | 80 |
|  | 30 | 24 | 86 |
|  |  | 48 | 60 |
|  |  | 132 | 52 |
| FR2 | 60 | 24 | 90 |
|  |  | 64 | 76 |
|  |  | 132 | 76 |
|  | 120 | 32 | 80 |
|  |  | 64 | 56 |
|  |  | 128 | 54 |

Proposal: Take simulation results in Table 1 and Table 2 into account for defining the accuracy requirements for TRS based PDC.

Conclusions

In this paper we provided our simulation results for TRS based PDC.

Proposal: Take simulation results in Table 1 and Table 2 into account for defining the accuracy requirements for TRS based PDC.

Reference

- R4-2207020, WF on NR_IIOT_URLLC_enh_RRM requirements, Nokia, Nokia Shanghai Bell