3GPP TSG-RAN WG4 Meeting #103-e                                      R4-229247

Electronic Meeting, May 16 – 27, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on REFSENS evaluation for band n28 supporting 25MHz

Agenda Item:	8.21.2.1

Document for:	Approval

Introduction

Regarding the WI [1] approved in RAN#85 meeting, it is tasked for RAN4 to add wider channel bandwidth in band n28. Based on the discussion for 30 MHz channel bandwidth, in this paper we would like to further provide our views on REFSENS evaluation for 25 MHz channel bandwidth.

Discussion

We think REFSENS should be evaluated with the same UL allocation used for 30MHz (25RB for 15kHz SCS, 10RB for 30kHz SCS) for symmetric 25MHz channel bandwidth. As for the UL/DL configuration, we tend to reuse what is specified for 30MHz.

Proposal 1: For evaluation/specification of 25MHz REFSENS, reuse the 30MHz REFSENS Uplink configuration.

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n28 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |  |

REFSENS for symmetric UL/DL channel bandwidth

We characterize REFSENS with symmetric UL/DL bandwidth 25MHz with the above assumptions. It is easy to find that if UL 25MHz is allocated in range 723~748MHz, and set UL RB allocation, e.g. 4.5MHz for 15kHz SCS, as 743.5~748MHz, then CIM5 will fall into the corresponding Rx frequency range 778~803MHz. Similar principle could be applied for 30kHz SCS case. Thus we do the calculation for 25MHz with following listed assumptions.

Table 1. Typical receiver performance parameters for MSD analysis

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n28 filter rejection from TX to RX | 50 | dB |
| n28 diversity receiver filter TX rejection | 40 | dB |
| Front-end loss | 4 | dB |
| Thermal noise at n28 RX ANT port | -164 | dBm/Hz |
| SNR requirement for QPSK | -1 | dB |

Besides, since non-zero delta MPR should be applied for 25MHz channel bandwidth, we propose:

Proposal 2: To consider 0.5dB Delta MPR for 25MHz n28 when RAN4 evaluate the REFSENS.

In conclusion, we have the following REFSENS result for 25MHz:

Proposal 3: Specified the following REFSENS for Band n28 25MHz channel bandwidth:

Table 7.3.2-1a: Two antenna port reference sensitivity QPSK PREFSENS for asymmetric channel bandwidth

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 | [-85.2] | -78.5 |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.6 | -93.6 | -91.0 | [-85.3] | -78.6 |  |  |  |  |  |  |  |

Conclusion

In this contribution we share our analysis and evaluation results on Band n28 25MHz channel band width REFSENS, we have the following proposals:

Proposal 1: For evaluation/specification of 25MHz REFSENS, reuse the 30MHz REFSENS Uplink configuration.

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n28 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |  |

Proposal 2: To consider 0.5dB Delta MPR for 25MHz n28 when RAN4 evaluate the REFSENS.

Proposal 3: Specified the following REFSENS for Band n28 25MHz channel bandwidth:

Table 7.3.2-1a: Two antenna port reference sensitivity QPSK PREFSENS for asymmetric channel bandwidth

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 | [-85.2] | -78.5 |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.6 | -93.6 | -91.0 | [-85.3] | -78.6 |  |  |  |  |  |  |  |

- References

- RP-192261, New WI proposal: Addition of wider channel bandwidth in NR band n28, CBN, Huawei, HiSilicon, RAN#85, Newport Beach, USA, September 16 – 20, 2019.

- 3GPP TR 38.888, v16.0.0, March, 2020.