3GPP TSG-RAN WG4 Meeting #103-e	R4-2209288

# Electronic Meeting, 9 - 20 May, 2022

Source:	Huawei, HiSilicon

Title:	TP for 38.717-03-01 CA_n1A-n28A-n38A

Agenda item:	8.6.2

Document for:	Approval

1 Background

This contribution provides text proposal on the NR CA band combination CA_n1A-n28A-n38A as defined in Revised WID: Rel-17 Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) RP-220738 [1].

2 Text Proposal

---Start of changes---

6.X 	CA_n1-n28-n38

6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n28-n38 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n28A-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |

6.X.3	Co-existence studies

For single uplink, the UE coexistence is already considered in the fallback combinations in TS 38.101-1.

6.X.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n1, n28 and n38, the TIB,c and RIB,c values are shown in table 6.X.4-1 and table 6.X.4-2, respectively.

Table 6.X.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n28-n38 | n1 | 0.5 |
|  | n28 | 0.6 |
|  | n38 | 0.5 |

Table 6.X.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n28-n38 | n1 | 0 |
|  | n28 | 0.2 |
|  | n38 | 0 |

6.X.5	REFSENS requirements

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

---End of changes---

Reference

[1]	RP-220738, “Revised WID for Rel-17 NR inter-band Carrier Aggregation for 3DL1UL”, CATT