3GPP TSG-RAN WG4 Meeting # 103-e                	R4-2209384

Electronic Meeting, May 9th – 20th, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on general RRM requirements for extension to 71 GHz

Agenda item: 	9.15.8.1

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to the operation between 52.6 GHz and 71 GHz.

Discussion

RX beam sweeping scaling factor

During the last RAN4 meeting, the following open issue was left regarding RX beam sweeping scaling factor [1]:

Rx beam sweeping scaling factor - Rx beam sweeping scaling factor is FFS - Option 1: The Rx beam sweeping scaling factor is increased for FR2-2 compared with FR2-1 - FFS: Scaling factor = (12 - 16) - Companies to provide the values for the scaling factor. - Option 2: Reuse the existing FR2-1 scaling factor for Rx beam sweeping for FR2-2.

which  followed previous agreements from RAN4 #101-bis

Rx beam sweeping scaling factor - Rx beam scaling factor for FR2-2 need to continue waiting for more RF conclusion on antenna size assumption and power class.

And RAN4#100

Rx beam sweeping scaling factor GTW agreements: - Rx beam sweeping scaling factor - Further study whether new scaling factor is needed for FR2-2 considering RF session conclusions on UE antenna array assumptions and UE power classes and the difference with FR2-1 assumptions - Rx beam sweeping factor from FR2-1 can be used as a starting point for analysis

One important point to consider is the impact of the Rx beam sweeping scaling factor. When considering the RLM requirements, the scaling factor is directly considered in the evaluation period formulas of table 8.1.2.2-1 in 38.133. Examples of these values are calculated in Table 1, where P is considered for the case where TSSB=TSMTC for simplicity. In the example the period TEvaluate_out_SSB assumes values in the range between 1.6 s, for no DRX, until 300 s for 1280 DRX cycle when the current Rx beam sweeping is considered. If we extend this rx beam sweeping scaling factor, this can be increased up to 600 s in the worst case of the RLM requirements. This poses serious restriction on any UE movement since the time for the UE to react after reaching out of sync would be very large.

Table 1Example of RLM out of sync evaluation periods considering different Rx beam sweeping scaling factors N

|  |  | P=PsharingFactor for overlapped Psharingfactor 1 or 3 | TEvaluate_out_SSB (s) |  |  |
| --- | --- | --- | --- | --- | --- |
| DRX | TSSB |  | N=8 | N=12 | N=16 |
| no | 20 | 1 | 1.6 | 2.4 | 3.2 |
| 320 | 160 | 1 | 38.4 | 57.6 | 76.8 |
| 320 | 160 | 3 | 115.2 | 172.8 | 230.4 |
| 640 | 160 | 1 | 51.2 | 76.8 | 102.4 |
| 640 | 160 | 3 | 153.6 | 230.4 | 307.2 |
| 1280 | 160 | 3 | 307.2 | 460.8 | 614.4 |

RX beam sweeping impacts directly RLM in sync and out of sync requirements, where if N=12 and N=16 TEvaluate_out_SSB could reach up to 460 s and 614 s respectively.

Furthermore, the Rx beam sweeping has a large specification impact. Table 2 lists all the requirements that are impacted by the Rx beam sweeping.

Rx beam sweeping scaling factor impacts the requirements listed on Table 2.

Do not change Rx beam sweeping scaling factor for FR2-2 in comparison to FR2-1.

If RAN4 agrees to define new Rx beam sweeping scaling factor for FR2-2, consider the requirements on Table 2 for defining CR work split.

Table 2 RRM requirements impacted by Rx beam sweeping

| Requirements |  | Impact due to Rx Beam Sweeping |
| --- | --- | --- |
| Cell reselection | Intra-frequency | Yes |
|  | Inter-frequency | Yes |
| Handover |  | Yes |
| RRC Connection Mobility Control | RRC re-establishment | Yes |
|  | RRC release with redirection | Yes |
| Signalling characteristics | SCell activation and deactivation delay | Yes |
|  | Interruption | No |
|  | PSCell addition and release delay | No |
|  | Active TCI state switching delay | No |
|  | Active BWP switching delay | No |
|  | PSCell change | No |
|  | Conditional PSCell change | Yes |
|  | Radio link monitoring | Yes |
|  | Link recovery procedures | Yes |
|  | Uplink spatial relation switch delay | Indirect impact from TL1_Measurement_Period in 9.5.4 |
|  | Pathloss reference signal switch delay | No |
| Measurement requirements | UE measurement capability | No |
|  | Intra-frequency | Yes |
|  | Inter-frequency | Yes |
|  | L1-RSRP measurements for reporting | Yes |
|  | L1-SINR measurements for reporting | Yes |
|  | CSI-RS based measurement | Yes |

SSB index detection

In the last meetings it has been discussed whether SSB index detection should be revised for FR2-2. In one contribution it was identified that the number of samples for index detection might have to be extended, however no agreement was reached on that topic. In table 3 we present our simulation results according to the simulation parameters in Table 4

From our simulation results 2 samples are enough to detect SSB index with TDL-A 30 and TDL-A 5 at -6dB for the following SCS: 120 kHz, 240 kHz, 480 kHz and 960 kHz.

Since there is no difference in the number of acquisition attempts at -6 dB for different subcarrier spacings, in our view it is not needed to extend the measurement period for time index detection.

RAN4 to reuse the SSB index identification time from FR2-1.

Table 3 Simulation results for index detection

| Propagation condition | SNR (dB) | Number of acquisition attempts |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 120 kHz | 240 kHz | 480 kHz | 960 kHz |
| TDL-A 5 | -8 | 3 | 3 | 3 | 3 |
|  | -7 | 3 | 2 | 3 | 3 |
|  | -6 | 2 | 2 | 2 | 2 |
|  | -5 | 2 | 2 | 2 | 2 |
|  | -4 | 2 | 2 | 1 | 1 |
|  | -3 | 1 | 1 | 1 | 1 |
| TDL-A 10 | -8 | 3 | 3 | 3 | 3 |
|  | -7 | 2 | 3 | 2 | 3 |
|  | -6 | 2 | 2 | 2 | 2 |
|  | -5 | 2 | 2 | 1 | 2 |
|  | -4 | 1 | 1 | 1 | 1 |
|  | -3 | 1 | 1 | 1 | 1 |
| TDL-A 20 | -8 | 3 | 3 | 3 | 4 |
|  | -7 | 3 | 3 | 2 | 3 |
|  | -6 | 2 | 2 | 2 | 2 |
|  | -5 | 2 | 2 | 2 | 2 |
|  | -4 | 1 | 1 | 1 | 1 |
|  | -3 | 1 | 1 | 1 | 1 |
| TDL-A 30 | -8 | 3 | 4 | 4 | 5 |
|  | -7 | 2 | 3 | 3 | 3 |
|  | -6 | 2 | 2 | 2 | 2 |
|  | -5 | 2 | 2 | 2 | 2 |
|  | -4 | 1 | 1 | 1 | 1 |
|  | -3 | 1 | 1 | 1 | 1 |
| TDL-C 60 | -8 | 3 | 3 | 4 | 6 |
|  | -7 | 2 | 2 | 3 | 3 |
|  | -6 | 2 | 2 | 2 | 2 |
|  | -5 | 2 | 2 | 2 | 2 |
|  | -4 | 1 | 1 | 1 | 1 |
|  | -3 | 1 | 1 | 1 | 1 |

Table 4: Parameters for basic SI reading (PBCH acquisition)

| Parameter | Unit | Value |
| --- | --- | --- |
| Carrier frequency | GHz | 60GHz |
| Subcarrier spacing | KHz | 120, 240, 480kHz;960kHz |
| Number of Tx antennas | - | 1 Assuming only 1Tx port is used |
| Number of Rx antennas | - | 2 |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| RB Utilization | % | 100 |
| PBCH symbols within the SS block |  | According to the RAN1 agreements |
| Data and Control Power offset with respect to PSS and SSS | dB | 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | 0 |
| PSS and SSS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS RE positions within the PBCH resource | - | According to the RAN1 agreements |
| PBCH Channel coding |  | Polar code with 512 length and 24bit DCRC |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit (CRC 24bit) |
| PBCH SNR | dB | -10 : 0 dB, at least including -6dB,-8 dB |
| Propagation Condition [38.808] | - | AWGN/TDL-A (5/30ns delay spread) |
| UE speed |  | 3km/h |
| Detection Method |  | Baseline: on one shot detection (No combination for different PBCHs,) FFS Soft- combining |
| Performance metric |  | Basic SI reading time = the number of SS-blocks required for 99% to successfully decode the PBCH |

Conclusion

This paper has presented Nokia’s views on RRM requirements for the extension to 71 GHz. From this discussion we have derived the following observations and proposals:

- RX beam sweeping impacts directly RLM in sync and out of sync requirements, where if N=12 and N=16 TEvaluate_out_SSB could reach up to 460 s and 614 s respectively.

Rx beam sweeping scaling factor impacts the requirements listed on Table 2.

- Do not change Rx beam sweeping scaling factor for FR2-2 in comparison to FR2-1.

If RAN4 agrees to define new Rx beam sweeping scaling factor for FR2-2, consider the requirements on the table below for defining CR work split.

| Requirements |  | Impact due to Rx Beam Sweeping |
| --- | --- | --- |
| Cell reselection | Intra-frequency | Yes |
|  | Inter-frequency | Yes |
| Handover |  | Yes |
| RRC Connection Mobility Control | RRC re-establishment | Yes |
|  | RRC release with redirection | Yes |
| Signalling characteristics | SCell activation and deactivation delay | Yes |
|  | Interruption | No |
|  | PSCell addition and release delay | No |
|  | Active TCI state switching delay | No |
|  | Active BWP switching delay | No |
|  | PSCell change | No |
|  | Conditional PSCell change | Yes |
|  | Radio link monitoring | Yes |
|  | Link recovery procedures | Yes |
|  | Uplink spatial relation switch delay | Indirect impact from TL1_Measurement_Period in 9.5.4 |
|  | Pathloss reference signal switch delay | No |
| Measurement requirements | UE measurement capability | No |
|  | Intra-frequency | Yes |
|  | Inter-frequency | Yes |
|  | L1-RSRP measurements for reporting | Yes |
|  | L1-SINR measurements for reporting | Yes |
|  | CSI-RS based measurement | Yes |

From our simulation results 2 samples are enough to detect SSB index with TDL-A 30 and TDL-A 5 at -6dB for the following SCS: 120 kHz, 240 kHz, 480 kHz and 960 kHz.

RAN4 to reuse the SSB index identification time from FR2-1.

References

- R4-2206919 WF on NR extension to 71 GHz RRM requirements (Part 1) Qualcomm

- R4-2202657, WF on NR extension to 71 GHz RRM requirements (Part 1), Qualcomm

- TS 38.133: “NR; Requirements for support of radio resource management”