3GPP TSG-RAN WG4 Meeting # 103-e                	R4-2209393

Electronic Meeting, February 21 – March 3, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on PRACH demodulation requirements for the extension to 71 GHz

Agenda item: 	9.15.10.3.3

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on PRACH demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Discussion

Ncs value

During the last meeting, the following options for NCS were discussed  with the tentative agreement as in [1]:

Issue 2-5-8: (Ncs, logical sequence index, v) - Consider the following PRACH parameters as starting point: Ncs = 69, logical sequence index = 0, v =0 as starting point - Other options are not precluded

From our point of view, it is reasonable to assume that the NCS is chosen such considering the equivalent cell size. For this reason, we calculate the maximum detectable timing offset based on NCS. Considering that:

- for each root Zadoff-Chu sequence the number of cyclically shifted sequences that can be obtained is

NSEQ=LRANCS

- that the sequence length for the short preamble format C0 is

Nu=2048∙κ∙2-μ

- the maximum detectable time offset can be calculated as

TMAX=NuNSEQTc

Additionally, the values of Ncs can be chosen from Table 6.3.3.1-7 of 38.211 [2]:

Table 6.3.3.1-7: for preamble formats with LRA∈139, 571, 1151. zeroCorrelationZoneConfig, msgA-ZeroCorrelationZoneConfig; NCS value. LRA=139; LRA=571; LRA=1151. 0; 0; 0; 0. 1; 2; 8; 17. 2; 4; 10; 21. 3; 6; 12; 25. 4; 8; 15; 30. 5; 10; 17; 35. 6; 12; 21; 44. 7; 13; 25; 52. 8; 15; 31; 63. 9; 17; 40; 82. 10; 19; 51; 104. 11; 23; 63; 127. 12; 27; 81; 164. 13; 34; 114; 230. 14; 46; 190; 383. 15; 69; 285; 575.

Considering the formulas above, the maximum detectable timing offset can be calculated as shown in Table 1 for the existing Rel 15 and Rel 16 requirements. In this table, the values of Ncs are chosen in accordance to 38.104. The results from the table show similar cell size for all SCS and LRA options of FR1. For FR2 the Tmax decreases with the SCS since all SCSs already use the Ncs value that provides larger Tmax, i.e. Ncs = 69.

Therefore, we understand that we can chose the same approach as the one used in FR2-1, where Ncs is chosen to provide good coverage are using zeroCorrelationZoneConfig = 15. The calculated TMAX in that case is as shown in Table 1 for FR1 in Rel15 + NRU and FR2-1 and in Table 2 with the proposed Ncs value.

Table 1 Calculated maximum timing offset for existing PRACH requirements

|  | FR1 (Rel 15 + NRU) |  |  |  | FR2-1 (Rel 15) |  |
| --- | --- | --- | --- | --- | --- | --- |
| LRA | 139 | 139 | 1151 | 571 | 139 | 139 |
| NCS | 23 | 46 | 164 | 190 | 69 | 69 |
| SCS | 15 | 30 | 15 | 30 | 60 | 120 |
| μ | 0 | 1 | 0 | 1 | 2 | 3 |
| κ | 64 | 64 | 64 | 64 | 64 | 64 |
| Tc | 0.509 | 0.509 | 0.509 | 0.509 | 0.509 | 0.509 |
| Nu | 131072 | 65536 | 131072 | 65536 | 32768 | 16384 |
| NSEQ | 6 | 3 | 7 | 3 | 2 | 2 |
| TMAX (us) | 11.1 | 11.1 | 9.5 | 11.1 | 8.3 | 4.2 |

Table 2 Calculated maximum timing offset for the proposed Ncs PRACH configuration

|  | FR2-2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| LRA | 139 | 571 | 1151 | 139 | 571 | 139 |
| NCS | 69 | 285 | 575 | 69 | 285 | 69 |
| SCS | 120 | 120 | 120 | 480 | 480 | 960 |
| μ | 3 | 3 | 3 | 5 | 5 | 6 |
| κ | 64 | 64 | 64 | 64 | 64 | 64 |
| Tc | 0.509 | 0.509 | 0.509 | 0.509 | 0.509 | 0.509 |
| Nu | 16384 | 16384 | 16384 | 4096 | 4096 | 2048 |
| NSEQ | 2 | 2 | 2 | 2 | 2 | 2 |
| TMAX (us) | 4.2 | 4.2 | 4.2 | 1.0 | 1.0 | 0.5 |

The NCS parameters used for FR2 requirements (Ncs=69) are the ones that provide the maximum coverage area for LRA=139.

The LRA=139 and Ncs=69 provide a similar coverage area to LRA=571 and Ncs=285 and LRA=1151 and Ncs=575.

RAN4 to define PRACH demodulation performance requirements using the following Ncs parameters:
      -SCS=120, LRA=139, NCS=69
      -SCS=120, LRA=571, NCS=285
      -SCS=120, LRA=1151, NCS=575
      -SCS=480, LRA=139, NCS=69
      -SCS=480, LRA=571, NCS=285
      -SCS=960, LRA=139, NCS=69

PRACH maximum detectable offset

As a part of the PRACH conformance testing, a timing offset scheme is used as shown in Figure 1. This scheme is used for all existing PRACH requirements, including SCS up to 120 kHz. This scheme uses a maximum offset of 0.8 us [3], which is fine for the Rel 15 SCSs, since for 120 kHz the maximum detectable offset is TMAX=4.2 us as indicated in Table 1. However, 0.8 us is very close to the achievable limit for 480 kHz, TMAX=1.0 us and above the limit for 960 kHz SCS, TMAX=1.0 us.

Figure 1 Timing offset scheme for PRACH preamble format A1 A2, A3, B4, C0 and C2 [3]

The achievable maximum detectable timing offset for 480 kHz and 960 kHz SCS is 1.0 us and 0.5 us respectively.

The Rel 15 requirements for PRACH include timing offset scheme with offset up to 0.8 us, which exceeds the maximum detectable offset for 960 kHz SCS.

RAN4 to specify PRACH performance with timing offset up to 0.4 us for 480 kHz and 960 kHz SCS.

Conclusion

This paper has presented Nokia’s views on PRACH demodulation requirements for the extension to 71 GHz. From this discussion we have derived the following observations and proposals:

- The NCS parameters used for FR2 requirements (Ncs=69) are the ones that provide the maximum coverage area for LRA=139.

The LRA=139 and Ncs=69 provide a similar coverage area to LRA=571 and Ncs=285 and LRA=1151 and Ncs=575.

- RAN4 to define PRACH demodulation performance requirements using the following Ncs parameters:
      -SCS=120, LRA=139, NCS=69
      -SCS=120, LRA=571, NCS=285
      -SCS=120, LRA=1151, NCS=575
      -SCS=480, LRA=139, NCS=69
      -SCS=480, LRA=571, NCS=285
      -SCS=960, LRA=139, NCS=69

The achievable maximum detectable timing offset for 480 kHz and 960 kHz SCS is 1.0 us and 0.5 us respectively.

The Rel 15 requirements for PRACH include timing offset scheme with offset up to 0.8 us, which exceeds the maximum detectable offset for 960 kHz SCS.

- RAN4 to specify PRACH performance with timing offset up to 0.4 us for 480 kHz and 960 kHz SCS.

References

- R4-2207223, WF on demodulation performance requirements definition for 52.6 - 71 GHz, Intel Corporation

- TS 38.211, NR; Physical channels and modulation

- TS 38.141-2, NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing

- RP-213540, Revised WID:  Extending current NR operation to 71 GHz, Qualcomm

- TR 38.808 v17.0.0, Study on supporting NR from 52.6 GHz to 71 GHz

- R1-2007982 "On NR operations in 52.6 to 71 GHz" Ericsson

- R1-2008615 "NR using existing DL-UL NR waveform to support operation between 52p6 GHz and 71 GHz" Qualcomm Incorporated.

- R1-2200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”

- R4-2202364, “Draft CR for TS 38.101-2: Introduction of system parameters for FR2-2”