3GPP TSG-RAN WG4 Meeting # 103-e		R4-2209413

# Electronic Meeting, May 09 – May 20, 2022

Source:	China Telecom

Title:	On CSI requirements for UE MMSE-IRC receiver for inter-cell interference suppression

Agenda Item:	9.11.2.1.2

Document for:	Discussion

- Introduction

In the RAN4#102-e meeting, we discussed the test parameters and test requirement definition for CQI reporting requirements for UEs supporting MMSE-IRC receiver for suppressing inter-cell interference, and the WF was approved in [1].

In this paper, our views on the remaining open issues are given.

- Discussion

Test setup methodology for signal power

Status in [1]:

  - Option 1: Define test based on SINR

  - Option 2: Define test based on SNR

In our understanding, when executing the CQI test with MMSE-IRC under ICI scenario, the test is mainly divided in 2 parts:

Part#1: Test the UE throughput under both ICI and white noise added, with SINR = x dB, in which case the SNR is smaller than x dB.

Part#2: Test the UE throughput with only white noise added, with SINR = SNR = x dB, which means the white noise power will be increased in the test part#2.

We observe that the SNR will be changed while executing the CQI test with MMSE-IRC under ICI scenario, therefore SINR based requirement definition is more preferable from our point of view.

Observation 1: The SNR will be changed while executing the CQI test with MMSE-IRC under ICI scenario.

Proposal 1: SINR based requirement definition is more preferable.

SINR requirement for 2Rx and 4Rx

Status in [1]:

Issue 2-2-2: SINR for 2 Rx

  - Option 1: -2 dB

  - Option 2: 0dB or -1 dB

Issue 2-2-5: SINR for 4 Rx

  - Option 1: -2 dB

  - Option 2: SINR requirement for 4Rx should be lower than that for 2Rx

  - Option 2a: Consider 3 dB difference for 2 and 4 Rx requirements

We prefer to consider different SINR requirements for 2Rx and 4Rx requirements, which is aligned with all the existing UE CQI reporting test requirements in TS38.101-4 as well as the IRC based CQI reporting test in LTE.

From companies’ simulation results summarized in [2], it can be observed that TP ratio is larger than 2.0 in the SINR range [2, -6] dB for 4Rx cases, and in the SINR range [-2, -6]dB for 2Rx cases.

Therefore, we propose to set SINR requirement as -2 or -3 dB for 2Rx, and -5 or -6 dB for 4Rx with TP ratio requirement as 2.0.

Observation 2: 3dB S(I)NR difference between 2Rx and 4Rx is considered in all the existing UE CQI reporting test requirements in TS38.101-4.

Observation 3: Lower SINR is used for LTE IRC based CQI reporting test for 4Rx (9.9.2.1 in 36.101) compared with that of 2Rx (9.3.5.1 in 36.101).

Observation 4: From companies’ simulation results summarized, TP ratio is larger than 2.0 in the SINR range [2, -6] dB for 4Rx cases, and in the SINR range [-2, -6]dB for 2Rx cases.

Proposal 2: Set SINR requirement as -2 or -3 dB for 2Rx, and -5 or -6 dB for 4Rx.

Proposal 3: TP ratio requirement as 2.0 for both 2Rx and 4Rx.

- Conclusions

The following proposals were given for CQI reporting requirements:

Observation 1: The SNR will be changed while executing the CQI test with MMSE-IRC under ICI scenario.

Proposal 1: SINR based requirement definition is more preferable.

Observation 2: 3dB S(I)NR difference between 2Rx and 4Rx is considered in all the existing UE CQI reporting test requirements in TS38.101-4.

Observation 3: Lower SINR is used for LTE IRC based CQI reporting test for 4Rx (9.9.2.1 in 36.101) campared with that of 2Rx (9.3.5.1 in 36.101).

Observation 4: From companies’ simulation results summarized, TP ratio is larger than 2.0 in the SINR range [2, -6] dB for 4Rx cases, and in the SINR range [-2, -6]dB for 2Rx cases.

Proposal 2: Set SINR requirement as -2 or -3 dB for 2Rx, and -5 or -6 dB for 4Rx.

Proposal 3: TP ratio requirement as 2.0 for both 2Rx and 4Rx.

- References

R4-2207243, WF on CQI reporting requirements for inter-cell interference MMSE-IRC, Ericsson, RAN4 #102-e, March 2022.

R4-2205509, Summary of PDSCH simulation results for inter-cell IRC CSI reporting, Ericsson, RAN4 #102-e, March 2022.