3GPP TSG-RAN WG4 Meeting # 103-e		R4-2209415

# Electronic Meeting, May 09 – May 20, 2022

Source:	China Telecom

Title:	Simulation results on PDSCH demodulation requirements for inter-cell interference MMSE-IRC

Agenda Item:	9.11.2.1.1

Document for:	Discussion

- Introduction

In the RAN4#102-e meeting, the test parameters for MMSE-IRC receiver for suppressing inter-cell interference were discussed. In this paper, our simulation result based on the agreed parameters in the approved WF [1] are given.

- Simulation results

In this section, our simulation result based on the agreed parameters in the approved WF[1] are given.

Since it has been proved in the previous meetings that MMSE-IRC can achieve enough performance gain over the baseline MMSE/MRC receiver, in this paper, only the MMSE-IRC performance results are provided in the below table.

Table 1. Ideal and impairment SNR @ 70% max TP of MMSE-IRC receiver

| Duplex Mode | Scenario | Channel model | INR1 | INR2 | Number of UE Rx antenna | Alignment Result | Impairment Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | HetNet | TDLA30-10 | 7.58 | N/A | 2Rx | 9.1 | 11.1 |
|  |  |  |  |  | 4Rx | 4.5 | 6.5 |
|  | HomNet | TDLC300-100 | 7.77 | 2.29 | 2Rx | 12.2 | 14.2 |
|  |  |  |  |  | 4Rx | 7.0 | 9.0 |
| TDD | HetNet | TDLA30-10 | 7.58 | N/A | 2Rx | 8.8 | 10.8 |
|  |  |  |  |  | 4Rx | 4.6 | 6.6 |
|  | HomNet | TDLC300-100 | 7.77 | 2.29 | 2Rx | 12.6 | 14.6 |
|  |  |  |  |  | 4Rx | 7.5 | 9.5 |

- References

R4-2207242, Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel, RAN4 #102-e, March 2022.