3GPP TSG-RAN WG4 Meeting #103e	R4-2209461

# Electronic Meeting, 9th – 20th May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0462 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on DMRS bundling measurements in TS 38.101-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UE RF requirmeent on DMRS bundling is introduced in Rel-17 in NR but measurement is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inroduce the annex F.8.2 and F8.3 on the DMRS measurement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No measurement on DMRS bundling in specificaitons. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F8.2, F.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < start of changes >

## F.8.2	Modified test signal

The post-FFT modulated signal before the equalization is modified according to:

Z''(t,f)=FFTz(v-Δt)⋅e-j2πΔfv.ej2πfΔt

where

z(v) is the time domain samples of the signal under test within the bundled time slots.

To minimize the error, the signal under test should be modified with respect to a set of parameters following the procedure explained below.

Notation:

Δt is the sample timing difference between the FFT processing window in relation to nominal timing of the ideal signal.

Δf is the RF frequency offset with the bundled time slots.

To minimize the error, the EVM window estimation is reused for the phase difference measurement for DMRS bundling.

In the following Δc represents the middle sample of the EVM window of length W or the last sample of the first window half if Wis even.

The test equipment shall

-	detect the start of each slot and estimate Δt and Δf,

-	determine Δc so that the EVM window of length W is centred

-	on the time interval determined by the measured cyclic prefix minus 16κ samples of the considered OFDM symbol for symbol l for subcarrier spacing configuration µ in a subframe, with l = 0 or l = 7*2^µ for normal CP, i.e. the first 16κ samples of the CP should not be taken into account for this step. In the determination of the number of excluded samples, a sampling rate of 1/Tc is assumed. If a different sampling rate is used, the number of excluded samples is scaled linearly.

-	on the measured cyclic prefix of the considered OFDM symbol symbol for all other symbols for normal CP and for symbol 0 to 11 for extended CP.

To determine the other parameters a sample timing offset equal to Δc is corrected from the signal under test. The test equipment shall then

-	correct the RF frequency offset Δffor each time slot within the bundled time slots, and

-	apply an FFT of appropriate size. The chosen FFT size shall ensure that in the case of an ideal signal under test, there is no measured inter-subcarrier interference.

The carrier leakage shall be removed from the evaluated signal; however, the removed relative carrier leakage power also has to satisfy the applicable requirement.

At this stage estimates of Δf, a(t,f), ϕ(t,f) and Δc are available. Δt is one of the extremities of the window W, i.e. Δtcan be Δc+α-W2 or Δc+W2, where α=0 if W is odd and α=1 if Wis even. The test equipment  shall then

-	calculate PhaseOffsetl with Δt set to Δc+α-W2,

-	calculate PhaseOffseth with Δt set to Δc+W2.

## F.8.3	Phase offset measurement

The phase difference ∆φtm,f for each subcarrier between a reference timeslot tref and the measurement timeslot tm is then calculated as defined below:

∆φtm,f= φtm,f-φ(tref,f)

The RMS phase offset with M bundled time slots and within one measurement time window is then calculated as below:

PhaseOffset=1N∙(M-1)m=1M-1f=1N∆φtm,f2

then, the PhaseOffset measurement shall be tested at the two FFT window W extremities of the measurements as described in Annex F.4:

- PhaseOffsetl is calculated with Δt set to Δc+α-W2 and

- PhaseOffsethis calculated with Δt set to Δc+W2.

Thus we get:

PhaseOffset=max(PhaseOffsetl ,PhaseOffseth)

The RMS average phase offset over K measurement time windows is then calculated as below:

PhaseOffset=1Kk=1KPhaseOffset(k)2

## << Unchanged part is omitted>>

## < end of changes >