3GPP TSG-RAN WG4 Meeting # 103-e 	R4-2209498

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2380 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on R17 RLM and BFD relaxation for UE power saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1.No hysteresis period for entering/exiting low mobility state, which may lead to frequent relaxed/normal state change. 2. No clarification on the conditions for entering relaxation when multiple RSs are configured. 3. The clarification for the case low mobility criterion is not configured is not right. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add hysterisis period for entering/exiting low mobility state. 2. Clarifies the conditions for entering relaxation when multiple RSs are configured. 3. Add/revised the note for explicit signaling 4. Add latest RRC signaling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Low mobility criterion may not work well for connected UE. 2. The applicability of relaxed requirements is not clarified. 3. The explanation of explicit signaling contradicts RAN2 spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1, 8.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

#### 8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria

For the UE supports [connected mode power saving] and configured with [goodServingCellEvaluationRLM] to enable RLM relxation:

-	when the UE is not performing intra-band carrier aggregation configured with CSI-RS based RLM and CSI-RS based BFD on SCell, and

-	UE has fulfilled good serving cell quality criterion defined in 5.7.13.2 of [2, TS 38.331], if the low mobility criteria is not configured, or

-	UE is also configured with low mobility criterion defined in 5.7.13.1 of [2, TS 38.331] and UE has fulfilled good serving cell quality criterion defined in 5.7.13.2 of [2, TS 38.331], and UE is in low mobility state based on evaluation of low mobility criterion.

-	when the UE is performing intra-band carrier aggregation configured with CSI-RS based RLM and CSI-RS based BFD on SCell, and

-	UE has fulfilled good serving cell quality criterion defined in 5.7.13.2 of [2, TS 38.331] for CSI-RS based RLM on SpCell and for CSI-RS based BFD on serving cell if the low mobility criteria is not configured, or

-	the UE is configured with low mobility criterion defined in 5.7.13.1 of [2, TS 38.331], and UE has fulfilled good serving cell quality criterion defined in 5.7.13.2 of [2, TS 38.331] for CSI-RS based RLM on SpCell and for CSI-RS based BFD on a serving cell, and UE is in low mobility state based on evaluation of low mobility criteria.

The UE is allowed to apply the relaxed requirements defined in clause 8.1.2. 4 for SSB based radio link monitoring and the relaxed requirements defined in clause 8.1.3. 4 for CSI-RS based radio link monitoring; otherwise, UE shall apply the requirements defined in clause 8.1.2.2 for SSB based radio link monitoring and the requirements defined in clause 8.1.3.2 for CSI-RS based radio link monitoring.

If the low mobility criteria is configured, UE enters low mobility state if it has fulfilled low mobility criterion for TDeltaP,Hys ms, and exits low mobility state if it has not fulfilled low mobility criterion for TDeltaP,Hys ms. TDeltaP,Hys is signalled to UE by [TBD].

If the low mobility criteria is not configured, but the explicit signaling [goodServingCellEvaluationRLM] is configured to enable RLM relaxation, UE is in low mobility state without evaluation of low mobility criteria.

The UE is not allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met:

-	The UE sends out-of sync indications to the higher layers,

-	The UE has triggered T310 timer

<End of Change #1>

<Start of Change #2>

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria

The relaxed beam failure detection procedures specified in this clause apply provided for the UE supports [connected mode power saving] and configured with [goodServingCellEvaluationBFD] to enable BFD relxation:

-	when the UE is not performing intra-band carrier aggregation configured with CSI-RS based RLM on SpCell and CSI-RS based BFD on SCell provided that:

-	UE has fulfilled good serving cell quality criterion defined in 5.7.13.2 of [2, TS 38.331] if the low mobility criteria is not configured, or

-	UE is also configured with low mobility criterion defined in 5.7.13.1 of [2, TS 38.331] and UE has fulfilled good serving cell quality criterion defined in 5.7.13.2 of [2, TS 38.331], and UE is in low mobility state based on evaluation of low mobility criterion.

-	when the UE is performing intra-band carrier aggregation configured with CSI-RS based RLM on SpCell and CSI-RS based BFD on SCell provided that:

-	UE has fulfilled good serving cell quality criterion defined in 5.7.13.1 of [2, TS 38.331] for CSI-RS based RLM on SpCell and for CSI-RS based BFD on serving cell if the low mobility criteria is not configured, or

-	the UE is configured with low mobility criterion defined in 5.7.13.1 of [2, TS 38.331], and UE has fulfilled both good serving cell quality criterion defined in 5.7.13.2 of [2, TS 38.331] for CSI-RS based RLM on SpCell and for CSI-RS based BFD on a serving cell, and UE is in low mobility state based on evaluation of low mobility criteria.

The scenario and RS resource configurations in the set  defined in section 8.5.1 apply for this section.

The UE is allowed to apply the relaxed requirements defined in clause 8.5.2.4 for SSB based beam failure detection and the relaxed requirements defined in clause 8.5.3.4 for CSI-RS based beam failure detection; otherwise, UE shall apply the requirements defined in clause 8.5.5 for SSB based beam failure detection and the requirements defined in clause 8.5.3 for CSI-RS based beam failure detection.

If the low mobility criteria is configured, UE enters low mobility state if it has fulfilled low mobility criterion for TDeltaP,Hys ms, and exits low mobility state if it has not fulfilled low mobility criterion for TDeltaP,Hys ms. TDeltaP,Hys is signalled to UE by [TBD].

If the low mobility criteria is not configured, but the explicit signaling [goodServingCellEvaluationBFD] is configured to enable BFD relaxation, UE is in low mobility state without evaluation of low mobility criteria.

The scenario and RS resource configurations in the set  defined in section 8.5.1 apply for this section.

The UE is not allowed to apply the relaxed link recovery procedures provided that at least one of the following conditions is met:

The UE sends a beam failure instance indication to the higher layers,

The UE has triggered T310 timer.

<End of Change #2>