3GPP TSG-RAN WG4 Meeting#103-e                               R4-2209588

E-meeting, May 9 – May 20,2022

Agenda item:	9.15.4.2

Source: 	ZTE Corporation

Title: 	Further discussion on BS Rx requirements for 52.6-71GHz

Document for:	Approval

Introduction

In the previous RAN4 meetings, some further progress on BS Rx requirement on 52.6-71GHz was made in [4], for Rx intermodulation requirement agreed in last meeting was still kept in square bracket since the spectral utilization requirements for FR2-2 is still pending in the main session. Spectral utilization for FR2-2 was agreed in [6] except for 2000MHz and 960kHz, therefore in this contribution, we want to share some further inputs on that requirements.

Discussion

First of all, spectral utilization requirements for FR2-2was agreed as following and 200MHz with 960kHz is still pending.

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 264 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33 | 66 | 132 | [156] |

Indeed for the following Rx intermodulation requirement definition, the above spectral utilization requirement has already been taken into account with additional assumption that wanted signal SCS and interference signal SCS could be different for 800, 1600, 2000MHz, therefore we propose to the remove the square bracket for FR2-2 intermodulation requirement except for 2000MHz case.

Proposal 1: to remove the square bracket for FR2-2 Rx intermodulation requirements except for 2000MHz, 960kHz.

| Frequency Range | BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- | --- |
| FR2-1 | 50 | ±7.5 | CW |
|  |  | ±40 | 50MHz DFT-s-OFDM NR signal (Note 1) |
|  | 100 | ±6.88 | CW |
|  |  | ±40 | 50MHz DFT-s-OFDM NR signal (Note 1) |
|  | 200 | ±5.64 | CW |
|  |  | ±40 | 50MHz DFT-s-OFDM NR signal (Note 1) |
|  | 400 | ±6.02 | CW |
|  |  | ±45 | 50MHz DFT-s-OFDM NR signal (Note 1) |
| FR2-2 | 100 | ±7.5 | CW |
|  |  | ±65 | 100MHz DFT-s-OFDM NR signal (Note 2) |
|  | 400 | ±6.28 | CW |
|  |  | ±70 | 100MHz DFT-s-OFDM NR signal (Note 2) |
|  | 800 | ±6.32 | CW |
|  |  | ±80 | 100MHz DFT-s-OFDM NR signal (Note 2) |
|  | 1600 | ±6.4 | CW |
|  |  | ±100 | 100MHz DFT-s-OFDM NR signal (Note 2) |
|  | 2000 | [±5.52] | CW |
|  |  | [±160] | [100MHz DFT-s-OFDM NR signal (Note 2)] |
| NOTE 1: Number of RBs is 64 for the 60 kHz subcarrier spacing, 32 for the 120 kHz subcarrier spacing. NOTE 2: Number of RBs is 64 with 120 kHz subcarrier spacing. |  |  |  |

Conclusions

In this contribution, further considerations on each BS Rx requirements for 52.6-71GHz and some proposals and observations are made as following:

Proposal 1: to remove the square bracket for FR2-2 Rx intermodulation requirements except for 2000MHz, 960kHz.

References

- R4-2106114, WF on BS RF RX requirements for 52.6 – 71 GHz, approved.

- TR 38.808

- R4-2203017,WF on BS RF Rx requirements, approved.

- R4-2207217	WF on >52.6 GHz BS RF Rx Requirements, approved.

- R4-2206536,WF on 60 GHz UE RF, approved.

- R4-2207222,Draft CR for TS 38.104 on introduction of BS RF Rx requirements for 57-71GHz in section 10.6 – 10.9, ZTE, endorsed.