3GPP TSG-RAN4 Meeting #103-e	R4-2209615

# Electronic Meeting, May 9 – 20, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2385 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on correction for RTT-based PDC measurement requirements in 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| - Reason for change: |  | - Update the measurement for PDC based on the RAN1/2 specification update |  |  |  |  |  |  |  |  |
| - Summary of change: |  | - Update the RRC tirgering signaling and the UE capaibility |  |  |  |  |  |  |  |  |
| - Consequences if not approved: |  | - Performance for UE Rx-Tx measurement for propagation delay compensation cannot be guaranteed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 9.12	Measurement for Propagation Delay Compensation

### 9.12.1	Introduction

The requirements in this clause are applicable for UE capable of RTT based propagation delay compensation based on PRS/TRS and SRS, where the UE is configured with measObjectRxTxDiff in MeasObjectToAddModList defined in TS38.331.

### 9.12.2	Requirements Applicability

The requirements in clause 9.12 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	If UE Rx-Tx time difference measurement is based on PRS, the related side conditions given in clause [TBD] are met for a corresponding band.

-	If UE Rx-Tx time difference measurement is based on TRS, the related side conditions given in clause [TBD] are met for a corresponding band.

-	SRS is configured on at least one of the PCell.

### 9.12.3	Measurement Capability

The gNB side Rx-Tx time difference measurement capability is as indicated by the UE in gNB-SideRTT-BasedPDC-r17, according to TS 38.306.

If UE Rx-Tx time difference measurement is based on PRS, the capability is as indicated by the UE in [TBD].

If UE Rx-Tx time difference measurement is based on TRS, the capability is as indicated by the UE in [TBD].

### 9.12.4	Measurement period requirements

#### 9.12.4.1 PRS Measurement Period

When UE is configured with prs-Ref-r17 for  MeasObjectRxTxDiff-r17 defined in TS 38.331, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_PRS, where

TUERxTx, PRS=NPRSslotN'Lavailable_PRSN*(Nsample-1)*Teffect+Tlast

where

Lavailable_PRS is the time duration of available PRS resources to be measured during TperPRSTPRS, and is calculated in the same way as PRS duration K defined in 9.1 of TS 38.214.

NPRSslot is the maximum number of DL PRS resources configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to [durationOfPRS-ProcessingSysmbols] in [TBD] processed every T ms corresponding to [durationOfPRS-ProcessingSymbolsInEveryTms] in [TBD] for a given maximum bandwidth supported by UE corresponding to [supportedBandwidthPRS ] in [TBD],

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to [maxNumOfDL-PRS-ResProcessedPerSlot] as specified in [TBD],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= 4,

- Tlast is the measurement duration for the last UE Rx-Tx time difference measurement, including the sampling time and processing time, Tlast = T + TperPRSTPRS ,

Teffect is periodicity of UE Rx-Tx time difference measurement:

Teffect= TTPRS*TperPRSTPRS

where

T	corresponds to [durationOfPRS-ProcessingSymbolsInEveryTms] defined in [TBD],

TperPRSTPRS is the PRS resource periodicity specific for RTT-based propagation delay compensation.

Note: the PRS measurement period TUERxTx, PRS can be revisited based on further agreement from RAN1/2 and further RAN4 discussion.

UE is only required to perform UE Rx-Tx time difference on PRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on PRS, the requirements apply provided that the SCS of the PRS is same as that of the active BWP on PCell.

#### 9.12.4.2 TRS Measurement Period

When UE is configured with csi-RS-Ref-r17 for  MeasObjectRxTxDiff-r17 defined in TS 38.331, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_TRS, where

TUERxTx,TRS=Nsample *TTRS

Where

-	Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample is [1 or 4],

-	TTRS is the TRS resource periodicity specific for RTT-based propagation delay compensation.

Note: the TRS measurement period TUERx-Tx_TRS can be revisited based on further agreement from RAN1/2 and further RAN4 discussion.

UE is only required to perform UE Rx-Tx time difference on TRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on TRS, the requirements apply provided that the SCS of the TRS is same as that of the active BWP on PCell.

<End of Change 1>